A ZONING RESOLUTION FOR THE UNINCORPORATED AREA OF

LAWRENCE TOWNSHIP

Proposed by the

Zoning Commission of Lawrence Township

Patrick Kline

Mark Scott

Joe Hafer

Sally Storad

Roxanne Kane

Alternates:

Robert Eddy

Nikki Moore

Prepared by the

Zoning Commission of Lawrence Township and

Special Thanks to Stan Koehlinger for his dedication

Adopted by the

Lawrence Township Board of Trustees

August 10, 2024

ADOPTED November 14, 1972

Includes Amendments Effective January 28, 1979

Includes Amendments Effective September 29, 1979

Includes Amendments Effective February 16, 2001

Includes Amendments Effective January 9, 2003

Includes Amendments Effective January 8, 2004

Includes Amendments Effective April 25, 2009

Includes Amendments Effective December 11, 2010

Includes Amendments Effective January 31, 2017

Includes Amendments Effective April 26, 2020

Includes Amendments April 4, 2021

Includes Amendments June 25, 2023

Includes Amendments August 10, 2024

TABLE OF CONTENTS

ARTICLE I ……………………………………………………………………………………………………………………………………………	  12

TITLE, AUTHORIZATION, PURPOSE ……………………………………………………………………………………………………	  12

SEC. 101	TITLE ………………………………………………………………………………….………………………………………..	  12

SEC. 102	AUTHORIZATION ……………………………………………………………..………………………………………….	  12

SEC. 103	PURPOSE ……………………………………………………………………………..………………………………………	  12

ARTICLE II …………………………………………………………………………………………………………………………………………..	  13

GENERAL PROVISIONS ………………………………………………………………………………………………………………………	  13

SEC. 201	ESTABLISHMENT OF DISTRICTS OR ZONES ……………………………………………………………………	  13

SEC. 201.1	Uniformity of Regulations ………………………………………………………………………………….	  13

SEC. 201.2	General Rules of Interpretation …………………………………………………………………………	  13

SEC. 201.3	Jurisdiction ……………………………………………………………………………………………………….	  14

SEC. 201.4	Interpretation and Conflict ………………………………………………………………………………..	  14

SEC. 201.5	Relationship with Private-Party Agreements ………………………………………………………	  14

SEC. 202	ZONING DISTRICT MAP ………………………………………………………………………………………………..	  14

SEC. 202.1	Interpretation of District Boundaries …………………………………………………………………	  15

SEC. 202.2	Vacation of Public Ways …………………………………………………………………………………….	  16

SEC. 203	REGULATIONS APPLICABLE TO ALL DISTRICTS OR ZONES ……………………………………………..	  16

SEC. 203.1	Uses Exempt from Regulations ………………………………………………………………………….	  16

SEC. 203.2	Required Compliance ………………………………………………………………………………………..	  20

SEC. 203.3	Nuisance Prohibited ………………………………………………………………………………………….	  21

SEC. 203.4	General Regulations of Lots ……………………………………………………………………………….	  21

SEC. 203.5	General Regulations of Structures and Construction ………………………………………….	  24

SEC. 203.6	Architectural and Landscaping Review ……………………………………………………………….	  36

SEC. 203.7	Fire Hazard ……………………………………………………………………………………………………….	  42

SEC. 203.8	Outdoor Display, Sales, and Storage ………………………………………………………………….	  43

SEC. 203.9	Standards for Outdoor Storage Areas ………………………………………………………………..	  44

SEC. 203.10	Pets ………………………………………………………………………………………………………………….	  45

SEC. 204	LOT MEASUREMENTS AND REQUIREMENTS ………………………………………………………………..	  45

SEC. 204.1	Distance Measurements ……………………………………………………………………………………	  45

SEC. 204.2	Measurement of Lot Area ………………………………………………………………………………….	  45

SEC. 204.3	Minimum Lot Width Requirement …………………………………………………………………….	  46

SEC. 204.4	Corner Lots ……………………………………………………………………………………………………….	  47

SEC. 204.5	Double Frontage Lots ………………………………………………………………………………………..	  47

SEC. 204.6	Dedicated Private Drives ……………………………………………………………………………………	  49

SEC. 204.7	Panhandle Lots …………………………………………………………………………………………………	  51

SEC. 204.8	Building Setback Lines ………………………………………………………………………………………	  52

SEC. 204.9	Vision Clearance Triangles …………………………………………………………………………………	  53

SEC. 204.10	Fence Standards ……………………………………………………………………………………………….	  55

SEC. 205	TUSCARAWAS COUNTY REGIONAL PLANNING ACKNOWLEDGEMENT ………………………….	  60

SEC. 206	TIME…………………………………………………………………………………………………………………………….	  60

SEC. 206.1	Computing Time ……………………………………………………………………………………………….	  60

SEC. 206.2	Computation of Time ………………………………………………………………………………………..	  60

SEC. 207	HUMAN REMAINS ……………………………………………………………………………………………………….	  60

SEC. 207.1	Human Remains Discovery ………………………………………………………………………………..	  60

SEC. 207.2	Prehistoric and Native American Human Remains………………………………………………	  61

SEC. 208	FIRE HYDRANT REGULATIONS ON PRIVATE PROPERTY………………………………………………….	  61

SEC. 208.1	Hydrant Installation ………………………………………………………………………………………….	  61

SEC. 208.2	Hydrant Clearance …………………………………………………………………………………………….	  61

SEC. 208.3	Hydrant Color ……………………………………………………………………………………………………	  62

SEC. 208.4	Fire Hydrant Protection …………………………………………………………………………………….	  62

SEC. 209 	BUFFERS ………………………………………………………………………………………………………………………	  62

SEC. 209.1	Purpose …………………………………………………………………………………………………………….	  62

SEC. 209.2	Applicability ………………………………………………………………………………………………………	  62

SEC. 209.3	Definition of Terms ……………………………………………………………………………………………	  62

SEC. 209.4	General Standards for Buffers ……………………………………………………………………………	  63

SEC. 209.5	Minimum Buffers for Adjoining Land Use …………………………………………………………..	  65

SEC. 209.6	General Landscaping Requirements. ………………………………………………………………….	  65

SEC. 209.7	Recommended Plant Materials and Maintenance ………………………………………………	  67

SEC. 209.8	Berming ……………………………………………………………………………………………………………	  67

SEC. 209.9	Maintenance …………………………………………………………………………………………………….	  68

SEC. 210 	STORM WATER MANAGEMENT ……………………………………………………………………………………	  68

SEC. 210.1	Erosion, Sedimentation, and Storm Water Management Controls ………………………	  68

SEC. 211	ENVIRONMENTAL PERFORMANCE STANDARDS …………………………………………………………..	  70

SEC. 211.1	Air Pollution ……………………………………………………………………………………………………..	  70

SEC. 211.2	Erosion ……………………………………………………………………………………………………………..	  71

SEC. 211.3	Water Pollution …………………………………………………………………………………………………	  71

SEC. 211.4	Electrical Disturbance ……………………………………………………………………………………….	  71

SEC. 211.5	Noise ………………………………………………………………………………………………………………..	  71

SEC. 211.6	Vibration …………………………………………………………………………………………………………..	  72

SEC. 211.7	Toxic or Noxious Matter ……………………………………………………………………………………	  73

SEC. 211.8	Radiation Hazards …………………………………………………………………………………………….	  73

SEC. 211.9	Fire and Explosive Hazards ………………………………………………………………………………..	  73

SEC. 211.10	Glare ………………………………………………………………………………………………………………..	  74

SEC. 211.11	Measurement Procedures …………………………………………………………………………………	  75

SEC. 211.12	Enforcement …………………………………………………………………………………………………….	  75

SEC. 212	OVERLAY DISTRICTS …………………………………………………………………………………………………….	  75

SEC. 213	HOME OCCUPATIONS ………………………………………………………………………………………………….	  75

SEC. 213.1	General Provisions …………………………………………………………………………………………….	  75

SEC. 213.2	Prohibited Home Occupations …………………………………………………………………………..	  76

SEC. 214	WASTE RECEPTACLES …………………………………………………………………………………………………..	  77

SEC. 215	FREESTANDING WIND ENERGY CONVERSTION SYSTEM (WECS) ……………………………………	  77

SEC. 215.1	Site Plan Requirements ……………………………………………………………………………………..	  79

SEC. 215.2	Decommissioning ……………………………………………………………………………………………..	  79

SEC. 216	SMALL STRUCTURE MOUNTED WIND ENERGY CONVERSION SYSTEMS (SSM-WECS) ……	  79

SEC. 216.1	Small Structure Mounted Wind Energy Conversion System (SSM-WECS), including			Blade Tip Power System Turbines (BTPS) ……………………………………………………………	  79

SEC. 216.2	Decommissioning ……………………………………………………………………………………………..	  80

SEC. 217	SMALL WIND PROJECTS (FARM) …………………………………………………………………………………..	  81

SEC. 217.1	Permits …………………………………………………………………………………………………………….	  83

SEC. 217.2	Application ……………………………………………………………………………………………………….	  83

SEC. 217.3	Decommissioning ……………………………………………………………………………………………..	  85

ARTICLE III ………………………………………………………………………………………………………………………………………….	  88

DISTRICT REGULATIONS ……………………………………………………………………………………………………………………	  88

SEC. 301	R-1 LOW DENSITY RESIDENTIAL (SURBURBAN) DISTRICT ……………………………………………..	  88

SEC. 301.1	Purpose …………………………………………………………………………………………………………….	  88

SEC. 301.2	Uses …………………………………………………………………………………………………………………	  88

SEC. 301.3	Area, Yard and Height Regulations …………………………………………………………………….	  89

SEC.301.4	Permitted Use Certificate Required……………………………………………………………………….  90

SEC. 302	R-2 PLANNED RESIDENTIAL DISTRICT ……………………………………………………………………………	  91

SEC. 302.1	Purpose …………………………………………………………………………………………………………….	  91

SEC. 302.2	Uses ………………………………………………………………………………………………………………… 	  91

SEC. 302.3	Area, Yard and Height Regulations …………………………………………………………………….	  92

SEC. 302.4	Site Development, Parking, Open Space and Other Requirements ………………………	  94

SEC. 302.5	Plan for Development of Land; Approval Procedures ………………………………………….	  95

SEC. 302.6	Residential Specification ……………………………………………………………………………………	  97

SEC. 302.7	Permitted Use Certificate Required ……………………………………………………………………… 100

SEC. 303	R-3 HIGH DENISTY RESIDENTIAL DISTRICT ……………………………………………………………………	 101

SEC. 303.1	Purpose …………………………………………………………………………………………………………….	 101

SEC. 303.2	Uses …………………………………………………………………………………………………………………	 101

SEC. 303.3	Area, Yard and Height Regulations …………………………………………………………………….	 102

SEC. 303.4	Site Plan Review and Conformation …………………………………………………………………..  104

SEC. 303.5	Permitted Use Certificate Required ……………………………………………………………………..  104

SEC 304	R-4 RESIDENTIAL DISTRICT ………………………………………………………………………………………….	 105

SEC. 304.1	Purpose …………………………………………………………………………………………………………..	 105

SEC. 304.2	Uses …………………………………………………………………………………………………………………	 105

SEC. 304.3	Area, Yard and Height Regulations (Without Central Sewage Treatment) …………..	 106

SEC. 304.4	Area, Yard and Height Regulations (With Central Sewage Treatment) ………………..	 108

SEC. 304.5	Permitted Use Certificate Required ……………………………………………………………………… 109

SEC. 305	MANUFACTURED HOME COMMUNITY DISTRICT ………………………………………………………….	 110

SEC. 305.1	Purpose …………………………………………………………………………………………………………….	 110

SEC. 305.2	Use Regulations ………………………………………………………………………………………………..	 110

SEC. 305.3	Procedure …………………………………………………………………………………………………………	 110

SEC. 305.4	General Requirements ………………………………………………………………………………………	 110

SEC. 305.5	Design Requirements ………………………………………………………………………………………..	 110

SEC. 305.6	Additional Requirements …………………………………………………………………………………..	 112

SEC. 305.7	Enlargement …………………………………………………………………………………………………….	 112

SEC. 305.8	Permitted Use Certificate Required …………………………………………………………………….. 112

SEC. 306	C-1 SERVICE DISTRICT ………………………………………………………………………………………………….	 113

SEC. 306.1	Purpose ……………………………………………………………………………………………………………	 113

SEC. 306.2	Uses …………………………………………………………………………………………………………………	 113

SEC. 306.3	Area, Yard and Height Regulations …………………………………………………………………….	 114

SEC. 306.4	Site Plan Review and Conformance ……………………………………………………………………	 114

SEC. 306.5	Permitted Use Certificate Required ……………………………………………………………………… 114

SEC. 306.6	Building Space ……………………………………………………………………………………………………… 114

SEC. 307	C-2 LOCAL COMMERCIAL DISTRICT ………………………………………………………………………………	 115

SEC. 307.1	Purpose ……………………………………………………………………………………………………………	 115

SEC. 307.2	Uses …………………………………………………………………………………………………………………	 115

SEC. 307.3	Area, Yard and Height Regulations …………………………………………………………………….	 116

SEC. 307.4	Site Plan Review and Conformance ……………………………………………………………………	 116

SEC. 307.5	Permitted Use Certificate Required ……………………………………………………………………… 117

SEC. 307.6	Building Space ……………………………………………………………………………………………………… 117

SEC. 308	C-3 HIGHWAY COMMERICAL DISTRICT …………………………………………………………………………	 118

SEC. 308.1	Purpose …………………………………………………………………………………………………………….	 118

SEC. 308.2	Uses …………………………………………………………………………………………………………………	 118

SEC. 308.3	Area, Yard and Height Regulations …………………………………………………………………….	119

SEC. 308.4	Site Plan Review and Conformance ……………………………………………………………………	120

SEC. 308.5	Supplementary Regulations ………………………………………………………………………………	120

SEC. 308.6	Permitted Use Certificate Required ……………………………………………………………………..120

SEC. 308.7	Building Space ……………………………………………………………………………………………………..120

SEC. 309	I-1 LIGHT INDUSTRIAL DISTRICT ……………………………………………………………………………………	121

SEC. 309.1	Purpose …………………………………………………………………………………………………………….	121

SEC. 309.2	Uses …………………………………………………………………………………………………………………	121

SEC. 309.3	Area, Yard and Height Regulations …………………………………………………………………….	123

SEC.309.4	Site Plan Review and Conformance ……………………………………………………………………	124

SEC. 309.5	Permitted Use Certificate Required ……………………………………………………………………..124

SEC. 309.6	Building Space ……………………………………………………………………………………………………..124

SEC. 310	T-1 WIRELESS TELECOMMUNICATIONS FACILTY …………………………………………………………..	125

SEC. 310.1	Intent ……………………………………………………………………………………………………………….	125

SEC. 310.2	Definitions ………………………………………………………………………………………………………..	125

SEC. 310.3	Permitted Locations ………………………………………………………………………………………….	126

SEC. 310.4	Co-location Covenant of Good Faith ………………………………………………………………..	127

SEC. 310.5	Standards Applicable to Wireless Telecommunications Tower Facilities ……………..	127

SEC. 310.6	Abandoned Telecommunication Facilities ………………………………………………………….	129

SEC. 310.7	FCC Compliance ………………………………………………………………………………………………..	129

SEC. 311	F-1 FLOOD PLAIN …………………………………………………………………………………………………………	130

SEC. 311.1	Letter of Review ……………………………………………………………………………………………….	130

SEC. 311.2	Flood Prone Areas Outside the FIRM Regulatory Area ………………………………………..	130

SEC. 311.3	Purpose of the District ………………………………………………………………………………………	130

SEC. 311.4	Restricted Uses …………………………………………………………………………………………………	130

SEC. 311.5	Development of Areas within a Flood Plain ……………………………………………………….	130

SEC. 311.6	Alternate Criteria for Determining Flood Plains ………………………………………………….	130

SEC. 311.7	Warning and Disclaimer of Liability ……………………………………………………………………	131

SEC. 312	PLANNED UNIT DEVELOPMENT (PUD) DISTRICTS …………………………………………………………	132

SEC. 312.1	Purpose and Intent ……………………………………………………………………………………………	132

SEC. 312.2	Planned Unit Development Districts Authorized …………………………………………………	132

SEC. 312.3	Establishment of Planned Unit Development Districts ………………………………………..	132

SEC. 312.4	Permitted Uses …………………………………………………………………………………………………	133

SEC. 312.5	Development Standards Applicable to all PUD Districts ………………………………………	134

SEC. 312.6	Development Standards Applicable to SFPUD Districts ……………………………………….	138

SEC. 312.7	Development Standards Applicable to MDPUD Districts …………………………………….	140

SEC. 312.8	Application Requirements and Procedures ………………………………………………………..	141

SEC. 312.9	Development Plan Requirements and Procedures ……………………………………………..	142

SEC. 312.10	Professional Assistance ……………………………………………………………………………………..	147

SEC. 312.11	Construction Inspection …………………………………………………………………………………….	147

SEC. 312.12	Fees and Deposits …………………………………………………………………………………………….	147

ARTICLE IV ………………………………………………………………………………………………………………………………………….	150

SIGN REGULATIONS ………………………………………………………………………………………………………………………….	150

SEC. 401	SIGN REGULATIONS ……………………………………………………………………………………………………..	150

SEC. 401.1	Purpose …………………………………………………………………………………………………………….	150

SEC. 401.2	Permitted Signs …………………………………………………………………………………………………	150

SEC. 401.3	Construction …………………………………………………………………………………………………….	152

SEC. 401.4	Maintenance …………………………………………………………………………………………………….	153

SEC. 401.5	Removal of Signs ……………………………………………………………………………………………….	153

SEC. 401.6	Fees ………………………………………………………………………………………………………………….	153

ARTICLE V …………………………………………………………………………………………………………………………………………..	155

PARKING SPACES, PARKING LAYOUT AND LOADING REQUIREMENTS …………………………………………………	155

SEC. 501	PARKING SPACES, PARKING LAYOUT AND LOADING REQUIREMENTS ………………………….	155

SEC. 501.1	Purpose …………………………………………………………………………………………………………….	155

SEC. 501.2	Applicability ………………………………………………………………………………………………………	155

SEC. 501.3	Exemptions ……………………………………………………………………………………………………….	156

SEC. 501.4	General Requirements ………………………………………………………………………………………	156

SEC. 501.5	Stacking Spaces …………………………………………………………………………………………………	157

SEC. 501.6	Off-Street Parking Spaces Required ……………………………………………………………………	158

SEC. 501.7	Employee Parking ……………………………………………………………………………………………..	164

SEC. 501.8	ADA Accessibility ………………………………………………………………………………………………	164

SEC. 501.9	Parking Lot Layout, Construction and Maintenance ……………………………………………	164

SEC. 501.10	Loading Space Requirements …………………………………………………………………………….	166

SEC. 501.11	Parking Space Standards ……………………………………………………………………………………	166

SEC. 501.12	Parking Lot Standards ……………………………………………………………………………………….	166

SEC. 501.13	Conflict with Other Provisions ……………………………………………………………………………	166

SEC. 501.14	Nonconforming Parking Continuance …………………………………………………………………	166

SEC. 501.15	Regulations for Fire Lanes …………………………………………………………………………………	167

SEC. 501.16	Parking and Loading Requirements for Use Non-Specified ………………………………….	167

SEC. 501.17	Severability ………………………………………………………………………………………………………….	168

TABLE 601 ………………………………………………………………………………………………………………………………………….	170

ARTICLE VI ………………………………….………………………………………………………………………………………………………	172

CONDITIONAL USE …………………………………………………………………………………………………………………………….	172

SEC. 601 	CONDITIONAL USE ZONING CERTIFICATES ……………………………………………………………………	172

SEC. 601.1	Purpose …………………………………………………………………………………………………………….	172

SEC. 601.2	Authority ………………………………………………………………………………………………………….	172

SEC. 601.3	Effect of Conditional Use Listing ………………………………………………………………………..	172

SEC. 601.4	Review Procedure for Conditional Use Applications ……………………………………………	172

SEC. 601.5	Coordination Review and Approval of Applications …………………………………………….	174

SEC. 601.6	General Considerations for Conditional Uses ……………………………………………………..	174

SEC. 601.7	Specific Criteria Pertaining to Conditional Uses ………………………………………………….	175

SEC. 601.8	Accessory Uses to Conditional Uses …………………………………………………………………..	177

SEC. 601.9	Additional Regulations Pertaining to Conditional Uses ……………………………………….	177

SEC. 601.10	Period of Validity ………………………………………………………………………………………………	179

SEC. 601.11	Appeal of Decisions …………………………………………………………………………………………..	179

SEC. 601.12	Additional Conditional Uses and Special Exceptions ……………………………………………	179

SEC. 601.13	Specific Regulations Pertaining to Certain Conditionally Permitted Uses …………….	180

SEC. 601.14	Dangerous, Exotic and Wild Animals ………………………………………………………………….	210

SEC. 601.15	Small Wind Farm ………………………………………………………………………………………………	211

ARTICLE VII …………………………………………………………………………………………………………………………………………	226

NONCONFORMING USES AND STRUCTURES ………………………………………………………………………………………	226

SEC. 700	REGULATIONS …………………………………………………………………………………………………………...	226

SEC. 701.1	Alterations ………………………………………………………………………………………………………..	226

SEC. 701.2	Extensions ………………………………………………………………………………………………………..	226

SEC. 701.3	Construction Approved Prior to Resolution ………………………………………………………..	226

SEC. 701.4	Displacement ……………………………………………………………………………………………………	226

SEC. 701.5	Discontinuance or Abandonment ………………………………………………………………………	226

SEC. 701.6	Restoration ……………………………………………………………………………………………………….	227

SEC. 701.7	Zoning Certificate for Nonconforming Uses ……………………………………………………….	227

ARTICLE VIII ……………………………………………………………………………………………………………………………………….	229

AMUSEMENT ARCADES …………………………………………………………………………………………………………………….	229

SEC. 801	PURPOSE AND FINDINGS ……………………………………………………………………………………………..	229

SEC. 801.1	Purpose …………………………………………………………………………………………………………….	229

SEC. 801.2	Findings ……………………………………………………………………………………………………………	229

SEC. 802	DEFINITIONS ………………………………………………………………………………………………………………..	229

SEC. 802.1	Operation; License Required ……………………………………………………………………………..	231

SEC. 802.2	Application Information …………………………………………………………………………………….	231

SEC. 802.3	Corporations, Trusts, and Partnerships ………………………………………………………………	231

SEC. 802.4	Affidavit Required …………………………………………………………………………………………….	231

SEC. 802.5	License Issuance; Effective Period; Fee ………………………………………………………………	232

SEC. 802.6	License Administration ………………………………………………………………………………………	232

SEC. 802.7	Submission of Source Code for Approval ……………………………………………………………	232

SEC. 802.8	License Validity and Display ……………………………………………………………………………….	233

SEC. 802.9	Operation of Arcade ………………………………………………………………………………………….	233

SEC. 802.10	License Revocation ……………………………………………………………………………………………	234

SEC. 802.11	Revocation Process ……………………………………………………………………………………………	235

SEC. 802.12	Transfer of License ……………………………………………………………………………………………	235

SEC. 802.13	Inspection …………………………………………………………………………………………………………	235

SEC. 802.14	Nuisance …………………………………………………………………………………………………………..	236

SEC. 802.15	Effect of Partial Invalidity ………………………………………………………………………………….	236

SEC. 802.16	Fee Collected ……………………………………………………………………………………………………	236

SEC. 802.17	Penalty ……………………………………………………………………………………………………………..	236

ARTICLE IX ………………………………………………………………………………………………………………………………………….	238

WELLHEAD PROTECTION OVERLAY DISTRICTS ……………………………………………………………………………………	238

SEC. 900	“WP-1” INNER ZONE OVERLAY DISTRICT ………………………………………………………………………	238

SEC. 900.1	Purpose …………………………………………………………………………………………………………….	238

SEC. 900.2	Permitted Uses …………………………………………………………………………………………………	238

SEC. 900.3	Conditional Use …………………………………………………………………………………………………	238

SEC. 900.4	Lot, Height, Bulk, Yard, and Buffer Regulations …………………………………………………..	238

SEC. 901	“WP-2” OUTTER ZONE OVERLAY DISTRICT …………………………………………………………………..	238

SEC. 901.1	Purpose …………………………………………………………………………………………………………….	238

SEC. 901.2	Applicability to Underlying Zoning Districts ………………………………………………………..	239

SEC. 901.3	Permitted Uses, Bulk and Yard Regulations …………………………………………...............	239

SEC. 901.4	Conditional Uses ……………………………………………………………………………………………….	239

SEC. 901.5	Prohibited Uses …………………………………………………………………………………………………	240

SEC. 901.6	Best Management Practices and Exemptions …………………………………………………….	240

SEC. 901.7	Enforcement …………………………………………………………………………………………………….	242

SEC. 901.8	Reporting Regulated Substance Spills, Leaks, or Discharges ………………………………..	244

SEC. 901.9	Exemption of Regulated Substances ………………………………………………………………….	244

SEC. 901.10	Clean Closure Requirements ……………………………………………………………………………..	245

ARTICLE X …………………………………………………………………………………………………………………………………………..	247

ADMINISTRATION AND ENFORCEMENT …………………………………………………………………………………………….	247

SEC. 1001	ADMINISTRATION ……………………………………………………………………………………………………….	247

SEC. 1001.1	Zoning Inspector ……………………………………………………………………………………………….	247

SEC. 1001.2	Submission of Application …………………………………………………………………………………	247

SEC. 1001.3	Zoning Certificates Required ……………………………………………………………………………..	247

SEC. 1001.4	Fees ………………………………………………………………………………………………………………….	248

SEC. 1001.5	Board of Zoning Appeals ……………………………………………………………………………………	248

SEC. 1001.6	Zoning Commission – Powers and Duties ……………………………………………………….....	251

SEC. 1001.7	Board of Trustees – Powers and Duties ……………………………………………………………..	254

SEC. 1001.8	Amendments ……………………………………………………………………………………………………	256

SEC. 1001.9	Permitted Use Certificate ……………………………………………………………………………………. 258

SEC. 1002	ENFORCEMENT…………………………………………………………………………………………………………….	258

SEC. 1002.1	Violations ………………………………………………………………………………………………………….	258

SEC. 1002.2	Inspection …………………………………………………………………………………………………………	258

SEC. 1002.3	Correction Period ……………………………………………………………………………………………..	258

SEC. 1002.4	Penalties …………………………………………………………………………………………………………..	258

SEC. 1002.5	Prevention of Violations ……………………………………………………………………………………	259

ARTICLE XI ………………………………………………………………………………………………………………………………………….	261

VALIDITY AND SEPARABILITY …………………………………………………………………………………………………………….	261

ARTICLE XII …………………………………………………………………………………………………………………………………………	262

REPEALER …………………………………………………………………………………………………………………………………………	262

ARTICLE XIII ………………………………………………………………………………………………………………………………………..	263

EFFECTIVE DATE ……………………………………………………………………………………………………………………………….	263

APPENDIX “A”		SCHEMATIC FOR FIRE LANES …………………………………………………………………………….	265

APPENDIX “B”		WP-1 ………………………………………………………………………………………………………………..	266

APPENDIX “C”		WP-2 ………………………………………………………………………………………………………………..	267

APPENDIX “D”		LOT AREAS ……………………………………………………………………………………………………….	268

APPENDIX “G”		 BASEMENT & STORY ………………………………………………………………………………………..	269

APPENDIX “I”		TYPES OF SIGNS ………………………………………………………………………………………………..	270

APPENDIX “J”		NONCONFORMING USE ……………………………………………………………………………………	271

APPENDIX “K”		LAWRENCE TOWNSHIP ZONING REGULATIONS ………………………………………………..	272

APPENDIX “L”		LOT TYPES ………………………………………………………………………………………………………..	274

APPENDIX “M”	LOCATION OF LOT LINES AND SETBACKS/YARD ON A TYPICAL LOT …………………..	275

DEFINITIONS 		………………………………………………………………………………………………………………………….	277

ARTICLE I

TITLE, AUTHORIZATION, PURPOSE

# Sec. 101	Title

This Resolution shall be known as the Lawrence Township Zoning Resolution.

Sec. 102	Authorization

The authority for establishing “The Lawrence Township Zoning Resolution” is derived from 			Section 519.02 to 519.99, inclusive, of the Ohio Revised Code.

Sec. 103	Purpose

In order to promote and protect the health, safety and morals of the residents of the 				unincorporated area of Lawrence Township, Tuscarawas County, Ohio and to insure orderly 			growth and development in said Township, the Board of Township Trustees has found it 				necessary and advisable to adopt a comprehensive plan of zoning which regulates the location, 			height, bulk, number of stories, and size of buildings and other structures, percentages of lot 			areas which may be occupied, building setback lines, size of yards, and other open spaces; the 			use of buildings and other structures and the uses of the land for trade, industry, residence, 			recreation, or other purposes; and for such purposes to divide the unincorporated area of 			Lawrence Township into districts or zones, and to provide for the administration and 				enforcement of such regulations.

ARTICLE II

GENERAL PROVISIONS

Sec. 201	Establishment of Districts or Zones

The unincorporated territory of Lawrence Township, Tuscarawas County, Ohio is 					hereby divided into districts or zones as follows:

R-1	 Low Density Residential (Suburban) District

R-2 	 Medium Density Planned Residential District

R-3	 High Density Residential District

R-4	 Rural Residential District

MHCD	Manufactured Home Community District

C-1	 Service District

C-2	 Local Commercial District

C-3	 Highway Commercial District

I-1	 Light Industrial District

F-1	 Flood Plain District

PUD	 Planned Unit Development District

Sec. 201.1	Uniformity of Regulations

All regulations shall be uniform for each class or kind of building or other 						structure, or use, throughout each district or zone.

Sec. 201.2	General Rules of Interpretation

For the purpose of this Resolution, certain terms or phrases used herein 						shall be interpreted as follows:

A.	Words used in the present tense shall include the future.

B.	The singular number shall include the plural and the plural the 							singular.

C.	Use of the word “shall” indicate a mandatory requirement; the 							word “may” is a permissive standard the word “should” is a 							preferred standard.

D.	The word “used” shall include the words “arranged”, “designed”, 							“constructed”, “altered”, “converted”, or “intended to be used”.

E.	A “person” shall, in addition to an individual, mean a firm, 								corporation, association, or any legal entity that may own and/or 							use land or buildings.

Sec. 201.3	Jurisdiction

The provisions of this Zoning Resolution shall apply to all land, land development, use of 				all structures, and uses of land within the unincorporated areas of Lawrence Township, 				Tuscarawas County, Ohio.

Sec. 201.4	Interpretation and Conflicts

A.	For purposes of interpretation and application, the provisions of this Resolution 					shall be held to be the minimum requirements for the promotion of the public 					health, safety, convenience, comfort, prosperity, or general welfare.

B.	When the provisions of this Zoning Resolution are inconsistent with one another 					or with the provisions found in another adopted resolution, the more restrictive 					provision shall govern.

C.	Where this Zoning Resolution imposes a greater restriction than imposed or 					required by other provisions of law or by other rules, regulations or resolutions, 					the provisions of this Zoning Resolution shall control provided it complies with   					the Ohio Revised Code.

D.	Where a conflict or disagreement exists, the current “Black’s law Dictionary” shall be used as the final determination.

Sec. 201.5	Relationship with Private-Party Agreements

A.	This Resolution is not intended to interfere with or abrogate any easements covenants, or agreements between parties, provide that wherever this Resolution proposes a greater restriction upon the use of buildings or land, upon the location or height of buildings or structures, or upon requirements for open areas than those that are imposed or required by such easements, covenants, or agreements between parties, the provision of this Resolution shall govern.

B.	In no case shall the Township be obligated to enforce the provisions of any 					easements, covenants, or agreements between private parties.

Section 202	Zoning District Map

A.	Replacement of the Official Zoning Map

The NEW official Zoning Map may correct drafting or other errors or omissions in the 				prior Official Zoning Map, but no such corrections shall have the effect of amending the 				original official Zoning Map or any subsequent amendment thereof.  The new Official 				Zoning Map shall be identified by the signature of the Chairman of the Board of Trustees 			attested by the Fiscal Officer and bearing the seal of the Township under the following 				words:

“This is to certify that this Official Zoning Map supersedes and replaces the Official 				Zoning Map adopted November 14, 1972 as part of Resolution Number 1 of the 					Township of Lawrence, County of Tuscarawas, and State of Ohio.”

B.	Preserving Records

Unless the prior Official Zoning Map has been lost, or has been totally destroyed, the 				prior map and/or significant parts thereof remaining shall be preserved, together with 				all available records pertaining to its adoption or amendment.

Sec. 202.1	Interpretation of District Boundaries

Where uncertainty exists with respect to the boundaries of any of the established 				districts as shown on the zoning map the following rules shall apply:

A.	Follows Streets, Alleys, or Highways

Where district boundaries are indicated as approximately following the center 					line or right-of-way line of streets, the center line or alley line of alleys, or the 					center line or right-of-way lines of highways, such lines shall be construed to be 					such district boundaries.

B.	Parallel Street Right-of-Way, Alley or Highway Right-of-Way

Where district boundaries are so indicated that they are approximately parallel 					to the centerlines or right-of-way lines of streets, the center lines or alley lines of 				alleys, or the center lines of right-of-way lines on highways, such district 						boundaries shall be construed as being parallel thereto and at such a distance 					there from as indicated on the map.  If no distance is given, such dimension shall 					be determined by the use of the scale shown on said zoning map.

C.	Parallel Lot Lines

Where district boundaries are so indicated that they approximately follow the lot 				lines, such lot lines shall be construed to be said boundaries.

D.	Parallel Streams, Lakes, or Other Bodies of Water

Where district boundaries follow a stream, lake, or other body of water, said 					boundary line shall be deemed to be at the limit of the jurisdiction of the 						Township unless otherwise indicated.

Sec. 202.2	Vacation of Public Ways

Whenever any street or public way is vacated in the manner authorized by law, the 				zoning districts adjoining each side of the street or public way shall be automatically 				extended to the center of such vacations and all area included in the vacation shall 				thereafter be subject to all regulations of the extended districts.

Sec. 203	Regulations Applicable to All Districts or Zones

Sec. 203.1	Uses Exempt from Regulations

A. 	AGRICULTURE EXEMPTED – CAUV (County Agricultural Use Valuation)

ORC §519.25, inclusive, confer no power on any Board of Township Trustees or Board of Zoning Appeals to prohibit the use of any land for agricultural purposes or the construction or use of buildings or structures incident to the use for agricultural purposes of the land on which such buildings or structures are located, and no zoning certificate shall be required for any such building or structure. Applicant must submit an “Agriculture Exemption” form.

A township zoning resolution, or an amendment to such resolution, may in any platted subdivision approved under ORC §711.05, 711.09, or 711.10, or in any area consisting of fifteen (15) or more lots approved under ORC §711.131 that are contiguous to one another, or some of which are contiguous to one another and adjacent to one side of a dedicated public road, and the balance of which are contiguous to one another and adjacent to the opposite side of the same dedicated public road regulate:

1.	Agriculture on lots of one (1) acre or less;

2.	Buildings or structures incident to the use of land for agricultural purposes on lots greater than one (1) acre but not greater than five (5) acres by: Set back building lines, height, and size,

3.	Dairying and animal and poultry husbandry on lots greater than one (1) acre but not greater than five (5) acres when at least thirty-five percent (35%) of the lots in the subdivision are developed with at least one (1) building, structure, or improvement that is subject to real property taxation or that is subject to the tax on manufactured homes under ORC §4503.06. After thirty-five percent (35%) of the lots are so developed, dairying and animal and poultry husbandry shall be considered nonconforming use of the land and buildings or structures pursuant to ORC §519.19.

This section confers no power on any Township Zoning Commission, Board of Township Trustees, or Board of Zoning Appeals to regulate agriculture, buildings, or structures, and dairying and animal and poultry husbandry on lots greater than five (5) acres.

Such sections confer no power on any Township Zoning Commission, Board of Township Trustees, or Board of Zoning Appeals to prohibit in a district zoned for agricultural, industrial, residential, or commercial uses, the use of any land for farm market where fifty percent (50%) or more of the gross income received from the market is derived from the produce raised on farms owned or operated by the market operator in a normal crop year. However, a Board of Township Trustees may regulate such factors pertaining to farm markets as size of the structure, size of parking areas that may be required, set back building lines, and egress and ingress, where such regulation is necessary to protect the public health and safety. A farm is defined by the “CAVU” tax schedule.

B.	Railroads

1.	Except as otherwise provided in division (B) or (C) of this section, sections 519.02 to 519.25 of the Revised Code confer no power on any board of township trustees or board of zoning appeals with respect to the location, erection, construction, reconstruction, change, alteration, maintenance, removal, use, or enlargement of any buildings or structures of any public utility or railroad, whether publicly or privately owned, or the use of land by any public utility or railroad, or the operation of its business.

C.	Telecommunications Towers Exceptions

As used in this division, “telecommunications tower” means any free-standing structure, or any structure to be attached to a building or other structure, that meets all of the following criteria:

1.	The free-standing or attached structure is proposed to be constructed on or after October 31, 1996.

2.	The free-standing or attached structure is proposed to be owned or principally used by a public utility engaged in the provision of telecommunications services.

3.	The free-standing or attached structure is proposed to be located in an unincorporated area of a township, in an area zoned for residential use.

4.	Free Standing Structure

a.	The free-standing structure is proposed to top at a height that is greater than either the maximum allowable height of residential structures within the zoned area as set forth in the applicable zoning regulations, or the maximum allowable height of such a free-standing structure as set forth in any applicable zoning regulations in effect immediately prior to October 31, 1996, or as those regulations subsequently are amended.

b.	Attached Structure

The attached structure is proposed to top at a height greater than either the height of the building or other structure to which it is to be attached, or the maximum allowable height of such an attached structure as set forth in any applicable zoning regulations in effect immediately prior to October 31, 1996, or as those regulations subsequently are amended.

5.	The free-standing or attached structure is proposed to have attached to it radio frequency transmission or reception equipment.

6.	Sections 519.02 to 519.25 of the Revised Code confer power on a board of township trustees or board of zoning appeals with respect to the location, erection, construction, reconstruction, change, alteration, removal, or enlargement of a telecommunications tower, but not with respect to the maintenance or use of such a tower or any change or alteration that would not substantially increase the tower’s height. However, the power so conferred shall apply to a particular telecommunications tower only upon the provisions of a notice, in accordance with division (B) (4) (a) of this section, to the person proposing to construct the tower.

7.	Any person who plans to construct a telecommunications tower in an area subject to township zoning regulations shall provide both of the following by certified mail:

a.	Written notice to each owner of property, as shown on the county auditor’s current tax list, whose land is contiguous to or directly across a street or roadway from the property on which the tower is proposed to be constructed, stating all of the following in clear and concise language:

b.	The person’s intent to construct the tower;

c.	A description of the property sufficient to identify the proposed located;

d.	That, no later than fifteen days after the date of mailing of the notice, any such property owner may give written notice to the board of township trustees requesting that sections 519.02 to 519.25 of the Revised Code shall apply to the proposed location of the tower as provided under division (B)(4)(a) of this section.

If the notice to a property owner is returned unclaimed or refused, the person shall mail the notice by regular mail. The failure of delivery of the notice does not invalidate the notice.

8.	Written notice to the board of township trustees of the information specified in divisions (B) (3) (a) (i) and (ii) of this section. The notice to the board also shall include verification that the person has complied with division (B) (3) (a) of this section.

a.	If the board of township trustees receives notice from a property owner under division (B)(3)(a)(iii) of this section within the time specified in that division or if a board member makes an objection to the proposed location of the telecommunications tower within fifteen days after the date of mailing of the notice send under division (B)(3)(b) of this section, the board shall request that the clerk of this township send the person proposing to construct the tower written notice that the tower is subject to the power conferred by and in accordance with division (B)(2) of this section. The notice shall be sent no later than five days after the earlier of the date the board first receives such a notice from a property owner or the date upon which a board member makes an objection. Upon the date of mailing of the notice to the person, section 519.02 to 519.25 of the Revised Code shall apply to the tower.

b.	If the board of township trustees receives no notice under division (B) (3) (a) (iii) of this section within the time prescribed by that division or no board member has an objection as provided under division (b) (4) (a) of this section within the time prescribed by that division, division (A) of this section shall apply to the tower without exception.

9.	Any person who plans to construct a telecommunications tower within one hundred feet of a residential dwelling shall provide a written notice to the owner of the residential dwelling and to the person occupying the residence, if that person is not the owner of the residence stating in clear and concise language the person’s intent to construct the tower and a description of the property sufficient to identify the proposed location. The notice shall be sent by certified mail. If the notice is returned unclaimed or refused, the person shall mail the notice by regular mail. The failure of delivery does not invalidate the notice.

10.	As used in division (F) of this section:

a.	“Residential dwelling” means a building used or intended to be used as a personal residence by the owner, part-time owner, or lessee of the building, or any person authorized by such a person to use the building as a personal residence;

b.	“Telecommunications tower” has the same meaning as in division (B) (1) of this section, except that the proposed location of the free-standing or attached structure may be an area other than an unincorporated area of a township, in an area zoned for residential use.

D.	Public Utilities

Section 519.02 to 519.25 of the Revised Code confer power on a board of township trustees or board of zoning appeals with respect to the location, erection, construction, reconstruction, change, alteration, maintenance, removal, use, or enlargement of any buildings or structures of a public utility engaged in the business of transporting person or property, or both, or providing or furnishing such transportation service, over any public street, road, or highway in this state, and with respect to the use of land by any such public utility for the operation of its business, to the extent that any exercise of such power is reasonable and not inconsistent with Chapter 4901, 4905, 4909, 4921 and 4923, of the Revised Code. However, this division confers no power on a board of township trustees or board of zoning appeals with respect to a building or structure of, or the use of land by, a person engaged in the transportation of farm supplies to the farm or farm products from farm to market or to food fabricating plants.

E.	Liquor Sales

Section 519.02 to 519.25 of the Revised Code confer no power on any township zoning commission, board of township trustees or board of zoning appeals to prohibit the sale or use of alcoholic beverages in areas where the establishment and operation of any retail business, hotel, lunchroom, or restaurant is permitted.

F.	Oil and Gas Production

Section 519.02 to 519.25 of the Revised Code do not confer any power on any township zoning commission, board of township trustees, or board of zoning appeals to prohibit the use of any land owned or leased by an industrial firm for the conduct of oil or natural gas well drilling or production activities or the location of associated facilities or equipment when such oil or natural gas obtained by the industrial firm is used for the operations of its own plants.

G.	Outdoor Advertising Classified as a Business Use (Billboards)

For the purpose of ORC §519.02 to 519 .25, outdoor advertising shall be classified as a business use and shall be permitted in all districts zoned for industry, business, or trade, and land used for agricultural purposes.

H.	Limitation of Restrictions of Family and Group Homes

Licensed family homes and licensed group homes for handicapped persons are provided for under ORC §5123.19. Such facilities shall be permitted and regulated in accordance with the Resolution and in accordance with the laws of the State of Ohio.

I.	Submission to Director of Transportation

According to ORC §5511.01, before any zoning permit is issued affecting any land within three hundred (300) feet of the centerline of a proposed new highway or a highway for which changes are proposed as described in the certification to local officials by the Director of Transportation of any land within a radius of five hundred (500) feet from the point of intersection of said centerline with any public road or highway, the Zoning Inspector shall give notice, by registered mail, to the Director of Transportation, and shall not issue a zoning permit for one hundred twenty (120) days from the date the notice is received by the office. If notified that the state is proceeding to acquire the land needed then a zoning permit shall not be issued. If notified that acquisition at this time is not in the public interest or upon the expiration of the one hundred twenty (120) day period or any agreed upon extension thereof, a permit shall be granted if the application is in conformance with all provisions of this resolution.

J.	Type B Family Day Care Homes

According to ORC §5104.054, any type B family day-care home, whether certified or not certified by the county director of human services, shall be considered to be a residential use of property for purposes of municipal, county, and township zoning and shall be a permitted use in all zoning districts in which residential uses are permitted. No municipal, county, or township zoning regulations shall require a conditional use permit or any other special exception certification for any such type B family day-care home.

Sec. 203.2	Required Compliance

No building or land shall be used in a manner that does not comply with all of the 				district provisions established by this Resolution.  Any building not being specifically 				permitted shall be considered prohibited until, by amendment, such use will be 					consistent with this Resolution.

Sec. 203.3	Nuisance Prohibited

No use shall be permitted or authorized to be established which, when conducted in 				compliance with the provisions of this Resolution, and any additional conditions and 				requirements prescribed, is or may become hazardous, noxious, or offensive due to 				emission of odor, dust, smoke, fumes, cinders, gas, noise, vibration, electrical 					interference, refuse matter, water-carried wastes, or which will interfere with adjacent 				landowners’ enjoyment of the use of their lands

Sec. 203.4	General Regulations of Lots

A. 	One Principle Building Per Lot

No more than one principal building shall be permitted on any one lot.  						Every principal building shall be located on a lot having frontage on a public 					dedicated street or a private street meeting the minimum construction 						standards required for public streets.

B.	Required Lots, Yard, and Open Space Unaffected by Change of Ownership

No space which, for the purpose of a building, has been counted or calculated as 					part of a side yard, rear yard, front yard, or other open space required by this 					Resolution, may, by reason of change in ownership or otherwise, be counted or 					calculated to satisfy the yard or other open space requirement of or for any 					other building.

C.	Projections into Yard Areas

1.	Every part of a required yard shall be open to the sky unobstructed, 						except one accessory building in a rear yard.

2.	Parking of automobiles as regulated by Article V herein.

3.	Fences constructed in compliance with Section 204.10 of this Resolution, 						terraces, uncovered porches, platforms and ornamental features which 						do not extend more than two feet above the level of the ground or 						first story, may project into a required side or rear yard, provided these 						projections are at least ten feet from the adjacent lot line.

An open, unenclosed porch or paved terrace may project into the front 						yard for a distance not to exceed ten feet.

D.	Substandard Lots

Any lot greater than five thousand square feet in area but not meeting minimum 					area requirements and being a lot of record or lot for which a land contract has 					been issued or any lot within an unrecorded allotment, of which at least one-half 				of said lots are recorded or have been sold on land contract on the effective date 				of these regulations, may be used for single-family dwelling irrespective of the 					area, depth, or width of said lot,  provided however, that in no instance shall the 					minimum dimensions of the side and rear yards be less than five  and twenty 					feet respectively.  Lots less than five thousand square feet are not buildable lots.

E.	Lot Storage and Lot Display

1.	Merchandise Display

Storage of materials, equipment, and supplies and displays of 							merchandise shall take place within a completely enclosed building 						except as otherwise provided in this Resolution.

2.	Disabled Vehicles

No person shall park, store or leave, or permit the parking or storing of 						any disabled vehicle in a rusted, wrecked, junk, partially dismantled, 						unlicensed, inoperative or abandoned condition, whether attended or 						not, upon any property within a Residential or Commercial District unless 						the same is completely enclosed within a building.

3.	Boats and Recreational Vehicles

a.	Location

1.	In R-1, R-2 and R-3 Districts, no boat or recreational 								vehicle, including travel trailers, shall be stored in front or 								side yards or between a dwelling and a street right-of-way 								line, except that a boat, or recreational vehicle located in a 							rear yard shall not be closer than five feet to any property 								line or ten feet to any street right-of-way, whichever is 								greater.

- In an R-4 Residential District, camping trailers, recreational vehicles and/or boats, may be parked in the driveway, or side or rear yard, provided that no living quarters shall be maintained, or any business conducted in connection therewith. All on-road recreational vehicles which require licensing by the State of Ohio must be currently licensed. All vehicles shall comply with setback regulations for that district.

b.	Limitations on Number and Duration of Parking – Outdoor Storage

No more than one recreational vehicle and/or one boat on 							a trailer may be temporarily parked, provided that no occupancy 							for human habitation is maintained or business conducted within 							said trailer or motor home.  Such recreational vehicle or boat may 						be temporarily parked for a period not to exceed three days in 							any ten-day period. Preparation or minor maintenance only 							shall be permitted during this three-day period.

c.	Ownership

The storage of motor vehicles, trailers, recreational vehicles or 							boats shall be owned by the residents of the premises only.  							Storage of any such items for non-residents whether or not a fee 							is charged shall be deemed a business.

d.	Occasional or temporary visitors may stay for a period of up to 							30 days, and shall follow the regulations as stipulated in a-1 and 							a-2 of this section. When an occasional or temporary visitor 							intends a longer than 	thirty-day visit, the owner and visitor must 							apply for one thirty-day extension with the Zoning Inspector, and 							the one thirty-day extension may be granted where the visitor 							satisfactorily demonstrates that the motor home, tent, trailer, 							camper or other vehicle is not being used as a permanent 								residence.

4.    	Storage in Vehicles

Mobile homes, trailers, utility trailers, commercial vehicles or other 	motor vehicles shall not be used for the storage of equipment or 	materials in any district.

5.	Garage and Yard Sales

a.	Conditional Approval Terms

Garage or yard sales are permitted on residential property 								provided that they meet the following requirements:

1.	No single sale shall last longer than three consecutive 								calendar days.

2.	Sales are held no more than one time within a three-								month period.

3.	Sales are conducted on the owner’s property.

4.	Multi-family sales are permitted if they are held on the 								property of one of the participants.

5.	All directional and advertising signs shall be freestanding 								and removed after completion of the sale.  Such signs shall 							not be located in the street right-of-way, in the curb strip 								or on township property, trees or utility poles.  Refer to 								Section 401.2A2 for temporary sign permit regulations.  A 								permit is issued and fee paid in accordance with the 								official fee schedule kept in the township office.

Sec. 203.5	General Regulations of Structures and Construction

A.	Construction Begun Prior to Establishment of Zoning

Nothing contained in these regulations shall hinder the construction of a building 				or prohibit its use where construction has started before the effective date of 					these regulations and provided further that such building shall be completed 					within one year from the date of passage of the regulations.  Completion shall 					be determined by the original plans as submitted for building permit.

B.	Height Measurement and Requirements

1.	Height Measurement

a.	Where specified in stories, building height shall be measured in 							number of complete stories above the finished grade for any 							elevation fronting on a public street including attics, half-stories, 							mezzanines, at-grade structured parking, but excluding features 							that are greater than one-half story or completely below grade, 							such as basements, cellars, crawl spaces, subbasements, and 							underground parking structures.

b.	Where specified in feet, building height shall be measured as the 							vertical distance from grade at the base of the structure to (See 							Figure 203.5):

1.	The highest point of a flat roof;

2.	The deck line of a mansard roof; or

3.	The mean height between the eaves and ridge on gable, 								hip, or gambrel roofs.

c.	The height will be measured at a point in the middle of the 							structure at the front of the building.  Where the lot has two or 							more front yards, the heights will be averaged.

2.	Height Exemptions

a.	Aerials, antennas, wireless telecommunications towers, telephone 						transmitters and towers, cooling towers, elevator bulkheads, fire 							towers, conveyors, silos, farm buildings, or necessary mechanical 							appurtenances, may be erected to any lawful and safe height.

b.	Public or public service buildings, hospitals (except as otherwise 							provided), institutions, or educational facilities, church spires, 							domes, flag poles, chimneys, belfries, monuments, stacks, stage 							towers or scenery lofts, tanks, water towers, when permitted in 							any zoning district, may be erected to a height not exceeding 							seventy-five feet if the building is set back from each required

yard line at least one foot for each foot of additional building

height above the height limit otherwise provided in the applicable

zoning 	district in which the building is built.

c.	The height of cell towers and other wireless communication towers are subject to the use-specific regulations of this Resolution.

Figure 203.5			Mansard Roof					       Flat Roof

Building Height

Deck Line

Building Height

Grade

Gambrel, Hip and Gable Roofs

1/2 roof height

Building Height

Grade

C.	Temporary Buildings, etc. Used for Construction Work

1.	Temporary buildings, tents, storage containers, construction trailers, 					equipment and material uses incidental to construction work may only 					be permitted in any district during the period construction work is in 					progress, as per Section 1001.3E.

2.	Such temporary facilities shall be removed upon completion or 						abandonment of the construction work.  Storage of such facilities or 					equipment beyond the completion date of the project shall require a 					special permit authorized by the Board of Zoning Appeals.	
			3.	Signs shall be permitted in compliance with Article VI.	
			4.	Parking shall be required in compliance with Article V.	
			5.	On-street parking shall not be permitted.							D.	Accessory Buildings and Uses											1.	General Provisions

a.	General Development Standards

Accessory Uses or structures shall be permitted provided:

1.	The building or use is incidental to and customarily found 								in connection with a principal building or use permitted in 								the district in which is located.

2.	It is subordinate to and serves the principal building or use.

3.	It is subordinate in size, area, extent, and purpose to the 								principal building or use.

4.	It is located on the same lot as the principal use which it 								serves.

5.	An owner applies for and receives a Zoning Certificate 								unless exempted by this Section.

6.	Accessory uses shall be prohibited in any open space area 								that is preserved by covenant including, but not limited to, 							open spaces in any planned development or open space 								residential development (See Exemptions in Section 								203.5F).

7.	Accessory buildings and uses established prior to the 								effective date of this Resolution which does not comply 								with the setback or coverage requirements of this section 								shall still be considered in compliance with this Resolution.  							Any further structure modification of pre-existing

accessory uses and buildings shall be in compliance with

this Resolution.

8.	No accessory buildings may occupy more of the lot than 								the footprint of the principal building.

9.	The total of all accessory buildings shall not occupy more 								than thirty percent of the rear yard.

10.	Accessory buildings and uses shall also meet any district 								and use specific standards in this and other articles of this 								Resolution.

b.	Accessory Buildings and Uses Not Requiring a Zoning Certificate

1.	Any accessory building or use that is ancillary or incidental 								to an agricultural use that is exempt from zoning 									requirements pursuant to Sec. 203.1 (Exemptions from 								Zoning Certificates).  Owners intending to build a structure 							that is accessory to an agricultural use shall apply to the 								Lawrence Township Zoning Department for a zoning 								waiver (Agriculture Exemption Form).

2.	The following uses are exempt from the Zoning Certificate 								requirement and may be located in any yard:

a.	Basketball hoops provided they are set back ten 									feet from any street right-of-way

b.	Bird baths

c.	Flag Poles (See Section 203.5B2)

d.	Fountains

e.	Statues

f.	Minor play structures

g.	Other structures or uses as permitted by the Zoning Commission.

c.	Location of Accessory Buildings and Uses

Unless otherwise stated in this Section or exempted below, 							detached accessory buildings and uses shall be located in the rear 							yard.

1.	Exemptions:

a.	On a corner lot, accessory buildings and uses may 									be located in the side yard as illustrated in Section									204.4 (Corner Lots).

b.	On a double frontage lot, accessory buildings may 									be located in the front yard located to the rear of 									the house as illustrated in Section 204.5 (Double 									Frontage Lots)													c.	Detached garages may be located in the side yard 									provided they are set back from the front building 									line of the principal structure.

d.	An accessory building or use may be located in any 									yard where such building or use is set back two 									hundred feet or more from a street right-of-way 									exclusive of panhandles.

d.	Attached Versus Detached Accessory Buildings

Accessory buildings shall be considered attached and subject to 							the site development standards of the applicable zoning district 							rather than the provisions of this section when they meet any of 							the following:

1.	The structure is an integral part of a principal building and 								not separated by an open porch, walkway, breezeway, or 								other similar open structure.

2.	The structure is connected by a permanently enclosed 								porch, walkway, breezeway, or other similar enclosed 								structure.

E.	Permitted Accessory Uses in Residential Districts.

The following are permitted accessory uses or structures that shall not count 					toward the maximum thirty percent coverage established in Section 203.4D9

and shall not require a Zoning Certificate unless otherwise noted:

1.	Gardens.

2.	Woodpiles provided the wood is neatly stacked and does not exceed six 						 feet in height.

3.	Patios that are not enclosed which require a zoning certificate.

4.	Fences, which require a Zoning Certificate.

5.	Swing sets and gym sets.

6.	Wood or solid surface decks, which require a Zoning Certificate.

7.	Compost piles or bins provided they meet the following provisions:

a.	The compost pile or bin is located in the rear yard and setback a							minimum of five feet from every lot line.

b.	The maximum area of a compost pile shall be twenty-four square 							feet.

c.	The compost pile or bin is setback a minimum of twenty feet 							from a dwelling.

d.	The compost pile or bin does not exceed six feet in height.

8.	Exterior enclosures or private kennels for household pets provided that 						the enclosure is located in the rear yard and is set back a minimum of 						 twenty feet from every lot line.  A private kennel is defined as a kennel

for pets of the resident.

9.	A satellite signal receiver, as defined in Definitions, restricted to the sole 						purpose of receiving and amplifying signals, for television reception shall 						be permitted in all Districts subject to the following conditions and 						restrictions:

a.	Applicability

The following Categories of satellite signal receiver shall be 							exempt from all zoning requirements and shall not require a 							zoning certificate:

1.	A satellite signal receiver that is two meters (78.74 									inches) or less in diameter and located or proposed to be 								located in a commercial or industrial zoning district.

2.	A satellite signal receiver that is one meter (39.37 									inches) or less in diameter and located in any zoning 								district.

b.	Site Plan

A plan for a wall or roof mounted satellite signal receiver or a 							ground mounted satellite signal receiver that is not exempted 							under Section 203.5E9a shall be submitted to the Zoning

Inspector indicating the proposed height, diameter, location, and

setbacks.  Foundation details, landscaping, and screening shall

also be required in the case of a ground mounted satellite signal receiver.

c.	Standards

1.	Approval of a wall or roof mounted satellite signal

receiver, larger than one meter in diameter and attached

to the main building shall be subject to the following

standards:

a.	Location.  In all zone districts wall or roof mounted 									satellite signal receiver shall be prohibited on the 									front elevation of the building.

b.	Setbacks.  In all zone districts wall or roof mounted 									satellite signal receiver shall not be permitted to 									project into any required side or year yard area.

2.	Approval of a ground mounted satellite signal receiver 								shall be subject to the following standards:

a.	Location

1.	In the R-1, R-2 and R-3 Districts satellite 										signal receiver shall be located in the rear 										yard of the property.

2.	In all other Zone District, ground mounted 										satellite signal receiver shall also be 											permitted in the side yard.

b.	Setbacks

Ground mounted satellite signal receiver shall 									provide the following minimum setbacks:

1.	Rear Yard – In all zone Districts, fifteen feet.

2.	Side Yard – In all zone Districts, fifteen feet.

c.	Landscape

Ground-mounted antennas shall be screened from 									ground view from the street and adjacent property 									owners by landscaping as shall be approved by the 									Zoning Inspector and have a minimum opacity 									percentage of seventy-three percent for screening.

d.	Diameter

The diameter of such antenna shall not exceed the 									following:

1.	In the R-1, R-2, and R-3 Districts, two 										meters (78.74 inches)

2.	In all other Districts, three meters 											(118.11 inches)

e.	Height

Ground-mounted antennas shall be limited to a 									maximum height of twelve feet above grade in the 									R-1, R-2, and R-3 Districts, and a maximum height 									of fifteen feet above grade in all other Districts.

f.	Ground Coverage

The ground coverage of satellite signal receiver 									shall be counted in computing the ground coverage 								for auxiliary and accessory use structures located 									upon the building site.

g.	Number Permitted

Only one satellite signal receiver shall be allowed 									for each dwelling unit.

h.	Installation

The installation or modification of a satellite signal 									receiver shall be in accordance with all applicable 									construction and safety codes and procedures.

i.	Changing Providers

Should a new service provider be contracted, it is 									the responsibility of the occupant changing 										providers to insure the old satellite signal receiver

is removed from dwelling unit.

10.	Noncommercial amateur radio antenna structures for use by a licensed 						amateur radio operator in all residential districts provided that:

a.	A Zoning Certificate shall be required prior to construction of the 							antenna.

b.	The height of the structure shall not exceed the lesser of the

width or depth of the property as determined by a line bisecting

such antenna and measured from one side of the lot line to the 							opposite lot line, or from the front lot line to the rear lot line.  A 							property owner shall be permitted to establish an antenna with a 							maximum height of sixty feet regardless of the established lot 							width or depth.

c.	Antennas shall not exceed one hundred feet in height; and

d.	The setback for the antenna shall be a minimum of ten feet 							from all lot lines for antennas up to forty-five feet in height and 							shall increase by one foot for every three feet of increased 								antenna height.

All other radio or other antenna structures, not classified as a 							noncommercial amateur radio or a telecommunication tower, per

Section 310, shall be considered a conditional use in all districts subject

to review pursuant to Section 601 and Section 1001.5B (Appeals,

Variance, and Conditional Uses).

11.	Solar panels, which require Zoning Certificate, are permitted in 							accordance with the following:

a.	Solar energy equipment shall be located in the least visibly 							obtrusive location where panels would be functional.  Solar roof 							panels which simulate typical roofing materials such as asphalt 							shingles or clay tiles and are compatible with the design of the 							structure are preferred.

b.	Rooftop solar panels shall be installed on the plane of the roof 							material (flush mounted) or made a part of the roof design (e.g., 							utilizing capping or framing compatible with the color of the roof 							or structure), but shall not extend above the ridgeline of the roof.

c.	A ground mounted solar panel shall be subordinate in size to the 							principal structure it serves, shall not exceed fifteen feet in height, 						and is subject to lot coverage limitations and accessory structure 							location requirements.

d.	For rooftop or wall mounted solar panels, all exterior electrical 							lines shall be in conduit and painted in a color scheme that 							matches as closely as reasonably possible the color of the 								structure and materials adjacent to the conduit (i.e. conduit on 							walls should be painted the color of the structure of the walls 							while conduit on roof should be the color of the roof).

e.	For rooftop or wall mounted solar panels, all exterior plumbing 							lines must be painted in a color scheme that matches as closely as 						reasonably possible the color of the structure and materials 							adjacent to the plumbing lines.

f.	For ground mounted solar panels, all exterior electrical lines must 							be in conduit and conduit and plumbing lines must be buried.

g.	Solar energy equipment shall meet setback and height 								requirements for the zone.

h.	Solar storage batteries shall be placed in a secure container or 							enclosure when in use.

i.	Solar energy equipment shall not block required parking.
j.	Repair and replacement of existing solar energy equipment is 		exempt from this chapter provided that there is expansion in the 		rooftop or ground area covered by solar panels.

k.	Nonfunctioning solar energy equipment shall be removed within 							three months of becoming nonfunctional and shall be disposed 							of in accordance with applicable laws and regulations.

F.	Permitted Accessory Buildings/Structures in Residential Districts

1.	The following are permitted accessory buildings or structures that shall 						count toward the maximum thirty percent coverage established in

Section 203.5D1a9 and shall require a Zoning Certificate.

a.	Accessory dwelling units under the following provisions:

1.	They are located fully within the principal structure;

2.	There is no separate entrance;

3.	There is no separate address; and

4.	Only one dwelling unit, in addition to the principal

dwelling unit, may be permitted on any lot in the R-2 and

R-3 Districts.

b.	Private garages.

c.	Storage garages in the R-2 Multi-Family District when the garages 							are related to a permitted multi-family dwelling use and are solely 							for the use of tenants of such multi-family dwellings.

d.	Tool sheds and other similar structures.

e.	Roadside stand under the following provisions:

1.	A roadside stand shall only be permitted where fifty 								percent or more of the gross income received from the 								stand is derived from produce raised on farms owned or 								operated by the market operator in a normal crop year as 								referenced in ORC Section 519.21(c)(1).

2	The structure shall not exceed eight hundred square feet.

3.	The structure and any related parking areas (Section 								501.4) shall be set back sixty feet from all lot lines.

4.	Sign may be permitted provided it is in accordance with 								Section 401.1A2d.

f.	Tennis courts or other ball courts.

g.	Any outdoor swimming pool, bathhouse, tennis court, or other 							recreational facility designed for the use of the occupants of the 							dwelling and their guests.  Such facilities shall comply with the 							following conditions and requirements:

1.	Indoor pools are excluded from these regulations when

the pool enclosure is attached to the principal building.

2	The facility, equipment, decks and associated structures 								shall meet the accessory use and building setbacks.

3.	The swimming pool, or the entire property, on which said 								pool is located, shall be walled or fenced with a minimum 								four-foot-high wall or fence.  Any gate shall be latchable 								and lockable.

4.	The fencing for an above ground pool under four feet 								tall may be located on the outer pool wall.

5.	Above-ground pools that have a wall height of four feet 								or higher shall not be required to have a wall surrounding 								the pool or property.

6.	Any pool for the use of the occupants of a multi-family 								dwelling containing over three dwelling units shall meet 								the structural and sanitary requirements of the Ohio 								Department of Health.

7.	The pool will be used solely for the enjoyment of the 								occupants and guests of the principal use of the property 								on which it is located.

2.	Accessory buildings/structure in residential districts shall have a 							maximum height of twenty feet, but in no case shall the accessory 							building/structure exceed the height of the principal building.

3.	Accessory buildings and uses in residential districts shall be located a 						minimum of ten feet from side and rear lot lines; however, if a 							greater side yard setback for the principal building or use is allowed for 						the district in which the accessory building or use is proposed, the 							minimum setback from side and rear lot lines for such accessory building 						or use shall be the same as the side yard setback for the principal building 					or use.

G.	Accessory Buildings and Uses in Business Districts

1.	Accessory Dwelling Units

a.	One accessory dwelling unit may be permitted in the C-1, C-2, C-3 							or I-1 District for the following uses:

1.	Funeral services.

2.	Storage facilities or warehouses that may require twenty-four-hour security.

3.	Other uses as reviewed and authorized by the Board of 								Zoning Appeals.

b.	All accessory dwelling units must meet the yard setback 								requirements for the zoning district in which it is located.

2.	Any use which is accessory and customarily found in conjunction with,

and required for, the economic viability of the principal use that meets

the definition of an accessory use, and which complies with the applicable standards of the district in which it is located, shall be permitted in the C-1, C-2, C-3 or I-1 District.

3.	The total gross floor area of all accessory buildings in a business district 						shall not exceed fifty percent of the gross floor area of the principal 						building or structure.

H.	Swimming Pools

1.	Public or private in-ground or above-ground swimming, wading or other 						pools containing over one and one-half feet of water depth, shall 							not be located to the front or side of a dwelling, and shall conform to all 						required yard setback lines, except that in-ground pools shall be located 						not closer than ten feet to the rear property line.  The construction, 						plumbing, and electrical requirements, inspection, and other safety 						facilities shall be regulated by the appropriate codes.

2.	Every pool defined as a structure shall be completely surrounded by a 						fence, wall, or solid enclosure not less than three feet in height.  Such 						fence shall be constructed so as to have no openings, holes, or gaps 						larger than three inches in any dimension, except for doors or gates 						that shall be equipped with suitable locking devices to prevent 							unauthorized intrusion.  An accessory building may be used in or part of 						such enclosure.  Pools above-ground having vertical surfaces of at least 						three feet in height shall be required to have fences and gates only 						where access may be had to the pool.

3.	For the purpose of these regulations, ponds that are used primarily for 						agriculture or domestic water supply, decoration, wildlife preservation, 						or fishing shall not be considered as structures or swimming pools.

4.	Any above ground residential or commercial swimming pool which 						has been abandoned for more than twenty-four months shall be 							removed from the property.

5.	Any in-ground residential or commercial swimming pool which has 						been abandoned for more than twenty-four months shall be filled in.

Sec. 203.6	Architectural and Landscaping Review

A.	Architectural and Landscaping Review Requirements

1.	Site Plans

No person shall commence any use or construct or alter any structure 						without first submitting a site plan for architectural and landscaping 						review and approval except as provide for in Section 203.6E 							“Exemptions from Architectural and Landscaping Review Requirements”.

2.	Required Permits

A site plan for development shall receive architectural and landscaping 						review approval prior to the issuance of zoning, building or occupancy 						permits.

3.	Approval Site Plan

No use shall be carried on; no structure shall be constructed or altered, 						and no other improvement or constructions undertaken except as shown 						on an approved site plan

4.	Applicability

This section shall apply to all zones that specifically require an approved 						site plan except where an R-2 Planned Residential Development is 						required or undertaken in accordance with Section 302 of this resolution.

B.	Site Plan Submission

1.	Zoning Inspector

Prior to the submission of a site plan, any owner, builder or developer 						shall first consult with the Zoning Inspector.

2.	Site Plan Submission

All site plans shall be submitted to the Lawrence Township Zoning 							Inspector.

3.	Fees and Attachments

The site plan shall be complete and accompanied by all required fees and 						data.  Fees shall be required per the provision of Section 1001.4.

4.	Copies of Documents

Four identical copies of the site plan shall be submitted.  The Zoning 						Inspector may request additional copies.  On all commercial or industrial 						site plans, one copy shall be forwarded to the Fire Chief of the Bolivar 						Volunteer Fire Department for review.

5.	Size of Site Plans

Site plans shall be on one or more sheets that are twenty-four x thirty-six 						inches in size and shall be clearly and legibly drawn.

C.	Site Plan Contents

1.	Signing of Site Plans

Every site plan shall be signed by the owner of the land to which the site 						plan applies; or if a corporation, by a duly authorized officer of the 						corporation.

2.	Every plan shall include the following:

a.	The names, addresses and telephone numbers of the owners, 							developers and designers.

b.	Location of the property by Tuscarawas County parcel ID number.

c.	The legal description of the property included in the site plan.

d.	The boundary lines of the property including bearings, dimensions 						and a reference to a section corner, quarter corner or a point 							which has been established on a record plat.

e.	The scale of the site plan, a north arrow, date, and title.

f.	The zoning classification of the property included in the site plan 							and all adjoining properties.

g.	A vicinity sketch showing the location of the property in relation 							to the surrounding roadway system.

h.	The names of all subdivisions and property owners and the 							location of all property lines within five hundred feet of the 							property included in the site plan.

i.	Locations, widths and names of all existing streets, railroad rights 							of way, easements, permanent buildings, and corporation, 								township and county lines within five hundred feet of the 								property included in the site plan.

j.	Existing and proposed grades, drainage systems and structures 							with topographic contours at intervals not exceeding:

1.	One foot for zero to six percent slopes.

2.	Two feet for six to eighteen percent slopes.

3.	Five feet for slopes over eighteen percent

k.	Natural features such as wooded lots, streams, lakes, marshes and 						an indication as to whether they are to be retained, removed or 							altered.

l.	Wetland boundaries as delineated by a qualified professional.  If 							the proposed development will impact or alter wetlands, the 							applicant must provide evidence that the Ohio Environmental 							Protection Agency (OEPA) has been contacted, and the required 							permits have been obtained.  										m.	The types of soils found on the site and seasonal wind directions.

n.	All flood prone areas using the one hundred (100) year flood plain 						as a standard.

o.	The existing and proposed uses of the property and all structures 							thereon.

p.	The shape, size, location, height and floor area of all existing and 							proposed structures on the property with their final ground flood 							elevations and an indication as to whether a structure is to be 							retained, removed or altered.

q.	Front, side and rear elevations of all proposed or altered 								structures.

r.	The location and associated dimensions of all proposed streets, 							driveways, parking areas and sidewalks with directional 								indications for one-way streets and driveways.  Design geometrics 						should be included.

s.	The location and size of all existing and proposed public and 							private utilities such as sewer, water, gas and electric facilities 							with an indication as to whether they will be retained, removed or 						altered.

t.	The location dimensions and other relevant data for all proposed 							landscaping, fences, walls, or similar structures.  A landscaping 							plan must be submitted that includes the name, common name, 							location and quantity of all existing and proposed landscaping.

u.	The location, dimension, lighting and description of all signs.

v.	The location, intensity and orientation of all exterior lighting.

w.	A plan that illustrates the manner in which surface drainage will 							be accommodated.  This plan shall include and temporary erosion 							and sediment control measures to be employed during on-site 							construction.  All drainage areas influencing or influenced by the 							site shall be identified.

x.	A time schedule that indicates the anticipated starting and 								completion dates for construction.  If the development is to be 							staged, indication shall be made as to how the staging is to 							proceed.

y.	In the case of industrial uses, adequate provisions shall be made 							for the disposal of industrial wastes.  Wastes containing 								poisonous, corrosive, flammable or explosive solids, liquids, oils or 						gases shall not be discharged into the sanitary sewer.

z.	Any additional information that may be deemed necessary for 							proper and complete review when a proposed development 							presents difficult or unusual problems.

D.	Architectural and Landscaping Review of the Site Plan

1.	The Lawrence Township Trustees, Zoning Commission, Board of Zoning 						Appeals, and the Bolivar Volunteer Fire Department Fire Chief shall have 						the ability to review all site plans.

2.	Review comments concerning any site plan may be solicited from local, 						state and federal agencies including, but not limited to, the Tuscarawas 						County Engineer, Tuscarawas County Health Department, Ohio 							Department of Transportation, Ohio Department of Natural Resources 						and the Soil Conservation Service and the Tuscarawas County Regional 						Planning Commission.  The Board may seek expert advice or cause special 					studies to be made.  The cost of securing such advice or studies shall be 						borne by the applicant.

3.	All site plans shall be reviewed for their impact upon the general public 						and the occupants of nearby properties.  The site plans will be evaluated 						using the following architectural and landscaping standards and zoning 						requirements.

a.	The adequacy, location, and arrangement of vehicular and 								pedestrian circulation facilities.

b.	The adequacy, location and arrangement of parking and loading 							facilities.

c.	On-site traffic circulation shall be designed to make possible 							adequate fire and police protection.  The site plan shall be 								approved by the Fire Chief of the Bolivar Volunteer Fire 								Department for location of fire lanes and adequate fire apparatus 							access.

d.	The locations, arrangement, size and placement of all buildings, 							lighting facilities and signs to ensure that potential adverse effects 						on adjacent development are minimized.

e.	The design and location of building, landscaping, fences and walls 							to ensure adequate screening, and maximize visual and auditory 							privacy for surrounding properties.  Parking areas and service 							areas should be fully screened to promote harmony with adjacent 						development.

f.	The adequacy and design of storm water drainage facilities.  							Grading and surface drainage shall be designed to minimize 							adverse effects on abutting properties, streams, and public 							streets, and to minimize the possibility of erosion.  The Township 							may require that the County Engineer review grading and 								drainage plans with any costs borne by the applicant.  The cost of 							any unusual means necessary to alleviate surface drainage 								problems onto adjacent properties due to development shall be 							borne by the applicant.

g.	The treatment of environmentally sensitive areas such as 								woodlands, steep slopes (those greater than eight percent), 							areas with highly erodible soils and aquifer recharge areas.  							Building location and placement should be developed with 							consideration given to minimizing the need for tree removal, cut 							and fill, and stream and wetland alteration.

h.	The architectural design of buildings should be developed with 							consideration given to the relationship of adjacent development 							in terms of building height, mass, texture, line and pattern, and 							character.

i.	Any other factors necessary for a complete review by the 								Lawrence Township Zoning Inspector.

4.	The Zoning Commission may approve, approve with conditions, or deny a 					site plan request.  Upon the satisfaction of approval conditions, if any, 						approval shall be complete.

5.	Upon approval of the site plan, the Lawrence Township Zoning Inspector 						shall endorse two copies, one to be retained by the Lawrence 							Township Zoning Inspector and one to be returned to the applicant.

6.	Minor modifications as determined by the Zoning Inspector, to an 							approved site plan may be approved by the Zoning Inspector.  The 						Lawrence Township Trustees shall approve all other modifications.  A 						change in the building, where a site plan has been previously approved 						may, at the discretion of the Zoning Inspector, be considered a minor 						modification.

7.	An approved site plan shall expire one year after the date of its 							approval unless construction of the project has started.

8.	An approved site plan shall run with the land and shall not expire due to 						change in land ownership, providing the site use remains the same and 						notification of new ownership is submitted to the Trustees.

E.	Exemptions from Architectural and Landscaping Review Requirements

1.	The lawful construction, removal or alteration of a single family or two-family dwelling and any associated structures shall be exempt from the architectural and landscaping plan review requirements of Section 203.6.

2.	Any lawful use, construction, removal or alteration on land use for 						agricultural purposes shall be exempt from the architectural and 							landscaping plan review requirements of Section 203.6.

3.	Any alteration to a building, when confined to the interior of the 							structure, shall be exempt from the architectural and landscaping plan 						review requirements of Section 203.6.

4.	The painting of any structure or regular maintenance shall not require the 					preparation or approval of an architectural and landscaping plan.

5.	Any alteration of any structure upon less than twenty five percent of its 						exterior surface area, and less than twenty five percent of its building 						area (square footage) as measured against the exact condition of an 						existing structure as of the effective date of this section, shall be exempt 						from the architectural and landscaping plan review requirements of 						Section 203.6.

F.	Conformance with the Approved Site Plan

A performance bond, or other financial guarantee, shall be placed on deposit 					with the Trustees to ensure that the landscaping is installed, that the hard-					surfacing of the private drives and parking areas are installed, and that the 					surface water drainage is managed, all in conformance with the approved plans.  				Such bond or guarantee shall be in the amount equal to the cost of the 						construction of the improvements based on an estimate approved by the 						Trustees.

Sec. 203.7	Fire Hazard

Any activity involving the use or storage of flammable or explosive materials shall be 				protected by adequate fire-fighting and fire-suppression equipment and by such safety 				devices as are normally used in the handling of any such material.  Such hazards shall be 				kept removed from adjacent activities to a distance that is compatible with the potential 			danger involved.

Sec. 203.8	Outdoor Display, Sales, and Storage

A.	Purpose

The purpose of these regulations is to ensure the proper use of land for outdoor 					displays, sales, and storage so as to minimize impacts on surrounding property 					owners and uses.

B.	Applicability

The provisions of this Section shall apply to all uses except single-family, two-					family dwellings, and roadside markets that are accessory to an agricultural use 					and are permitted pursuant to Section 519.21 of the Ohio Revised Code and 					Section 203.5F (Accessory Buildings and Uses) of this Resolution.

C.	Exemptions

The following uses where the outdoor displays and sales are the permitted 					principal use of the lots shall be exempt from these regulations:

1.	Outdoor display, sales, and storage areas approved as part of a site plan 						or final development plan prior to the effective date of this Resolution.

2.	Automotive sales or rental in operable condition.

3.	Greenhouses, nursery and landscape supplies.

4.	Rental or sales for tools and machinery in operable condition.

5.	Similar uses as determined by the Board of Zoning Appeals.

D.	General Standards

1.	All outdoor sales, display, and storage areas shall require a Zoning 							Certificate and shall be illustrated on the corresponding site plan.

2.	Outdoor sales, display, and storage areas shall not be located in any 						required setback, parking and circulation area, right-of-way, or required 						landscape or buffer area.

3.	Such sales, display, and storage areas shall be prohibited if they will 						create any safety hazard for pedestrians.  A minimum pathway in areas 						used for outdoor displays, sales, and storage shall be provided to allow

for the flow of pedestrian traffic outside of designated vehicular traffic

drives. Such pathways shall have a minimum clearance width of five feet.

4.	Where screening or security fencing is provided or required, decorative 						cast iron, aluminum, wood material, or materials used in the principal 						building, shall be used for the fencing.  Other materials may be permitted 					with the administrative approval of the Zoning Commission.

5.	Chain link fencing, barbed wire fencing, and other wire mesh fencing shall 					be permitted only where the fencing is not visible from any public right-

of-way.

6.	All outdoor sales, display, and storage areas shall be maintained free of 						garbage and other debris.

7.	Outdoor sales, display, and storage areas shall be limited to five 							percent of the gross floor area of the principal structure with a maximum 						area of two thousand five hundred square feet.

E. 	Standards for Outdoor Sales and Displays

Outdoor sales and displays may be permitted where such sales and display areas 					comply with the following regulations:

1.	Outdoor sales and displays are prohibited on vacant lots unless approved 						in advance by the Zoning Commission and only retail commercial uses are 					permitted in the applicable zoning district.

2.	Only those goods and material associated with the existing on-site use 						may be sold or displayed.

3.	Outdoor sales and displays may be permitted provided that the 							merchandise is displayed along the sidewalk, the walkway adjacent to the 					building, or in the side yard.

4.	Outdoor displays and sales of bulk or large products that exceed twenty						pounds, including, but not limited to mulch (bag or bulk), concrete, salt,

or other similar products that cannot be easily carried into the store for 						purchase shall be required to meet the requirements of Section 203.9 						(Standards for Outdoor Storage).

5.	Outdoor cafes and food service areas may be permitted when they 						comply with the follow regulations:

a.	Outdoor cafes or food service areas shall be located along a 							sidewalk or between a building and parking area.

b.	Outdoor café and food service areas wider than four feet shall 							be surrounded by railings that separate the eating area from the 							sidewalk or vehicular traffic.

c.	Umbrellas that shelter diners from the elements shall be secured 							so as not to create a hazard in windy conditions.

d.	Enclosing outdoor cafes or food service areas either by a 								permanent roof or to expand the existing structure shall meet all 							the requirements of a building within the applicable zoning

district and shall require the issuance of a Zoning Certificate.

Sec. 203.9	Standards for Outdoor Storage Areas

Outdoor storage areas may be permitted where such storage areas comply with the 				following regulations:

A.	Outdoor storage shall be prohibited on vacant lots.

B.	Only those goods and materials associated with the existing on-site use may be 					stored or sold in outdoor storage areas.

C.	Outdoor storage may be permitted provided that the storage areas are located in the side or rear yard.

D.	Storage of any goods or materials shall not exceed six feet in height.

E.	All outdoor storage areas shall be screened from view from the public right-of-					way by a six-foot fence in conformance with Section 204.10 (Fence Standards). 				  	Screening shall not be required if the outdoor storage area is located out of the 					view from any public right-of-way.

Sec. 203.10	Pets

A.	The keeping of exotic wildlife as a pet is prohibited in all residential districts and 					uses.

B.	Household pets are permitted in all districts in accordance with the following:

1.	The yard shall be kept free of pet feces and other pet debris.

2.	All outdoor pens, enclosures, doghouses, and kennels shall be kept in 						good repair, provide adequate protection from the environment, and 						shall be located as required by Section 203.5E8.

Sec. 204	Lot Measurements and Requirements

Sec. 204.1	Distance Measurements

Unless otherwise expressly stated, all distance specified in this Resolution are to be 				measured as the length of an imaginary straight line joining those points.

Sec. 204.2	Measurement of Lot Area

A.	The lot area is the total area within the lot lines of a lot, excluding any street 					right-of-way, access easements or other public dedication.

B.	Panhandles and other narrow appendages to lots, with less than the minimum 					required frontage for the particular zoning district at any point, shall not be 					included in the measurement of a lot area or contribute to the minimum lot area 				requirement of the applicable zoning district, nor shall they contribute to the 					measurement of any setback for the location of any buildings or structures.  See 					Figure 12-1

Figure 12-1   Areas not contributing to the Minimum Lot Area Requirement

Areas of a lot with less than the minimum required frontage for the particular zoning district shall not count toward the minimum lot requirement.

Sec. 204.3	Minimum Lot Width Requirement

All portions of every lot, including panhandle lots, shall have a minimum forty feet of lot 				width at every point of the lot, including the street frontage, unless otherwise approved 				through a PD District or if a reduction is permitted in accordance with the Tuscarawas 				County Subdivision Regulations.  In addition, all lots must meet the minimum site 				development standards established for each zoning district.

Sec. 204.4	Corner Lots

A.	Where a lot is considered a corner lot, the required minimum front yard setback 					shall be provided on each street or section thereof. (See Figure 12-2)

B.	The narrowest lot line opposite the public right-of-way shall be the rear lot line 					and the minimum rear yard setback shall be applied. (See Figure 12-2)

C.	The longest lot line opposite the public right-of-way shall be the side lot line and 					the minimum side yard setback shall be applied. (See Figure 12-2)

D.	All corner lots created after the effective date of this Resolution shall have a 					minimum lot area that shall not be less than one hundred fifty percent of the 					minimum lot area requirement of the applicable zoning district.

E.	The minimum required lot width of a corner lot in an R-1, R-2, R-3, or R-4 District 					shall be increased by an amount equal to the depth of the required rear yard 					setback of the applicable zone district.

Figure 12-2   Corner Lots

S  T  R  E  E  T

SIDE YARD

REAR YARD

FRONT

YARD

FRONT YARD

S  T  R  E  E  T

Sec. 204.5	Double Frontage Lots

A.	Front of house is defined as part facing street of legal address of dwelling as 					defined by the Tuscarawas County Auditor.

B.	Where a lot is considered a double frontage lot, the required minimum front yard setback shall be provided on both streets. (See Figure 12-3)

C.	The remaining lot lines not abutting a public road right-of-way shall be 						considered as side yards and shall have the required minimum side yard setback 					provided for each side lot line.  (See Figure 12-3)

Figure 12-3   Double Frontage Lots

S  T  R  E  E  T

F  R  O  N  T               Y  A  R  D

S I D E    Y A R D

S I D E     Y A R D

F  R  O  N  T                 Y  A  R  D

S  T  R  E  E  T

Sec. 204.6	Dedicated Private Drives

Dedicated private drives that provide access to two to five lots shall be considered

public streets for the 	purposes of establishing minimum setbacks and yard 					requirements.  Such drives shall be subject to the following provisions:

A.	Corner lots with one private street frontage and one public street frontage 					shall not be included in the calculation for the purposes of determining the 					number of lots served by the private drive and shall not be subject to 						requirements of this Section unless the use gains access from the private drive.

B.	There shall be a minimum setback, equal to the minimum front yard setback of 					the applicable zoning district, between the edge of pavement of a private drive 					and the principal building.  (See Figure 12-4)

C.	There shall be a minimum setback, equal to the side yard setback of the 						applicable zoning district, between the principal building and the side lot line.  					(See Figure 12-4)

D.	In all cases, the area between the building and the private drive shall be 						considered the front yard.  (See Figure 12-4)

E.	The creation of a private drive or a public street shall be designed to ensure that 					the minimum setback requirements of any existing dwelling shall be maintained 					and the new drive or street shall not create nonconformity because of 						setbacks.

Figure 12-4 Dedicated Private Drives

Dedicated Private Drive

REAR YARD

FRONT YARD

SIDE YARD

.

FRONT YARD

Min. Side Yard Setback

Minimum

Front

Yard

Setback

S  T  R  E  E  T

Sec. 204.7	Panhandle Lots

A.	The panhandle portion of the lot shall have a minimum frontage as regulated by 					the Tuscarawas County Regional Planning Subdivision Regulations Section 715.

B.	Flag or panhandle lots shall not be used to avoid the construction of a street.

C.	The minimum front yard setback requirement shall be measured from the lot

line that creates the rear lot line of the adjacent lot as illustrated in Figure 12-5.

D.	For panhandle lots that are not subject to the panhandle regulations of Section					204.6 (Dedicated Private Drives), there shall be no front yard setback 						requirement from the driveway or private drive that access less than two to five 					lots.  (See Figure12-5)

E.	The panhandle portion of the lot shall not be used for storage nor shall any 					structures be permitted in such portion of the lot.

Figure 12-5 Panhandle Lots Not Subject to Private Drive Provisions

REAR YARD

SIDE YARD

SIDE YARD

FRONT YARD

PANHANDLE

S  T  R  E  E  T

Sec. 204.8	Building Setback Lines

Building setback lines indicating the front, rear, and side yards shall be approved by the 				Zoning Inspector and indicated on all proposed panhandle, pie-shaped, corner, double 				frontage, or other irregularly shaped lots where the front of the lot is not apparent 				within the definition of this Resolution.  These setbacks shall be a part of, and recorded 				with a plat designating the division of land.

A.	Every part of a required yard shall be open to the sky and unobstructed except:

1.	As otherwise provided in this section.

2.	For accessory buildings in a rear yard.

3.	For the ordinary projections of skylights, sills, belt courses, cornices and 						ornamental features projecting not to exceed three feet in R-1, R-2, R-						3, and R-4 Districts and not to exceed twelve inches in all other Districts.

4.	Open or lattice-enclosed fire escapes, fireproof outside stairways, and 						balconies opening upon fire towers projecting into a yard not more than 						five feet.

B.	Terraces, uncovered porches, platforms and ornamental features which do not 					extend more than three feet above the floor level of the ground (first) story 					may project into a required front or rear yard, but shall not be closer to any side 					lot line than the side yard requirement.

C.	An open unenclosed porch, or paved terrace may project into a front yard for a 					distance not exceeding ten feet.

D.	For new construction on a residential lot where the adjacent lots are already 					developed and where building within one hundred feet of the subject lot are set 					back less than the minimum required front yard setback, the minimum front

yard setback for the new construction shall not be less than the average depths

of the front yards of the two buildings on each side and within one hundred feet

of such lot.

Sec 204.9	Vision Clearance Triangles

A.	Corner Lots at Intersections

1.	All corner lots at street intersections shall maintain, for safety vision purposes, a vision clearance triangle.  The vision clearance triangle shall consist of the area bounded by the right-of-way of the adjacent intersecting streets extending along those right-of-way’s centerlines thirty feet from the point of the right-of-way intersection, and a straight line connecting said latter points.

2.	Nothing within the vision clearance triangle shall be erected, placed, 						planted or allowed to grow in such a manner as to materially impede 						vision between the heights of two and one half and ten feet 							above the centerline of grades of intersecting streets.  (See Figure 12-7)

Figure 12-7   Intersection Vision Clearance Triangle

30 FEET

30 FEET

STREET RIGHT-OF-WAY

STREET RIGHT-OF-WAY

B.	Driveway Curb Cut

1.	A vision clearance triangle shall be maintained at all driveways and curb 						cuts for vision safety purposes.  The vision clearance triangle shall be 						formed by extending one line twenty-five feet along the street right of 						way from the edge of the driveway pavement and another line from that 						same point twenty feet along the driveway and then connecting the two 						endpoints.

2.	No sign or associated landscaping shall be placed within this triangle so 						as to materially impede vision between the heights of two and one half 						 and ten feet above the centerline grade of the streets.  (See 							Figure 12-8)

Figure 12-8   Driveway Vision Clearance Triangle

25 FEET

DRIVEWAY

20 FEET

STREET RIGHT-OF-WAY

C.	New Access Connections

1.	For State Routes

All access connections shall meet minimum requirements in regard to sight distance from the State of Ohio.  All applications shall include an approved permit from the State of Ohio Department of Transportation.

2.	For County and Township Routes

All new access connections shall meet the minimum guidelines as set in 						the Tuscarawas County Access Management Regulations.  All applications 					shall include an approved Sight Distance Survey from the Tuscarawas 						County Engineers office.

Sec. 204.10 	 Fence Standards

A.	Fence Administration

1.	Permit Required

No fence shall be constructed, or altered without a permit from the 						Zoning Inspector and after such application has been approved.

2.	Inspections

It shall be the duty of each property owner to determine property lines 						and easements and to ascertain that the fence thus constructed does not 						deviate from the plans approved by the Township, and such fence does 						not encroach upon another lot or parcel of land.  The Township shall 						furnish such inspection as it deemed necessary to determine that the 						fence is constructed in accordance with plans submitted for permit, 						provided, however, that the issuance of the permit by the Township shall 						not be 	construed to mean that the Township has determined that the 						fence is not encroaching upon another lot, nor shall it relieve the 							property owner of the duty imposed upon him therein.

B.	General Fence Standards for all Zoning Districts (except Agricultural Use) shall be:

1.	No barbed wire fence or similar sharp point fence shall be permitted in any district except for agricultural uses or as part of security fence in an Industrial or Commercial District.

2.	‘Razor’ wire fencing shall be prohibited in all Districts.

3.	Electrically charged fences shall be forbidden in all districts except on

sites used to confine livestock.

4.	All fences, gates, and ground between the fence and property line shall be maintained in good condition.

5.	Fences that are minimum fifty percent open may be erected to a 							maximum height of three feet in side and front yards.  (See Figure 12-9)

6.	All fences and walls must present the finished non-structural face 							outward.

7.	No fence or wall may be placed such that they interfere with street level 						sight visibility for a distance of fifty feet from any approach to an 							intersection.  No fence or wall shall be permitted to encroach upon public 					rights-of-way or easements.

8.	Mesh wire fence shall be permitted if incorporated into or an integral 						part of a wood rail fence.

9.	All fences on a parcel shall have a unified style.

10.	Guard rails shall not be used as fencing.

11.	Must have an Ohio Utility Protection Service ticket.

12.	Shall comply with O.R.C. 971 where applicable.

13.	Privacy fence may be placed on property line with the recommendation that fences between adjoining properties be placed a lawnmower’s width off of the property line so as to be able to maintain both sides of the fence while staying on one’s own property.

C.	Fence Standards for Residential Districts:

1.	Front Yard.

All front yard fences require a Fence Permit.  In front yards only the 						following fences shall be permitted:

a.	Decorative fences to a maximum height of three feet provided 							that said fence complies with Standards otherwise found in 							Section 204.10B.

b.	Open fences designed to only partially enclose an area.  An 							example of this would be a short length of white picket fencing 							incorporated into a landscape treatment along a walkway to a 							porch.

2.	Side and Rear Yards.

Permitted are:  fences to a maximum height of six feet, including 							privacy fences unless the property line abuts property in a commercial or 						industrial district, then a maximum height of eight feet.

3.	Other

The following shall be permitted:

a.	Chain link fences shall be permitted in residential districts for 						sport applications.  Chain link fences for full size tennis courts, 						basketball courts, baseball diamonds, softball diamonds and other 					similar applications are limited to ten feet in height.

b.	Fencing around Swimming Pools.  See Section 203.5H.

D.	Fence Standards for Commercial and Industrial Districts:

1.	Chain link fences for full size tennis courts, basketball courts, baseball 						diamonds, softball diamonds and other similar applications as required 						by applicable sports authority standards.

2.	Fences that are seventy-five percent open, and used for security 						purposes, may be erected in the front, rear, or side yards, to a 						maximum height of eight feet, but shall not encroach into the minimum 					required front building setback.

Figure 12-9	Permitted Fencing

REAR YARD

6 FOOT FENCE

25% of Side Wall Length

SIDE

YARD

SIDE

YARD

FRONT YARD

3 FOOT FENCE

(50% OPEN)

Sec. 205	Tuscarawas County Regional Planning Acknowledgement

When Lawrence Township Zoning Regulations conflict with Tuscarawas County Regional 				Planning Guidelines (TCRPC), Lawrence Township Zoning regulations shall have precedence

Sec. 206	Time

Sec. 206.1	Computing Time

A.	The time within which an act is required by law to be done shall be computed by 					excluding the first and including the last day; except that when the last day falls 					on Saturday, Sunday or a legal holiday, then the act may be done on the next 					succeeding day which is not Saturday, Sunday or a legal holiday.

B.	When Township offices are closed to the public for the entire day which 						constitutes the last day of doing an act required by law, or if Township offices are 				closed before the usual closing time on such day, then such act may be 						performed on the next succeeding day which is not Saturday, Sunday or a legal 					holiday.

C.	“Legal Holiday” as used in this section means the days set forth in Lawrence 					Township Policy and Procedures Manual Section 7.2.

D.	If any day designated in Lawrence Township Policy and Procedures Manual 					Section 7.2 as a legal holiday falls on a Sunday, the next succeeding day is a legal 					holiday.

Sec. 206.2	Computation of Time

If a number of months is to be computed by counting the months from a particular day, 				the period ends on the same numerical day in the concluding month as the numerical 				day of the month from which the computation is begun, unless there are not that many 				days in the concluding month, in which case the period ends on the last day of the 				concluding month.

Sec. 207	Human Remains

Sec. 207.1	Human Remains Discovery

When human remains are unearthed in the course of excavation work in any district, 				the work in the immediate vicinity of such human remains shall cease forthwith, and the 			owner or owner’s representative shall call the Tuscarawas County Sheriff’s Office and 				notify the Zoning Inspector.  If the Sheriff, with whatever consultations he deems 				appropriate determines that the human remains do not constitute evidence of a crime, 				the Township shall report the remains to the Ohio Historical Preservation Office and the 				owner or owner’s representative shall consult at the owner’s expense with an 					archeologist designated by the Township to determine whether the human remains are 				prehistoric or Native American.  If the human remains are determined not to be 					prehistoric or Native American, they shall be reburied in accordance with applicable law.

Sec. 207.2	Prehistoric and Native American Human Remains

If the human remains are determined to be prehistoric or Native American, the Zoning 				Inspector shall immediately notify the Ohio Council for Native American Burial Rights 				(the “Council”) or The Ohio Center for Native American Affairs (the “Center”), or any 				successor to the Council or the Center deemed bona fide by the Zoning Inspector, that 				at its option the human remains (and any artifact physically attached thereto or 					determined by the archeologist to have been attached at the time such human remains 				were originally buried) are available for ceremonial reburial if claimed and receipted for 				within thirty days by a duly authorized representative thereof.  No work is to be 					resumed prior to the human remains and/or any artifacts being released by the Sheriff’s 			Office and/or the Council or Center and removed from the location where found.

During the thirty-day period, the human remains and any such artifacts that remain at 				the location where they were unearthed shall be accorded reasonable protection by the 			owner or owner’s representative.

If the human remains and any such artifacts are not so claimed and receipted for, they 				shall be donated to the Ohio Historical Society.

Sec. 208	Fire Hydrant Regulations on Private Property

Sec. 208.1	Hydrant Installation

Hydrants should be set plumb with a finish grade, which shall measure approximately 				eighteen inches from ground to the center of the steamer cap.  When hydrants are 				installed before grading is completed, the final grade line and accessibility must be 				considered. Most hydrants have a grade line indicated on the barrel of the hydrant.

Sec. 208.2	Hydrant Clearance

Fire hydrants shall be visible from the roadway and shall not be blocked from Fire 				Department view.  A minimum six-foot clearance area shall be maintained around 				and above the fire hydrant.  Tree branches, rocks, landscaping items, electrical or cable 				lines, advertising or information signs, and building roofs or overhangs shall not be 				permitted in the required clearance area.

Sec. 208.3	Hydrant Color

Except as otherwise provided in this section, fire hydrants shall not be painted without 				expressed written consent from the Bolivar Volunteer Fire Department or the 					Tuscarawas County Sewer & Water Department.  To help protect fire hydrants located 				close to streets and in or near parking lots from vehicles, a YELLOW line will be painted 				on the curb or parking lot near the hydrant.  (Refer to Appendix “A” for drawing.)

Sec. 208.4	Fire Hydrant Protection

There shall be four steel posts, six inches in diameter and a minimum of six 					feet long mounted in the ground to form a rectangle around fire hydrants in commercial 			and industrial districts.  The posts shall be fifty-four inches apart when facing the fire 				hydrant, and shall be forty-eight inches apart to the rear.  The top of the posts shall be 				thirty-six inches above finished grade level.  The post shall be set in concrete and filled 				with concrete.  (Refer to Appendix “A” for drawing).

Sec. 209	Buffers

Sec 209.1	Purpose

The purpose of this Article is to require buffering between non-compatible land uses 				and to protect, preserve and promote the character and value of surrounding 					neighborhoods, to promote the public health and safety through the reduction of noise 				pollution, air pollution, visual pollution, air temperature, and artificial light glare by 				providing for the installation and maintenance of buffer areas in accordance with the 				standards and requirements of this Article.

Sec. 209.2	Applicability

No structure on land that abuts a right-of-way or boundary between two non-					compatible uses shall be developed, or redeveloped unless a buffer area is established 				in accordance with the requirements of this Article.

Sec. 209.3	Definition of Terms

For the purposes of this Article the following terms shall have the following meanings:

A.	Buffer or Buffer Yard

An area of natural or planted vegetation adjoining or surrounding a land use and 					unoccupied in its entirety by any building structure, paving or portion of such 					land use, for the purposes of separating, screening and softening the effects of 					the land use, no part of which buffer is used for active recreation or parking, or 					interior access drives.  A buffer may include a wall, fence or berm as provided in 					accordance with the provisions of this Article.

B.	Canopy Tree

A deciduous tree with an expected height of at least thirty-five feet at maturity that, at time of planting has a minimum two- and one-half inch DBH, (diameter at breast height) of four- and one-half feet at ground level.

C.	Screen, Landscape

A method of reducing the impact of noise and unsightly visual intrusions with 					less offensive or more harmonious elements such plants, berms, fences, walls, 					or any appropriate combination thereof.

D.	Shrub

A woody plant which is smaller than a tree that has several main stems growing 					close to the ground and does not exceed six feet in height.

E.	Types of Buffers

The following types of buffers shall be required, as applicable, in accordance 					with the provisions of this Article:

1.	Boundary Buffer

The purpose of a boundary buffer is to separate adjacent non-compatible 					land uses and screen and soften the detrimental impacts of such non-						compatible use upon one another and upon the surrounding 							neighborhood.

2.	Streetscape Buffer

The purpose of a Streetscape Buffer is to shield or enhance views into a 						parking lot, establish coordination among diverse buildings, setbacks and 						uses, to retain the quality of the environment by providing appropriate 						means to diminish the effect of wires/poles, lights and other clutter along 					the public right-of-way.

F.	Plan Review

For any buffer required by this Article, a development plan shall be submitted 					with the application for the zoning certificate to the Lawrence Township Zoning 					Commission to review for compliance with these regulations and any other 					applicable regulations.  The development plan shall be prepared in accordance

with Section 203.6

Sec. 209.4	General Standards for Buffers

A.	Responsibility for Installation of Buffer

The boundary buffer area shall be provided by the person in charge of or in 					control of developing the property whether as owner, lessee, tenant, occupant 					or otherwise (hereinafter referred to as “Owner”).

B.	Submission of Plan

A plan of the proposed buffer shall be submitted to the Lawrence Township 					Zoning Commission for review.

C.	Review

The plan will be evaluated on the efficacy of the screening of the nonresidential 					use.  Screening should include planting of various sizes and species and comply 					with Section 209.6D.

D.	Location

Boundary buffers shall be located along the rear and side boundaries of a lot or 					parcel.  Buffers shall not be located on any portion of an existing, or dedicated or 				reserved public or private right-of-way.  On sloped areas the buffer shall be 					located to maximize its effectiveness.  Streetscape buffers shall be located 					adjacent to the public right-of-way and may be required along private street 					easements.

E.	Constructions within Required Buffer

No constructions shall be permitted within a required buffer other than a wall, 					fence or berm, or a driveway in the front yard connecting a parking area on the 					lot to a street or to a parking area on an abutting lot.  A driveway in the side yard 				that connects a paving area on the lot to the street shall not encroach into the 					buffer area.  Sub-surface constructions are permitted provided the required 					buffer yard is placed above said sub-surface constructions and further provided 					no part of a sub-surface construction protrudes above the ground surface.

F.	Adjacent Parcels Owned by the Same Owner

When both parcels are owned and being developed by the same owner, the 					buffer may be placed on either adjoining parcel or stride the boundary, however 					a buffer astride boundaries shall be filed with the Lawrence Township Zoning 					Commission and recorded on chain of title.

G.	Adjacent Parcels Owned by Different Owners

When adjoining parcels have different owners the buffer shall be placed on the 					parcel being developed.  However, a buffer that meets the requirements of both 					parcels may be placed astride the boundary if a written agreement, signed by 					both owners, is filed with the Lawrence Township Zoning Commission and 					recorded on the chain of title.

H.	Existing Fence, Wall or Berm on Adjacent Parcel

When the adjoining parcel has an existing fence, wall or berm adjacent to a 					developing parcel boundary, the existing landscape material on the adjacent 					parcel may not be used toward the requirements for the boundary buffer 					required for that boundary edge.

I.	Existing Development on Both Sides

Where development already exists on both sides of a property line, a buffer shall 				be established as a condition of any new development.  The property owner 					shall provide the maximum buffer possible under the standards of this Article 					given the location of existing buildings and driveways.

Sec. 209.5	Minimum Buffers for Adjoining Land Use

TABLE OF MINIMUM BUFFERS FOR ADJOINING LAND USE (IN FEET)

|  | Residential | C-1 | C-2 | C-3 | I-1 | MHCD |
| --- | --- | --- | --- | --- | --- | --- |
| Residential | 0 |  |  |  |  |  |
| C-1 | 10 | 0 |  |  |  |  |
| C-2 | 20 | 20 | 0 |  |  |  |
| C-3 | 40 | 30 | 20 | 0 |  |  |
| I-1 | 60 | 40 | 40 | 40 | 0 |  |
| MH | 60 | 40 | 40 | 40 | 40 | 0 |

Adjoining right-of-ways in all districts require Streetscape buffers except in Residential 				Districts.  Streetscape buffers in Residential Districts are required for parking area of five 			 or more vehicles within fifty feet of the right-of-way.  Buffers shall be provided on the 				more intense use parcel.

Sec. 209.6	General Landscaping Requirements

A. 	Parking Lots

In parking lots of five spaces or more, in residence districts R-1 through R-4, 					and in all other zoning districts, these requirements shall apply:

1.	Use deciduous shade trees with ground cover or low shrubs as the 						primary landscape material within the parking lots.  Avoid tall shrubs or 						low-branching trees that will restrict visibility.

2.	When planting islands that are parallel to parking spaces, islands shall be 						a minimum of nine feet wide.

3.	When planting islands that are perpendicular to parking spaces, islands 						shall be a minimum of eight feet wide.

4.	Mechanical equipment, trash, and loading areas shall be screened by 						using landscaping features.

5.	The use of porous pavement and/or specially designed brick or block is 						encouraged to increase on-site water detention for plant material and 						ground water supplies and to reduce problems associated with run-off.

6.	Waste receptacles shall be screened in accordance with Section 214. 				B.	Interior Landscaping

1.	General

The purpose of landscaping the interior of lots is so important to 							moderate heat, glare, wind and other climatic effects produced by 						pavement areas.  The minimum number of trees and shrubs for interior 						landscaping areas shall be calculated as follows:

a.	Required Parking

One tree and three shrubs per fifteen spaces or fraction 								thereof

b.	Parking in excess of code requirements

Two trees and six shrubs per fifteen spaces or fraction 								thereof

The trees and shrubs should be distributed throughout the parking lot to 						decrease the appearance of a single expanse of pavement and to create a 					canopy effect.  The trees should be of a variety to provide the shade 						canopy and have a clear trunk height of at least six feet.

2.	Minimum Area

The landscape area shall be a minimum of fifteen percent of the total 						ground area occupied by parking spaces.

C.	Site Perimeter Landscaping

An evaluation of the building site shall be made to determine the extent of tree 					stands, ponds or other existing natural features that can be preserved and 					incorporated into the landscaping plan.

D.	Screening

Between a building or parking lot and an adjacent residential use, a minimum of 					six-foot high evergreen hedge, wall, berm or wooden fence shall be provided.  					This screen may consist of a masonry wall, wooden fence, or a row of evergreen 					shrubs that will be six feet in height and seventy three percent opacity within 					two years.

E.	Grade Changes

In cases where grading is necessary that results in a parking lot lower in elevation 				that the surrounding area or adjacent right-of-way, the resulting embankment 					shall be planted with low shrubs and shade or ornamental trees.  The type and 					the variety of plantings shall be based on the steepness of the slope.  A 						maximum slope of one to two and one half shall be provided for landscaping 					between the lot line and the parking lot.

F.	Landscaped Berms

The use of earthen berms is encouraged to provide screening.  When berms are 					provided, plantings shall be combined with berms to provide effective 						landscaping features.

Sec. 209.7	Recommended Plant Materials and Maintenance

A.	Tree and Shrub Selection

Trees, shrubs, and perennials shall be selected on the basis of climate, natural 					habitat and visual screen.  No plants listed by the Ohio Department of 						Agriculture as invasive to Ohio shall be utilized.  See Definitions section.

B.	Minimum Size of Trees

Minimum size of trees, when installed, shall be as follows:

1.	Deciduous – Two and one-half inch DBH

2.	Evergreen – Six feet high

3.	Flowering - Six feet high.

C.	Minimum Landscape Requirements

Once the minimum landscape requirements have been met, any size tree may be 				installed on a lot to supplement the minimum requirements.

Sec. 209.8	Berming

Berming used for landscape and/or screening purposes shall be in accordance with the 				following:

A.	Mounding shall be located between the right-of-way and the building setback 					lines.

B.	Earth mounding shall vary in height, width, and length, to create a free form, 					naturalistic effect.

C.	The height of berms shall range between three and four feet in front yards 					along major arterial and between four and seven feet adjacent to 							residential districts; slope shall not exceed a three to one slope ration.

Sec 209.9	Maintenance

All landscaping shall be maintained in a healthy growing condition, neat and orderly in 				appearance, and free of refuse and debris; and all dead plant materials shall be removed 			and replaced within thirty days as required in order maintaining an attractive landscape 				at all times unless:

A.	Ground is frozen, or

B.	Water ban effect, or

C.	Drought conditions.

Sec. 210	Storm Water Management

Sec. 210.1	Erosion, Sedimentation, and Storm Water Management Controls

A.	Storm Water Management Control Plan Applicability

1.	An erosion, sedimentation and storm water management control plan 						shall be included with an application for a zoning certificate for the 						construction of all permitted buildings including accessory or conditional 						buildings, structures, off-street parking areas or loading/unloading areas 						allowed by this resolution and any amendments thereto.  The plan shall 						be forwarded to the Tuscarawas County Soil and Water Conservation 						District (TWCD) by the Zoning Inspector for review and approval.  The 						issuance of a Zoning Certificate will be contingent on written 							documentation from the TWCD that an erosion, sedimentation and storm 					water management control plan has been submitted and approved for 						the subject lot(s).

2.	Single lot residential construction with no contiguous land undergoing 						development by the same owner, builder, or developer may be exempt 						from this requirement if total site impervious cover does not exceed 						fifteen percent of the lot size and total land area disturbed during 							construction is less than twenty thousand square feet.  Land area that is 						disturbed from septic construction may be subtracted from the total 						disturbed area provided it is revegetated.  Owners, builders and 							developers who wish to apply for exemption under this provision must 						submit a signed Single Lot Residential Storm Water Management Plan 						along with an application for a Zoning Certificate.

3.	No erosion, sedimentation and storm water management control plan 						shall be required for sites containing less than three hundred square feet 						in area.

B.	Storm Water Management Plan Preparation

1.	An erosion, sedimentation, and storm water control plan shall be 							prepared by a professional engineer certified with the State of Ohio, or a 						soils scientist, and must address measures for controlling erosion, 							sedimentation and storm water during and after construction.  The 						content of the plan and such temporary and permanent measures to 						control erosion, sedimentation, and storm water shall meet all 							requirements of sediment, erosion and storm water control rules of the 						Ohio Division of Soil and Water Conservation as contained within Section 						1501:15-1-01 to 04 of the Ohio Administrative Code.

2.	ORC 4733.161 covers “Unauthorized Practices” for any person who 						performs labor or who provides services pursuant to a construction 						contract may offer or provide engineering services in connection with the 					person’s own labor or services.

C.	Storm Water Management Plan Submission

1.	Except as otherwise provided in this section, an erosion, sedimentation, 						and storm water control plan shall be submitted to the Zoning Inspector 						as a part of an application for a zoning certificate or a conditional use 						zoning certificate.   The Zoning Inspector shall advise the applicant that 						the Zoning Inspector may forward the plan to the Tuscarawas County Soil 					and Water Conservation District for technical assistance and review.

2.	Erosion and sediment control practices used to satisfy the standards shall 					meet the specifications in the current edition of “Rainwater and Land 						Development, Ohio’s Standards for Storm Water Management, Land 						Development and Urban Stream Protection” (Department of Natural 						Resources, Natural Resources Conservation Service, and Ohio 							Environmental Protection Agency), which is available to all Ohio County 						and Water Conservation Districts.

D.	Stabilization Measures for Disturbed Earth

All areas affected by earth disturbing activities on a lot shall be permanently 					seeded and mulched pursuant to the erosion, sedimentation and storm water 					control plan within forty-five days after the commencement of construction.  					Temporary stabilization measures shall be utilized when construction is 						scheduled to occur between November 1 and April 15 of the calendar year 					following zoning certificate issuance.

E.	The use of porous pavement and/or specifically designed brick or block is						encouraged to increase on-site water detention for plant material and ground 					water supplies and to reduce problems associated with run-off.

F.	Oversight Authority

The Zoning Inspector has the authority to ensure erosion, sedimentation, and 					storm water management control have been installed and maintained according 					to the approved plan required by this section.

Sec. 211	Environmental Performance Standards

This section provides performance standards for all non-residential uses permitted by this 			resolution.  These standards are intended to apply in addition to all other requirements of this 			resolution.  Non-residential uses shall at all times comply with the performance 					standards set forth herein:

Sec. 211.1	Air Pollution

A.	Odor

No odor shall be permitted at any lot line exceeding the lowest amount set forth 					in Table III, “Odor Thresholds”, of Chapter 5, “Physiological Effects”, “The Air 					Pollution Abatement Manual” of the Manufacturing Chemists’ Association, 					according to the latest edition of such table for the compounds therein 						described.

B.	Smoke

No activity, operation, or use shall, during normal operations, emit smoke at a 					density that exceeds number two on the Standard Smoke Chart in a quantity 					in excess of ten Smoke Units per hour per stack.

C.	Heat and Humidity

No use, operation or activity shall produce intense heat or excessive humidity in 					the form of steam or moist air that has a perceptible impact beyond the lot lines 					of the property.

D.	Dust and Particulate Matter

No use, operation, or activity shall exhaust or discharge into the air any quantity 					of fly ash, dust, dirt, or other particulate matter except in conformance with 					current air pollution standards of the Ohio Environmental Protection Agency 					(OEPA) and pursuant to a valid discharge permit issued by OEPA.  In no event 					shall there be any emission of solid or liquid particles in concentrations 						exceeding three-tenths grains per cubic foot neither of the conveying gas or air, 					nor of acid gases in excess of two tenths percent by volume.

Sec. 211.2	Erosion

No erosion, by either wind or water, shall be permitted which will carry objectionable 				substances onto neighboring properties.

Sec. 211.3	Water Pollution

No use, operation, or activity shall emit or discharge solids, liquids, or other matter into 				or onto any bodies of water, streams, or the ground except in conformance with the 				water pollution control standards established by the Ohio Environmental Protection 				Agency (OEPA) and pursuant to a valid discharge permit issued by the OEPA.

Sec. 211.4	Electrical Disturbance

No use, operation, or activity shall generate or emit any electrical disturbance that 				interferes with normal radio and television reception or the use of electrical equipment 				beyond the property boundaries of such use, operation, or activity.

Sec 211.5	Noise

Reference Lawrence Township Noise Resolution 22-07.

Sec. 211.6	Vibration

No activity or operation shall cause or create earthborn vibrations at the property 				boundary line in excess of the displacement amounts for frequencies set forth below:

Vibration Displacement

Frequency (in inches)

Cycles per Second			          Steady State				Impact

| Under 10 | .0008 | .0016 |
| --- | --- | --- |
| 10-19 | .0005 | .0010 |
| 20-29 | .0003 | .0006 |
| 30-39 | .0002 | .0004 |
| 40 and over | .0001 | .0002 |

Sec. 211.7	Toxic or Noxious Matter

A.	No use, operation or activity shall emit or discharge toxic or noxious matter in 					any form which may be detrimental to the public health, safety, or general 					welfare, or which may endanger the natural environment.

B.	The use or storage of any hazardous or regulated material shall be reported to 					the Bolivar Volunteer Fire Department using the appropriate Material Safety 					Data Sheets.

C.	Provisions for proper storage, use, and disposal of hazardous and/or toxic 					materials shall conform to the standards and requirements for such material as 					established by the Ohio Environmental Agency and shall be implemented in 					consultation with the Bolivar Volunteer Fire Department Chief.

Sec. 211.8	Radiation Hazards

A.	Unsealed radioactive material shall not be manufactured, utilized or stored 					(unless such materials are stored in a fireproof and concussion proof container at 				or below ground level) in excess of one million times the quantities set forth in 					Column 1 of the table in Section 38-2 of the Industrial Code Rule No. 38, relating 					to Radiation Protection of the New York State Department of Labor.

B.	None of the following fissionable materials shall be assembled at any one point, 					place, or work area on any parcel in a quantity equal to or in excess of the 					amount set forth herein:

| Material | Quantity |
| --- | --- |
| Uranium – 233 | 200 grams |
| Plutonium – 239 | 200 grams |
| Uranium – 235 | 350 grams |

Sec. 211.9	Fire and Explosive Hazards

A.	Storage, utilization, and/or manufacture of materials or products which are slow 					to moderate burning including those with open cup flash points above one 					hundred eighty-two degrees Fahrenheit are permitted in accordance with the 					standards set forth in 	the National Fire Protection Association’s Fire Protection 					Handbook.

B.	Storage, utilization, and/or manufacturing of materials or products which are 					free burning and/or intense burning, including those which have open cup flash 					points between one hundred degrees and one hundred eighty-two degrees 					Fahrenheit are permitted provided that:

1.	The material or products are stored, manufactured, and/or utilized only 						within completely enclosed buildings having noncombustible exterior 						walls.

2.	The buildings are set back at least forty feet from any lot line and one 						hundred feet from any residential zoning district boundary.

3.	The buildings shall be protected throughout by an automatic fire-							extinguishing system meeting the criteria set forth in the Ohio Basic 						Building Code for such systems.

C.	Storage, utilization, and/or manufacture of materials or products which are 					flammable, combustible liquids, produce flammable or explosive vapors or gases, 				or decompose by detonation shall not be permitted, except in conformance with 				the following:

1.	Storage of such materials, exclusive of finished products in original sealed 					containers, shall be underground.

2.	Any activity or process involving the use of such materials shall take place 					solely within a completely enclosed building which is protected with an 						automatic fire-extinguishing system and contains such other safety and 						fire-prevention equipment as required by the Ohio Basic Building Code, 						the Fire Protection Code, and the Lawrence Township Fire Chief.

3.	No such material shall be used or stored within fifty feet of any property 						boundary line, or within one hundred feet of any residential zoning 						boundary.

Sec 211.10	Glare

Any activity, operation, or use which produces glare or intense light emissions shall be 				adequately shielded, screened, or enclosed as not to be directly visible from any lot line.  			No glare or light emissions shall be of such intensity as to constitute a nuisance for 				adjoining properties.

A.	This means to the extent reasonably possible the reduction or prevention of glare and light pollution, the conservation of energy, and promotion of safety and security.	
B.	The following shall not be regulated by this Article:

1.	Lighting required by State or Federal regulations

2.	Illuminated poles for governmental or institutional flags

3.	Seasonal holiday lighting

4.	Athletic field and amphitheater lighting (when extinguished within one hour following games/programs)

5.	Emergency lighting

Sec. 211.11	Measurement Procedures

Methods and procedures for the determination of the existence of any dangerous and 				objectionable elements shall conform to applicable standard measurement procedures 				published by the American Standards Association, Inc., the Manufacturing Chemist 				Association, Inc., the United States Bureau of Mines, the National Fire Protection 				Association, and the Ohio Environmental Protection Agency, and any other generally 				accepted methods of collection and standard methods of chemical analysis.

Sec. 211.12	Enforcement

A.	The application for any non-residential zoning permit to the Zoning Inspector 					shall be accompanied by a statement setting forth the proposed use’s ability to 					comply with these performance standards and describing any materials, 						processes, or activities which constitute potential hazards, as set forth herein, 					and the proposed methods for mitigating those potential hazards.

B.	The Zoning Inspector may, from time to time, monitor a use’s performance to 					determine its continued compliance with these standards.  The Zoning Inspector 					shall have the authority to investigate complaints relating to alleged 						noncompliance with the standards set forth herein.

C.	The Zoning Inspector may take such appropriate action as may be deemed 					necessary to protect the public health, safety, and general welfare and to compel 				compliance with these performance standards.

Sec. 212	Overlay Districts

The provisions of the Overlay districts, including the Flood Plain and Wellhead Protection 			overlay districts, shall be applicable to all lands shown as being located within the boundaries of 		such overlay district and shall be supplemental to the regulations of the underlying zoning 			district.  Where the requirements of the overlay district are in conflict with the regulations of 			the underlying zoning district, the more restrictive regulations shall apply.

Sec. 213	Home Occupations

Sec. 213.1	General Provisions

A home occupation shall comply with the following provisions:

A.	The person running the home occupation lives at the residence.

B.	The use of a dwelling unit for the home occupation shall be clearly incidental and 				subordinate to the use of a dwelling for residential proposes.

C.	The use shall not utilize more than twenty percent of the gross floor area of the 					dwelling unit.

D.	There shall be no change to the exterior appearance of the dwelling unit, or any 					other visible evidence of the home occupation.

E.	One wall sign may be permitted provided that it shall not exceed four square 					feet in area, shall not be illuminated, and shall be mounted flat against the wall 					of the principal building.											F.	The home occupation shall not employ any person on the premises other than 					members of the immediate family and permanent occupants of the dwelling 					unit.

G.	It shall not utilize mechanical, electrical, or other equipment which produces 					noise, electrical or magnetic interference causes fluctuation in line voltage, 					vibration, heat, glare or other nuisances outside the dwelling unit in which it is 					located.

H.	The use shall not generate pedestrian or vehicular traffic in greater volumes than 				would normally be expected in a residential neighborhood.

I.	No expansion of existing off-street parking shall be permitted.  The home 						occupation shall not create any additional off-site parking demands.

J.	No commodity or stock in trade shall be sold, displayed or stored outdoors.

K.	It shall not require the use of common carrier vehicles for delivery of materials to 				or from the premises in greater volumes than would normally be expected in a 					residential neighborhood.

L.	Separate entrance into the dwelling unit for the sole purpose of home 						occupation access shall be prohibited.

M.	No accessory building or outdoor space shall be used in conjunction with the 					home occupation.

N.	The provision for home occupations provided in this section is intended to 					secure flexibility in the application of the requirements of this Resolution.  Such 					provisions are not intended to allow the essential residential character of 						residential districts, in terms of use and appearance, to be changed by the 					occurrence of activities associated with home occupations.

Sec. 213.2	Prohibited Home Occupations

The following home occupations shall be prohibited:

A.	Uses and activities that require a gathering point or dispatch center for 						employees where business is conducted off the premises.  These may include, 					but are not limited to the following:

1.	Landscaping.

2.	Contracting and Trucking.

B.	Home occupations that require fire safety inspections, precautions or permits or 					other regulatory inspections or permits.

C.	Home occupations that require the use of mechanical ventilation systems to 					exhaust the by-products of the home occupation.

D.	Home occupations that involve the use of controlled substances.

E.	Sexually Oriented Businesses.

Sec. 214	Waste Receptacles

Trash receptacles, dumpsters, or other containers thirty-two gallons or greater in size and 			intended for the temporary holding of trash, refuse, garbage or other discarded materials prior 			to collection, shall be screened from agricultural or residential properties as follows:

A.	Such waste receptacles shall only be permitted in the side or rear yard.

B.	Waste collection and storage facilities shall be screened from view from any public right-				of-way and/or adjoining residential property by an enclosure consisting of a solid 				masonry wall or fence with a minimum height of six feet.

C.	No waste receptacles shall be placed in the front yard more than one day prior to 				collection, and such receptacles must be removed from the front yard by midnight of 				the collection day.

Sec. 215	Freestanding Wind Energy Conversion System (WECS)

A.	Freestanding Wind Energy Conversion System (WECS) for energy uses shall be permitted 			as an accessory to a principal use within the R-4 Rural Residential Districts on lots five 				 acres or greater and comply with the following regulations:

1.	Primary purpose shall be to provide power for the principal use and accessory 					uses of the property and is not for the generation of power for commercial 					purposes.

2.	Placement of towers at a density of no greater than one per five acres.

3.	No WECS shall be located in any required front yard area.

4.	Maximum height shall be one hundred feet measured from the natural grade 					surrounding the support pad to the tip of the blade in a vertical position 						measured along the vertical axis of the tower.

5.	Minimum setback from all property lines, structures and above ground utility 					lines shall be two hundred feet.

6.	Anchor points for guy wires for the tower shall be located no closer than twenty-					five feet to the property lines and not on or across any above ground electric 					transmission or distribution line.

7.	Minimum height from the base of the tower to the lowest part of the blade tip 					or rotor system shall be twelve feet.

8.	Blade color shall be white or light gray.

9.	Lighting of the tower for aircraft and helicopter will conform to Federal Aviation 					Administration (FAA) standards for wattage and color, when applicable.

10.	The tower should have either:

a.	Tower climbing apparatus located no closer than twelve feet to the 						ground level at the base of the structure.

b.	A locked anti-climb device installed on the tower.

c.	Shall be completely enclosed with a locked fence at least six feet in 						height to prevent uncontrolled access from unauthorized personnel.

11.	A sign shall be posted at the base of the tower warning of electrical shock or high 				voltage.

12.	An automatic braking, governing or feathering system shall be required to 					prevent uncontrolled rotation.

13.	No variance shall be issued for the placement of a WECS so close to a property 					line as to result in any portion of the WECS at any time, whether erect or in the 					event the WECS should fall or be toppled, to overhang, cross or otherwise 					extend beyond the property line.

14.	The audible noise radiation due to wind turbine operations shall not cause the 					noise level at the boundary of the proposed project site to 	exceed thirty dB(A) 					for more than five minutes out of any one hour time period, or to exceed thirty 					five dB(A) for any time period; provided, however, if the Natural Ambient Noise 					Level without the commercial wind turbine generators is greater than thirty 					dB(A), the audible noise radiation shall not exceed five dB(A) above the Natural 					Ambient Noise Level.  A commercial wind energy facility shall not be operated so 				that impulsive sound below twenty Hz adversely affects the habitability or use of 				the dwelling unit, hospital, school, nursing home, or other sensitive noise 						receptor.

B.	Windmills eighteen feet in height or less shall be permitted in any residential district and 			shall have a minimum setback of fifteen feet from any building, rear or side property 				line, or equal to twice the height of the windmill, whichever is greater and twenty-five 				feet from any right-of-way line.

Sec. 215.1	Site Plan Requirements

Site Plan Requirements include but are not limited to:

A.	Property lines and physical dimensions of the site.

B.	Location for WECS tower, guy wires, and setbacks from property lines, above 					ground utility lines and any structure on the property.

C.	Location of signage.

D.	Elevation of the proposed WECS tower.

E.	Location of trees within a one hundred feet radius of the proposed WECS.

F.	Make, model, picture, and manufacturer’s specification, including noise decibels.

Sec. 215.2	Decommissioning

A.	The WECS owner/applicant shall complete decommissioning within twelve 					months after the end of the WECS useful life.  The term “useful life” is defined as 				zero electricity generation for a period of twelve consecutive months from a 					particular WECS.

B.	All decommissioning expenses are the responsibility of the Owner/Applicant.

C.	The Board of Zoning Appeals may grant an extension of the decommissioning 					period based upon request of the Owner/Applicant.  Such extension period shall 					not exceed one year.

D.	If the WECS Owner/Applicant fails to complete the act of decommissioning 					within the period described in this Section, the Lawrence Township Trustees may 				deem the WECS a public nuisance.

E.	Following removal of the WECS and all related structures, the site shall be 					graded, stabilized and seeded to prevent soil erosion.

F.	A performance bond, or other financial guarantee, shall be placed on deposit 					with the Trustees to insure that the decommissioning is completed and the area 					is replaced back to its natural state.  Such bond or guarantee shall be in the 					amount equal to the cost of the construction of the improvements based on an 					estimate approved by the Trustees.

Sec. 216	Small Structure Mounted Wind Energy Conversion System (SSM-WECS)

Sec 216.1	Small Structure Mounted Wind Energy Conversion Systems (SSM-WECS), including 				Blade Tip Power System Turbines (BTPS)

Shall be permitted in any Zoning District as an accessory to a principal use and comply 				with the following regulations:

A.	A permit shall be required before any site work is performed.

B.	The lowest extension of rotor blade or other exposed moving component of a 					SSM-WECS shall be at least fifteen feet above the ground, as measured from the 					highest point of grade within thirty feet of the base of the SSM-WECS.  In 						additional, the lowest extension of any rotor blade or other exposed moving 					component of a SSM-WECS shall be at least fifteen feet above any outdoor areas 				intended for human use that are located below the SSM-WECS.  Examples

include balconies, roof gardens, etc.

C.	A SSM-WECS shall not produce vibrations that are perceptible to humans beyond 				any property line upon which a SSM-WECS is located.

D.	Guy wires or similar apparatus shall not be allowed as part of a SSM-WECS or 					installation.

E.	The mounting height of a SSM-WECS shall not exceed fifteen feet above the 					highest point of the roof or structure, or a maximum of forty eight feet.

F.	A SSM-WECS shall be setback a minimum the height of the tower including the 					blade radius plus ten feet from any property line, public right-of-way, and public 					easement or overhead utility lines.

G.	If the SSM-WECS is affixed by an extension to a structure’s walls, roof, or other 					elevated surface then the setback from property lines, public right-of-way, and 					public easement or overhead utility lines shall be measured from the furthest 					outward extension of moving SSM-WECS components.

H.	If more than one SSM-WECS is installed on a property, then a distance equal to 					the mounted height of the adjacent SSM-WECS must be maintained between the 				bases of each SSM-WECS.

I.	Minimum lot size is ten thousand five hundred square feet for roof mounted, `				and five acres for tower mounted systems.

J.	Maximum tower height shall be forty-eight feet if ground mounted.

K.	Maximum Rotor Blade Length shall be six feet, and maximum turbine 						diameter shall be twelve feet.

L.	No portion of the turbine, including rotor blades, may extend over parking areas, 				driveways, or sidewalks.

M.	A site drawing showing the location of the unit in relation to existing structures 					on the property, roads and other public right-of-ways, and neighboring property 					lines.

N.	All wires should be concealed in conduit and/or underground.

.

Sec. 216.2	Decommissioning

A.	The SSM-WECS owner/applicant shall complete decommissioning within twelve 					months after the end of the SSM-WECS useful life.  The term “useful life” is 					defined as zero electricity generation for a period of twelve consecutive months 					from a particular WECS.

B.	All decommissioning expenses are the responsibility of the owner/applicant.

C.	The Board of Zoning Appeals may grant an extension of the decommissioning 					period based upon request of the owner/applicant.  Such extension period shall 					not exceed one year.

D.	If the SSM- WECS owner/applicant fails to complete the act of decommissioning 					within the period described in this Section, the Lawrence Township Trustees may 				deem the WECS a public nuisance.

E.	Following removal of the SSM- WECS and all related structures, the site shall be 					graded, stabilized and seeded to prevent soil erosion.

F.	A performance bond, or other financial guarantee, shall be placed on deposit 					with the Trustees to ensure that the decommissioning is completed and the area 				is replaced back to its natural state.  Such bond or guarantee shall be in the 					amount equal to the cost of the construction of the improvements based on an 					estimate approved by the Trustees.

Sec. 217	Small Wind Projects (FARM)

Two or more Small Wind Energy Conversion Systems (SWECS) less than five megawatts in 			capacity shall be permitted, with an approved Zoning Certificate, in R-4 Residential Districts 			subject to all requirements as provided herein.  Wind power systems five megawatts in 				capacity or greater are regulated by the Ohio Public Utilities Commission (PUCO).  Permissible 			SWECS shall include both Horizontal Axis Wind Turbines (HAWTs) and Vertical Axis Wind 			Turbines (VAWTs), and Blade Tip Power System Turbines (BTPs).  SWECS shall be subject to the 			conditions listed below:

|  | HAWT | VAWT |
| --- | --- | --- |
| Minimum Lot Size | Fifty acres/SWECS | Fifty acres/SWECS |
| Maximum Tower Height | One hundred fifty feet |  |
| Turbine Clearance | No portion of the turbine, including rotor blades, shall extend within twenty feet of the ground, except that the generator of a VAWT may be located on the ground at the base of the system. No portion of the turbine, including rotor blades, may extend over parking areas, driveways, or sidewalks. |  |
| Maximum Rotor Blade Length | Maximum rotor diameter shall be forty-three feet | Twenty feet less than the maximum height of the tower, as defined above, in order to provide for the twenty-foot turbine clearance |
| Minimum Setbacks | Two hundred feet. No part of the wind energy conversion system structure, including, but not limited to, guy wire anchors and any necessary ground-mounted conversion equipment, may extend closer than twenty-five feet to the property line. |  |

Figure 217

HORIZONTAL AXIS WIND TURBINE			VERTICAL AXIS WIND TURBINE

Sec. 217.1	Permits

A.	A permit shall be required before construction can commence on an individual 					wind turbine project.

B.	As part of the permit process, the applicant shall inquire with the FAA as to 					whether or not additional height restrictions are applicable due to the unit’s 					location in relation to any local airports, and provide documentation from the 					FAA clearly stating restrictions, if applicable, or no restrictions.

Sec. 217.2	Application

A.	The applicant shall provide the Township Zoning Inspector with the 						following items and or information when applying for a permit:

1.	Written approval, or notice that approval is not needed, from the FAA.

2.	A report that shows:

a.	The total size and height of the unit.

b.	Data specifying the kilowatt size and generating capacity in 							kilowatts of the particular unit.

c.	The maximum decibel level of the particular unit.  This 								information shall be obtained from the manufacturer of the 							turbine unit.

d.	Schematics and engineering drawings with a report									signed by a licensed engineer.

3.	A site drawing showing the location of the unit in relation to existing 						structures on the property, roads and other public right-of-ways, and 						neighboring property lines.  The site drawing must meet the site plan 						standards of Section 203.6 of the Zoning Resolution.

4.	A shadow flicker analysis demonstrating impact on adjacent and nearby 						residential properties.  (Small Wind Projects should be constructed in 						locations that minimize the impacts of shadow flicker on residents.)

B.	Sound.  Small wind energy conversion systems shall not exceed fifty-five dBA, 					measured five feet above ground level at the closest property line.  The sound 					level, however, may be exceeded during short-term events such as utility

outages and/or severe wind storms with sustained winds of fifty-eight miles per

hour or fifty knots.

C.	Automatic Over-Speed Controls.  All small wind energy conversion systems shall 					be equipped with manual (electronic or mechanical) and automatic over-speed 					controls to limit the blade rotation speed to within the design limits of the small 					wind energy conversion system.

D.	Utility Notification.  No small wind energy conversion system shall be installed 					until evidence has been given that the electrical utility company has been 					informed of, and approves of customer’s intent to install an interconnected 					customer-owned generator.  Off-grid systems shall be exempt from this 						requirement.

E.	Tower color.  Tower colors shall be white, off-white, gray, or neutral subdued 					tones, such as earth tones of green or brown.  Towers shall not be finished in 					bright or vivid colors, nor shall the tower be used for advertising of any kind.

F.	Lighting.  Small wind energy conversion systems shall not be artificially lighted, 					except as required by the FAA.

G.	Climb prevention.  Small wind energy conversion systems shall not be climbable 					up to fifteen feet above the ground surface.

H.	Compliance with other regulations.  The applicant or owner shall be responsible 					for acquiring all necessary approvals from other applicable agencies, including 					but not limited to the FAA.

I.	Maintenance.  Small wind energy conversion systems shall be maintained in 					working order, structurally sound, and with any surface treatments intact.

J.	Abandoned Facilities.  Any small wind energy conversion system that is not 					operated on a functional basis for a period of twelve consecutive months shall

be deemed abandoned.  The Zoning Inspector may order the repair or 						decommissioning of said small wind energy conversion system, in accordance 					with these provisions.  The applicant, owner, or other person responsible for the 					facility shall repair or decommission the same within sixty days of receipt of 					notification by certified mail.  If said facility is not either operational or removed 					after sixty days, the Township may decommission the system at the owner’s 					expense.

K.	A performance bond, or other financial guarantee, shall be placed on deposit 					with the Trustees to ensure that the site area is returned to its original state 					following decommissioning.  Such bond or guarantee shall be in the amount 					equal to the cost of the construction of the improvements based on an estimate 					approved by the Trustees.

L.	All wires should be concealed in conduit and/or underground.

Sec. 217.3	Decommissioning

A.	The SWECS owner/applicant shall complete decommissioning within twelve 					months after the end of the SWECS useful life.  The term “useful life” is defined

as zero electricity generation for a period of twelve consecutive months from a 					particular SWECS.

B.	All decommissioning expenses are the responsibility of the owner/applicant.

C.	The Board of Zoning Appeals may grant an extension of the decommissioning 					period based upon request of the owner/applicant.  Such extension period shall 					not exceed one year.

D.	If the SWECS owner/applicant fails to complete the act of decommissioning 					within the period described in this Section, the Lawrence Township Trustees may 				deem the SWECS a public nuisance.

E.	Following decommissioning of the SWECS and all related structures, the site shall 				be graded, stabilized and seeded to prevent soil erosion.

ARTICLE III

DISTRICT REGULATIONS

Sec. 301	R-1 Low Density Residential (Suburban) District

Sec. 301.1	Purpose

The purpose of this district is to accommodate residential development at densities of 				one to four units per net acre in areas that are, or can be at the same time of 					development serviced by existing or extensions of existing public central water and 				sewer facilities and paved streets with curbs and gutters.

Sec 301.2	Uses

Within an R-1 Residential District, no building, structure or premises shall be used, 				arranged to be used or designed to be used except for one or more of the following 				uses:

A.	Permitted Uses

1.	Single-Family dwelling.

2.	Accessory buildings.

a. 	Detached Garages and/or storage buildings containing one thousand two hundred square feet or less.

b.	Detached Garages and/or accessory buildings on parcels of 2 							(two) acres or more containing no more than two thousand four 							hundred square feet, provided the total square footage of the 							primary residence and all accessory buildings do not exceed 40% 							of the total square footage of the parcel.  All setbacks in 								accordance with Appendix K must be met without the need for a 							variance.

3.	Permanently sited manufactured homes.

4.	Adult Group Home, Small

5.	Type B Family Day Care Homes

6.	Recreational Facility/Use, Type A

7.	TEMPORARY STORAGE POD/ Portable Storage Container

B.	Conditionally Permitted Uses

The Board of Zoning Appeals may issue Conditional Use Zoning 							Certificates for uses listed herein, subject to Article VI and Table 601.

1.	Wireless Facilities.

2.	Agriculture, in conjunction with Sec. 203.1 A and Sec. 601.13 N

3.	Accessory Buildings

a.	Detached Garages and/or Storage Buildings containing more than one thousand two hundred square feet shall require a conditional use zoning permit.

4.	Family Day Care Home Type ‘A’

5.	BED AND BREAKFAST, RESIDENTIAL

6.	Churches

7.	Recreational Facility/Use, Type B

8.	Home Occupation

9.	Granny Cottage

10.	Gas/Oil Wells

Sec. 301.3	Area, Yard and Height Regulations

A.	Minimum Lot Area

Eleven thousand two hundred fifty square feet for single-family dwellings.

B.	Minimum Lot Width at Building Line

Seventy-five feet for single-family dwellings.

C.	Minimum Lot Width at Street

Fifty feet except forty-five feet on cul-de-sac turn around.

D.	Minimum Front Yard Depth from Street Right-of-Way

1.	Thirty feet except that a residence fronting on a state roadway shall be 						set back from the street right-of-way not less than forty feet.  If there is 						no established street right-of-way, said minimum front yard depth shall 						be deemed to be sixty feet from the centerline of the road.

2.	Double frontage lots shall provide the required front yard on both street 						frontages.  No accessory building shall project into any required front 						yard.

3.	Corner lots shall provide the required front yard adjacent to each street 						frontage, except that in no case shall the buildable width of a lot be 						reduced to less than forty feet.  No accessory building shall project into 						any required front yard line.

E.	The two side yards shall total not less than twenty-five feet.  The width of the 						narrower of the two side yards shall not be less than ten feet.

F.	Minimum Rear Yard Depth

Forty feet

G.	Maximum Height of Buildings

Thirty-five feet

H.	Minimum Living Area per Dwelling Unit

1.	Minimum Living Area

No structure shall be erected, reconstructed, or converted for use as a dwelling unless the following minimum living floor area per dwelling unit is provided:

Single- Family:

Dwelling Unit				With or Without Basement

One or Two Bedroom			One thousand two hundred Sq. Ft.

Three Bedroom			One thousand four hundred Sq. Ft.

Four Bedroom				One thousand six hundred Sq. Ft.

Five or More Bedrooms		One thousand eight hundred Sq. Ft.

2.	Not included in Living Floor Area

Minimum living floor area per family shall not include porches, steps, terrace breezeways, attached or built-in garages, unfinished basements or basements not having windows exposed to the outside, or other attached structures not intended for human occupancy.

3.	Foundation

A residential structure shall have a complete foundation installed to a depth 					below frost line.

Sec. 301.4	Permitted Use Certificate Required

According to Section 1001.9 a permitted use certificate shall be required prior to occupancy of any nonresidential use permitted or conditionally permitted in an “R” District.

Sec 302	R-2 Planned Residential District

Sec. 302.1	Purpose

It is the intent of this District to recognize and accommodate in a unified project, 					creative and imaginative planned residential developments that provide 						flexibility in housing types while retaining the best interests of the Township.  In 					order to accomplish this purpose, it is the intention in establishing these 						regulations:

A.	To permit in a planned residential development, a variety of dwelling 						types including single-family detached, clustered and attached.

B.	To permit the flexible spacing of lots and buildings in order to encourage 						the separation of pedestrian and vehicular circulation, the conservation 						of the natural amenities of the landscape, and the creation of functional 						and interesting residential areas.

C.	To permit the economical distribution and use of such facilities as 							sanitary sewer, central water supply and public and private streets.

Sec. 302.2	Uses

A.	Permitted Uses

1.	Single-family detached, attached and clustered dwellings.

Single-family attached dwellings, where no more than four dwellings 						are contained in any one grouping of attached or contiguous structures.

2.	Accessory buildings.

a.	Detached Garages and/or Storage Buildings containing one thousand two hundred square feet or less.

b.	Detached Garages and/or accessory buildings on parcels of 2 							(two) acres or more containing no more than two thousand four 							hundred square feet with a single-family residence, provided the 							total square footage of the primary residence and all accessory 							buildings do not exceed 40% of the total square footage of the 							parcel.  Clustered or attached dwelling units will remain held to 							the standard of 302.2 A.2. a. per dwelling unit, so long as the 							dwelling units building and all accessory buildings total square 							footage does not exceed 40% of the parcel square footage.  All 							setbacks in accordance with Appendix K must be met without 							the need for a variance.

3.	Permanently sited manufactured homes.

4.	Adult Group Home, Small.

5.	TYPE B FAMILY DAY CARE HOMES

6.	Agriculture, in conjunction with Sec. 203.1 A and Sec. 601.13 N

7.	Recreational Facility/Use, Type A

8.	TEMPORARY STORAGE POD/ Portable Storage Container

B.	Conditionally Permitted Uses

The Board of Zoning Appeals may issue Conditional Use Zoning 							Certificates for uses listed herein, subject to Article VI, Section 301.2B1 and

Table 601.

1.	Agriculture

2.	Adult Group Home, Large.

3.	Accessory Buildings

a.	Detached Garages and/or Storage Buildings containing more than one thousand two hundred square feet shall require a conditional use zoning permit.

4.	Churches

5.	Congregate Living Development

6.	Congregate Living Facility

7.	Recreational Facility/Use, Type B

8.	Home Occupation

9.	Granny Cottage

Sec 302.3	Area, Yard and Height Regulations

A. 	Minimum area for development

An R-2 Planned Residential District shall contain a minimum of five acres.  The 					acreage shall be contiguous in that it shall not be divided into segments by any 					limited access highway or any tract of land (other than roads, right-of-way for 					pipelines, or electric transmission lines) not owned by the developer of the R-2 					development.

B.	Maximum overall density

Six dwelling units per acre.  The density of the cumulative total of all phases 					of a development, having been given final development plan approval, shall 					never exceed the maximum permissible density for the planned residential 					development.

C.	Minimum lot area for single-family units

Minimum lot area for single-family units on individual lots is ten thousand eight 					hundred square feet.

D.	Minimum yard requirements for single-family units

Minimum yard requirements for single-family units on individual lots:

1.	Front yard depth is thirty feet.

2.	Rear yard depth is forty feet.

3.	Side yard width – The two side yards shall total not less than twenty-five 						feet.  The width of the narrower of the two side yards shall not be less 						than ten feet.

4.	Minimum lot frontage – All lots shall have a minimum frontage on a 						public thoroughfare of fifty feet.

E.	Minimum lot area and yard requirements for single-family attached dwellings.

1.	Seven thousand two hundred sixty square feet per dwelling unit.

2.	Front yard depth is thirty feet.

3.	Rear yard depth is forty feet.

4.	Side yard width – Ten feet adjacent to the non-attached side of 							dwellings, with combined two side yards being twenty five feet minimum.

5.	Minimum lot frontage.  Dwellings on individual lots shall have a frontage 						of fifty feet and dwellings grouped on a common parcel shall have a 						parcel frontage of fifty feet on a public roadway or on a private street 						built to county paving material standards.

F.	Minimum floor area.

Minimum floor areas for dwelling units permitted in the R-2 Residential 						Developments are as follows:

1.	Single-family dwellings – attached or unattached:

a.	One or two bedrooms – One thousand two hundred 								square feet

b.	Three bedrooms – One thousand four hundred square feet

c.	Four or more bedrooms – One thousand six hundred 								square feet

2.	Not included in Living Floor Area

Minimum living floor area per family shall not include porches, steps, terrace breezeways, attached or built-in garages, unfinished basements or basements not having windows exposed to the outside, or other attached structures not intended for human occupancy.

3.	Foundation

A residential structure shall have a complete foundation 								installed to a depth below frost line.

G.	Maximum Height

The maximum height for dwelling units permitted in R-2 Residential 						Developments is thirty-five feet (As listed in Appendix K).

Sec. 302.4	Site Development, Parking, Open Space and Other Requirements

A.	Development Layout

1.	When a proposed R-2 Planned Residential District abuts existing lots 						containing single-family dwellings, proposed structures of the R-2 							Development abutting those lots shall be restricted to single-family 						detached dwellings.

2.	Dwelling units shall be grouped or clustered to break up the development 					arrangements, maximize privacy, collect and maximize common open 						space, and promote the individual character and coordinated layout of 						each lot, cluster, and grouping.

3.	Streets and cul-de-sacs shall be laid out to utilize natural contours and 						discourage through and high-speed traffic.

B.	Off-Street Parking

There shall be provided outside the public right-of-way and off the primary 					access drives an average of three parking spaces per dwelling unit for the 						total development.

C.	Streets and Driveways

All streets and driveways, whether public or private, shall be designed and 					constructed to or greater than the county standards.

D.	Open Space

The minimum area devoted to open space in a development permitted by the R-					2 District shall be provided as follows:

1.	Ten percent open space shall be required for a development with a 						density of three or fewer single-family dwelling units per acre or five 						 or fewer attached single-family dwelling units per acre.

2.	Twenty percent open space shall be required for a development 							with a density of three to four single-family dwelling units per acre 						or five to six attached single-family dwelling units per acre.

3.	Forty percent open space shall be required for a development with 						a density of greater than seven attached single-family dwelling units 						per acre.

E.	Sanitary Sewer and Water Facilities

Sanitary sewer and water facilities shall be provided at the time of development.  				Underground utilities, including telephone and electrical systems, are required 					within the limits of all planned unit developments.  Appurtenances to these 					systems, which can be effectively screened, may be exempted from this 						requirement if the planning commission finds that such exemption will not 					violate the intent or character of the proposed planned unit development.

F.	Topography, Natural Features and Site Planning

Topography, natural features and site planning shall be designed to take 						advantage of the topography of the land in order to utilize the natural contours, 					and to minimize destruction of watercourses, natural vegetation, trees, and 					topsoil.  The natural features and other distinctive characteristics of the site shall 				be integrated into the plan to create variations in the arrangements of buildings, 					open spaces, and site features.

G.	Development

Development must be in compliance with other applicable sections of the 					Township Zoning Resolution and County Subdivision Regulations.

Sec. 302.5	Plan for Development of Land; Approval Procedure

A.	General Plan Approval

At the time of the application to the Township Zoning Commission for rezoning 					to R-2, a general plan for the development of the land shall have been filed with 					the Township and Regional Planning Commission by the owners of the land 					involved.  The general plan shall show the following:

1.	The boundaries of the development.

2.	A tabular summary of the total acreage of the proposed development, 						the maximum allowable number of dwelling units by type, area and 						acreage, and the number of acres proposed for recreational/open space 						development.

3.	The proposed street system, parking areas, access areas, access drives, 						and general pedestrian circulation system for the proposed development.

4.	The proposed locations of all areas for single-family dwellings, both 						attached and detached, and all accessory and other uses, such as 							recreational service buildings.

5.	The general plan for proposed sewer and water facilities.  Storm water 						drainage must be taken into consideration.

6.	The proposed reservations for recreational areas, including parks and 						playgrounds, open spaces, and other community facilities, with a 							statement of the proposed methods to be employed to preserve and 						maintain recreational areas and open spaces.

7.	The plan shall show the location of flood hazard areas from the current 						Tuscarawas County Flood Insurance Rate Map and Corp. of Engineers 						flowage easements.

B.	Final Approval of Uses in a Planned Residential District

Before such uses may begin as provided for by this chapter, the owner shall file a 				final development plat for any specific area within the project, or the overall 					project, with the Township and Regional Planning Commission, together with a 					letter of application for such approval.  Such final development plat shall show 					the following:

1.	The area to be developed and the area to be devoted to open spaces for 						the use of all residents of the area with accurate acreage, courses, and 						distances, as determined by a licensed surveyor who shall sign such plan 						and certify to accuracy thereof.

2.	The location of all buildings, descriptive data as to the type of buildings, 						the number of dwelling units in each separate residential type, and the 						number of bedrooms for each singe family dwelling.

3.	A detailed plan setting forth the manner and means whereby all open 						space, properties held in common, rental properties, and private 							properties, including all facilities of the storm water management and 						sedimentation control systems, shall be properly maintained.  Such plan 						shall be in the form of a homeowner’s association or some such similar 						instrument.

4.	Approved plans for sanitary sewer, water supply facilities, and other 						utilities.

C.	Conditions for Final Approval of Uses

The Township Zoning Commission shall give the final approval of uses only on 					finding that the following conditions are met:

1.	No applicable, general, or specific requirements of the Township Zoning 						Resolution and the County Subdivision Regulations, as existing at the 						time of general plan approval, is violated by the final development plat.

2.	The final development plat is substantially in accordance with the general 					plan that has been previously filed with and approved by the Township 						and Regional Planning Commission as the basis for establishing an R-2 						Planned Residential District.

3.	The density of dwelling units in any area does not exceed that shown on 						the general plan and that the overall density of the development has not 						been exceeded with respect to the total figure shown on the general 						plan.

4.	The area reserved for open space and recreation in the sum of all areas 						for which final development plat approval has been given or is requested 						shall never be less than four percent of the cumulative acreage of all 						areas for which final development plat approval has been given or is 						requested.

5.	The zoning certificate for a planned unit development shall be for a 						period of five years to allow the preparation and recording of the 							required subdivision plat and the development of the project.  If no 						development has occurred to effectuate the plan within five years 						after approval is granted, the approval shall be voided and the land shall 						revert to the district regulations in which it is located.  An extension of 						the limit or modification of the approval development plan may be 						approved if the planning commission and the board of zoning appeals 						find that such extension or modification is not in conflict with the public 						interest.

Sec. 302.6	Residential Specifications

A. 	Building Setback, Ownership’s, and Specifications

All setback dimensions are minimums unless otherwise specified.

1.	For the purpose of this section the following definitions shall apply unless 					the context clearly indicates or requires a different meaning.

a.	Condominium Association involves mandatory membership and 							support.

b.	Homeowners Association involves voluntary membership and 							member support.

2.	Single-family detached.  A separate single-family dwelling on its own, 						legally described lot.

Individual Lot				Rental

Ten thousand eight hundred sq. ft.	Ten thousand eight hundred sq. ft.

Thirty ft. Front Yard			Thirty ft. Front Yard

Forty ft. Rear Yard			Forty ft. Rear Yard

Ten feet Side yard minimum with sum of both sides being twenty-five 						feet minimum fifty feet drainage

Condominium

Yes, Limited to lot; common lands

3.	Single-family attached

Independent dwellings with separate identities, entrances, and services, 						but with contiguous or attached structural elements.  These dwellings 						may be located on separate lots or grouped on commonly owned and 						maintained acreage.  For purposes of flexibility, variety, and identity, no 						more than five dwellings shall be in an attached or contiguous 							grouping.

Individual Lot				Rental

Eight thousand sq. ft.			Eight thousand sq. ft.

Thirty ft. Front Yard			Thirty ft. Front Yard

Forty ft. Rear Yard			Forty ft. Rear Yard

Ten feet Side yard minimum with combined two sides being twenty-five 						feet minimum.

Fifty feet frontage

Condominium

Yes, limited to lot; common lands.

4.	Town Houses

Independent dwelling with separate identities and entrances but 							structurally connected or having common walls or roofs.  All dwellings 						have direct access to the outside at grade, and no dwellings are located 						one over the other.  Townhouse buildings may include up to six 							dwellings and may be located on acreage that is either under single 						ownership or commonly owned and 	maintained.

Individual Lot				Rental

No					Maximum six dwellings/ac.

Front Yard is thirty ft.

Rear Yard is forty ft.

Side Yard Minimum is ten ft.

Frontage is twenty ft. bldg. Sep.

Condominium – Same as Rental

5.	Garden Apartment

A multi-family structure containing four self-contained dwellings, which 						may be located one over the other and which may have common building 					entrances, stairway, and other common services and facilities.  Garden 						apartment buildings may have two floors above ground.

Individual Lot				Rental

No					Maximum six dwellings/ac.

Front Yard is forty ft.

Rear Yard is twenty ft.

Side Yard Minimum is ten ft.

Bldg. Sep. is twenty ft.

Frontage is fifty ft.

Condominium – Same as rental

B.	Open Space

Land not occupied by buildings, parking lots, or other structures and set aside for 				the preservation, provision, and maintenance of the natural environment, such 					as unusual, interesting, or significant areas of vegetation, wildlife, habitat, 					natural panoramas, unusual land forms, water bodies and water courses, 						wetland, and floodplains, and other natural amenities.

C.	Recreation Areas

Open space developed for or intended to be developed for recreational activities 				requiring land areas outside of buildings and other structures, including such 					facilities as play fields, ball fields, golf courses, hiking and riding trails, and picnic 					areas.

D.	Disposition of Open Space

The amount of open space reserved under a planned-unit development shall 					either be held in corporate ownership by the owners of the project area building 					sites for the use of each owner who buys property within the development or 					dedicated to the Township and retained as open space for parks, recreation, and 					related uses.  All land dedicated to the Township must meet the planning 						commission’s requirements as to shape, size, and location.  Public utility and 					similar easements and right-of-way for watercourses and other similar channels 					are not acceptable for open space dedication to the Township unless such land 					or right-of-way is usable as a trail or similar way and approved by the planning 					commission.

E.	Recreation Facilities

Building, equipment, and their structures developed for recreational activities, 					including tennis, basketball, volleyball, and other type of court facilities, 						swimming pools, play equipment, party and meeting rooms and buildings, and 					landscaping, flower beds and other aesthetic features.

F.	Public Facility Sites

Land designated for future facilities or land containing facilities such as water 					and sewerage pump stations, storm water systems and detention basins, school 					and fire station sites, and sites for other publicly owned or operated facilities and 				services.

Sec. 302.7	Permitted Use Certificate Required

According to Section 1001.9 a permitted use certificate shall be required prior to

occupancy of any nonresidential use permitted or conditionally permitted in a “R”

District.

Sec 303	R-3 High Density Residential District

Sec. 303.1	Purpose

The purpose of this district is to encourage relatively high-density residential 					development in area adjacent to community facilities, neighborhood shops and major 				highways.  Development is to consist primarily of townhouses and garden apartments at 			densities up to sixteen units per acre in groupings that will provide for efficient 					development and utilization of community facilities including central water and sewer.

Sec. 303.2	Uses

Within an R-3 High Density Residential District no building, structure or premises shall 				be used, arranged to be used or designed to be used except for one or more of the 				following uses:

A.	Permitted Uses

1.	 Single-family dwelling including home occupation.

2.	Two-family dwellings.

3	Multi-family dwellings.

4.	Accessory buildings

5.	Signs as regulated by Article IV hereof.

6.	Permanently sited manufactured homes.

7.	Adult Group Home, Small.

8.	TYPE B FAMILY DAY CARE HOMES				                                  		 		9.	Agriculture, in conjunction with Sec. 203.1 A and Sec. 601.13 N.

10.	Recreational Facility/Use, Type A

11.	TEMPORARY STORAGE POD/ Portable Storage Container

B.	Accessory permitted use

1.	Parking and loading facilities as regulated by Article V of this Resolution.

2.	Detached Garages and/or Storage Buildings containing one 							thousand two hundred square feet or less.

3.	Detached Garages and/or accessory buildings on parcels of 2 (two)

acres or more containing no more than two thousand four hundred

square feet with a single-family residence, provided the total square

footage of the primary residence and all accessory buildings do not

exceed 40% of the total square footage of the parcel.  Two-family or

multi-family dwelling units will remain held to the standard of 303.2 						B.2., per dwelling unit, so long as the dwelling units building and all 						accessory buildings total square footage does not exceed 40% of the 						parcel square footage.  All setbacks in accordance with Appendix K must 						be met without the need for a variance.

C.	Conditionally Permitted Uses

The Board of Zoning Appeals may issue Conditional Use Zoning Certificates for 					uses listed herein, subject to Article VI and Table 601.

1.	Publicly owned and/or operated buildings and facilities subject to Article 						VI.

2.	Planned unit developments subject to Section 312.

3.	Agriculture

4.	Adult Group Home, Large.

5.	Accessory Buildings

a.	Detached Garages and/or Storage Buildings containing more than one thousand two hundred square feet shall require a conditional use zoning permit.

6.	Family Day Care Home Type ‘A’

7.	Congregate Living Development

8.	Congregate Living

9.	Churches

10.	Licensed Group Home for Handicapped Persons

11.	Recreational Facility/Use, Type B

12.	Granny Cottage

13.	Home Occupation						                                  				14.	Office (excluding Medical)

15.	Temporary Storage/Pods

Sec 303.3	Area, Yard and Height Regulations

A.	Minimum Lot Area

1.	Single-family dwellings seven thousand five hundred square feet.

2.	Two-family dwellings ten thousand square feet.

3.	Multi-family dwellings, twelve thousand square feet for the first three 						units, plus two thousand five hundred square feet for each dwelling unit 						over three.

B.	Minimum Lot Width at Building Line

1.	Two-family dwelling eighty feet.

2.	Multi-family dwelling one hundred feet.

C.	Minimum Lot Width at Street

Fifty feet except forty feet for single family dwellings on cul-de-sac turn around.

D.	Minimum Front Yard Depth from Street Right-of-Way

1.	Forty feet plus one foot for each two feet of building height in excess 						of thirty-five feet, except that uses fronting on a state roadway shall be 						set back from the street fifty feet plus one foot for each two feet of 						building height in excess of thirty-five feet.  If there is no established 						right-of-way, said line shall be deemed to be thirty feet from the 							centerline of the road.

2.	Double frontage lots shall provide the required front yard on both street 						frontages.  No accessory building shall project into any required front 						yard.

3.	Corner lots shall provide the required front yard adjacent to each street 						frontage, except that in no case shall the buildable width of a lot be 						reduced to less than forty feet.  No accessory building shall project into 						any required front yard line.

E.	Minimum Side Yard Width

1.	Single-family dwelling.  The two side yards shall total not less than twenty 					feet.  The width of the narrower of the two side yards shall not be less 						than eight feet.

2.	Two family dwelling, ten feet.

3.	Multi-family dwellings, ten feet plus one foot for each two 								feet of building in excess of thirty-five feet above grade.

F.	Minimum Rear Yard Depth

Forty feet

G.	Maximum Height of Buildings

Fifty feet

H.	Minimum floor areas for dwelling units permitted in the R-3 Residential 						Developments are as follows:

1.	Single-family dwellings attached or unattached:

One bedroom			Seven hundred fifty square feet

Two bedrooms		Eight hundred fifty square feet

Three bedrooms		Nine hundred fifty square feet

Additional one hundred square feet per each additional bedroom

a.	Not included in Living Floor Area

Minimum living floor area per family shall not include porches, steps, terrace breezeways, attached or built-in garages, unfinished basements or basements not having windows exposed to the outside, or other attached structures not intended for human occupancy.

b.	Foundation

A residential structure shall have a complete foundation installed to a depth below frost line.

Sec 303.4	Site Plan Review and Conformance

All apartment uses specified under Section 303.3 shall be permitted only after the 				review and approval of the site plans by the Board of Zoning Appeals and upon finding 				by the Zoning Commission that the specific standards set forth in this article and the 				requirements of Article II, Section 203.6 will be met.

Sec.  303.5	Permitted Use Certificate Required

According to Section 1001.9, a permitted use certificate shall be required prior to occupancy of any nonresidential use permitted or conditionally permitted in an “R” District.

Sec. 304	R-4 Rural Residential District

Sec 304.1	Purpose

The purpose of this district is to accommodate residential development at a low density 				that will promote the continuation of the predominately rural residential character of 				this zone, which may or may not include central sewage treatment.

Sec 304.2	Uses

Within an R-4 Residential District, no building, structure, or premises shall be used, 				arranged to be used, or designated to be except for one or more of the following uses:

A.	Permitted Uses

1.	Single-family dwelling including home occupation.

2.	Detached Garages and/or Accessory Buildings to any square footage

provided the total square footage of the Primary Residence and all

Accessory Buildings do not exceed 40% of the total square footage of 						the parcel.  In instances where the parcel is less than 2 (two) acres, the

Detached Garage and/or Accessory Building may not exceed one

thousand two hundred square feet in size.  All setbacks in accordance

with Appendix K must be met without the need for a variance.

3.	One roadside stand offering for sale only agricultural products, which are 						produced on the premises, is permitted.  Such stands shall not be erected 					closer than twenty feet from the road right-of-way nor closer than							sixty (60) feet from any lot line.

4.	Signs as regulated by Article IV hereof.

5.	Permanently sited manufactured homes.

6.	Manufactured homes on individual lots in a manufactured home park.

7.	Agriculture, in conjunction with Sec. 203.1 A and 601.13 N.

8.	Adult Group Home, Small

9.	TYPE B FAMILY DAY CARE HOMES

10.	Recreational Facility/Use, Type A

11.	Recreational Facility/Use, Type B

12.	TEMPORARY STORAGE POD/ Portable Storage Container

B.	Accessory Permitted Uses

1.	Parking and loading facilities as regulated by Article V of this Resolution.

2.	Freestanding Wind Energy Conversion Systems.

C.	Conditionally Permitted Uses

The Zoning Board of Appeals may issue conditional zoning certificates for uses 					subject to Article VI and Table 601 per other Districts as well.  Conditional uses 					shall include:

1.	Non-commercial recreation

2.	Home occupation

3.	Manufactured Home Park

4.	Small Wind Projects (FARM) in accordance with Section 217

5.	Extraction

6.	Bed & Breakfast, residential

7.	Congregate Living Facility

8.	Churches

9.	Licensed Group Home for Handicapped Persons

10.	Family Day Care Home – type ‘A’

11.	Recreational Facility/Use Type ‘C’

12.	Recreational Facility/Use Type ‘D’

13.	Granny Cottage

14.	Adult Group Home – Large

15.	Restaurant – Accessory to Agriculture Use

16.	Golf Course

Sec. 304.3	Area, Yard and Height Regulations (Without Central Sewage Treatment)

A.	Minimum Lot Size

One and one-half acre

B.	Minimum Lot Width at Building Line

One hundred fifty feet

C.	Minimum Lot Width at Street

Seventy-five feet except fifty feet on cul-de-sac turn around.

D.	Minimum Front Yard Depth from Street Right-of-Way

1.	Seventy feet except those residences fronting on a state roadway shall be 					set back from the street right-of-way not less than ninety-feet.

2.	If there is no established street right-of-way, the minimum front yard 						depth shall be deemed to be one hundred feet from the center of the 						road.

3.	Double frontage lots shall provide the required front yard on both street 						frontages.  No accessory building shall project into any required front 						yard.

4.	Where a lot is located at the intersection of two or more streets, there 						shall be a front yard on each street side of a corner lot, except that the 						buildable width of a lot of record shall not be reduced to less than forty						feet.  No accessory building shall project beyond the front yard line on 						either street.

E.	Minimum Side Yard Width

1.	Two side yards shall total not less than fifty feet.  The width of the 						narrower of the two side yards shall not be less than twenty feet.

2.	Double frontage lots shall provide the required front yard on both street 						frontages.  No accessory building shall project into any required front 						yard.

3.	Corner lots shall provide the required front yard adjacent to each street 						frontage, except that in no case shall the buildable width of a lot be 						reduced to less than forty feet.  No accessory building shall project into 						any required front yard line.

F.	Minimum Rear Yard Depth

Forty feet

G.	Maximum Height of Buildings

Thirty-five feet

H.	Minimum Living Area

1.	No structure shall be erected, reconstructed, or converted for use as a dwelling unless the following minimum living floor area per dwelling unit is provided:

Single- Family:

Dwelling Unit				With or Without Basement

One or Two Bedroom	One thousand two hundred Sq. Ft.

Three Bedroom	One thousand four hundred Sq. Ft.

Four Bedroom	One thousand six hundred Sq. Ft.

Five or More Bedrooms	One thousand eight hundred Sq. Ft.

2.	Not included in Living Floor Area

Minimum living floor area per family shall not include porches, steps, terrace breezeways, attached or built-in garages, unfinished basements or basements not having windows exposed to the outside, or other attached structures not intended for human occupancy.

3.	Foundation

A residential structure shall have a complete foundation installed to a depth below frost line.

Sec. 304.4	Area, Yard and Height Regulations (With Central Sewage Treatment)
			A.	Minimum Lot Area

Thirteen thousand six hundred square feet

B.	Minimum Lot Width at Building Line

Eighty-five feet

C.	Minimum Lot Width at Street

Fifty feet

D.	Minimum Front Yard Depth from Street Right-of-Way

1.	Thirty feet except those residences fronting on a state roadway shall be 						set back from the street right-of-way not less than seventy feet.

2.	If there is no established street right-of-way, the minimum front yard 						depth shall be deemed to be sixty feet from the center of the road.

E.	Minimum Side Yard Width

Two side yards shall total not less than twenty-five feet.  The width of the 					narrower of the two side yards shall not be less than ten feet.
		F.	Minimum Living Area

1.	No structure shall be erected, reconstructed, or converted for use as a dwelling unless the following minimum living floor area per dwelling unit is provided:

Single- Family:

Dwelling Unit				With or Without Basement

One or Two Bedroom	One thousand two hundred Sq. Ft.

Three Bedroom	One thousand four hundred Sq. Ft.

Four Bedroom	One thousand six hundred Sq. Ft.

Five or More Bedrooms	One thousand eight hundred Sq. Ft.

2.	Not included in Living Floor Area

Minimum living floor area per family shall not include porches, steps, terrace breezeways, attached or built-in garages, unfinished basements or basements not having windows exposed to the outside, or other attached structures not intended for human occupancy.

3.	Foundation

A residential structure shall have a complete foundation installed to a 						depth below frost line.

Sec.  304.5	Permitted Use Certificate Required

According to Section 1001.9, a permitted use certificate shall be required prior to occupancy of any nonresidential use permitted or conditionally permitted in an “R” District.

Sec. 305	Manufactured Home Community District

Sec. 305.1	Purpose

The regulations set forth in this Article, or set forth elsewhere in this Resolution, when 				referred to in this Article, are the district regulations in the “MH” Manufactured Home 				Community District.  It is the purpose of this district to provide sites for manufactured 				homes and mobile homes at appropriate locations in relation to the existing and 					potential development of their surroundings and in relation to other uses and 					community facilities to afford a proper setting for such uses and proper relation to other 			land uses and the land use plan.

Sec 305.2	Use Regulations

Land or premises within the “MHC” Manufactured Home Community District shall be 				used only for manufactured homes and mobile homes and accessory buildings and uses 				customarily incident thereto.

Sec. 305.3	Procedure

The owner or owners of a tract of land (except land situated within the “FPM” Flood 				Plain Management Overlay District) comprising not less than ten acres may submit 				a plan for the use and development of the tract of land for a Manufactured Home 				Community as provided herein.  Such plan for development of the area shall be filed 				with the Zoning Inspector and shall be referred to the Lawrence Township Zoning 				Commission for study and report.  The Lawrence Township Zoning Commission shall 				recommend the approval or denial of the plan or approval of some modifications and 				submit the plan, together with a report stating its findings and recommendations to the 				Board of Trustees for a public hearing and determination.  If the report of the Lawrence 				Township Zoning Commission recommends approval of the plan, it shall state the 				reasons for approval and shall include specific evidence and facts showing that the 				proposed manufactured home community has met the requirements of this section.

Sec. 305.4	General Requirements

Each manufactured home community shall comply with the rules of the Ohio 					Department of Health, Public Health Council, for Mobile Home Parks, as adopted 				pursuant to Chapter 4781.36 to 4781.52 of the Ohio Revised Code, and such other 				applicable requirements imposed by the Tuscarawas County Health Department.

Sec 305.5	Design Requirements

A.	The location and planning of the site and the amount, arrangement and 						treatment of open space shall be designed to ensure a satisfactory living 						environment and shall be carried out in consideration of property adjacent to 					the area included in the plan and ensure that such adjacent property not be 					adversely affected.

B.	Buffers shall be established and maintained in accordance with the provisions of 					Section 209.  The Lawrence Township Zoning Commission may modify buffer 					requirements to the extent warranted by topography or other physical 						conditions, provided the surrounding property and the public welfare are 						adequately protected.

C.	The number of manufactured or mobile homes shall not exceed seven such 					units per net usable acre of this site.  The net usable acreage shall be deemed to 					be the total area of the site, excluding any public street right-of-way and 						excluding the buffer required in Section 209.

D.	All manufactured or mobile homes, accessory buildings and uses, including the 					recreation areas required herein, shall be located within the area determined 					and defined as the net usable area.

E.	Permitted accessory buildings and uses shall include management offices, 					laundry facilities, recreation areas and, where specifically approved as a part of 					the community plan, other recreation facilities and the sale of convenience 					goods and services exclusively for and to occupants of the manufactured home 					community.

F.	Every manufactured home community shall provide one or more recreation 					areas easily accessible to all residents of the community.  The aggregate size 					of such areas shall be not less than one hundred square feet for each lot and no 					individual recreation areas shall be less than three thousand square feet.  Such 					recreation areas shall be graded and arranged and provided with appropriate 					equipment for full recreational use of the area.  No such recreation area shall be 					located in any part of a required buffer area.

G.	Each manufactured home community shall have frontage on a public street, and 					each home lot shall have direct access to the private internal road system, either 					by direct frontage or by means of a ten-foot, hard surfaced driveway.  Such 					internal road system shall be constructed to provide a permanent pavement of 					at least twenty-six feet, including curbs and gutters.  Off-street parking spaces 					shall be provided in the ratio of two spaces for each home lot; such spaces shall 					be within twenty feet from the home or homes served.  No parking space shall 					be located within any part of a required buffer area.

H.	A common walk system shall be provided and maintained between locations 					where pedestrian traffic is concentrated, and all home lots shall be connected by 				walks to the common walk system, to the parking spaces, to the paved streets 					and to all service buildings.

I.	Each lot, designed for mobile home occupancy as defined in Article III, shall be 					equipped with a concrete slab or with concrete ribbons of adequate thickness 					and size to support the mobile home load during all season.  Where concrete 					ribbons are used, the area between such ribbons shall be filled with crushed 					rock.

J.	Each manufactured home community shall be adequately lighted for safety at 					night; all such lights shall be so located and shielded to prevent direct 						illumination of any area outside the community.

K.	Each manufactured home community shall be provided with public water supply 					and a water distribution system installed in accordance with Tuscarawas County 					Sewer and Water District specifications.  Where a public sanitary sewer is 						reasonably accessible, the community shall be provided with sanitary sewerage 					connected thereto, including a lateral connection to each home lot, subject to 					the review and approval of the Tuscarawas County Sewer and Water District, the 				Tuscarawas County Health Department and the State Department of Health.  					Where a public sanitary sewer is not available and not reasonably accessible in 					the combined judgment of the Lawrence Township Zoning Commission and the 					Tuscarawas County Sewer & Water District, an alternate means of sewage 					disposal, such as a community sewage treatment plant may be considered, 					subject to review and approval of officials having jurisdiction.  Individual sewage 					disposal systems shall not be permitted.

L.	Each community shall be graded and drained to prevent the standing of storm 					water and the method of drainage, including treatment of both paved and 					unpaved areas shall be subject to approval of the Tuscarawas County Engineer.

Sec. 305.6	Additional Requirements

In addition to the foregoing, the Lawrence Township Zoning Commission may impose 				such other conditions, requirements or limitations concerning the design, and 					development and operation of such manufactured home community as it may deem 				necessary for the protection of adjacent properties and the public interest.

Sec. 305.7	Enlargement

Any enlargement or extension of an existing manufactured home community shall be 				treated as if it were a new establishment and shall be subject to the provisions of 				Section 209 and all provisions of this Article.  No enlargement or extension of a 					manufactured home community shall be permitted unless the existing community is 				made to conform substantially to all the requirements.

Sec.  305.8	Permitted Use Certificate Required

According to Section 1001.9, a permitted use certificate shall be required prior to occupancy of any nonresidential use permitted or conditionally permitted in an “R” District.

Sec. 306	C-1 Service District

Sec. 306.1	Purpose

The purpose of this district is to provide for a variety of services and administrative 				establishments in unified groupings, which will encourage safe pedestrian movement 				between uses while at the same time, provide adequate off-street parking and servicing 				facilities.  Residential development is prohibited.

Sec. 306.2	Uses

Within a C-1 Local Commercial District, no building structure or premises shall be used, 				arranged to be used or designed to be used except for one or more of the following 				uses:

A.	Permitted Uses

1.	Establishments engaged in providing a variety of services to individuals 						and business establishments e.g.:

a.	Credit agency other than bank

b.	Investment firm

c.	Insurance carrier

d.	Real estate and insurance company

2.	Miscellaneous business services such as advertising news syndicates and 						employment agency.

a.	Engineering and architectural services

b.	Legal services

c.	Accounting, auditing and bookkeeping services

d.	Nonprofit professional, charitable and labor organizations

3.	Accessory uses clearly incidental to the principal uses permitted on the 						same premises.

4.	Other uses similar in character to those listed herein above.

5.	Signs as regulated by Article IV hereof.

6.	Parking and loading as regulated by Article V hereof.

7.	Wireless Facilities (See Sec. 310).

8.	Recreational Faciltiy/Use, Type A

B.	Conditionally Permitted Uses

The Board of Zoning Appeals may issue Conditional Use Zoning Certificates for 					uses listed herein subject to the general requirements of Article VI and Table 					601

1.	Churches

2.	Publicly owned and/or operated buildings and facilities subject to Article 						VI.

3.	Outside music or entertainment.

4.	Temporary Storage Pod/ Portable Storage Container

5.	Recreational Facility/Use, Type B
				6.	Clubs

7.	Shipping Containers

Sec 306.3	Area, Yard and Height Regulations

A.	Minimum Lot Area

None

B.	Minimum Lot Width at Building Line

None

C.	Minimum Lot Width at Street

None

D.	Minimum Front Yard Depth from Street Right-of-Way

Equal to the setback of the residential uses in the immediate vicinity

E.	Minimum Side Yard Width

None, except thirty feet when adjacent to a residential district and on the side 					adjacent to the residential district only

F.	Minimum Rear Yard Depth

Twenty-five feet

G.	Maximum Height of Buildings

Fifty feet

Sec. 306.4	Site Plan Review and Conformance

All business uses specified under Section 306.2 shall be permitted only after the review 				and approval of the site plans by the Board of Appeals and upon finding by the Zoning 				Commission that the specific standards set forth in this article and the requirements of 				Article II, Section 203.6 will be met.

Sec. 306.5	Permitted Use Certificate Required

A Permitted Use Certificate shall be required to be obtained prior to occupancy of any permitted or conditional permitted use for all businesses in accordance with Section1001.9. Any change in use from one use to another, addition of a use or from one owner/tenant to another will require a new Permitted Use Certificate.

Sec. 306.6	Building Space

Wherever there is more than one principal building on a lot in a C-1 district, the 					minimum distance between the buildings shall be at least thirty (30) feet, measured 				from the building overhang or attached permanent structure, whichever is the lesser 				distance.

Sec. 307	C-2 Local Commercial District

Sec. 307.1	Purpose

This district is established to provide for uses principally to accommodate the sale of 				retail goods and personal services purchased frequently for daily or weekly needs.  It is 				intended that the design of this district will encourage groupings of establishments 				located on a unified site providing adequate off-street parking facilities as well as an 				efficient and safe method of handling vehicular and pedestrian traffic.  Residential 				development is prohibited.

Sec. 307.2	Uses

Within a C-2 Local Commercial District, no building, structure, or premises shall be 				used, arranged to be used, or designed to be used except for one or more of the 					following uses:

A.	Permitted Uses

1.	All uses permitted in the C-1 Local Commercial District.

2.	Establishments engaged in providing a variety of services to individuals 						and business establishments e.g.:

a.	Bank, credit union, etc.

b.	Personal services such as barber and beauty shops, tailor, shoe 							repair, laundries and dry cleaning.

c.	Medical and other health services.

d.	Dance studio and school.

e.	Veterinarian hospital or clinic.

3.	Establishments engaged in retail trade, e.g.:

a.	Book and stationery store

b.	Apparel store

c.	Florist shop and gift shop

d.	Antique store

e.	Sporting goods store

f.	Jewelry store

g.	Optical goods store

h.	Furniture, home furnishing and office equipment and office 							supplies

i.	Food stores including supermarkets

j.	Restaurant not including drive-thru

k.	Preparation and processing of food and drink to be retailed on 							premises including bakery, delicatessen, and meat market

l.	Taverns, lounges and bars

m.	Games of skill in accordance with Article VIII.

4.	Accessory uses clearly incidental to the principal uses permitted on the 						same premises.

5.	Other uses similar in character to those listed herein above.

6.	Signs as regulated by Article IV hereof.

7.	Parking and loading as regulated by Article V hereof.

8.	Recreational Facility/Use Type B

B.	Conditionally Permitted Uses

The Board of Zoning Appeals may issue Conditional Use Zoning Certificates for 					uses listed herein subject to the general requirement of Article VI referred to 					below:

1.	All conditionally permitted uses in the C-1 Local Commercial District.

2.	Mini-Storage Facility

3.	Clubs, lodges, fraternal, charitable, or social organizations subject to 						Article VI

4.	Outside music or entertainment

5.	Restaurant with Drive-Thru

6.	Congregate Living Facility

7.	TEMPORARY STORAGE POD/ Portable Storage Container

8.	Golf Course

9.	Shipping Containers

10.	Recreational Facility Type C

Sec. 307.3	Area, Yard and Height Regulations

A.	Minimum Lot Area

None

B.	Minimum Lot Width at Building Line

None

C.	Minimum Lot Width at Street

None

D.	Minimum Front Yard Depth from Street Right-of-Way

None.  If there is no established street right-of-way, said line shall be deemed to 					be forty feet from the centerline of the road

E.	Minimum Side Yard Width

None, except the minimum side yard width abutting a residential district shall 					not be less than thirty feet

F. 	Minimum Rear Yard Depth

None, except the minimum rear yard depth abutting a residential district shall 					not be less than the yard requirements of said residential district

G.	Maximum Height of Building

Fifty feet

Sec. 307.4	Site Plan Review and Conformance

All business uses specified under Section 307.2 shall be permitted only after the review 				and approval of the site plans by the Board of Zoning Appeals and upon finding by the 				Zoning Commission that the specific standards set forth in this Article and the 					requirements of Article II, will be met.

Sec. 307.5	PERMITTED USE CERTIFICATE REQUIRED

A Permitted Use Certificate shall be required to be obtained prior to occupancy of any permitted or conditional permitted use for all businesses in accordance with Section1001.9. Any change in use from one use to another, addition of a use or from one owner/tenant to another will require a new Permitted Use Certificate.

Sec. 307.6	Building Space

Whenever there is more than one principal building on a lot in a C-2 district, the 					minimum distance between the buildings shall be at least thirty (30) feet, measured 				from the building overhang or attached permanent structure, whichever is the lesser 				distance.

Sec. 308	C-3 Highway Commercial District

Sec. 308.1	Purpose

This district is established to provide for uses in addition to those specified for the 				local and general business district in order to provide additional service and sales 				opportunities in support of the primary business activities in the community.  This 				district includes activities which, because of their nature, such as their tendency to 				encourage traffic congestion and parking problems, storage problems or certain other 				inherent dangers create special problems and are, therefore, best distinguished from 				other commercial activity.  Their location is advantageous at specified points on major 				thoroughfares.  Residential development is prohibited.

Sec. 308.2	Uses

Within a C-3 Highway Commercial District, no building, structure or premises shall be 				used, arranged to be used or designed to be used except for one or more of the 					following uses:

A.	Permitted Uses

1.	All uses permitted in the C1 and C2 commercial districts

2.	Radio and television broadcasting station

3.	Display or show room where merchandise sold is stored elsewhere

4.	Fuel, food and goods distribution station but excluding coal and coke and 						bulk storage

5.	Plant, greenhouse and garden supply sales

6.	Hotel and motel

7.	Mortuary, funeral home not including crematorium

8.	Monument sales and display

9.	Automobile, truck, trailer, boat and farm implement sales and services 						and storage both new and used

10.	Tool and equipment rental

11.	Drive-in establishments, restaurant and refreshment stands with or without Drive-Thru, but excluding drive-in theaters and commercial

amusement parks

12.	Gasoline service station, garage and automotive repair

13.	Drug stores

14	Bowling alley

15.	Motion picture and theatrical playhouse

16.	Beverage sale

17.	The following uses may be conducted not closer than fifty feet of any R-						District.  Where the C-3 District abuts upon but is separated from the R-						District by a street, the width of the street may be considered as part of 						the required setback.

a.	Carpenter, cabinet, upholstering, sheet metal, plumbing, heating, 							roofing, air conditioning, sign painting, painting and other similar 							establishments

b.	Repair services for machinery and equipment including rear 							garages and specialty establishments such as motor, body and 							fender, radiator, motor tune-ups, muffler shops, tire repairing 							sales and service including vulcanizing

18.	Accessory uses clearly incidental to the principal uses permitted on the 						same premises

19.	Other uses similar in character to those listed herein above

20.	Signs as regulated by Article IV hereof

21.	Parking and loading as regulated by Article V here

22.	Recreational Facility/Use Type C

B.	Conditionally Permitted Uses

The Board of Zoning Appeals may issue Conditional Use Zoning Certificates for 					uses listed herein subject to the general requirements for Section 601.1 through 					601.2A inclusive and other sections of Article VI referred to below:

1.	All conditionally permitted uses in the C-1 and C-2 commercial districts

2.	Mini-Storage Facility

3.	Recreational Facility/Use Type D

4.	Temporary Storage Pod/Portable Storage Container    
				5.	Clubs								
				6.	Skill-based Amusement Machine(s)

7.	Shipping Containers

8.	Congregate Living Facility    
9.	Golf Course

10.	Gas/Oil Wells

Sec. 308.3	Area, Yard and Height Regulations

A. 	Minimum Lot Area

None

B.	Minimum Lot Width at Building Line

None

C.	Minimum Lot Width at Street

None

D.	Minimum Front Yard Depth from Street Right-of-Way

Fifty feet.  If there is no established right-of-way, said line shall be deemed to be 					sixty feet from the centerline of the road.

E.	Minimum Side Yard Width

None, except fifty feet when adjacent to a residential district and on the side 					adjacent to the residential district only.

F.	Minimum Rear Yard Depth

None, except the minimum rear yard depth abutting a residential district shall 					not be less than the yard requirements of said residential district.

G.	Maximum Height of Building

Fifty feet.

Sec. 308.4	Site Plan Review and Conformance

All business uses specified under Section 308.2 shall be permitted only after the review 				and approval of the site plans by the Zoning Commission and upon finding by the 				commission that the specified standards set forth in this article and the requirement of 				Article II, Section 203.6 will be met.

Sec. 308.5	Supplementary Regulations

A.	Outdoor Display Areas

Merchandise to be sold at retail on the premises may be displayed out of door 					except that no such display area shall be within fifty feet of any residential 					district.  Display areas shall be screened from abutting residential uses by 						landscaping sufficient to limit to a minimum all visual effects of such display 					area; such landscaped buffer shall be maintained in a neat and orderly fashion, 					and in compliance with Section 203.8.

Sec. 308.6	Permitted Use Certificate Required

A Permitted Use Certificate shall be required to be obtained prior to occupancy of any permitted or conditional permitted use for all businesses in accordance with Section1001.9. Any change in use from one use to another, addition of a use or from one owner/tenant to another will require a new Permitted Use Certificate.

Sec. 308.7	Building Space

Whenever there is more than one principal building on a lot in a C-3 district, the 					minimum distance between the buildings shall be at least thirty (30) feet, measured 				from the building overhang or attached permanent structure, whichever is the lesser 				distance.

Sec. 309	I-1 Light Industrial District

Sec. 309.1	Purpose

This district is established to provide for and accommodate, industrial uses, such as, 				manufacturing, processing, wholesaling, distribution, and research facilities.  The uses 				allowed are those, which because of their normally unobjectionable characteristics can 				be in relatively close proximity to residential and commercial districts.

Sec. 309.2	Uses

Within an I-1 Light Industrial District, no building, structure or premises shall be used, 				arranged to be used or designed to be used except for one or more of the following 				uses:

A.	Permitted Uses

1.	Parking lot and garage

2.	Plant greenhouse

3.	Warehouse

4.	Wholesale establishments

5.	Crematorium

6.	Printing, blueprinting, newspaper printing, telegraphic service

7.	The following types of manufacturing, processing, cleaning, servicing, 						testing or repair activities which will not be materially injurious or 							offensive to the occupants of adjacent premises or the community at 						large by reason of the emission or creation of noise, vibration, smoke, 						dust or other particulate matter, toxic and noxious materials, odors, fire 						or explosive hazards, glare or heat or electromagnetic disturbances.

a.	Bakery goods, candy, cosmetics, pharmaceuticals, toiletries and 							food products; except fish or meat products, sauerkraut, yeast 							and rendering of refining of fats or oils.

b.	Products from the following previously prepared materials: bone, 							canvas, cellophane, cloth, cork, feathers, fiber, fur, glass, hair, 							horn, leather, plastics, precious or semi-precious metals or stones, 						metal, shell, textiles, tobacco, wax and wood (where saw and 							planning mills are employed within a completely enclosed 								building).

c.	Pottery and figurines, using previously pulverized clay and kilns 							fired only with gas or electricity.

d.	Musical instruments, toys, novelties, rubber metal stamps and 							other small rubber products.

e.	Electrical and electric appliances, instruments and devices, 							television sets, radios, phonographs, household appliances.

f.	Laboratories and processing including, film or testing provided no 							operation shall be conducted or equipment used which would 							create hazards, noxious, or offensive conditions.

8.	The following uses, provided that storage is within an enclosed building, 						or surrounded on all sides by a solid masonry wall or fence measuring 						eight feet in height, or the minimum height needed to obscure all 							materials, whichever is taller.  Fence may have an opening of no greater 						than fifteen percent when on a lot adjacent to land in a non-residential 						zoning 	district.  A solid masonry wall or fence shall be required on any lot 						adjacent to land in a residential zoning district.

a.	Building materials, sales yard, and lumber yard including millwork 							when within completely enclosed building

b.	Contractor’s equipment storage yard or plant, or storage yard or 							plant, or storage and rental of equipment commonly used by 							contractors

c.	Fuel, food and goods distribution station, warehouse and storage, 							but excluding coal and coke.  Inflammable liquids, if located not 							less than three hundred feet from any R-District.

d.	Public storage garage and yards

1.	To include: cans, carton, containers, pallets, equipment, finished goods, work in progress, raw materials, and distressed materials, but be enclosed.

9.	The following uses may be conducted not closer than one hundred feet 						of any R-District.   Where the I-District is separated from any R-District by 						a street, the width of the street may be considered as part of the 							required setback.

a.	Bag, carpet and rug cleaning, provided necessary equipment is 							installed and operated for the effective precipitation or recovery 							of dust

b.	Blacksmith, welding or other metal working shops, including 							machine shop operations of the tool, die and gauge types

c.	Carpenter, cabinet, upholstering, sheet metal, plumbing, heating, 							roofing, air conditioning, sign painting, painting and other similar 							establishments

d.	Ice manufacturing and cold storage plant; creamery and bottling 							plant

e.	Laundry, cleaning and dyeing plant

f.	Repair service for machinery and equipment including repair 							garages and specialty establishments such as motor, body and 							fender, radiator, motor tune-ups, muffler shops, tire repairing 							sales and service including vulcanizing

g.	Store or monument works not employing power tools, or if 							employing such tools then within a completely enclosed building

10.	Accessory uses clearly incidental to the principal uses permitted on the 						same premises

11.	Signs as regulated by Article IV hereof

12.	Parking and loading as regulated by Article V hereof

13.	Wireless Facilities Section 310

14.	Small Wind Projects (FARM) in accordance with Section 601.15

15.	Recreational Facility/Use Type A

16.	Recreational Facility/Use Type B

17.	Recreational Facility/Use Type C

B.	Conditionally Permitted Uses

The Board of Zoning Appeals may issue Conditional Use Zoning Certificates for 					uses listed herein, subject to the general requirements of Section 601 through 					601.2A inclusive and other Sections of Article VI referred to below:

1.	Publicly owned and/or operated buildings and facilities subject to Article 						VI

2.	Waste Transfer Station and Truck terminals subject to Section 601.13

3.	Transportation Terminal subject to Section 601.13C

4.	Recycling Facilities and Junkyards subject to Section 601.13B

5.	Extraction use subject to Section 601.13A

6.	Sexually Oriented Business/Uses

Sexually oriented business/uses are permitted only in the I-1 Industrial 						District under a Conditional Use Permit authorized by the Board of Zoning 						Appeals.  (See Resolution #19-09)

7.	Dangerous, Exotic and Wild Animals in accordance with Article VI.

9.	Temporary Storage Pod/ Portable Storage Container

12.	Shipping Containers

13.	Recreational Facility/Use Type D

Sec. 309.3	Area, Yard and Height Regulations

A.	Minimum Lot Area

Two acres

B.	Minimum Lot Width at Building Line

One hundred feet

C.	Minimum Lot Width at Street

Eighty feet

D.	Minimum Front Yard Depth from Street Right-of-Way

1.	There shall be a front yard having a depth of not less than fifty feet from 						the street line

2.	All watercourses shall be retained and preserved in their natural states 						unless otherwise approved by the Board of Zoning Appeals

E.	Minimum Side Yard Width

Twenty-five feet except that the minimum side yard width abutting a residential 					district shall be one hundred feet.  The area abutting the residential boundary, to 				a depth of twenty feet, shall be landscaped and maintained so as to limit to a 					minimum, any visual effects on adjacent residential uses; the balance of the yard 				area shall be used for open space or vehicular parking only

F.	Minimum Rear Yard Depth

Twenty-five feet

G.	Maximum Height of Building

Fifty feet

Sec. 309.4	Site Plan Review and Conformance

All industrial uses specified under Section 309.2 shall be permitted only after the review 				and approval of the site plans by the Board of Appeals and upon finding by the Zoning 				Commission that the specific standards set forth in this article and the requirements of 				Article II, Section 203.6 will be met.

Sec. 309.5	Permitted Use Certificate Required

A Permitted Use Certificate shall be required to be obtained prior to occupancy of any permitted or conditional permitted use for all businesses in accordance with Section1001.9. Any change in use from one use to another, addition of a use or from one owner/tenant to another will require a new Permitted Use Certificate.

Sec. 309.6	Building Space

Whenever there is more than one principal building on a lot in an I-1 district, the 					minimum distance between the buildings shall be at least thirty (30) feet, measured 				from the building overhang or attached permanent structure, whichever is the lesser 				distance.

Sec. 310	T-1 Wireless Telecommunications Facility

Sec. 310.1	Intent

These regulations are established to provide for the construction and use of wireless 				telecommunication towers and facilities as permitted uses and conditional uses 					depending on the specific land areas of the Township in which they are proposed to be 				located.  The purpose of these regulations is to balance the competing interests created 				by the federal Telecommunications Act of 1996, Public Law 104-104, and the interests of 			the Township in regulating wireless telecommunication towers and related facilities for 				the following purposes:

A.	To protect property values

B.	To regulate a commercial use so as to provide for orderly and safe development 					within the Township

C.	To provide for and protect the health, safety, moral and general welfare of the 					residents of the Township

D.	To protect residential properties, parks, open spaces and the non-intensive 					commercial zoning districts which are characteristic of the Township from the 					adverse effects of towers and related facilities

E.	To promote co-location of wireless telecommunication facilities in order to 					decrease the number of towers in the Township

F.	To maintain, where possible, the integrity of the existing regulations contained in 				the Zoning Resolution

Sec. 310.2	Definitions

A.	Co-location:  The use of a wire telecommunications facility by more than one 					wireless telecommunications provider or by one provider for more than one type 				of telecommunication technology.

B.	Lattice tower:  A support structure constructed of vertical metal struts and cross 					braces forming a triangular or square structure, which often tapers from the 					foundation to the top.

C.	Monopole:  A support structure constructed of a single, self-supporting hollow 					metal tube securely anchored to a foundation.

D.	Telecommunications:  The technology which enables information to be 						exchanged through the transmission of voice, video or data signals by means of 					electrical or electromagnetic systems.

E.	Wireless telecommunications antenna:  The physical device through which 					electromagnetic, wireless telecommunication signals authorized by the Federal 					Communications Commission are transmitted or received.  Antennas used by 					amateur radio operators are excluded from this definition.

F.	Wireless telecommunications facility:  A facility consisting of the equipment and 					structures involved in receiving telecommunications or radio signals from a 					mobile radio communications source and transmitting those signals to a central 					switching computer which connects the mobile unit with the land-based 						telephone lines.

G.	Wireless telecommunications tower:  A structure intended to support equipment 				used to transmit and/or receive telecommunications signals including 						monopoles, guyed and lattice construction steel structures.

Sec. 310.3	Permitted Locations

A wireless telecommunications tower or facility may be located in the following areas, 				under the following circumstances and upon an application for a zoning certificate and 				issuance of such certificate from the Zoning Inspector.  Efforts shall be made to locate in 			the areas listed in the order of priority listed.

A.	Locations:  Efforts shall be made to locate wireless telecommunication tower 					and facilities in the following areas, in the order of priority listed.

1.	First Priority – In a” C” or” I” District located at least two hundred feet 						from an existing residential dwelling.

2.	Second Priority – In an R-4 or Residential PUD District located at least two 					hundred feet from an existing residential dwelling.

3.	Third Priority – In an R-1, R-2, or R-3 District located at least two hundred 						feet from an existing residential dwelling.

B.	In order for the Board of Zoning Appeals to consider the location of a wireless 					telecommunication tower and facility as a conditional use, the applicant shall 					demonstrate that:

1.	There is no technically suitable space for the applicant’s antenna(s) and 						related facilities reasonably available in a permitted location as set forth 						in Section 310.2.

2.	If another tower, building or structure set forth in Section 310.2 is 						technically suitable, the applicant must show that it has requested to 						co-locate on the existing tower, building or structure and the co-location 						request was rejected by the owner of the tower, building or structure.

3.	If an area set forth in Section 310.2 is technically suitable, the applicant 						must show that it has requested all property owners with technically 						suitable locations to permit it to locate a tower facility in all technically 						suitable area(s) set forth in Section 310.2 under reasonable terms and 						that each request was rejected.

C.	With the conditional use permit application, the applicant must demonstrate 					that a technically suitable location is not available in any area set forth in Section 				310.2 and shall list the location of every tower, building or structure and all of 					the areas set forth in Section 310.2 that could support the proposed antenna(s) 					so as to allow it to serve its intended function, and the reasons why such tower, 					building or structure or area has been determined not to be technically suitable.

Sec. 310.4	Co-location Covenant of Good Faith

A.	All towers, owned by a licensed carrier, upon which this Chapter permits 						co-location, of additional antennae, shall be made available for use by the owner 				or initial user thereof, together with as many other licensed carriers as can be 					technically co-located thereon.  However, such licensed carrier may charge a 					reasonable fee for the co-location of additional antennae upon said tower.

B.	All licensed carriers shall cooperate with each other in co-locating additional 					antennae upon such towers.  All licensed carriers shall exercise good faith in 					co-locating with other licensed carriers and in the sharing of towers, including 					the sharing of technical information to evaluate the feasibility of co-location.  In 					the event that a dispute arises as to whether a licensed carrier has exercised 					good faith in allowing other licensed carriers to co-locate upon its tower, the 					Zoning Inspector may require a third-party technical study to evaluate the 					feasibility of co-location at the expense of either or both licensed carriers.  This 					covenant of good faith and fair dealing shall be a condition of any permit issued 					pursuant to this Chapter for a new tower.

C.	Any licensed carrier that allows co-location upon a tower permitted pursuant to 					this Chapter may condition said co-location to assure that the co-location 					antennae does not cause electronic or radio-frequency interference with its 					existing antennae. In the event that the co-location licensed carrier is unable to 					remedy the interference, the owner of the tower shall be relieved of its 						obligation to allow co-location of the interfering antennae upon its structure.

Sec. 310.5	Standards Applicable to Wireless Telecommunications Tower Facilities

All wireless telecommunication towers and facilities shall comply with the following 				standards and conditions:

A.	Towers shall be located, to the extent possible, to minimize any adverse impacts 					on residential property.

B.	The minimum setback of the tower from all property lines shall be:

1.	The height of the tower plus twenty-five feet, or

2.	When it is demonstrated, because of its design and construction, that in 						case of collapse, adjacent property shall not be affected, the minimum 						setback shall be:

a.	Forty feet from any property line abutting a nonresidential lot

b.	Seventy-five feet from any property line abutting a residential lot, 							provided that the base of the tower, including any guy wire 							anchors and required enclosure shall comply with the front yard 							setbacks for the district in which it is located

C.	Towers located as a permitted use in accordance with Section 310.3 shall not 					exceed a height of two hundred feet and may be either monopole structures or 					lattice-type structures.

D.	Towers located as a conditional use in accordance with Section 310.4 shall not 					exceed a height of two hundred feet and shall be monopole structures.

E.	Any accessory structure related to the wireless telecommunication facility shall 					not exceed a height of ten feet and shall not exceed fifty square feet in area, 					either above or below ground.

F.	The applicant of a proposed tower shall demonstrate that the proposed tower is 					the minimum height necessary to accommodate the antennae and is no higher 					than existing towers housing similar antennae.

G.	Prior to approving a new tower, a tower height greater than those prevailing in 					the area or a tower in a location not in compliance with these regulations, the 					applicant shall demonstrate to the Township that such new tower or additional 					height is needed to meet the reasonable service requirements of the applicant.  					This assessment shall include consideration of alternative sites and the 						operational implications of such alternatives with respect, but not limited to: 					height, opportunities for co-location, impact on residents, impact on service 					levels, etc.  The Township may retain consultant(s) to review the information 					with the reasonable costs for such consultation being borne by the applicant(s).

H.	The base of the tower, including any guy wires, and all related facilities shall be 					completely enclosed with a secure fence having a minimum height of eight 					feet.  The fence shall include three strands of barbed wire along the top and 					shall be equipped with a locked gate.  The fence shall be completely screened 					from view by at least one of the following:

1.	A row of evergreen trees spaced not less than ten feet on center.  							The initial planting shall be no less than six feet tall.

2.	Exiting vegetation, inclusive of trees and shrubs, shall be preserved to the 					maximum extent possible.

3.	Other appropriate landscaping that achieves the screening objective.

I.	The tower shall be painted a non-contrasting gray or similar color minimizing its 					visibility, unless otherwise required by the Federal Communications Commission 					(FCC) or Federal Aviation Administration (FAA).

J.	The tower shall be equipped with an appropriate anti-climbing device or shall 					have all climbing pegs from the lower twenty feet of the tower removed and 					separately secured from the public.

K.	The tower shall not be artificially lighted except as required for security and 					safety, or by the Federal Aviation Authority.  Any lighting so required shall be 					installed to minimize the impact on adjoining properties.

L.	Any accessory buildings shall comply with the location regulations set forth for 					the district in which the tower is located.

M.	“No Trespassing” signs and a warning sign shall be posted on the required fence 					in clearly visible locations.  The warning sign shall include a local or toll free 					telephone number of whom to contact in the event of an emergency.  The 					warning sign shall be twelve inches by twelve inches.  No other signs or 						advertising shall be located anywhere on the facility.

N.	The applicant shall submit a reclamation plan at the time of the development 					plan review.  All towers, structures and equipment shall be removed by the 					owner of the tower or facility, and the site restored to its original state within six 					 months following the date that the tower is no longer operational.

O.	An access drive shall be installed and maintained from the road to the tower site.

Sec. 310.6	Abandoned Telecommunication Facilities

A.	The owner or operator of a tower shall, on an annual basis, submit a written 					report to the Zoning Inspector, signed under penalty of perjury, which 						demonstrated whether or not there has been a cessation in use of the tower for 					a period of three months during the prior year.   Annual compliance reports 					shall be submitted by January 1st of each calendar year.  Provided, however, that 					a tower permitted and installed within nine months prior to January 1st shall 					not be required to submit the first compliance report until the following January 					1st.

B.	Any tower that has had no antenna mounted upon it for period of six months, 					or if the antenna mounted thereon are not operated for a period of three 					months, shall be considered abandoned, and the owner thereof shall remove the 				tower within one hundred eighty days after receipt of a notice from the Zoning 					Inspector to do so.

C.	In the event that more than one wireless communication service provider is 					using the antenna support structure, the antenna support structure shall not be 					considered abandoned until all such users cease using the structure as provided 					in this Section.

D.	If a tower and facilities are not removed within ninety days after receipt of a 					notice from the Township requiring said removal, the Township may seek and 					obtain a court order directing such removal and imposing a lien upon which such 				tower is situated in an amount equal to the cost of removal.

E.	Any small parcel(s) created for the use of the tower shall revert back to the 					original owner or contiguous property owners so as not create a landlocked 					parcel.

Sec. 310.7	FCC Compliance

Prior to receiving final inspection by the Zoning Inspector, documentation shall be 				submitted to the Zoning Inspector certifying that the wireless telecommunication facility 			complies with all current Federal Communications Commission (FCC) regulations for 				Non-Ionizing Electromagnetic Radiation (NIER).

Sec. 311	F-1 Flood Plain

Sec. 311.1	Letter of Review

In any area located in the U.S. Army Corps of Engineers flowage easement, the 					developer must provide a letter of review and comment from the Army Corps that 				explains their position on the proposed subdivision.

Sec. 311.2	Flood Prone Areas Outside the FIRM Regulatory Areas

Flood prone areas outside the FIRM (Flood Insurance Rate Maps) regulatory areas will 				be reviewed based on local engineering studies or approval of the U.S. Army Corps of 				Engineers.  If the subdivision is located in an area having poor drainage or other adverse 				physical characteristics, the Commission may approve the subdivision provided the 				sub divider agrees to perform such improvements as will render the area safe for the 				intended use.  In lieu of the improvements, the sub divider shall furnish security bond or 			certified check covering the cost of the required improvements, subject however, to the 				approval of the County Board of Health, and the County Engineer and/or the County 				Flood Plain Administrator.

Sec. 311.3	Purpose of the District

The purpose of this district is to show all areas under restrictions set forth by the 					Muskingum Conservancy and U.S. Army Corps of Engineers and to allow limited uses as 				set forth in Section 311.4.

Sec. 311.4	Restricted Uses

The purpose of this district is to restrict uses within the Flood Plain District.

A.	Permitted Uses

No structure shall be erected, reconstructed, altered or moved onto a premise 					unless a site plan for such use has been approved in accordance with Section 					203.6 of this resolution.  Permitted uses are limited to Non-Commercial 						Recreation and Agriculture Public Park (Non-Commercial Recreation).

Sec. 311.5	Development of Areas Within a Flood-Plain

The development of areas within a flood plain shall be in accordance with the 					Tuscarawas County Floodplain Regulations and shall be approved prior to the approval 				of a final plat for a subdivision.

Sec. 311.6	Alternate Criteria for Determining Flood Plains

A.	Where a sub divider determines there is sufficient doubt as to the flooding of a 					particular portion of land, which is specified as flood plain, he may have a flood 					hazard or other appropriate study prepared by a professional engineer.  This 					data will be submitted to the Tuscarawas County Floodplain Administrator who, 					after consultation with the Ohio Department of Natural Resources, will make a 					final determination as to whether the land in question shall conform to the flood 				plain regulations.

B.	Flood plain information reports approved by the Tuscarawas County Floodplain 					Administrator shall take precedence over all other reports in determining flood 					plains.

Sec. 311.7	Warning and Disclaimer of Liability

The degree of flood protection required by these regulations is considered reasonable 				for regulatory purposes and is based on engineering and scientific methods of study.  				Larger floods may occur or flood heights may be increased by manmade or natural 				causes such as ice jams and bridge openings restricted by debris.  These regulations do 				not imply that areas outside flood plain areas will be free from flooding or flood 					damages.  These regulations shall not create liability on the part of Lawrence Township 				or any officer or employee thereof for any flood damage that result from reliance on 				these regulations or any administrative decision lawfully made there under.

Sec. 312	Planned Unit Development (PUD) Districts

Sec. 312.1	Purpose and Intent

The purpose of this chapter is to establish provision for planned unit development 				districts subject to the regulations and procedures contained herein in conformance 				with the provisions of Section 519.021 of the Ohio Revised Code.  Planned Unit 					Development districts are intended to promote the general public welfare, encourage 				the efficient use of land and resources, promote greater efficiency in the provision of 				public services and utilities, and encourage innovation in the planning and building of 				developments by providing opportunities for creative design and planning of 					developments using flexible zoning guidelines and site design criteria.

It is the intent of this chapter to make planned unit developments subject to the 					established and negotiated development standards as proposed and accepted by the 				developer as part of the approval process in order to:

A.	Encourage creative, high quality site design practices in the development of 					residential areas.

B.	Promote harmony and integration with existing land uses and protect adjoining 					properties from adverse impacts.

C.	Promote safe and efficient pedestrian and vehicular movement.

D.	Permit the clustering of housing units.

E.	Promote efficient layout of infrastructure.

F.	Provide tracts of permanently preserved open space.

G.	Implement the Lawrence Township Comprehensive Plan.

Sec. 312.2	Planned Unit Development Districts Authorized

To achieve the purposes of this chapter, the following Planned Unit Development 				Districts are hereby created:

A.	Single Family Planned Unit Development (SFPUD) Districts

Single Family Planned Unit Development (SFPUD) Districts are intended to 					provide for clustering of single-family dwellings in appropriate locations in 					furtherance of the stated purposes of this chapter.

B.	Moderate Density Planned Unit Development (MDPUD) Districts

Moderate Density Planned Unit Development (MDPUD) Districts are intended to 					provide for more intensive housing options including multi-family dwellings in 					well-planned developments which have condominium style ownership, exhibit 					cohesive design principles, have privately owned and maintained streets, and are 				of limited size and scale.

Sec. 312.3	Establishment of Planned Unit Development Districts

The following criteria shall govern the establishment of Planned Unit Development 				(PUD) Districts:

A.	No PUD District shall be stablished except subsequent to petition by the 						owner(s) of land proposed to be included within the district.

B.	No PUD District shall be created unless a determination is made that such 					development will be in conformance with the goals and objectives of the 						Lawrence Township Comprehensive Plan.

C.	Each PUD District shall be served by an approved public water supply system and 				an approved public sanitary sewer system.

D.	Each Planned Unit Development shall be developed in conformance with an 					approved development plan, which has been reviewed and approved by the 					Township Trustees in accordance with the provisions, set forth herein.

E.	Each Single Family Planned Unit Development (SFPUD) District shall have a 					minimum area of not less than ten contiguous acres, provided however, that 					after a district is established, additional contiguous areas of any size may be 					added as modifications and extensions of the original district.

F.	Each moderate Density Planned Unit Development (MDPUD) District shall consist 				of a gross area of not less than five or more than twenty-five contiguous acres.

G.	Each Moderate Density Planned Unit Development (MDPUD) District shall have 					frontage on a Primary Route as designated in the Lawrence Township 						Comprehensive Plan.

Sec. 312.4	Permitted Uses

Within a Planned Unit Development District, no building, structure or premises shall be 				used, arranged to be used, or designed to be used, in whole or in part, except for one or 				more of the following uses specifically enumerated as a permitted, conditionally 					permitted or accessory use within such planned unit development district and further 				provided that each such use is identified on and approved as part of the development 				plan.  The Zoning Commission and Township Trustees shall determine the 					appropriateness of each proposed use in reviewing the development plan, giving 				consideration to the standards established herein and any other considerations deemed 			by the Commission and Trustees to be significant in determining the appropriateness of 				a use or uses.  The Commission and Trustees shall have the authority to deny approval 				for inclusion of any proposed use where such use is determined to be inappropriate for 				the development.

A.	Single Family Planned Unit Development (SFPUD) Districts

1.	Permitted Uses

a.	Detached Single Family Dwellings

b.	Attached Single Family Dwellings, provided that not more than 							four such dwelling units shall be attached in any single building

c.	Public or private parks and recreation facilities, including golf 							courses, club houses, swimming pools, and tennis courts

d.	Public facilities

2.	Conditionally Permitted Uses

a.	Private and public schools

b.	Day care facilities and nursery schools

3.	Accessory Uses

a.	Detached garages

b.	Common and/or guest parking areas

c.	Detached storage buildings provided that the maximum size of 							such buildings shall be as established by the Zoning Commission 							and Township Trustees as part of the Final Development plan 							approval.

d.	Fences

e.	Home occupations subject to the conditions and requirements set 						forth in Section 213 of this Zoning Resolution.

B.	Moderate Density Planned Unit Development (MDPUD) Districts

1.	Permitted Uses

a.	Attached single family dwellings, provided that no more than 							eight such dwelling units shall be attached in any single 								building.

b.	Multi-family dwellings provided that no more than eight such 							dwelling units shall be contained within a building.

c.	Parks and recreation facilities

d.	Public facilities

2.	Accessory Uses

a.	Detached Garages

b.	Common and/or guest parking areas

c.	Detached storage buildings, provided that the maximum size of 							such buildings shall be established by the Zoning Commission and 							Township Trustees as part of the Final Development Plan 								approval.

d.	Fences

e.	Home occupations subject to the conditions and requirements set 						forth in Section 213 of this Zoning Resolution.

Sec. 312.5	Development Standards Applicable to all PUD Districts

The following development standards shall apply to all Planned Unit Development 				Districts:

A.	Open Space Requirements

1.	Open space and recreation areas shall be as set forth on the final 							approved development plan provided, however, that the land area 						designated for open space and recreational use shall not be less than 						forty percent of the total land area of the Planned Unit Development 						District.

2.	Common Open Space shall be located and designed to:

a.	Be integrally related to the overall design of the development

b.	Be beneficial to the residents of the PUD

c.	Conserve and protect significant natural features such as 								wetlands, woodlands, streams, lakes, historic features, and 							environmentally sensitive areas.

3.	Land areas devoted to streets, drives, parking areas, rights-of-way, 						required setbacks from streets and rights-of-way, required spacing 						between buildings, and areas within individual building lots shall not be 						included in the calculation of open space for the purpose of meeting the 						minimum area requirement.  Open space areas within required buffers 						and/or setbacks from property lines might be counted as common open 						space area.

4.	Water surfaces of lakes, ponds, or other open bodies of water which are 						under the direct ownership and control of the applicant may be included 						as open space but shall not be considered to constitute more than one-						half of the required common open space requirement for any 							Planned Unit Development District.

5.	The ownership of all common open space areas, vegetative buffers, 						fences, ponds, parking areas, and similar common spaces shall be 							identified and a perpetual maintenance plan for said areas submitted to 						the Township for review and approval.  Said perpetual maintenance plan 						shall set forth responsibility for maintenance of all such areas and 							describe the method of financing for said maintenance program.  The 						perpetual maintenance plan shall become part of the development plan 						and development agreement and shall be placed on record with the 						County Recorder as covenant on the land within the PUD district.  The 						perpetual maintenance plan shall identify Lawrence Township as a 						beneficial party thereto with rights, but no obligation, to enforce the 						provision contained therein.

B.	Utilities

All utilities shall be located underground, except that utility appurtenances may 					be constructed above-ground as approved by the Township as part of the 					development plan approval.

C.	Parking

Off-street parking shall be provided in accordance with the following:

1.	Each dwelling unit shall be provided with a minimum of two- and one-half off-street parking spaces, at least one of which shall be within a completely enclosed garage.

2.	Parking for recreation facilities and other permitted non-residential uses 						shall be as identified on the development plan and approved by the 						Township Trustees.

3.	Illumination shall be provided for common parking lots, walkways, and 						drive intersections.  Lighting shall be designed and located so as not to 						cause glare to permit the source of light to be visible beyond the 							boundaries of the development.

4.	All common parking areas shall be improved with a compacted base, not 						less than four inches in depth and shall be surfaced with a minimum 						of two inches of asphaltic concrete or comparable all-weather 							dustless surface material.  Every parking area designed for storage of five 						 or more vehicles shall have permanent pavement markings to 							delineate spaces.

5.	Parking areas shall be graded and drained to minimize standing water, to 						provide positive drainage away from buildings, and to prevent runoff on 						to adjacent properties.

6.	Parking areas containing more than thirty spaces shall contain planting 						strips or islands to interrupt the mass of paved area, aid in controlling 						traffic flow, and provide visual quality.

7.	Handicapped parking and access shall be provided in accordance with the 					requirements of the Americans with Disabilities Act.

8.	One guest parking space shall be provided for every four dwelling units.  						Driveways of individual dwelling units shall not be counted toward 						compliance with this requirement.

9.	Pedestrian walkways shall be provided between common parking areas 						and dwelling units.

10.	Each parking space shall be a minimum of nine feet in width and shall 						have a minimum area of not less than one hundred eighty square feet, 						exclusive of access drives and aisles.

D.	Access and Street Requirements

1.	All Planned Unit Developments shall be designed to provide access for 						proposed uses internally within the development and to minimize access 						points and intersections onto existing public streets.  No dwelling unit or 						non-residential use that is part of a PUD shall have a driveway access 						directly onto an existing public street.

2.	Each dwelling unit and each non-residential use shall have access to a 						street internal to the PUD in a manner approved by the Township and 						said access shall be clearly defined on the development plan.

3.	Planned Unit Developments shall be designed to permit adequate access 						by emergency vehicles, promote the safety of motorists and pedestrians, 						minimize traffic conflicts and congestion, and promote the safe, efficient 						flow of vehicular traffic.

4.	Street identification signs and traffic control devices shall be provided 						and installed by the developer as directed by the Township and in 							accordance with the standards of the Tuscarawas County Engineer and 						the Ohio Department of Transportation.  Where such signs, markings, or 						devices are located on private streets, the owner’s association shall be 						responsible for their maintenance.

E.	Landscaping and Buffers

1.	All disturbed areas within a Planned Unit Development which are not 						covered by permitted structures or pavement shall be landscaped with 						grass, trees, shrubbery and other appropriate ground cover or 							landscaping materials.  All landscaping shall be in conformance with the 						approved landscaping plan for the development.

2.	Entry points shall be landscaped in an attractive manner using plant 						materials, which are consistent throughout the project.  Street trees shall 						be provided along the main access streets that are sufficient to create a 						tree-lined image.  Trees and shrubs shall be planted so as not to obstruct 						the view of drivers at driveway entrances and/or street intersections.

3.	Screening and buffering shall be provided within the required setback 						from existing public street right-of-way.  Where existing vegetation is 						inadequate to provide an appropriate buffer, as determined by the 						Zoning Commission and Township Trustees, supplemental landscaping 						shall be provided to create an adequate screen.  Said landscape 							improvements may include mounding and/or screen wall or fences if 						approved as part of the landscape plan.

4.	Buffers may be required within setbacks from adjoining properties where 					the Zoning Commission and Township Trustees determine that such 						screening is necessary to mitigate anticipated visual or auditory impacts.

F.	Architectural Design

Architectural treatments shall demonstrate a cohesive design concept which 					promotes compatibility among structures while incorporating a diversity of 					materials, styles, and features conductive to an attractive and desirable 						residential environment.  Concrete block or other foundation materials shall not 					be left exposed above finished grade, but shall be covered with brick, stone, or 					other finished building façade material approved by the Township.

G.	Drainage

All PUD Developments shall provide for storm water management in accordance 					with the provision of Section 210.

H.	Erosion and Sedimentation Control Erosion

Erosion and Sedimentation Control Erosion and sedimentation control shall be 					required pursuant to Section 211.2 Water Quality and Erosion and 						Sedimentation Control.

I.	Water Quality Standards

Water quality shall be required pursuant to Section 211.3 Water Quality and 					Erosion and Sedimentation Control.

J.	Fire Protection

All PUD Developments shall make provisions for fire protection that shall include 					the following:

1.	Installation of fire hydrants on all streets at intervals of not more than 						three hundred feet.

2.	Hydrants shall have two- and one-half-inch outlets and one large pumping connection and shall be placed with all outlets either facing or parallel to the street.

3.	The type of hydrant and control valves shall be as approved by the 						Bolivar Volunteer Fire Department Fire Chief and the Tuscarawas County 						Sanitary Engineer.

4.	Water lines serving fire hydrants shall be a minimum of eight inches in 						diameter and shall be looped except as specifically approved by the BVFD 					Fire Chief.

K.	Trash Receptacles

Outdoor trash collection and storage facilities shall be screened from view from 					any public right-of-way and/or adjoining residential property by an enclosure 					consisting of a solid wall or fence at least six feet in height.

Sec. 312.6	Development Standards Applicable to SFPUD Districts

The following development standards shall apply to all Single Family Planned Unit 				Development Districts:

A.	Density of Dwelling Units

The maximum density of dwelling units shall be as set forth on the approved 					development plan, but shall in no case be greater than two dwelling units per 					gross acre of the Singe Family Unit Development District.

B.	Maximum Height

No building or structure shall exceed two stories or thirty-five feet in height 					except as specifically authorize by the Zoning Commission and Township 						Trustees as part of the development plan approval.  Accessory buildings shall not 				exceed twenty feet in height except as specifically authorized by the Zoning 					Commission and Township Trustees as part of the development plan approval.

C.	Minimum Floor Area

Dwelling units within a Single Family Planned Unit Development shall comply 					with the following standards for minimum livable floor area:

1.	Each Detached Single-Family Dwelling shall contain not less than one 						thousand eight hundred square feet of livable floor area.

2.	Each Attached Single Family Dwelling shall contain not less than one 						thousand two hundred square feet of livable floor area.

D.	Setbacks and Separations

Dwelling units within a Single Family Planned Unit Development may be on 					individual lots and/or part of a condominium arrangement in accordance with 					Chapter 5311 of the Ohio Revised Code.  Building setbacks and separations shall 					be as established on the approved final development plan.  In establishing said 					separations, the Zoning Commission and Township Trustees shall consider the 					spacing necessary for adequate visual and acoustical privacy, adequate light and 					air, fire and emergency access, building configurations, energy-efficient lighting, 					and the relationships of building sites to circulation patterns.  In no instance shall 				the established setbacks and/or separations be less than the following:

1.	No building, structure, or parking area shall be located closer than one 						hundred feet to the right-of-way line of an existing public street.

2.	No building, structure, or parking area shall be located closer than fifty 						feet from any project boundary line of the Single Family Planned Unit 						Development.

3.	No building, structure, or parking areas shall be located closer than fifty						feet from the right-of-way line of any new public right-of-way 							constructed as part of the SFPUD.

4.	No building or structure shall be located closer than thirty-six feet from 						the centerline or twenty-five feet to the edge of pavement or edge of 						easement of any private street whichever is the greatest.

5.	The minimum distance between buildings, which contains less than three 					 dwelling units, shall be twenty feet.

6.	The minimum distance between buildings where one or more of the 						buildings contain three or more dwelling units shall be twenty-five feet.

7.	Where lot lines are created as part of a Single Family Planned Unit 						Development, there shall be no setback requirement from such lot lines 						provided that:

a.	Such arrangement is shown on the development plan

b.	The building separations comply with the requirements contained 						herein, and

c.	Such arrangement is specifically approved by the Township as 							part of the development plan approval.

E.	Access and Street Requirements

1.	All public streets within a SFPUD shall be designed and constructed in 						accordance with the standards and specifications of the Tuscarawas 						County Subdivision Regulations and the County Engineer.

2.	Private streets within a SFPUD District shall be a minimum of twenty-two 						feet in width.  Private cul-de-sac streets shall be designed with sufficient 						turn around areas to adequately accommodate emergency vehicles and 						service vehicles such as snowplows and garbage trucks.

3.	Private streets shall meet the same base and pavement design criteria as 						public streets.

F.	Minimum Building Lot Area

Where individual building lots are established within a Single Family Planned 					Unit Development, the sizes of said lots shall be as established on the 						development plan and as approved by the Township.

Sec. 312.7	Development Standards Applicable to MDPUD Districts

The following development standards shall apply to all Moderate Density Planned Unit 				Development Districts:

A.	Density of Dwelling Units

The maximum density of dwelling units shall be as set forth on the approved 					development plan, but shall in no case be greater than six dwelling units per 					gross acre of the Moderate Density Planned Unit Development District.

B.	Maximum Height

No building or structure shall exceed three stories or forty feet in height except 					as specifically authorized by the Zoning Commission and Township Trustees as 					part of the development plan approval.  Accessory buildings shall not exceed 					twenty feet in height except as specifically authorized by the Zoning Commission 					and Township Trustees as part of the development plan approval.

C.	Minimum Floor Area

Dwelling units within a Moderate Density Planned Unit Development shall 					comply with the following standards for minimum livable floor area:

1.	Each Attached Single Family Dwelling shall contain not less than one 						thousand two hundred square feet of livable floor area.

2.	Each Multi-family Dwelling shall contain not less than eight hundred 						square feet of livable floor area.

D.	 Setbacks and Separations

Dwelling units within a Moderate Density Planned Unit Development shall be part of a condominium arrangement in accordance with Chapter 5311 of the Ohio Revised Code.  Building setbacks and separations shall be established on the approved final development plan.  In establishing said separations, the Zoning Commission and Township Trustees shall consider the spacing necessary for adequate visual and acoustical privacy, adequate light and air, fire and emergency access, building configurations, energy-efficient lighting, and the relationships of building sites to circulation patterns.  In no instance shall the established setbacks and/or separations be less than the following:

1.	No building, structure, or parking area shall be located closer than one 						hundred feet to the right-of-way line of an existing public street.

2.	No building, structure, or parking area shall be located closer than fifty 						feet to any project boundary line of the Moderate Density Planned Unit 						Development.

3.	No building or structure shall be located closer than 36 feet from the 						centerline or twenty-five feet to the edge of pavement or edge of 							easement of any private street whichever is the greatest.

4.	The minimum separation between buildings containing less than five 						dwelling units shall be twenty-five feet.

5.	The minimum separation between buildings where one or more of the 						buildings contains five or more dwelling units shall be thirty-five feet.

E.	Access and Street Requirements

1.	All streets within a MDPUD shall be privately owned and maintained and 						shall be designed and constructed in accordance with the base and 						pavement design standards and specifications of the Tuscarawas County 						Subdivision Regulations and the County Engineer.

2.	Private streets within a MDPUD District shall be a minimum of twenty-two feet in width.  Private cul-de-sac streets shall be designed with sufficient turn around areas to adequately accommodate emergency vehicles and service vehicles such as snowplows and garbage trucks.

3.	Where the Lawrence Township Comprehensive Plan indicates future 						right-of-way, roadway connections, or frontage roads in the area of a 						proposed MDPUD District, the development plan for the MDPUD shall be 						designed to accommodate the proposed routing of such planned 							roadways and the access provisions for the MDPPUD shall be designed 						and coordinated to be compatible with such future roadways.  The 						Zoning Commission and Township Trustees may require such special 						conditions as they may deem necessary and appropriate to manage the 						future access to and from the MDPUD in association with any proposed 						future rights-of-way, road connections, or frontage roads, as approved by 					the Tuscarawas County Planning Commission.

Sec. 312.8	Application Requirements and Procedures

A.	Property owners who wish to have their land zoned for a Planned Unit 						Development District shall make application for both a Zoning Map amendment 					and general development plan approval.  The request for rezoning and 						application for general development plan approval shall occur simultaneously 					and the approval of one shall be dependent on the approval of the other.

B.	Applications for rezoning to a Planned Unit Development District shall be made 					by filing an application to the Zoning Commission pursuant to the provision of 					Section 602 of this Zoning Resolution.  Said application shall be heard and action 					taken in accordance with the procedures and provisions set forth in Section 602.  				The following additional submission requirements shall apply to all applicants for 				rezoning to a Planned Unit Development District:

1.	A General Development Plan conforming to the requirements of Section 						601 of this Resolution

2.	A traffic impact analysis projecting the nature and volumes of vehicular 						traffic to be generated by the proposed development and evaluating the 						capacity of the existing roadway system to accommodate that traffic

3.	A utility impact analysis addressing the proposed project’s demand for 						water and sanitary sewer services and accessing the availability of 							adequate treatment and transmission capacities to meet the projected 						needs

4.	A storm drainage management plan, which addresses the proposed 						methods of controlling storm run-off and mitigating erosion and 							sedimentation, impacts

5.	Proposed covenants and restrictions intended to govern the 							development and future use of the Planned Unit Development including 						a perpetual maintenance plan setting forth the proposed ownership 						arrangement, maintenance responsibility, and financing method for all 						common open space, recreation facilities, common parking areas, private 					streets, and other commonly owned facilities

6.	Any proposed development agreement proffered as inducement for the 						rezoning of the property

C.	After the Board of Township Trustees approves both an application for rezoning 					to a Planned Unit Development District and the General Development Plan for 					said PUD District, the applicant shall submit a Final Development Plan, or plans if 					the development is to be accomplished in phases, in conformance with Section 					602 hereof.  Final Development plans shall be submitted concurrently to 						appropriate County reviewing agencies.

Sec. 312.9	Development Plan Requirements and Procedures

A.	Pre-application Meeting

Prior to the submission of an application and development plan for a Planned 					Unit Development, applicants shall notify the Zoning Inspector of their intent to 					file an application and request a meeting with the Zoning Inspector and other 					pertinent Township and County Officials at which time the applicant may present 				a preliminary sketch or concept plan of his proposed Planned Unit Development.  				The purpose of this pre-application meeting is to discuss the criteria and 						standards contained herein, to familiarize the applicant with the PUD process, to 					familiarize the applicant with the relevant portions of the Lawrence Township 					Comprehensive Plan, and to review the applicant’s proposed general approach 					to development of the site.

B.	Development Plan Required

Submission of development plans is required for all Planned Unit Development 					projects.  A general Development plan for the entire project shall be submitted 					and reviewed simultaneously with the application for rezoning for the PUD 					District, which shall also include any floodplain Corp of Engineers controlled 					inundation, and any wetland delineation.  If the rezoning and General 						Development Plan is approved, subsequent development of the property shall 					be made only in substantial conformance to said approved General Development 				Plan.  After a parcel is rezoned to a Planned Unit Development District and a 					General Development Plan has been approved for the entire project, the 						applicant shall submit a Final Development Plan for review and approval.

C.	General Development Plan Submission Requirements

Each application for General Development Plan review shall include twelve 					copies of the plan for the entire PUD area, drawn to scale and shall include, at a 					minimum, the following data:

1.	The name of the development, the name of the owner or developer, 						north arrow, date and scale

2.	The owners and zoning classification of adjoining parcels

3.	A boundary survey

4.	Existing topography and proposed finished grade with a maximum two-foot contour interval

5.	Proposed building locations

6.	Location of all minimum setback lines

7.	Vehicular and pedestrian circulation plans

8.	All off-street parking areas indicating the number of parking spaces 						provided and the number required

9.	A storm drainage plan: including preliminary arrangements for storm 						detention facilities

10.	All existing and proposed water facilities including the location and sizes 						of water mains, and the location of fire hydrants

11.	All existing and proposed sanitary sewer facilities

12.	Location and size of all recreation and open space areas

13.	A general planting and landscaping plan

14.	Architectural plans of proposed structures including the number and 						minimum floor area of dwelling units

15.	The location, width, names, and grades of existing and proposed streets

16.	Typical sections for all proposed streets

17.	Proposed phases if the project is to be developed in stages

18.	The location and sizes of any proposed fee simple building lots with 						SFPUD Districts

19.	A summary table showing the total acres of the proposed development, 						the number of acres devoted to open space, streets, and contained 						within lots, and the number of dwelling units by type.

D.	Review Criteria for General Development Plans

When reviewing an application for a Planned Unit Development, the Zoning 					Commission and Township Trustees shall consider, but shall not be limited to 					consideration, of the following characteristics of the proposed development:

1.	The comprehensive nature and design of the General Development Plan, 						including appropriate design of the physical, aesthetic, and economic 						relationships among its parts

2.	The suitability of the site proposed for zoning as a Planned Development 						District, including its location, area, relationship to existing development 						in the community, natural features, relationship to community plans, and 					other such characteristics

3.	The anticipated effects of the proposed development upon the Township 						and upon adjoining and proximate neighbors and properties, including 						the impacts of traffic, storm water, noise, lighting, utilities, aesthetic 						values and other impacts

4.	The adequacy of existing and planned roads, drives, and parking areas to 						meet the projected demand for such facilities and to integrate with 						existing and planned facilities in the Township

5.	The adequacy of planned pedestrian and bicycle facilities to meet the 						demand for such facilities, to integrate with existing and planned facilities 					in the Township and to promote use of such transportation modes

6.	The suitability of the location, dimensions, access to streets and utilities 						of each proposed dwelling unit within the District

7.	The adequacy of utilities to serve the proposed development and the 						suitability of the proposed utility design within the District

8.	The proper orientation and relationship of the proposed elements of the 						development with natural and historic features and resources both on 						and off site, the degree to which the development has been designed to 						protect and enhance such features and resources, and the measures 						taken to mitigate negative impacts on such features and resources both 						on and off site

9.	The relationships of the architectural and site design characteristics 						among the areas of the development and with surrounding properties

10.	The availability of recreation and open space sites and facilities proposed 						for use by the residents of the development

11.	The nature and extent of proposed landscaping, existing vegetation and 						landform to be retained, and of proposed screening and buffering

12.	The suitability of the proposed separations between buildings, including 						any proposed setbacks or yards

13.	The suitability of the total acreage and floor area proposed for each type 						of dwelling unit, and the number of bulk of buildings proposed

14.	The suitability of proposed condominium or homeowners’ association 						agreements, deed restrictions, protective covenants and other legal 						statements or devices intended to provide for future use, ownership, 						operation and maintenance of areas of the Planned Unit Development 						and its improvements

15.	The ability of each proposed phase of the development, or of any group 						of developed phases, to meet the standards established in this Zoning 						Resolution

E.	Final Development Plan Requirements

1.	Final Development Plans submitted to the Zoning Commission for review 						shall be based on a previously approved General Development Plan and 						may be for portions or phases of the entire project.  Final Development 						Plans shall be submitted at least ten working days prior to the 							meeting at which said plans will be reviewed by the Zoning Commission.  						A minimum of twelve copies shall be submitted.  Submission shall include 					a fee as established by the Township Trustees.  Final Development Plan 						submissions shall be accompanied by performance bonds as required 						herein to guarantee completion of required improvements.

2.	Final Development Plans shall be prepared by persons professionally qualified to do such work.  Final Development Plans shall be certified by an architect, engineer or land surveyor duly registered by the State of Ohio.  Final Development Plans shall be prepared at an appropriate scale, but not less than one-inch equals one hundred feet.  Profiles must be submitted on standard plan profile sheets.

3.	Final Development Plans shall include detailed design information for all 						of the items contained on General Development Plans but shall also 						include detailed construction drawings for proposed improvements 						including such items as:

a.	Detained street improvement plans including proposed traffic 							control provisions such as signage, pavement markings, and 							signalization

b.	Detailed utility improvement plans including all pipe sizes, types, 							grades, and invert elevations, and the location of manholes for 							sanitary and storm sewers and the location and sizes of water 							mains, and the location of fire hydrants

c.	A detailed landscaping plan including a listing of all plant material 							by type, size and number

d.	Provisions for the adequate control of erosion and sedimentation

e.	The location, type, size and height of all fencing, screening, and 							retaining walls

f.	The location, width, size and intended purpose of all easements 							and right-of-way and whether they are to be publicly or privately 							maintained

g.	A site lighting plan

h.	Detailed site grading and drainage plans including storm 								detention calculations and pipe sizing analyses

F.	Bonding of Required Improvements

A performance bond or other financial guarantee as approved by the Township Trustees and the Township’s legal counsel shall be placed on deposit with the Township to ensure that the landscaping, hard surfacing of private streets, drives and parking areas, improvements within public rights-of-way or easements, water lines, sanitary sewer lines, storm sewers, and surface water drainage, and other improvements integral to the proposed project shall be installed in conformity with approved plans. Such bond or guarantee shall be in an amount equal to the cost of the construction of the improvements, based on an estimate certified by the applicant’s design engineer and approved by the Township, and shall be for a period not to exceed two years and provide for the complete construction of the improvements within that period.  Where performance guarantees are provided to other public agencies, such as the Tuscarawas County Engineer for roads, and the Tuscarawas County Metropolitan Sewer and Water district for sewer and water, for certain required improvements, such arrangements shall be deemed to meet the 	requirement of this provision for such improvements so that no dual bonding of specific improvements is required.  All private roads shall be bonded by Lawrence Township, and all public roads shall be bonded by the Tuscarawas County Regional Planning Commission.

G.	Approval of Final Development Plans

1.	The Zoning Commission shall review each Final Development Plan and 						shall make a recommendation to the Township Trustees regarding same, 						within sixty days of the date at which such final development plan is first 						heard by the Zoning Commission unless such time is extended with the 						consent of the applicant.  The Zoning Commission may suggest, and the 						Township Trustees may attach, such conditions to the approval of the 						Final Development Plan as may be reasonable required by the public 						health, safety and welfare, deemed appropriate to carry out the 							purpose and intent of this Zoning Resolution, and consistent with the 						implementation of the Township’s Comprehensive Plan.  All Final 							Development Plans must have the approval of applicable County 							agencies.

2.	The Township Trustees shall act upon each Final Development Plan 						referred by the Zoning Commission within sixty days of receipt of the 						Zoning Commission’s recommendation provided, however, that said time 					period may be extended by the Trustees with the consent of the 							applicant.

H.	Compliance Required

Subsequent to the approval of a Planned Unit Development District, all 						subdivision plats, site plans, building permits, zoning certificates, and other plans 				for improvements and any development or construction within the District shall 					be in substantial compliance with the approved Final Development Plan and any 					conditions of such approval adopted by the Township in approving the PUD 					District and Final Development Plan.  Any departure from the approved Final 					Development Plan and any conditions or development agreements attached 					thereto, shall be deemed to be a violation of this Zoning Resolution.  When the 					Zoning Inspector determines that a proposed plan, request for permit, 						development or construction may not be in compliance with the Final 						Development Plan, he shall take appropriate action as authorized by this Zoning 					Resolution to compel compliance.

I.	Amendments to General or Final Development Plans

1.	The owner of a property within a Planned Unit Development District may 						submit plans for amendment of either the General or Final Development 						Plan approved for the District.  The Zoning Commission and Township 						Trustees shall review such amended plan and may approve the 							amendment if it is determined that the amendment is substantially in 						conformance with the form, nature, and intent of the General 							Development Plan approved for the district as part of the rezoning.

2.	If it is determined that the amendment is not substantially in 							conformance with the form, nature, or intent of the approved General 						Development Plan for the PUD District, then the amendment shall be 						disapproved and the applicant directed to proceed as if considering an 						amendment to the Zoning Map.

Sec. 312.10	Professional Assistance

The extent and complexity of certain applications for Planned Unit Developments will 				require that the Zoning Commission and/or Township Trustees obtain review assistance, 			statements of opinion, and reports from qualified professionals such as civil engineers, 				planners, appraiser, architects, and attorneys.  The Zoning Commission and/or Township 			Trustees shall determine when such studies or expert advice are necessary to evaluate a 			proposed Planned Unit Development relative to the requirements of this Section.  The 				Zoning Commission and/or Township Trustees shall advise the applicant if such studies 				are required and provide an estimate of whether the anticipated costs of such studies 				will exceed the base deposit set forth in Section 312.12.  The applicant shall 					immediately upon such notification deposit with the Township sufficient funds to pay 				for such studies.

Sec. 312.11	Construction Inspection

During construction the Township’s representatives shall be afforded adequate 					opportunity to inspect the development to confirm proper installation of required 				improvements and compliance with the provisions of this Zoning Resolution, the 					approved standards and conditions for the PUD, and such other regulations as may be 				applicable.

Sec. 312.12	Fees and Deposits

A.	All applications for development plan review and approval shall be accompanied 					by a non-refundable fee in an amount established by the Township Trustees.

B.	All applications for development plan review and approval shall also be 						accompanied by a cash deposit for professional consultant services in an amount 				established by the Township Trustees.  Any balance of unused deposit funds 					shall be refunded to the applicant within sixty days of the Township’s final action 				on the application.  Prior to commencement of construction, the developer of a 					PUD shall deposit with the Fiscal Officer a sum as determined by the Township 					Trustees, which funds shall be used by the Township to pay for project 						inspections and administration during construction as provided in Section 312.9.

Article IV

Sec. 401	Sign Regulations									REVISED

Sec. 401.1	Purpose

The purpose of this article is to provide for the use, location and size of signs and to 				regulate their installation and maintenance.

Sec. 401.2	Permitted Signs

Unless otherwise provided herein, a Zoning Certificate shall be required for all signs.  No 			signs except highway safety signs shall extend in to any street right-of-way.  All signs 				must be restricted to the premises.

A.	The following signs are permitted in any district:

1.	Permanent signs requiring sign fee or Zoning Certificate:

a.	Memorial signs or tablets, names of buildings, and/or signs 							indicating the date of erection.  Such signs or tablets shall have an 							area not to exceed six square feet facing each street and shall 							be affixed to the building.

b.	Awnings and canopies are permitted for public convenience with 							a minimum height requirement of eight foot clearance.  No 							advertising shall be placed on any awning or canopy, except that 							the name of the owner and the business industry or pursuit 							conducted within the premises may be painted or otherwise 							permanently placed in a space not exceeding eight inches in 							height of the front and side portions thereof.

c.	Nameplates not exceeding one square foot in area, containing 							only name of the resident and/or title of person practicing a 							profession and/or name of building and/or the name of the agent 							and/or the identification of a “home occupation”.

d.	Signs appropriate to a public or semi-public building for the 							purpose of displaying the name and activities or services therein 							provided, not larger than a total of twenty square feet and 							restricted to the premises.

e.	One permanent sign not more than twenty square feet in area 							identifying a subdivision or neighborhood at each entrance to said 						subdivision or neighborhood.  Such signs, exclusive of decorative 							walls, fences, or base, shall not exceed six feet in height, shall 							not be located closer than twenty five feet to any street right-of-							way line and shall not obstruct visibility at street corners.

2.	Temporary Signs

a.	Real Estate and Development Signs

1.	One unlighted real estate sign not exceeding six square 								feet in area pertaining only to the sale, lease, or rent of the 							particular building, property or premises upon which 								displayed such sign shall be removed within thirty days 								after the sale, lease or rent of the building or property.  No 							sign fee or permit shall be required.

2.	One temporary real estate sign advertising the 									development of the premises upon which it stands or the 								opening of a new subdivision within which such sign is 								located and maintained upon the issuance of a temporary 								six months renewable Zoning Certificate and shall be 								removed from the premises within thirty days of the sale 								or rental thereof.  Such sign shall not exceed one hundred 								square feet in area and be set back one foot per square 								foot of sign area, with a minimum of twenty five feet from 								the street right-of-way.

b.	Special Events Signs

1.	The Zoning Inspector may permit the placement of 									temporary signs within the Township for a period not to 								exceed thirty days which signs shall be limited to 									advertising public activities and activities or organizations 								within the township.  No sign fee shall be required.  Such 								signs shall be removed within five days subsequent to said 								thirty day period.

2.	The Zoning Inspector shall permit the placement of signs 								promoting candidates for public office.  Such signs shall 								not be placed on or over public property or right-of-way.  								Such signs shall be placed no earlier than thirty days prior 								to the date of election to which they relate, and such signs 								and all supporting materials shall be removed within ten  								days following the election.

c.	Construction Signs

One sign denoting the architect, engineer or contractor when 							placed upon work under construction, and not exceeding twelve 							square feet in area.  No sign fee or Zoning Certificate shall be 							required.

d.	Signs for Roadside Stands

One unlighted sign not more than twenty feet in area in 								conjunction with a roadside stand which shall be used only to 							advertise agricultural products produced on the premises.  Such 							signs shall be removed at the conclusion of the seasonal sales.  No 						sign fee or Zoning Certificate shall be required.

B.	The following signs are permitted in “C” and “I” Districts

1.	The following signs are permitted in all “C” and “I” Districts except that 						no sign shall be located closer than twenty five feet to any lot located in 						any “R” District.

a.	One exterior wall sign on each street frontage may be erected 							which advertises a business or service conducted upon the 							premises and/or advertises products, merchandise or 								commodities stocked and sold on the premises.  Wall sign shall be 						affixed flat against the wall of buildings and shall not project 							there from more than one foot.  No such sign shall extend 								beyond the width or height of the wall to which it is attached.  No 							wall sign shall exceed fifty square feet in area.

b.	One free standing sign on each frontage which advertises a 							business or service conducted on the premises and/or advertises 							products, merchandise and/or commodities stocked and sold on 							the premises shall be permitted provided such sign conforms to 							the following requirements:

1.	No freestanding sign shall exceed fifty square feet in area 								per side

2.	No sign shall exceed twenty feet in height

3.	In districts where setbacks are required, no sign shall be 								located closer than ten feet from the street right-of-way

2.	The following signs are permitted in all C districts except that no sign					shall be located closer than fifty feet to any lot located in any “R” District.

a.	All signs permitted in Section 401.2B

b.	One and only one pole-type high rise sign advertising the 								name of the business located on the property.  The maximum 							area per each of two permitted sides shall be one hundred square 						feet.  Such signs shall not exceed forty feet in height.

Sec. 401.3	Construction

A.	All signs shall be constructed in conformance with the appropriate Building Code 					and other applicable requirements of the Township, and shall display the “UL” 					label.

B.	All signs shall be rigidly secured and no sign shall swing from bar, crane, awning, 					or other sign, nor shall all or part of any sign be revolving, oscillating or 						otherwise designed to move to attract attention.

C.	No sign shall be so erected as to obstruct any window, door, fire escape, 						balcony, platform, stairway, ladder, vent or other means of ingress of any 					building.

D.	Signs shall be erected so as to not obstruct traffic sight lines or traffic control 					lights at street intersections or signals at railroad grade crossings.

E.	Signs visible from a street shall not contain an arrow or words such as “stop”, 					“go”, “slow”, etc. or otherwise resembles highway traffic or directional signals.

F.	No sign shall be located on the roof on any building.

G.	Signs, which are illuminated, shall use indirect lighting only.  The source of light 					shall not be visible from the street and shall be shaded.  No flashing, revolving or 					intermittent illumination shall be employed.

H.	Any business sign illuminated with electric lights (including neon or other 						gaseous type tubes or incandescent lamps) effected within one hundred feet of 					an intersection where an illuminated devise has been provided for the control of 					traffic shall not be duplicated in the electric light of such sign, any colors 						appearing in the traffic control signs.

Sec. 401.4	Maintenance

The Zoning Inspector may order any sign to be painted or refurbished at least once each 				year.  All supports, guys, braces and anchors for such signs shall be maintained in a safe 				condition.

Sec. 401.5	Removal of Signs

A.	Any sign not or hereafter existing which no longer advertises a bona fide 						business conducted, shall be taken down and removed by the owner, agent or 					person having the beneficial use of the building or structure upon which such 					sign may be found, within ten  days after written notification from the 						Zoning Inspector, and upon failure to comply with such notice within the time 					specified in such order, the Zoning Inspector is hereby authorized to cause 					removal of such sign, and any expense incident thereto shall be paid by the 					owner of the property on which such sign is located.

B.	If the Zoning Inspector shall find that any sign or other advertising structure 					regulated herein is unsafe or insecure or is a menace to the public or has been 					constructed or erected or is being maintained in violation of the provision of this 					Resolution, notice shall be given in writing by the Zoning Inspector to the 						permittee thereof.  If the permittee fails to remove or alter the structure so as to 				comply with the standards herein set forth within ten days after such notice, 					such sign or other advertising structure may be removed or altered to comply 					with this Resolution at the expense of the permittee or owner of property upon 					which it is located.  The Zoning Inspector shall refuse to issue a permit to any 					permittee or owner who refuses to pay cost so assessed.  The Zoning Inspector 					may cause any sign or other advertising structure that is an immediate peril to 					persons or property to be removed summarily and without notice.

Sec. 401.6	Fees

See the Fiscal Officer or Zoning Inspector for the current fee schedule.

Article IV

Sec. 401	Sign Regulations									REVISED

Sec. 401.1	Purpose

The purpose of this article is to provide for the use, location, and size of signs and to

regulate their installation and maintenance.

A sign is defined as any word, letter, model, sign, pattern, placard, board, notice, device or representation employed wholly or partly for the purposes of advertisement, announcement or direction including any structure used or adapted for the display of advertisements.

Sec. 401.2	Permitted Permanent Signs

Unless otherwise provided herein, a Zoning Certificate shall be required for all signs.  No

signs shall extend in to any street right-of-way.  All signs must be restricted to the

premises.

A.	The following signs are permitted in any district:

1.	Commemorative plaque.

a. 	Commemorative plaque refers to a memorial plaque, sign, plate or tablet which is permanently affixed to or near the structure, object or event it is intended to commemorate and which displays no advertising.

b.	Such signs or tablets shall have an area not to exceed six (6) square feet facing each street and shall be affixed to the building.

2.	Awnings and canopies have a minimum height requirement

of eight (8) feet above finished grade.  The name of the business, address, and/or owner may be printed on the canopy or awning front.  No other

advertising shall be affixed in any way to the canopy or awning.

3.	Nameplate, no greater than two (2) square feet, affixed to the exterior

wall, identifying a home occupation.  The light from any light source shall be shaded, shielded, or directed that the light intensity or brightness shall not adversely affect surrounding or facing residential districts, or affect the safe vision and operation of vehicles moving on public or private roads, highways, or parking areas.

4.	Signs appropriate to a public or semi-public building for the purpose of

displaying the name and activities or services therein provided, not larger

than twenty (20) square feet and restricted to the premises.  If located in

a residential district, the light from any light source shall be shaded, shielded, or directed that the light intensity or brightness shall not adversely affect surrounding or facing residential districts, or affect the safe vision and operation of vehicles moving on public or private roads, highways, or parking areas.

5.	One sign not more than twenty (20) square feet in area (on

each side of sign) identifying a subdivision or neighborhood, at each

entrance to said subdivision or neighborhood.  Such signs shall not

exceed six (6) feet in height, shall not be located closer than twenty-five

(25) feet to any street right-of-way line and shall not obstruct visibility

at street corners.

B.	The following signs are permitted in Commercial and Industrial districts.  No

sign shall be located closer than twenty-five (25) feet to any parcel line of a

parcel located in any Residential district.

1.	One exterior wall sign on each street frontage may be erected upon the

building.  The sign shall not project more than one (1) foot from the wall

on which it is affixed.  It shall not extend beyond the width or height of

the wall on which it is affixed.  It shall not exceed fifty (50) square feet

in area.

2.	One free standing sign on each frontage which advertises a business or

service conducted on the premises provided such sign conforms to the

following requirements:

a.	No freestanding sign shall exceed fifty (50) square feet in area per

side.

b.	No sign shall exceed ten (10) feet in height.

c.	No sign shall be located closer than ten (10) feet from a street

right-of-way.

d.	Sign shall conform to a monument style as depicted in figure 							401.2 B.2.d (1) below.

Sec 401.3	Temporary Signs.

A temporary sign is defined as any sign displayed for not more than 30 days.

A.	Temporary signs general standards.

1.	No temporary sign shall be mounted, attached, affixed or otherwise

secured by any permanent means to any building, permanent sign, other

structure or improvement, or to the ground upon which it is erected.

2.	No temporary sign shall be mounted, attached, affixed, installed or

otherwise secured so as to protrude above the roof of a structure.

3.	No temporary sign that is six (6) square feet or less in size shall be

illuminated by anything other than non-reflected daylight.

4.	Temporary signs shall be set back as follows:

a.	A minimum of ten (10) feet from the edge of street pavement or

a street right-of-way, whichever is the greater setback;

b.	A minimum of twenty (20) feet from all parcel lines adjacent to

a residential zoning district;

c.	A minimum of five (5) feet from all other parcel lines; and

d.	A minimum of five (5) feet from any other temporary sign.

B.	Temporary signs in a Residential Zoning District.

1.	The maximum height of all temporary signs in a Residential Zoning

District shall be four (4) feet.

2.	A maximum of two (2) temporary signs are permitted on each property

with a maximum sign area of six (6) square feet per sign.

3.	Said sign(s) of 401.3B.1 and 401.3B.2 shall not require a zoning certificate or fee.

4.	Temporary signs greater than six (6) square feet may be permitted during

the construction of a development under the following provisions:

a.	The owner of the property where the sign will be located applies

for, and receives a Zoning Certificate and pays the fee for a

Temporary Sign as set forth in the Zoning Fee Schedule approved

by the Township Fiscal Officer for such sign;

b.	There shall be a limit of one (1) sign per development and such

sign shall not exceed thirty-two (32) square feet per side with a

maximum of two (2) sides;

c.	The sign is maintained in good condition and appearance as

determined by the Zoning Inspector on inspection;

d.	The temporary sign may be posted during construction and/or

development of the subject property without a specified time

limit.  The sign shall be removed within fourteen (14) days of

completion of construction of the development.

C.	Temporary signs in a Commercial or Industrial District.

1.	The maximum height of all temporary signs shall be eight (8) feet unless

they are attached to a building as a wall sign, in which case they shall not

be attached in a manner as to exceed the height of the structure.

2.	A maximum of two (2) temporary signs are permitted on each property

with a maximum sign area of six (6) square feet per sign.

3.	Said signs of 401.3C.1 and 401.3C.2 shall do not require a Zoning Certificate or fee.

4.	Temporary signs greater than six (6) square feet in area may be permitted under the following provisions:

a.	The owner of the property where the sign will be located applies

for, and receives a Zoning Certificate and pays the fee for a

Temporary Sign as set forth in the Zoning Fee Schedule approved

by the Township Fiscal Officer for such sign;

b.	There shall be a limit of one (1) sign per premises and such sign

shall not exceed thirty-two (32) square feet per side with a

maximum of two (2) sides;

c.	The time limit for the Zoning Certificate shall comply with the

following:

(1)	A temporary sign under this section may be posted during

construction and/or development of the subject property

without a specified time limit.  The sign shall be removed

within fourteen (14) days of completion of the

construction or development.

(2)	The sign is maintained in good condition and appearance

as determined by the Zoning Inspector on inspection;

(3)	New businesses shall be permitted a promotional display

period of up to thirty (30) days, during which a temporary

sign may be displayed.  This display period permitted for

new businesses shall not run concurrently with display

periods otherwise permitted in this section, but signage

is subject to the same limitations of this section.

Sec 401.4	Maintenance

All on-site advertising signs as herein permitted shall be constructed and maintained

and illuminated in a safe manner, shall comply with all applicable codes, and shall be

kept in good repair.

A.	The Zoning Inspector shall have the authority to order the repair or removal of

any sign or sign structure that defective, damaged, or substantially deteriorated.

B.	The Zoning Inspector may order any sign to be painted or refurbished at least

once a year, if needed, to keep the sign in a safe, presentable, and good

structural condition.

C.	Maintenance may include upgrading of parts or materials only if such upgrades

do not take the sign out of conformance with this Resolution or, in the case of a

legal non-conforming sign as described in Sec 401.5, if such upgrades do not

increase the degree of non-conformity.

D.	Should any sign be or become unsafe or be in danger of falling, the owner or

tenant or lessee shall, upon receipt of written notice from the Zoning Inspector

as described in Sec 1002.2 Inspection, proceed at once to correct the unsafe

condition and/or remove the sign in question.

E.	Signs shall not be constructed, maintained and/or illuminated in such a manner

as to create or allow the obstruction of vision of drivers, pedestrians, or the

general public, or create a fire or safety hazard.

Sec 401.5	Non-Conforming Signs

The following constitutes legal Nonconformity of signs:

A.	All signs legally existing before the effective date of this resolution, even though such signs may not conform with the provisions of this resolution, must be

maintained, to include safety and repairs.

B.	Such signs shall be issued a Zoning Certificate of Non-Conformance by the

Zoning Inspector.

C.	Portable signs shall not be designated a legal non-conforming sign and shall be

removed within one hundred twenty (120) days of the effective date of this

Resolution.  Portable signs altered to be made non-portable shall still be

considered portable for the purposes of this resolution.

Sec 401.6	Loss of Legal Non-Conforming Status

A legal non-conforming sign loses the legal non-conforming status designation if:

A.	The sign is relocated.

B.	The sign is replaced.

C. 	The structure or size of the sign is altered in any way except towards compliance

changeable marquees, or face and copy changes.

D.	The sign is part of an establishment that discontinues its operation for a 						period of 2 or more years.

Sec 401.7	Maintenance and Repair of Non-Conforming Signs

A.	Legal non-conforming signs as determined under Sec 401.5 of this Resolution,

are subject to all the requirements of this Resolution in regard to safety,

maintenance and repairs.  However, if the sign suffers damage to an extent

greater than 51% of the estimated replacement value, the sign shall be replaced

with a sign that complies with this Article of this Resolution.

B.	The Zoning Inspector shall be authorized to make a determination of whether

a legally non-conforming sign is properly maintained and does not endanger the

public or constitute a nuisance as described in Sec 203.3 of this resolution.

Sec 401.8	Removal

Any sign that no longer advertises a bona fide business shall be removed or taken down

by the owner of, agent or person having beneficial use of, the building or structure.

Written notification shall be provided by the Zoning Inspector in compliance with Sec

1002.2 and the owner, agent or person having beneficial use of the building or structure

will have 30 days from the date of written notice to remove the sign in compliance with

Sec 1002.3 of this resolution.  Any refusal to remove the sign or comply with the written

order will result in further action in compliance with Sec 1002.3 and Sec 1002.4 of this

Resolution.

Sec 401.9	Fees

Fees set for signs in this Article of this Resolution can be found in the current Zoning Fee

Schedule, and can be requested from the Zoning Inspector or Fiscal Officer.

Article V

Sec. 501	Parking Spaces, Parking Layout and Loading Requirements

Sec. 501.1	Purpose

This Article is intended to regulate on-and off-street parking requirements for new 				construction, major alterations of existing structures, and to preserve the amount of off-				street parking already provided for use on the effective date of this Resolution.

A.	To implement comprehensive parking management policies throughout the 					Township, and to achieve a balance of transportation alternatives by including 					potential mass transit and walking in parking calculations.

B.	To acknowledge and protect historic and existing uses which do not meet the 					Township’s parking standards.

C.	To identify areas within the Township that requires more specific parking 						requirements.

D.	To reduce the creation of new impervious surfaces through lower required 					parking ratios, establishment of parking limits and shared parking facilities.

E.	To reduce traffic congestion and hazards.

F.	To provide accessible, attractive, well-maintained and screened off-street 					parking facilities.

G.	To provide aesthetically pleasing parking facilities in proportion to individual land 				use needs.

H.	To provide a proactive approach to large events which may require additional 					parking for short periods of time.

I.	To enable further study of unusual or unique uses in order to determine 						reasonable parking standards.

J.	To ensure the maneuverability of emergency vehicles.

Sec. 501.2	Applicability

A.	Off-street parking in conjunction with all land and building uses established after 					the enactment of the Resolution in this Article shall be provided for to the 					satisfaction of the Zoning Inspector after the issuance of a certificate of 						occupancy.

B.	Off-street parking may be either provided by an individual action parking 						program or carried out through public action by a special assessment district or 					otherwise.

C.	This Article shall be in addition to the provisions of the State of Ohio Traffic 					Manual.  This Article shall be construed in a manner that the provisions herein 					are separate and independent of the State of Ohio Traffic Manual and in addition 				thereto and where there is a conflict between the provisions of the State of Ohio 				Traffic Manual and this Article, the regulations of this Article are controlling.

Sec. 501.3	Exemptions

Provisions for the number of off-street parking spaces specified by this Article shall be 				required as a condition of issuance of a certificate of occupancy for any structure which 				is constructed, erected, relocated or expanded within Lawrence Township, except as 				follows:

A.	Change of Use or Expansion of Use

1.	Whenever a use requiring off-street parking is increased in floor area by 						more than fifty percent of the existing building, the minimum number of 						additional parking spaces shall be based on actual square footage of the 						increase.  Special exceptions may be granted by the Board of Zoning 						Appeals through a variance hearing upon finding that a specified lesser 						number of parking spaces will adequately serve the reasonably 							anticipated increase in parking needs.

2.	Within the C-1, C-2, C-3 and I-1 Districts, whenever a change in is use 						occurs within a structure existing at the time of this amendment that 						would increase the average daily number of trips by more than fifty 						percent (regardless of a change in floor areas), the applicant must

provide additional parking spaces or demonstrate that a sufficient number of credits are available within three hundred feet of the use.

B.	Existing Parking Lots

1.	Any area once designated as required off-street parking shall not be 						changed to any other use unless and until equal parking spaces which 						conform to the requirements of this Article are provided elsewhere 						within three hundred feet of subject property or as otherwise approved 						by the Zoning Inspector.

2.	Off-street parking existing at the effective date of the Resolution in this 						Article in connection with the operation of an existing building or use 						shall not be reduced to an amount less than hereinafter required for a 						similar new building or use.

Sec. 501.4	General Requirements

A.	Off-street parking for other than residential use shall be either on the same lot 					or within three hundred feet of the building it is intended to serve, except that in 				those cases where Township parking has been developed for a special area, this 					distance may be extended by the Zoning Inspector.  This distance may also be 					extended if the Zoning Inspector finds that the pedestrian route of travel is well 					lit and sufficiently protected from inclement weather; provided also, that ADA 					parking is provided in close proximity to the use, as per ADA standards.

B.	Residential off-street parking space shall consist of a parking strip, driveway, 					garage, parking lift, or a combination thereof and shall be located on the lot they 				are intended to serve.

C.	Two or more buildings or uses may collectively provide the required off-street 					parking, in which case the required number of parking spaces shall not be less 					than the sum of the requirements for the several individual uses computed 					separately.  Legally binding documents shall be approved by the Township 					Attorney and filed with the Township Clerk and recorded at the expense of the 					applicant in cases of mutual use of parking area.

D.	The required off-street parking shall be for occupants, employees, visitors and 					patrons and shall be limited in use to parking unless other uses are shown on an 					approved site plan.

E.	So long as operating hours do not materially overlap, the Zoning Inspector may 					grant an exception to the separate computation of individual uses in mutual use 					parking areas or the use of parking spaces.

F.	The Zoning Inspector, in consultation with the Zoning Commission, may 						determine the number of parking spaces required for uses not included in this 					Article.  For any use other than single-family residential development, the Zoning 				Inspector may require that the applicant provide a professional assessment of 					anticipated parking requirements and peak hours of operation.

G.	Facilities that use benches, pews, or booths, etc., shall provide one parking space 				for each three feet of seating area.

Sec. 501.5	Stacking Spaces

A.	All businesses which maintain drive-in facilities which are intended to serve 					customers who remain in their motor vehicles during business transactions shall 					be required to design their parking facilities to accommodate stacking space as 					provided in Table 1: Vehicle Stacking Requirements.

Table 1 Vehicle Stacking Requirements

| Type of Operation | Minimum Number of Vehicles |
| --- | --- |
| Financial institution, with drive-up tellers | Four vehicles per window/kiosk |
| Financial institution, with drive-up ATM (if separate from drive up tellers) | Four vehicles per window/kiosk |
| Car wash, self-service | Two vehicles per bay at entrance One vehicle per bay at exit |
| Car wash, automatic | Six vehicles per bay at entrance One vehicle per bay at exit |
| Restaurant, fast food and/or coffee shops with indoor seating | Six, including three vehicles behind each menu board and three vehicles behind first window, or a combination of the two |
| Restaurant, fast food and/or coffee shops with no indoor seating | Eight vehicles – one vehicle at window, seven in stacking lane |
| Photo processing/pharmacy prescription pick-up window | Four vehicles per window – one vehicle at window, three in line |
| Dry cleaning | Two vehicles per window |
| Retail gasoline/stations | Two vehicles per gasoline pump |
| Wholesale gasoline/stations | Four vehicles per gasoline pump |

B.	Minimum Dimensions of Stacking Space.  A stacking space is twelve feet wide by 					twenty feet long.  The minimum inside turning radius of stacking lanes is 						seventeen feet, six inches.

C.	Stacking lanes shall not interfere with parking spaces, parking aisles, loading 					spaces, internal site circulation, or points of ingress and egress for vehicles or 					pedestrians, and shall not stack into any right-of-way.  To meet this requirement, 				the Zoning Inspector, or their designee may require that one or more signs be 					posted restricting vehicle stacking beyond a certain point.

Sec. 501.6	Off-Street Parking Spaces Required

A.	The minimum number of off-street parking spaces to be provided shall be 					determined in accordance with Table 2.  Where parking spaces are based on the 					square footage of a building, the applicant shall provide the Township with a 					calculation based on gross floor area, as defined by this title.  The type and 					number of ADA-compliant parking spaces shall be subject to applicable state law, 				and each ADA parking space shall be considered as one parking space for the 					purposes of calculating required parking.

B.	Off-street parking facilities shall not occupy any part of any required front or side 				yard in C-1 and R-Districts.  In all C-Districts and in all I-Districts open off-street 					parking facilities may be located in the required front yard provided that at least 					a ten-foot-wide landscaped strip is located between the parking area and the 					street right-of-way line.  In all districts, open off-street parking facilities may 					occupy the required rear yard providing that a five-foot landscaped strip 						separates parking from all property lines abutting the rear yard.

Table 2 Off-Street Parking Requirements

| Use | Minimum Parking Requirements | Minimum Permissible Parking Spaces | Also See Section: |
| --- | --- | --- | --- |
| PERSONAL TRANSPORTATION |  |  |  |
| Automobile Sales |  | 4 | 501.4 |
| Automotive Recycling, Junkyards, Impound Yards, or similar operation for storage of vehicles | One space for each employee |  | 501.7 |
| Automobile Service Establishments | Three spaces for each employee | 4 |  |
| Bicycle or motorcycle rental and or sales | Two- and one-half spaces for each employee |  | 501.7 |
| Park and Ride | 501.4 |  |  |
| Transportation Center (includes bus, rail, and taxi stations) | 501.4 |  |  |
| ENTERTAINMENT AND DINING |  |  |  |
| Dance, Exhibition Hall, or Place of Assembly | One space per every two seats |  | 501.4 501.7 |
| Drive-in Restaurant | One space per each one hundred square feet |  | 501.7 |
| Restaurant, tavern, or any other establishment for the sale and consumption of food, alcoholic beverage, or refreshment | One space per every two seats |  | 501.4 501.7 |
| Restaurant, tavern, or any other establishment which includes sale and consumption of food combined with dance facilities | One space per every two seats |  | 501.4 501.7 |
| Theatre (indoor) | One space per every two seats |  | 501.7 |
| Outdoor event space (concerts, outdoor theater, etc.) |  |  | 501.4 |
| RESIDENTIAL USE |  |  |  |
| Accessory dwelling units | Two spaces per unit |  |  |
| Complete life care retirement community | One space for each five beds |  | 501.7 |
| Cooperative or Group Homes | One space per bed |  | 501.7 |
| Duplex | Two spaces per unit |  |  |
| Dwelling, Single Family | Two spaces per unit |  |  |
| Dwelling, Multi-Family | Two spaces per unit |  |  |
| Dwelling, Multi-Family in PUD District | Two spaces per unit, plus one guest space for every four units |  |  |
| Rooming House or Lodge House | One space per room |  |  |
| Mobile/Manufactured Home Park | Two spaces per unit, plus one guest space for every four units |  |  |
| Cottage Housing | Two spaces per unit |  |  |
| RECREATIONAL AND CULTURAL USE |  |  |  |
| Batting cages, racquetball and tennis courts, driving ranges, miniature golf, shooting ranges | One space per each batting cage, driving range tee, miniature golf hole, shooting range station, and two spaces for each racquetball or tennis court |  | 501.7 |
| Bowling Alley | Five spaces per each alley |  | 501.7 |
| Dance or Video game Establishment (not including restaurants/taverns with dance facilities) | One space per each seventy-five feet |  |  |
| Billiard Parlor | Four spaces per one hundred square feet |  | 501.7 |
| Fitness Center | One space per each one hundred square feet |  | 501.7 |
| Skating Rink | One space for each one hundred square feet |  | 501.7 |
| Campground | Two spaces for each camping space, plus two spaces for caretaker |  | 501.7 |
| Golf Course | 501.4 |  |  |
| Public and Private Parks including Trails | 501.4 |  |  |
| Community Clubhouse | 501.4 |  |  |
| Playfields | 501.4 |  |  |
| Outdoor event space (concerts, outdoor theatre, etc.) | 501.4 |  |  |
| Waterfront Park | 501.4 |  |  |
| Stadium Sports Area, auditorium, or other place of assembly with fixed seats | One space for each two- and one-half seats |  | 501.7 |
| Public Pool | 501.4 |  | 501.7 |
| Senior Center Activity center | One space per each four hundred square feet |  | 501.7 |
| Motion Picture Center | One space per each two seats |  | 501.7 |
| Library or Museum | One space per each two hundred square feet |  | 501.7 |
| Church, Mortuary, or Funeral Home | One space per each three seats in the chapel or nave |  | 501.7 |
| Clinic | One- and one-half spaces per each examination room |  | 501.7 |
| Convalescent Center | One space for each four residents |  | 501.7 |
| Hospital, sanitarium or home for the aged | One space for each two beds |  | 501.7 |
| Mental Health Facility | One space per each five beds |  | 501.7 |
| Substance Abuse Treatment Facility | One space per each three hundred square feet | 4 | 501.7 |
| Veterinary Office | Five spaces per each veterinarian |  | 501.7 |
| SERVICES USES, LODGING |  |  |  |
| Bed and Breakfast | One space per guest room, plus two spaces for residents |  |  |
| Boarding House | One space per guest room, plus two spaces for residents |  |  |
| Hotel | One space per unit |  | 501.7 |
| Motel | One space per unit plus two spaces for residence |  | 501.7 |
| RETAIL USES, COMMERCIAL |  |  |  |
| Retail, unless otherwise specified | One space per one hundred fifty square feet |  | 501.7 |
| Farm Equipment Sales & Service | Three spaces per employee | 4 | 501.7 |
| Grocery Store or Shopping Center less than one thousand six hundred square feet of retail space, three thousand two hundred square feet total space (neighborhood grocery store) | Eight spaces |  | 501.5 501.7 |
| Plant Nursery | One space per each one hundred fifty square feet of indoor space, plus two spaces for each acre of outdoor space. Maximum of 20 required | 8 | 501.7 |
| Retail Service Shops | One space per every one hundred fifty square feet |  | 501.7 |
| Shopping Center between three thousand two hundred and five thousand square feet | One space per each one hundred fifty square feet |  | 501.7 |
| Supermarket or Shopping Center over five thousand square feet | One space per each one hundred fifty square feet |  | 501.7 |
| Wholesale Store | One space per each one hundred fifty square feet |  | 501.7 |
| Barber/Beauty Salon | Two spaces per station | 4 | 501.7 |
| EDUCATION & CHILD CARE |  |  |  |
| All educational facilities must provide adequate parking spaces to accommodate temporary parking onsite during peak hours |  |  |  |
| Day Care or Day Nursery (including family day care) |  |  |  |
| Nursery School | One space per every five students, or fraction |  | 501.7 |
| Kindergarten through Middle School | One space per every five students, or fraction |  | 501.7 |
| High School | One space per ten students (auditoriums and stadiums counted separately) |  | 501.4 501.7 |
| Institution of Higher Education | One space per six students (auditoriums and stadiums counted separately) |  | 501.4 501.7 |
|  | 501.4 |  |  |
| STORAGE USES |  |  |  |
| Kennel |  |  |  |
| Moving and Storage Company | One space per each twelve animal enclosures; One space per each two hundred fifty square feet of retail sales area; Two spaces per each dwelling unit |  | 501.7 |
| Mini-Storage facility | One space per each three hundred square feet of office | 4 | 501.7 |
| Warehouse | One space for each two thousand square feet less than twenty thousand total square feet; One space for each two thousand five hundred square feet for twenty thousand one to ninety nine thousand nine hundred ninety nine square feet; One space for each three thousand square feet for greater than one hundred thousand square feet | Ten spaces for twenty thousand one to ninety nine thousand nine hundred ninety nine square feet; forty spaces for greater than one hundred thousand square feet | 501.7 |
| ESSENTIAL PUBLIC SERVICES |  |  |  |
| Correction Facility | 501.4 |  | 501.7 |
| MANUFACTURING USES |  |  |  |
| Food Processing | 501.7 |  |  |
| Junkyard | One space for each one thousand square feet of office space | 4 | 501.7 |
| Laboratory – Research & Development (For offices, see Professional Offices) |  |  | 501.7 |
| Manufacturing Use | 501.7 |  |  |
| OFFICE USE |  |  |  |
| Bank and Financial Institutions | Two spaces for each employee or station |  | 501.5 501.7 |
| Business Services | One space for each one hundred square feet | 4 | 501.7 |
| Professional Offices | One space for each one hundred square feet |  | 501.7 |
| OTHER USES |  |  |  |
| Convention Center, including Banquet Facilities and/or Meeting Hall | 501.4 |  |  |
| Home occupation | 501.4 |  |  |
| Service Station | 501.4 |  |  |
| Towing Service |  |  | 501.4 501.7 |

Sec. 501.7	Employee Parking

Any employer shall provide one parking spot for each employee, or if more than one

shift shall provide one parking spot for each employee for a two-shift operation, or two 		parking spots for each three employees for three shift operations.  The number of 		parking spaces will be determined by the total of the two largest shifts.

Sec. 501.8	ADA Accessibility

Parking within Lawrence Township shall conform to the requirements of the ANSI 17, as 				required by law.

Sec. 501.9	Parking Lot Layout, Construction and Maintenance

Off-street parking lot layout, construction and maintenance requirements shall be as 				follows:

A.	The party developing an off-street parking area shall submit to the Zoning 					Inspector for approval a plan of the parking area showing adjacent streets, 					proposed circulation of traffic, proposed location, type, and size of individual 					parking spaces, proposed drainage, lighting and landscaping, fencing and 						screening planting.  The plan shall also include a table listing the number and 					type of parking spaces.

B.	Off-street parking area lot dimensions and layout shall be guided by the design 					standards approved by the Institute of Traffic Engineers insofar as not otherwise 					prescribed by this title.

C.	Ingress and egress to the parking lot or area shall be provided at locations 					approved by the Tuscarawas County Engineer or his designee and the Ohio 					Department of Transportation.  In no case shall improved roadways be used for 					maneuvering into or out of the parking spaces.  Alleyways may be used for 					maneuvering into or out of parking spaces.

D.	Construction.  All required off-street parking areas shall be graded and paved to 					a standard comparable to the street which serves the parking area as

determined by the Zoning Inspector and shall be developed and completed to the required standard before a certificate of occupancy for the building is issued.  Pervious pavement is encouraged.  All traffic-control devices such as parking strips, designated care stalls, directional arrows or signs, rails, curbs and other developments shall be installed and completed as shown on the approved plans.  Hard surfaced parking areas shall use paint or similar devices to delineate car stalls and direction of traffic.  Where pedestrian walks are used in parking lots for the use of foot traffic only, they shall be raised six inches above the lot surface and shall be a minimum of four feet wide.

E.	Parking lot landscaping shall be as per the landscaping Section 203.6.

F.	Any parking spaces abutting a landscaped area on the drive or passenger side of 					the vehicle shall provide an additional eighteen inches above the minimum space 				width requirement to provide a place to step other than in the landscaped area.  					The additional width shall be separated from the adjacent parking space by a 					parking space division stripe.

G.	Driveway Dimensions. When off-street parking is provided in the rear of a 					building and a driveway or land alongside the building provides access to the

rear parking area, such driveway shall have a minimum width of twelve feet and 		provisions for a four-foot minimum width sidewalk accessing the rear of the 		building either abutting or in close proximity to the driveway, raised six inches 		above the driveway surface, shall be included.

H.	When due regard to public safety so indicates, the Zoning Inspector may require 					that up to fifty percent of the required off-street parking area must be clearly 					marked as customer parking with signs as necessary directing the public to the 					premises from adjacent street or streets.  During business hours, customer 					vehicles only may occupy parking spaces so marked.

I.	With regard to parking lot design, the applicant shall eliminate vehicle travel 					directly in front of the primary entrance wherever possible, so as to establish a 					“vehicle free zone” between the parking lot and the facility.  This zone shall be 					protected with bollards or other devices which may be removed in emergencies.

J.	Parking garages either above or below ground shall observe a minimum height 					clearance of seven feet, six inches.

K.	Developments may provide parking for recreational vehicles or other oversized 					vehicles in appropriately-sized vehicle spaces.  Parking lot camping is prohibited 					except in designated spaces, and no vehicles may be permitted to stay more

than seventy-two hours.  It is the responsibility of the developments’

management to maintain a log of such vehicles, including license plate numbers

and arrival and departure times.

L.	Parking areas shall be of usable shape, improved with bituminous, concrete or 					equivalent surfacing and so graded and drained as to dispose of all surface water 				accumulation within the area.  All lighting used to illuminate such parking areas 					shall be so arranged as to direct the light away from adjoining premises or 					streets and no open light sources such as the stringing of light bulbs shall be 					permitted.  Wheel guards, including bumper guards as may be necessary, shall

be provided in connection with any off-street parking area of five cars or more,

and shall be constructed so as to confine the storm water surface drainage to the 				premises; to contain the cars on sloping surfaces, and to prevent bumper 						overhang.

Sec. 501.10	Loading Space Requirements

An off-street loading space having access to a public thoroughfare shall be required 				adjacent to each building hereafter erected or enlarged to the extent specified in

Section 501.1 if the use of such building entails deliveries or shipments to or from it.

Such loading space shall be adequate size to accommodate the number and size of

vehicles simultaneously loaded or unloaded in connection with the business conducted

in such building.  No part of the truck or van shall extend into the public right-of-way

when using the loading space.

Sec. 501.11	Parking Space Standards

Automobile Parking

A.	Each parking space shall be ten feet wide and twenty feet long.

B.	Parking space depth may be reduced when vehicles overhang a walkway 						under the following conditions:

1.	Wheel stops or curbs are installed.

2.	The remaining walkway provides a minimum of sixty inches of 							unimpeded passageway for pedestrians.

3.	The amount of space depth reduction is limited to a maximum of 							one- and one-half feet for standing parking spaces.

Sec. 501.12	Parking Lot Standards

Parking lot design standards shall be subject to the most current version of the

Lawrence Township Section 203.6 design standards.

Sec. 501.13	Conflict with Other Provisions

Where there is a conflict between the regulations of this Article and the regulations of 				any other section of this code, the regulations of this Article shall prevail; provided, 				however, that the regulations of other sections shall prevail in the following cases:

A.	Where the regulations of any other section are more restrictive.

B.	Where a Planned Unit Development (PUD) or other similar private development 					has been established in accordance with procedures set forth in Section 302; 					provided, that any such Planned Unit Development regulations shall include 					comprehensive parking regulations encompassing the entire Planned Unit 					Development area, and there is no risk to life, safety, and general welfare.

Sec. 501.14	Nonconforming Parking Continuance

Any existing parking lot that was allowed use prior to the effective date of the ordinance 			codified in this Article may be continued in its current configuration and be maintained 				after said effective date and shall become a legal nonconforming use provided:

A.	The configuration of any parking stall or parked vehicle shall not be changed in 					any manner that increases its noncompliance with the provisions of the 						ordinance codified in this Article.

B.	Normal maintenance and upkeep of nonconforming parking lots, including 					restriping, repaving, and the addition or replacement of vehicle stops; provided, 					that the total number of parking spaces is not changed, and installation or 					replacement of lighting may be allowed; provided, that the work does not 					increase the nonconformity of the parking lot.

C.	Activities that are prohibited by this Article shall have ninety days after the 					effective date of the Resolution in this Article or after annexation of property

into the Lawrence Township to be brought into conformity with the Article.

D.	When a parking lot is altered, reducing or increasing the number or size of 					parking spaces, access points, etc., it ceases to be a legal nonconforming lot and 					must be brought into conformance with the provisions of this Article.  Any act 					which enlarges the size of the parking lots shall be subject to the provisions of 					this Article.

E.	When a business utilizing a parking space or parking lot changes or expands to a 					point that additional parking is required, it shall be the responsibility of the 					business to create additional parking according to the requirements of this 					Article.

Sec. 501.15	Regulations for Fire Lanes

A.	Fire lanes shall be twenty feet wide.

B.	Traffic zone yellow paint shall be used for all fire lane markings.  A solid traffic 					zone yellow line six inches wide running the entire distance of the fire lane shall 					be painted where the curb or sidewalk meets the parking lot.  This line shall be 					painted on the parking lot.  (See Figures 1 and 2, Appendix A)

C.	All lettering on the parking lot pavement shall be twelve-inch block letters.

D.	Fire lane signage shall conform to Ohio Department of Transportation sign 					regulations and standards for parking control zones as contained in the Ohio 					Manual of Uniform Traffic Control Devices.  Signs shall be posted a minimum of 					seventy-five feet and must be posted at the end of each fire lane.

1.	Loading docks or loading zones shall be posted as tow away zones.  Signs 						shall read “Fire Lane Loading Zone”.

2.	All hydrants and sprinkler standpipe connections shall have a fifteen-foot 						fire lane around them.  (See Figures 3 and 4)

3.	Fire hydrant signage shall be posted on a post and placed in the ground 						behind the fire hydrant.  Where protective posts are installed, the fire

lane sign can be placed on these posts.

Sec. 501.16	Parking and Loading Requirements for Use not Specified

Where the off-street parking and loading requirements for a use are not specifically 				defined herein, the parking and loading facilities for such use shall be developed so as to 			the sufficient to meet all the parking and loading needs of the proposed use as 					determined by the Board of Zoning Appeals in consultation with the Zoning Commission.  			No parking, loading or servicing shall be done on the right-of-way of any publicly 					dedicated thoroughfare.

Sec. 501.17	Severability

If any section, sentence, clause or phrase of this Article shall be held to be invalid or 				unconstitutional by a court of competent jurisdiction, such invalidity or 						unconstitutionality shall not affect the validity or constitutionality of any other section, 				sentence, clause or phrase of this Article.

| TABLE 601 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CONDITIONAL USES |  |  |  |  |  |  |  |  |  |  |
| SPECIFIC USES | CONDITIONAL USES BY DISTRICT |  |  |  |  |  |  |  |  | SPECIFIC CRITERIA |
|  |  |  |  |  |  |  |  |  |  | REFER TO |
|  | R-1 | R-2 | R-3 | R-4 | C-1 | C-2 | C-3 | I-1 | MHP | Sec. 601.7 |
| RESIDENTIAL USES |  |  |  |  |  |  |  |  |  |  |
| Bed & Breakfast | C |  |  | C |  |  |  |  |  | F, H, M, P2, S, X |
| Day Care Type "A" | C | C | C | C |  |  |  |  | C | E, G, H, I, N, P1, R, S, Y |
| Granny Cottage | C | C | C | C |  |  |  |  |  | SEC 601.13E Sec. 601.7 DD APPLIES |
| Adult Group Home, Large | C | C | C | C |  |  |  |  |  | SEC 601.13F APPLIES |
| Restaurant, as accessory to |  |  |  | C |  |  |  |  |  | B, F, G, H, P2, S, GG |
| agricultural use in R-4 Only |  |  |  |  |  |  |  |  |  |  |
| Agriculture | C | C | C |  |  |  |  |  |  | A, J, L, N, Q |
| Home Occupation | C | C | C | C |  |  |  |  |  | Compliance with SEC 213 |
| Small Wind Farm |  |  |  | C |  |  |  |  |  | SEC 601.15 APPLIES |
| COMMERCIAL USES |  |  |  |  |  |  |  |  |  |  |
| Office (excluding medical) |  |  | C |  |  |  |  |  |  | F, G, H, N, O1, 03, P2, S, Y |
| Restaurant or Bar as accessory |  |  |  |  | C | C | C |  |  | O2, P1, Q |
| use to Office use |  |  |  |  |  |  |  |  |  |  |
| INDUSTRIAL USES |  |  |  |  |  |  |  |  |  |  |
| Mini-Storage Facility |  |  |  |  |  | C | C |  |  | E, G, O2, 03, P2, U |
| Telecommunications Tower | C | C | C |  |  |  |  |  |  | D, N, O2, O3, U, EE & (ORC 519.211) |
| Vehicle Storage Yards |  |  |  |  |  |  |  | C |  | O2, O3, P2, Y, AA1 |
| Warehouse as Accessory Use |  |  |  |  |  |  | C |  |  | E, H, P1, Q, S |
| Extraction |  |  |  |  |  |  |  | C |  | SEC 601.13A APPLIES |
| Recycling / Junkyards |  |  |  |  |  |  |  | C |  | SEC 601.13B APPLIES |
| Transfer Station |  |  |  |  |  |  |  | C |  | SEC 601.13C APPLIES |
| Sexually Oriented Businesses/Uses |  |  |  |  |  |  |  | C |  | H, L ,P4, P5, P6, S, U, V, EE, HH, II, JJ,KK,LL LAWRENCE TOWNSHIP RESOLUTION 19-09 |
| Gas/Oil Wells | C |  |  | C | C | C | C | C |  | SEC 601.13D APPLIES |
| Dangerous/Exotic and Wild Animals |  |  |  |  |  |  |  | C |  | SEC 601.14 APPLIES |

| TABLE 601 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CONDITIONAL USES |  |  |  |  |  |  |  |  |  |  |
| SPECIFIC USES | CONDITIONAL USES BY DISTRICT |  |  |  |  |  |  |  |  | SPECIFIC CRITERIA |
|  |  |  |  |  |  |  |  |  |  | REFER TO |
|  | R-1 | R-2 | R-3 | R-4 | C-1 | C-2 | C-3 | I-1 | MHP | Sec. 601.7 |
| INSTITUTIONAL USES |  |  |  |  |  |  |  |  |  |  |
| Correctional Facility, Halfway House |  |  |  |  |  |  |  | C |  | B, G, I, O2, O3, P2, S, T, U, V, Y |
| Child Day Care Center | C | C | C |  | C | C | C |  |  | H, I, L, O1, O3, P2, R, S |
| PUBLIC SERVICE USES |  |  |  |  |  |  |  |  |  |  |
| Park & Ride Facility |  |  |  |  |  | C | C | C |  | E, G, H, I, L, O2, O3, P2, S, Y |
| Churches | C | C | C | C | C |  |  |  |  | E, G, H, I, O2, O3, P3, S |
| RECREATIONAL, CULTURAL AND ENTERTAINMENT USES |  |  |  |  |  |  |  |  |  |  |
| Campground, Public or Private |  |  |  |  |  |  | C | C |  | B, I, L, P3, U, Z, FF |
| CULTURAL |  |  |  |  |  |  |  |  |  |  |
| Botanical Garden | C | C | C | C |  |  |  |  |  | E, F, H, O1, O3, P3, S |
| Museum |  |  |  |  | C | C | C |  |  | D, G, H, L, O1, O3, P3, S, Y |
| Outdoor Drama Theatres (Excluding Drive-Ins) |  |  |  |  |  |  | C | C |  | A, D, H, L, O1, O3, P3, S, V |
| Zoo |  |  |  |  |  |  | C |  |  | B, G, H, L, O1, O3, P3, S, U, V, W, Y, Z, AA2 |
| RECREATIONAL FACILITY, Public or Private |  |  |  |  |  |  |  |  |  |  |
| Club, Private |  |  |  |  |  | C |  |  | C | F, G, H, N, O2, O3, P3, S, Y, BB |
| Golf Course |  |  |  | C | C | C | C |  |  | B, E, F, H, J, P3, S, X |
| Recreation Center, Internal | C | C | C | C |  |  |  |  | C | H, L, N, O1, O3, P2, S, BB |
| Summer Camp |  |  |  | C |  |  |  |  |  | B, I, L, P3, S, U, Z, FF |
| Swim/Tennis Facility Type B Type C Type D | C | C | C | C | C C | C | C | C |  | E, H, I, L, N, O2, O3, P3, R, S, U, W, Y H,L,N,01,03,P2,S,BB, |

ARTICLE VI

Sec. 601	Conditional Use Zoning Certificates

Conditional uses are those uses having some special uniqueness, which requires a careful 			review of their location, design, configuration and special impact to determine against fixed 			standards, the desirability of permitting their establishment on a specific site.  They are 				considered provisional in nature and established at the discretion of the Board of Zoning 			Appeals.

Sec. 601.1	Purpose

The purpose of this Article is to enable new uses which may be appropriate in a 					particular district after consideration of the public benefit against the local impact, the 				reduction of any adverse impact through special site planning, development techniques, 			and contributions to the public improvements and rights-of-ways.

Sec. 601.2	Authority

The Board of Zoning Appeals may, in accordance with the procedures and standards set 				out in this Article, and other regulations applicable to the district in which the subject 				property is located, approve by resolution those uses listed as conditional uses in the 				Table 601, or in any other part of this Resolution.

Sec. 601.3	Effect of Conditional Use Listing

A.	Compliance with Zoning Requirements

The listing of a conditional use in the Table 601 or in any other part of this 					Resolution does not constitute an assurance or presumption that such 						conditional use will be approved except as provided in Section 601.3B.  Rather 					each proposed conditional use shall be evaluated by the Board of Zoning Appeals 				on an individual basis.  This shall be done in relation to its compliance with the 					standards and conditions set forth in this Article and with the district in which it 					is located, in order to determine whether approval of the conditional use is 					appropriate at the particular location and in the particular manner proposed.

B.	Compliance with Other Requirements

Nothing in this Article shall be deemed to prohibit or unreasonably limit any use 					guaranteed by state or federal law.  However, reasonable conditions may be 					required by Board of Zoning Appeals in order to satisfy the purposes as outlined 					in Section 601.1

Sec. 601.4	Review Procedure for Conditional Use Applications

A. 	Applicant

An application for a Conditional Use Zoning Certificate may be filed with the 					Board of Zoning Appeals by the owner, lessee, or other person having a legal or 					equitable interest in the subject property.

B.	Application

An Applicant for a conditional use shall file a plan and an application on forms provided by the Board of Zoning Appeals. The plan for the use and development of the tract shall demonstrate that the general and specific criteria have been met.

C.	Scheduling of Hearing and Transmittal of Application

Within five days after filing of an application determined to be complete 						pursuant to Article VI, Section. 601.5, the Board of Zoning Appeals Chairman 					shall:

1.	Set a date for a public hearing not less than thirty, nor more than 							sixty-two days after filing of a complete application.

2.	Transmit a copy of the application and plan, together with the date of the 					scheduled hearing, to the Lawrence Township Zoning Commission for 						staff review and recommendation.

D.	Report

Within twenty days after the filing of a complete application, the Zoning 						Inspector shall prepare and transmit to the Board of Zoning Appeals a written 					report incorporating or summarizing comments of other departments, agencies, 					and officials as may be appropriate to the scope of development.  A 						recommendation shall be included, setting forth whether the application for a 					conditional use should be approved, approved with modifications, or denied and 				reasons for such recommendation.

E.	Notice and Hearing

The Board of Zoning Appeals (BZA) shall hold a public hearing in accordance with the adopted Procedures, Rules and Regulations of the BZA.  Notice of the hearing shall be given at least ten days before the hearing by notice in writing sent by the BZA administrator to:  the applicant, The Board of Trustees, and the owners within two hundred feet in all directions.  Notice shall also be published in one or more newspapers of general circulation in the County.  Upon the hearing, any party may appear in person or by an attorney.

F.	Decision

Within twenty-one days after the close of the public hearing, the BZA shall

1.	Approve the conditional use

2.	Approve the conditional use subject to further specified approvals or 						modifications necessary to achieve full compliance with all standards

3.	Disapprove the condition use

G.	Notification of Decision

Following the decision of the Board of Zoning Appeals, the BZA shall return to 					the applicant one copy of the resolution and submitted plans permanently 					marked to show either:

1.	Approval of the conditional use

2.	Approval of the conditional use subject to either specified approvals or 						modifications necessary to achieve full compliance with all standards

3.	Disapproval of the conditional use.

Sec. 601.5	Coordination Review and Approval of Applications

When an application for a Conditional Use Zoning Certificate is filed, applications shall 				be filed with the Board of Zoning Appeals for all other required approvals, including 				variances.

A.	Notice of Applications for Additional Approvals

Whenever an applicant files an application for other approvals pursuant to this 					Section, all required notices shall include reference to the request for any and all 					additional approvals.

B.	Procedures and Action by Board of Zoning Appeals

1.	Whenever an applicant files applications for other approvals pursuant to 						this Section, the Board of Zoning Appeals shall review and process all 						such applications at the same public hearing.  In reviewing such 							combined applications the Board of Zoning Appeals shall, except as 						hereinafter provided with respect to limitations on the time for taking 						action, comply with all of the provisions of this Resolution applicable to 						each of the applicants.

2.	The Board of Zoning Appeals shall act on any such combined application 						within the longest time period applicable to anyone of the individual 						applications for with such further time as may be consented to by the 						applicant.  The Board of Zoning Appeals shall issue notices and 							certificates of such action in accordance with the provisions of this 						Resolution applicable to the various applications involved.

Sec. 601.6	General Considerations for Conditional Uses

In approving an application for a Conditional Use Zoning Certificate, the Board of Zoning 				Appeals shall make a finding that the proposed conditional use is appropriate in the 				location proposed.  The finding shall be based upon the general considerations set forth 				below, as well as the designated specific criteria for specific uses contained in Section 				601.7

A. 	Spirit and Intent

The proposed use and development shall comply with the spirit and intention of 					the Zoning Resolution and with district purposes.  The proposed use and 						development will be in harmony with the general and specific purposes for 					which this Resolution was enacted and for which the regulations of the district in 				question were established and complies with all additional standards imposed 					on it by the particular provisions of this Resolution authorizing such use.

B.	No Adverse Effect

The proposed use and development shall not have an adverse effect upon 					adjacent property, or the public health, safety, morals, and general welfare.

C.	Protection of Public Interest

The proposed use and development should respect, to the greatest extent 					practicable, the natural, scenic, and historic features of significant public 						interest.

D.	Consistent with Adopted Plans

The proposed use and development shall, as applicable, be consistent with 					objectives, policies and plans related to land use adopted by the Board of 						Trustees.

Sec. 601.7	Specific Criteria Pertaining to Conditional Uses

In addition to the general considerations contained in Section 601.6, each conditional 				use is subject to one or more specific criteria as identified in the Table 601.  The 					following list contains all the specific criteria with each preceded by a letter for 					reference in Table 601.

A.	Site shall contain a minimum of five acres and all buildings shall not occupy 					over ten percent of the total area of the site.

B.	Site shall contain a minimum of twenty acres.

C.	Site shall contain a minimum of thirty acres.

D.	Any structure (except fences), parking area, or storage area shall be set back a 					least one hundred feet from every property line.

E.	Setbacks from any adjacent residential property line shall be a minimum of fifty 					feet for all buildings and twenty-five feet for all parking areas.

F.	Parking shall not be permitted in the area defined as the front yard setback of 					the existing zone district.

G.	Use shall have direct access to a collector or arterial street.

H.	The vehicular use area shall be located and designed so as to minimize impact on 				the neighborhood.

I.	Any use for which drop-off or pick-up of children, residents, visitors, products, or 				emergency vehicles is a common occurrence shall provide for the separation of 					incoming and outgoing vehicles so as not to impede other traffic.

J.	No building shall be located within the floodplain.

K.	The facility shall be reasonable accessible, either by its location or transportation 				provided by the applicant to medical recreational and retail services as well as to 				employment opportunities that may be required by its residents.

L.	Measures shall be taken to minimize the impact of potential nuisances such as 					noise, odor, vibration, and dust on adjacent properties.

M.	No exterior alterations of an existing structure shall be made that depart from 					the residential character of the building.  All new structures shall be compatible 					in residential design with the surrounding neighborhood.  However, any 						improvement required by code or necessitated by licensing requirements shall 					not be deemed incompatible.

N.	The architectural design and site layout of the structure and the location, nature, 				and height of any walls, screens, and fences are to be compatible with adjoining 					land uses and the residential character of the neighborhood.

O.	Landscaping shall be installed in accordance with one of the following buffers:

1.	Boundary buffer of ten feet with canopy trees and shrubs at ten	 						per one hundred linear feet.

2.	Boundary buffer of twenty feet with canopy trees and shrubs at eight per 						one hundred linear feet.

3.	Street side buffer of ten feet with canopy trees and shrubs at ten 							 per one hundred linear feet.

P.	Signage shall be regulated as follows:

1.	No signs shall be erected for purposes of identification except as 							permitted street address sign.

2.	One sign permitted at a maximum of twelve square feet and non-							internally illuminated.

3.	One sign permitted at a maximum of thirty-two square feet.

4.	Subject to sign standards in Article IV.

5.	No display of promotional materials shall be visible to the public from 						pedestrian sidewalks or walkways.

6.	No signage shall contain any lewd or offensive language, or any sort of 						sexually explicit graphics.

Q.	The conditional use shall be subordinate to the principal permitted use with 					regard to usage and character.

R.	Outdoor playgrounds, tot lots, exercise areas, and pools shall be fully enclosed 					by a fence.

S.	All exterior lighting shall be directed away from adjacent residential properties.

T.	Documentation indicating the need for a facility, the specific clientele it will 					serve and the location and type of similar facilities operated by the applicant 					shall be submitted as part of the application.

U.	Security measurers shall be submitted as part of the application.

V.	The applicant shall provide a plan indicating the manner in which the facility will 					maintain contact with neighborhood residents along with a structured procedure 				whereby their grievances may be filed and resolved.

W.	A refuse collection plan shall be submitted as part of the application.

X.	Meals shall be served only to guests or residents of the facility and not to the 					general public.

Y.	The intensity of the particular use shall be evaluated with regard to the location, 					size, and configuration of the tract.

Z.	An emergency response plan shall be submitted detailing safety measures and 					response procedures.

AA.	No structure, storage area, or vehicular use area shall be located closer than:

1.	One hundred feet to a residential use or district

2.	Two hundred feet to a residential use or district

BB.	The facility shall be centrally located and surrounded on all sides by the 						development the facility serves.

CC.	Coverage of the required rear yard by the Accessory Use shall not exceed ten 					 percent, and coverage of the entire lot by the Accessory Use and the Principal 					Use shall not exceed twenty percent.

DD.	The Accessory Structure shall contain a maximum of nine hundred square feet 					and not exceed twenty feet in height.

EE.	The terms of continuation of this use and those under which it shall eventually 					be removed or terminated are to be specified in the application and contained 					within the approved Resolution.

FF.	There shall be central management of the use to assure seasonal occupancy 					only.

GG.	Ownership and management of restaurant and operator of the agricultural use 					shall be related.  Commodities produced in the agricultural use shall be featured 					and served at the restaurant.

Sec 601.8	Accessory Uses to Conditional Uses

Any use or structure that is accessory to a conditional use shall be processed in the 				same manner as prescribed in this Article for conditional uses.  If an application for an 				accessory use is made concurrently with an application for the primary conditional use, 				they may be considered together as one application.  Whether processed in conjunction 			with a primary conditional use or as a later supplement to an existing primary 					conditional use, the accessory use shall meet the appropriate specific criteria listed in 				Section 601.6 as well as the general conditions contained in Section 601.7.

Sec. 601.9	Additional Regulations Pertaining to Conditional Uses

In addition to the general standards contained in Section 601.6, the specific criteria 				contained in Section 601.7, all conditional uses are subject to the following regulations:

A.	Additional Conditions

The Board of Zoning Appeals may impose additional conditions and limitations 					concerning use, construction, character, location, landscaping, screening, timing 					of implementation, and other matters relating to the purposes and objectives of 					this Resolution upon the premises benefited by a conditional use.  Such 						conditions are intended to prevent or minimize adverse effects upon other 					property and improvements in the vicinity of the subject property or upon public 				facilities and services or to assure compliance with general or specific standards.  				However, such conditions shall not be used as a device to authorize as a 						conditional use that which is intended to be temporary in nature.  All such 					conditions, including the designated specific criteria for a particular use, shall be 					expressly set forth in the resolution granting the Conditional Use Zoning 						Certificate.  Violation of any such condition, limitation, or specific criteria shall be 				violation of this Resolution and shall constitute grounds for revocation of the 					Conditional Use Zoning Certificate.

B.	Effect of Approval of Conditional Use

The approval of the proposed conditional use by the Board of Zoning Appeals 					shall not authorize the establishment or extension of any use nor the 						development, construction, reconstruction, alteration or moving of any building 					or structure, but shall merely authorize the preparation, filing and processing of 					applications for any permits or approvals that may be required by the 						regulations of the County, including but not limited to, a Conditional Use Zoning 					Certificate, a building permit, a certificate of occupancy and subdivision 						approval.

C.	Certification of Conditional Use Plan Compliance

Upon receipt from the applicant of an application for a Zoning Compliance Plan 					certification, the Administrative Official shall review the application to determine 				if it is complete pursuant to Section 601.3, including any additional conditions 					required in conjunction with the approval by the Board of Zoning Appeals.  					Within seven days of receipt of the completed application, the Administrative 					Official shall either:

1.	Certify that the Zoning Compliance Plan complies with the BZA approval 						or

2.	Refuse to certify the Zoning Compliance Plan for lack of compliance with 						the BZA approval.

D.	Affidavit of Compliance with Conditions

Whenever any proposed conditional use authorized pursuant to this Article is 					made subject to conditions or limitations to be met by the applicant, the 						applicant shall, upon meeting such conditions, file an affidavit with the Township 				Zoning Inspector so stating.  Such affidavit shall be accompanied by a 						nonrefundable fee as established by the Board of Trustees upon 							recommendation of the Administrative Official, to recover the Township’s actual 					direct cost of an inspection to verify that such conditions and limitations have 					been met.

E.	Limitations on Conditional Use Approval

The approval of a proposed conditional use by the Board of Zoning Appeals shall 					be deemed to authorize only the particular use for which it was issued, and such 					authorization shall automatically expire and cease to be of any force or effect if 					such use shall, for any reason, be discontinued for period of twenty-four 						consecutive months or more.

F.	Amendments to Conditional Uses

A conditional Use Zoning Certificate may be amended, varied, or altered only 					pursuant to the procedures and subject to the standards and limitations 						provided in this Article for its original approval.

Sec. 601.10	Period of Validity

Subject to an extension of time granted by the Board of Zoning Appeals, no Conditional 				Use Zoning Certificate shall be valid for a period longer than one year unless a building 				permit is issued.

Sec. 601.11	Appeal of Decisions

Any party aggrieved by the decision of the Board of Zoning Appeals may be appeal to 				the Court of Common Pleas of Tuscarawas County on the ground that such decision was 				unreasonable or unlawful.

Sec. 601.12	Additional Conditional Uses and Special Exception

In addition to permitting the conditional uses and special exceptions herein before 				specified, the Board shall have the power to permit the following conditional uses and 				special exceptions:

A.	Nonconforming uses.

1.	The substitution for a nonconforming use exists at the time of enactment 						of this Zoning Resolution, or another nonconforming use, if no structural 						alterations except those required by law or ordinance are made; 							provided, however, that in any “R” District, no change shall be authorized 					by the Board to any use which is not a permitted or conditional use in the 					“R” District, and in any “C” District, no change shall be authorized to any 						use which is not permitted or conditional use in any “C” District.

2.	The extension of a nonconforming building upon the lot occupied by such 					building or on an adjoining lot.

B.	Extension of use on border of districts.

The extension of a use or building into a more restricted district immediately 					adjacent thereto, but not more than twenty-five feet beyond the dividing line of 					the two districts, under such conditions as will safeguard development in the 					more restricted district.

C.	Temporary structures and uses.

The temporary use of a structure or premises in any district for a purpose or use 					that does not conform to the regulations prescribed elsewhere in this Zoning 					Resolution for the district in which it is located, provided that such use is of a 					temporary nature and does not involve the erection of a substantial structure.  A 				Zoning Certificate for such use shall be granted in the form of a temporary and 					revocable permit, for not more than a twelve-month period, subject to such 					conditions as will safeguard the public health, safety, convenience and general 					welfare.

Sec. 601.13	Specific Regulations Pertaining to Certain Conditionally Permitted Uses

A.	Extraction Regulations

1.	General

In recognition of the deposits of sand, gravel and other minerals in 						Lawrence Township, these Extraction Regulations are hereby established.  					As a result of the unique characteristics associated with extraction uses, 						such uses have been designated as conditionally permitted uses and, 						therefore, are subject to the requirements of this Resolution and review 						by the Board of Zoning Appeals through the issuance of a “Conditional 						Use Certificate”, as set forth in Article VI of this Resolution.

a	A conditional use permit for surface mining or topsoil removal shall not be required when a Zoning Certificate has been issued for construction of a structure or structures on a development area provided those plans specify the affected areas approved for said structure or structures, including parking, driveway or driveways and roadways in compliance with Sec. 203.6 of the Lawrence Township Zoning Regulations.

b.	A conditional use permit for surface mining or topsoil removal shall not be required when the removal of peat and minerals is from those areas that are needed to develop public or private roadways on a final subdivision plat and the plat has been approved by the Tuscarawas County Commissioners for development of residential housing, business, or industrial complex development

2.	Purpose

The purpose of these requirement is to ensure that the mineral 							resources of Lawrence Township and the lands that have significant 						gravel, sand or other mineral deposits for mining or extraction purposes 						are properly managed in a manner that complies with the regulations of 						this Resolution, would not constitute a hazard to the public health and 						safety, and would result in reclamation of the land in a suitable manner 						for other purposes.  Extraction uses shall be subject to the following 						requirements:

3.	Minimum Lot Area

Twenty-five acres

4.	Minimum Lot Width at Street

Five hundred feet

5.	Site Plan Information

The following information shall be provided on a site plan and/or 							submitted as part of an application for a conditional use certificate:

a.	General Information

1.	Name and address of the person, firm or corporation who 								or which will be conducting the actual operation

2.	Name and address of the owner of the property

3.	Name and address of the owner of the surface rights of 								the property

4.	Location of any processing plant and related buildings and 								structures including storage facilities, whether on-site or 								off-site

5.	A list of the types of minerals or resources to be removed 								or to be brought to the site

6.	Proposed method of removal, backfilling, and whether 								blasting or other use of explosives will be required

7.	General description of equipment to be used for 									excavating, processing, and/or transporting evacuated 								mineral resources

8.	The estimate time to complete total operations

9.	The total area (expressed in acres) proposed to be 									evacuated or mined

10.	Location and height of proposed fencing

11.	A plan for the control of erosion and sedimentation during 								and after development, construction, extraction, and 								other use of the site.  The plan shall be submitted with 								evidence of review and approval of the Tuscarawas Soil 								and Water Conservation Service.

b.	Schedule

1.	A proposed schedule for the initiation and progression of 								mining on the site

2.	Estimate the location, description, and size of the area to 								be disturbed in the first year, and subsequent years

c.	Maps

1.	Vicinity maps, drawn at a scale of one inch equal to 								one thousand feet illustrating:

a.	The site in relation to surrounding existing and 									proposed land uses

b.	The proposed primary means of ingress to and 									egress from the extraction operation

c.	Surrounding zoning districts

d.	An aerial photograph showing the site location 									and immediate vicinity.  The aerial photograph 									shall not have been taken more than one year 									prior to the submission of the site plan.

2.	Contour maps at a scale of at least one inch equal two 								hundred feet illustrating:

a.	Existing contours at intervals of five feet or less

b.	Any existing structures

c.	Any public utilities or easement on the property

3.	Surface water map – flow rates and direction

d.	Studies

The following studies shall be conducted by a qualified 								professional engineer registered in the State of Ohio, expert in the 						applicable field, or as otherwise may be required, and as 								approved by the Board of Zoning Appeals, at the applicant’s cost.

1.	A study of excavations and the existing water table shall 								address the following:

a.	Anticipated depth of excavation

b.	Effect on water table

c.	Data on wells in the area

d.	Existing ground water conditions at the site, 									demonstrate that the source of any public or 									private water supply shall not be adversely affected 								due to the lowering of the water table or 										contamination of the supply

2.	A hydrologic study shall address and/or include the 									following:

a.	Determine and describe surface water in the 									vicinity

b.	Information relating to flow rates and direction

3.	Soils analysis

A soils analysis of the property shall be required.  The 								analysis shall be prepared by an independent testing 								laboratory, prior to excavation and at the end of the 								excavation, to verify topsoil depth and to ensure no 								contaminants were brought to the site during operation.

e.	Reuse Plan

A plan for the reuse and restoration of the site as specified in this 							Section.  The reuse plan for the site shall be reviewed for 								compliance at the end of each twelve-month period by the Board 							of Zoning Appeals or their designated representative.  A reuse 							plan, drawn to a scale of one-inch equals fifty feet placed on a 							standard sheet and shall contain, at a minimum, the following 							information and meet the following requirements:

1.	A proposed grading plan showing future contours at 								intervals of five feet or less, including the surrounding 								area within five hundred feet of the property boundary 								line

2.	The depth and type of the proposed cover shall be 									sufficient to meet the proposed intended use

3.	The backfill material used in restoration shall be identified

4.	The angle of slope of all earthen banks shall not exceed 								one foot vertical to three feet horizontal.  In areas 									where at the commencement of evacuation a greater 								angle existed, the angle of slope shall be no greater than 								that which existed at the commencement of excavation

5.	The angle of slope of all banks consisting of rock and the 								required cover

6.	The location of fences or effective planting in those 								locations where the Board of Zoning Appeals determines 								that such angles of slope are not physically or 									economically feasible to reduce

7.	The number of trees and shrubs, and the type of ground 								cover to be provided.  The type and number per acre of 								trees, shrubs, ground cover or legume to plant shall be 								determined in consultation with a Tuscarawas County 								Agricultural Extension agent.   A statement that vegetation 							shall be restored by the spreading of sufficient soil and by 								appropriate seeding of grasses or planting of shrubs and 								trees in all parts of said reclamation area where the same 								is not submerged under water

8.	A description of the land use activities and any structures 								proposed to be located on the site upon completion of 								mining or extraction operations

9.	A description and location of the street, drainage courses, 								public utilities or easements, and water and sanitary sewer 							facilities required to serve the uses, as determined in 								consultation with the County Engineer

6.	Setbacks

The following minimum setback requirements shall apply:

a.	Mineral extraction operations shall not be conducted less than 							two hundred feet from any residential district or use.  All 								structures, machinery, processing, and stockpiling of material 							shall be located a minimum of three hundred feet from any 							residential district or use.

b.	All other mineral extraction operations, including structures, 							machinery, processing, and stockpiling of material shall not be 							conducted closer than six hundred feet from any residential 							district or use.

7.	Site, Sound and Dust Control Berms

a.	All sand and gravel excavating activities shall be located behind 							berms for sound, sight and dust control varying in height from ten 						 feet to twenty feet or as approved by the Ohio Department of 							Natural Resources (ODNR) or governmental agency having control 						over Special Flood Hazard Area activities.

b.	Berms will be sloped at one to four on the front side, and one to 							two on in the inside, and vegetation with native grasses during 							the first growing season following their construction.  Vegetative 							cover shall be mowed on a regular basis so as to control the 							growth of noxious or other harmful weeds.

c.	Silt fencing shall be used to prevent soils from leaving the site.  							Rip rap may be used to stabilize berms where necessary.  Gaps in 							the berms as well as the mine entrance shall be gated to control 							public ingress and egress from the property.

d.	Initial earthwork operations will construct the berms for sound, 							sight and set control.  Overburden and graded topsoil will be used 						to form the berming system.  A minimum one hundred foot and 							maximum of two-hundred-foot setback, measured from the foot 							of the inside berm, will be maintained for adjacent property lines 							and road right-a-ways to allow for adequate sloping and 								revegetation during the reclamation process.

e.	Berms will be maintained at a fifty-foot setback around the 							perimeter of wetlands adjacent to the active mining area.  Silt 							fences will be constructed at the base of the berms (on the 							wetland side) to further contain any sediment.  Berms will be 							monitored by-monthly for the duration of the operation in 							accordance with storm water permitting.  At the completion of 							operations, the overburden and topsoil will be replaced on site for 						restoration purposes.

8.	Safety and Signage

For the protection of public safety, the site shall be enclosed with a six-						foot fence with a locking access gate.  Such fences shall be placed no 						closer than fifty feet to the top or bottom of any slope.  The Board of 						Zoning Appeals shall approve all fence materials to ensure that such 						fences adequately preserve public safety.  The owner or operator shall 						place signs around the site advising the public of the operation contained 						therein and advising that no trespassing is permitted.  There shall be no 						more than one directional sign adjacent to the primary means of 							ingress and egress from the proposed operation.

9.	Reuse Plan

Reclamation and rehabilitation of mining areas in accordance with the 						Reuse Plan shall be accomplished as soon as practicable following the 						mining or excavation of an area.  Where possible, such rehabilitation and 						reclamation shall be accomplished concurrently with the mining or 						excavation operations.  All equipment, buildings and structures for which 						no future use is contemplated shall be removed from the mining area 						within six months after termination of mining or excavation activity.  						Substantial completion of all other reclamation and rehabilitation actions 						shall be affected no later than one year after termination of mining or 						excavation activity (inactivity for a twelve-month consecutive period shall 					constitute termination of mining activity).

10.	Access and Circulation

The following access and circulation requirement shall apply:

a.	Per ORC 303.141(B)(1), specific roads to be used as the primary 							means of ingress to and egress from the proposed activity shall be 						determined by recommendation of the County Engineer, and 							approved by the Board of County Commissioners.  The Board of 							Township Trustees may send written notice of appeal of the 							decision of the County Commissioners within ten days after 							the County Commissioners’ action.

b.	All rights-of-way used for the purpose of ingress and egress shall 							be kept dust free by hard topping with cement, bituminous 							substance or chemical treatment.  Trucks shall be cleaned on site 							before using public rights-of-way.  Sweeping equipment must be 							used and kept on site.

c.	In no case shall overburden deposit interfere with safe roadway 							sight distance, as determined by the Zoning Inspector.

d.	No more than two access drives shall be permitted directly 							from any public right-of-way.  If the site fronts on two or more 							roads, the driveways shall be located as far from the road 								intersections as possible.

e.	Truck parking areas, maneuvering lanes and access ways to public							rights-of-way shall be designed to cause no interference with the 							safe and convenient movement of automobile and pedestrian 							traffic on and adjacent to the site.

f.	At no time shall an emergency vehicle be restricted from access to 						a public right-of-way for any reason including but not limited to:  							on road parking, deliveries, business operations, moving of 							equipment, etc.

g.	As provided for under ORC 1514.024, the applicant shall deposit a 							bond with the Trustees for the repair and maintenance of 								Township rights-of-way used for hauling equipment or materials 							brought to or taken from the site.

h.	No material shall be hauled to the site, except for materials as 							may be necessary to adequately pave roads for ingress and egress 						to the site.

11.	Noise

Noise shall be kept to a minimum and shall in no case exceed the 							maximum sound levels set forth in Section 211.5 of this Resolution.  						Mufflers must be maintained on all equipment.  The use of compression 						release engine brakes shall be prohibited on or near the site.

12.	Hours of Operation

Operations, including blasting operations and the start-up and/or warm-						up of any extraction, processing, or related equipment, shall not 							commence before 8:00 a.m. local time, and shall terminate no later than 						5:30 p.m. local time.  Operations on Sundays and recognized Federal 						holidays shall be prohibited.

13.	Erosion Control and Backfilling

The following erosion control and backfilling requirements shall apply:

a.	Silt and sediment shall not be permitted to leave the site.  The 							smallest practicable area of land shall be exposed at any one-time 							during operations and for the shortest practicable period of time.  						Sediment basins, debris basins, desilting basins, silt traps, and 							other methods shall be installed to control sediment from runoff.

b.	Wherever feasible natural vegetation shall be retained and 							protected.

c.	The slopes of the banks of the excavation shall in no event exceed 							seven feet horizontal and one foot vertical.

d.	All excavations shall either be graded and/or backfilled with 							materials approved by the OEPA or appropriate authority to 							ensure that the excavated area will not collect and retain stagnant 						water.

e.	Spoil banks shall be graded to a level suiting the existing terrain 							and planted with trees, shrubs, legumes, or grasses where 								revegetation is possible.

f.	No topsoil shall be removed from the site, and all topsoil shall be 							redistributed properly upon completion of the extraction 								activities, or phase thereof.

g.	Increased runoff from operations shall be controlled by 								appropriate storm water management facilities, both during and 							after operations.

14.	Lake Creation

Creation of a lake or pond shall only be permitted where the applicant 						can demonstrate using engineering and hydrological studies that the 						water can be maintained in a non-polluted condition, and that the 						applicant meets any requirements of the State.  If such mining area is left 						as a permanent lake, all sides must be sloped to no greater than one 						foot vertical to three feet horizontal extending to a six-foot depth,	or a 						stable bank be established.

15.	Explosives

The use of explosives, if applicable shall conform to the following 							standards:

a.	The operation of stationary, mobile equipment, and explosives, if 							applicable, shall not cause vibrations in excess of that which is 							permitted by Section 211.6 of this Resolution.  In the event that 							State requirements set forth in ORC 1514.12(B), as now existing or 						hereafter amended, are stricter than Township requirements, the 							more stringent standard shall apply.

b.	Blasting shall be done in accordance with and conform to the 							requirements of ORC 1514.12 and shall be carried out by persons 							certified to be knowledgeable and competent in the sizing and 							placing of the explosive to be used for blasting.

c.	The operator shall maintain complete records on a daily basis of 							all blasting operations including records of the time, the date, the 							location, and complete description of weather conditions such as 							time, temperature, humidity, and wind velocity relating to each 							such blast.  Such records shall be made available to the Zoning 							Inspector upon request.

d.	All properties within two thousand five hundred feet of a blast 							shall be notified twenty-four hours in advance of a planned blast 							when the blast is within five hundred feet of a high-pressure gas 							pipeline.  The operator shall provide a blasting schedule to the 							Township and Fire Department having jurisdiction over the site.

16.	Water

The following requirements related to preservation of surface and 						ground water quality shall apply:

a.	In order to protect water wells and the water supply of the 							Township and County, the pumping or drainage of water from 							such mining operations is absolutely prohibited.

b.	The quality of water leaving the site of an extraction activity shall 							not be degraded due to the activities.

c.	During mining and reclamation, drainage shall be controlled so as 							to prevent flooding, landslides, and flood hazards to adjoining 							lands resulting from the mining.  Ponds shall be left in such 							condition as to avoid their constituting a hazard to adjoining 							lands.

d.	Ground water testing to the requirements and specifications of 							the Tuscarawas County General Health District, the OEPA, or 							appropriate authority shall be performed annually after the date 							of approval of the conditional use certificate, if granted, or as 							required by the Tuscarawas County General Health District, OEPA, 						or appropriate authority.  Permanent groundwater testing wells 							shall be established by the applicant to a depth determined by the 						Tuscarawas County General Health District, the OEPA, or 								appropriate authority.

e.	During mining and reclamation, contamination of ground water 							supplies shall be prevented.  Materials contaminating to ground 							water shall not be used for filling or grading at any time.

f.	If the mining operation causes the water table to drop and 							prevents an adequate supply of water to surrounding properties 							or causes existing wells to become contaminated, the owner 							and/or the operator of the land and/or operation shall be 								responsible for the cost of drilling new wells or for providing a 							source of water to the properties deprived of water.

17.	State and Federal Requirements

Proof of all required outside agency approvals or permits, including but 						not limited to those referenced below, shall be provided to the Township 						prior to the start of work on the site.

a.	In addition to the requirements of this Resolution, all mining 							operations shall be subject to and in conformance with ORC 							Chapter 1514 and ORC 1563.11.  The applicant shall provide a 							complete copy of the surface or in-stream mining permit 								application required by ORC 1514.02(A) and any amendments 							thereto.

b.	A copy of the Annual Report required by the ORC 1514.03 shall be 						provided to the Zoning Inspector on an annual basis within thirty							days of its filing with the Chief of the Ohio Division of Mineral 							Resources.

c.	A copy of the bond or acceptable financial guarantee as required 							by the ORC 1514.04 shall be provided to the Zoning Inspector, 							within thirty days of its filing with the Chief of the Ohio 								Department of Mineral Resources.

18.	Enforcement

Per ORC 1514.023, the requirements of this Section shall be subject to 						the enforcement provisions of Section 1002 of this Resolution.  In the 						event that it is established as a matter of fact that there has been a 						failure to adequately comply with the provisions of this Section, said 						operators shall take immediate steps to provide full compliance 							herewith.  In the event the operator does not comply, the Ohio Division 						of Mineral Resources shall be notified of the violation and 								nonconformance with the Township Zoning Resolution.

B.	Recycling Facilities and Junkyards

1.	Recycling facilities and junkyards, if approved by the Board of Zoning 						Appeals, shall be located no closer than two hundred feet to any R- 						District and/or public street right-of-way line, and shall otherwise have 						front, side and rear setbacks of at least one-hundred-foot-wide strip in 						the two-hundred-foot setback shall be planted for camouflaging purposes 				according to the following specifications:

2.	The one-hundred-foot-wide strip shall be planted with evergreens or 						other plants of similar screening value.

3.	Said trees shall be planted on a staggered pattern with no more than ten 						 feet between trees.

4.	Trees should be planted that are at the optimum transplanting size and 						age while still being as large as possible.

5.	A minimum of ten acres shall be required for this use.

6.	Recycling Facilities and junkyards are prohibited in all Residential Districts.

7.	There shall be no burning of refuse, garbage or other waste materials.

8.	The area of use shall be completely enclosed by a six-foot-high fence 						with openings no greater than ten percent of the fence area.

9.	Every portion of the property used for buildings shall be located not 						closer than one hundred feet to any “R” District; at least fifty feet 							(nearest the residential zone) of the one hundred feet shall be 							landscaped and orderly fashioned and a solid board fence, masonry wall, 						or other type fence approved by the Board shall be constructed along the 					interior line of the required landscaped area to a height of six feet 						effectively screening truck loading, unloading, and maneuvering activities 					from the view of any abutting “R” zone.

10.	Access for motor freight vehicles shall be by way of streets of adequate 						width, construction, and existing or planned function according to the 						Land Use and Thoroughfare Plan of current adoption.  In additional to 						adequate area within the site for docking, manipulation and maneuvering 					of motor freight vehicles, a reservoir of parking area for motor freight 						vehicles waiting to be loaded or unloaded, shall be provided at the rate of 					one parking space sufficient to park a motor freight vehicle for every 						four loading docks.

11.	The site shall be designed in such a manner as to permit forward 							movement of all vehicles both upon entering and upon leaving the site.  						The number, location and width of entrances to and exits from the site 						shall be determined by the Board.  The Board may obtain expert opinion 						on the specific site proposal.

C.	Transportation Terminal, Truck Terminal and Waste Transfer Stations

1.	There shall be no burning of refuse, garbage or other waste materials.

2.	The area of use shall be completely enclosed by a six-foot-high fence 						with openings no greater than ten percent of the fence area.

3.	Every portion of the property used for buildings shall be located not 						closer than one hundred feet to any “R” District; at least fifty feet 							(nearest the residential zone) of the one hundred feet shall be 							landscaped and orderly fashioned and a solid board fence, masonry wall, 						or other type fence approved by the Board shall be constructed along the 					interior line of the required landscaped area to a height of six feet 						effectively screening truck loading, unloading, and maneuvering activities 					from the view of any abutting “R” zone.

4.	Access for motor freight vehicles shall be by way of streets of adequate 						width, construction, and existing or planned function according to the 						Tuscarawas County Access Management Regulations.

5.	In addition to adequate area within the site for docking, manipulation 						and maneuvering of motor freight vehicles, a reservoir of parking area for 					motor freight vehicles waiting to be loaded or unloaded, shall be 							provided at the rate of one parking space sufficient to park a motor 						freight vehicle for every four loading docks.

6.	The site shall be designed in such a manner as to permit forward 							movement of all vehicles both upon entering and upon leaving the site.  						The number, location and width of entrances to and exits from the site 						shall be determined by the Board.  The Board may obtain expert opinion 						on the specific site proposal.

7.	Transfer terminals and Transfer Stations are prohibited in all residential districts.

D.	Gas and Oil Regulations

1.	Purpose

Lawrence Township is known to be the location of gas and oil reserves.  						This section sets forth requirements to ensure that any operation 							incidental to exploration, production or storage of gas and oil takes place 						in a manner not endangering public health, safety and welfare and 						consistent with zoning and land use regulations in the Township.  In the 						event of conflict between these regulations and Ohio Revised Code 						Chapter 1509, the Ohio Administrative Code, and applicable Federal 						Regulations, the most stringent regulations shall apply.

2.	Definitions

The following definitions apply only to Section 601.13D of the Lawrence 						Township Zoning Resolution:

a.	Well – any borehole, whether drilled or bored, within the 								Township, for production, extraction or injection of any gas or 							liquid mineral, excluding potable water to be used as such, but 							including natural or artificial brines and oil field waters.

b.	Oil – crude petroleum oil and all other hydrocarbons, regardless

of gravity, that are produced in liquid form by ordinary production methods, but do not include hydrocarbons that were originally in a gaseous phase in the reservoir.

c.	Gas – all-natural gas and all other fluid hydrocarbons not defined 							above as oil, including condensate.

d.	Waste -

1.	Physical waste, as such term is generally understood in the 								oil and gas industry

2.	Inefficient storing of oil or gas

3.	Locating, drilling, equipping, operating or producing an oil 								or gas well in a manner that reduces or tends to reduce

the quality of oil or gas ultimately recoverable under

prudent and proper operation from the pool into which it

is drilled, or that causes or tends to cause unnecessary or excessive surface loss or destruction of oil or gas

4.	Other underground or surface waste in the production or 								storage of oil, gas or condensate, however caused

e.	Owner – the person who has the right to drill on a tract or drilling 							unit and to drill into the produce from a pool and to appropriate 							the oil or gas that he produces therefrom either for himself or for 							others

f.	Pool – an underground reservoir containing a common 								accumulation of oil or gas, or both, but does not include a gas 							storage reservoir.  Each zone of a geological structure that is 							completely separated from any other zone in the same structure 							may contain a separate pool.

g.	Drilling Unit – the minimum acreage on which one well may be 							drilled, but does not apply to a well for injecting gas into or 							removing gas from a gas storage reservoir.

h.	Tract – A single, individually taxed parcel of land appearing on the 							tax list.

i.	Brine – All saline geological formation water resulting, obtaining 							or produced in connection with the exploration, drilling, or 							production of oil and gas.

3.	Filing Requirements

Not less than seven days prior to any drilling operation, the driller shall 						file and deposit with the Zoning Inspector:

a.	There shall be four copies of the Draft Plan submitted to the 							Board of Zoning Appeals for approval, and all other pertinent 							information per Section 203.6, “Architectural and Landscaping 							Review”.  The ‘Draft Plan’ must also fully evaluate all of the 							potential impacts to water supplies, including from 									“hydrofracking”, including but not limited to water and chemical 							usage in ‘hydrofracking’ fluids; risks associated with casing and 							cementing well bores; all risks associated with ‘hydrofracking’ not 							just those occurring during pressurized fracturing operations; 							impacts on ground water and surface water consumption; pre- 							and post-monitoring of groundwater and drinking water for 							residents; risks associated with seasonal weather patterns; 							wastewater generation, treatment and disposal; land disturbance 							and the addition of impervious surfaces, and storm water runoff.  							The plat, drawn to normal engineering scale, showing the location 							of:

1.	Ingress and egress points

2.	The well

3.	All known wells within one thousand feet

4.	Storage tanks

5.	Separator units

6.	Power shut-offs

7.	Transmission lines

8.	Oil flow shut-offs

9.	Permanent and temporary pits

10.	Access roads

11.	All dikes and swales for erosion control and spill

prevention

b.	A list of emergency telephone numbers of all parties responsible 							for any work on the tract

c.	A copy of the state permit

d.	A copy of the state-approved brine and waste disposal plan.  The 							plan shall include a description of the method of disposal of brine, 						frac-water, sludge and any other oil field wastes, the name, 							address and telephone number of the person, corporation or firm 							other than the owner disposing of the waste, the location of the 							disposal site being used and the name, address and telephone 							number of the owner of the disposal site and proof of liability 							insurance carried by the person, corporation or firm disposing of 							the waste and a copy of registration certificate required of brine 							transporters by the State of Ohio, ODNR, Division of Oil and Gas.

e.	A copy of the Spill Prevention, Control, and Counter-measure Plan 						(SPCC) as required by Title 40 Code of Federal Regulations, Part 							112.

f.	A schedule of the proposed drilling operation

g.	Proof of liability insurance for all operations related to drilling, 							production, storage and transmission of all products, byproducts 							and wastes.

h.	A bond for overweight vehicles in the amount of five thousand 							dollars and in the form of a bank check or money order made 							payable to Lawrence Township to be held and disposed of as 							herein provided.

i.	A copy of certified test results from the Tuscarawas County Health 						Department of private water supplies that are located within one 							thousand feet radius of the drilling site.

4.	Minimum Distances

No gas well, oil well, storage tank or separator unit shall be placed within 						Three hundred feet of any building except storage sheds at the drilling 						site, or within five hundred feet of any known source of water.

5.	Placing of Equipment

No equipment shall be placed within one hundred feet of any property 						line unless both properties are part of the same drilling unit.  In no case 						shall any equipment be placed directly on the property line.

6.	Clearing

The immediate areas surrounding all permanent production facilities shall be kept mowed and cleared of combustible materials for a distance of fifteen feet in all directions and shall be enclosed by a chain link fence not less than six feet in height topped with a double outward strand of barbed wire.

7.	Shut-Off Valves

All shut-off valves shall be painted the same conspicuous color for ease of 					identification in emergencies.

8.	Burial of Lines

All gas and oil lines shall be buried at a minimum depth of thirty inches.  						The location of such lines shall be drawn on a map and a copy deposited 						with the Township Zoning Inspector and Township Fire Department.

9.	Flaring

All non-commercial shows of natural gas shall be flared through a flow 						line at least sixty feet long into a separate flare pit.  The flare pit shall 						contain a smudge pot which shall remain lit at all times.  The flow line 						shall be directed away from all inhabited structures.  No gas shall be 						flared into storage and/or brine pits or into any lined drilling pits.

10.	Access Roads

Prior to the beginning of drilling, all access roads shall be paved with slag, 					gravel, crushed stone or other suitable material and shall be a minimum 						of ten feet wide.  All access roads shall be clearly marked and shall 						have a paved turnaround of sufficient size for rescue vehicles used by the 					Township.  All access roads shall be kept in repair and maintained at all 						times and shall be kept plowed and free of snow to allow access by safety 					vehicles.  Where access roads cross natural or man-made drainage 						channels, culverts shall be installed.  Culverts shall be installed as per 						“Tuscarawas County Access Management Regulations”, and maintained 						from the road to the well site and to any storage facility from which a 						natural resource will be removed from the site.  Erosion rills, gullies, and 						tire ruts that are four inches deep or deeper shall be immediately 							repaired to a level surface, or as instructed by the township.  Roadside 						drainage, including culverts, shall be maintained, cleaned, and repaired 						when approximately one third of their capacity is lost or whenever 						instructed to do so by a representative of Lawrence Township or the 						Tuscarawas County Engineer’s office.

11.	Fence and Locks

All gates, storage tank manholes, discharge valves, fill valves, shut-off 						valves and fence gates shall be locked.  All locks at a given well shall 						utilize a master key.  A master key marked with the well number shall be 						provided to the Zoning Inspector, the Fire Chief and the Tuscarawas 						County Sheriff.  The owner shall provide a master key for each of its wells 						located in the Township.  All well facilities – service, storage, and 							otherwise – shall be enclosed by an eight-foot-high chain link fence, 						and shall conform to all applicable rules in Section 204.10 (Fencing).  All 						facilities must meet appeals board restrictions.

12.	Signs

Prior to the beginning of drilling, a permanent, weatherproof sign shall be 					posted and maintained at the site at all times.  Said sign shall use letters 						and numerals at least five inches in height and shall show:

a.	Access street name, number, or both

b.	Owner

c.	Lease name

d.	Well number

e.	Permit number

f.	All emergency telephone numbers as required by Subsection C 							and all local emergency numbers including, but not limited to, 							Bolivar Volunteer Fire Department, Tuscarawas County Sheriff, 							etc.

13.	Parking

All truck loading and parking areas shall be located outside of any road 						right-of-way.

14.	Storage Pits and Liners

All cuttings and fluids produced during drilling operation must be 							contained in storage pits.  Cuttings and fluid storage pits shall be either 						equipped with a baffle to deflect cuttings or constructed to submerge 						cuttings as they enter the pit in order to preserve the integrity of the pit 						liner during the entire drilling process. All gas and oil production storage 						and brine storage pits shall be surrounded by perimeter dikes to prevent 						contamination of surface or ground water.   All gas and oil production 						storage and brine storage pits shall be liquid tight and constructed and 						maintained so as to prevent escape of any produced brines or waters.  All 					storage pits shall have at least two layers of liner.  Such liners shall be 						reinforced, a minimum of six millimeters thick, single piece and defect 						free.  Each liner shall be separated by a minimum two-inch layer of 						clay.  The liners shall be laid in opposite directions to assure their 							integrity.  Dikes surrounding storage facilities shall have a capacity three 						times that of the storage vessel.

15.	Diversity Swales

In locations where dikes may be damaged by storm runoff, a diversity 						swale shall be constructed to prevent damage to the containment dikes.

16.	Subsurface Drainage

If, during construction of any temporary or permanent pit or containment 					dike, a subsurface drainage system is encountered, said subsurface 						drainage system shall be removed to a distance of twenty feet from the 						pit or containment dike and shall be plugged at that point.

17.	Storage Tanks

If a well is located on a steep slope or in a flood plain, storage tanks shall 						be used; no open storage pits shall be used.  All tanks shall be adequately 						and permanently anchored to resist slippage or flotation.  All tanks shall 						be liquid tight.  Storage tanks, both at and off the well site, shall be 						located on a prepared site and shall be diked or otherwise designed, 						constructed, and maintained so as to prevent any seepage or drainage 						beyond a five-foot distance from each tank.  Such seepage or drainage 						shall, under no circumstances, be allowed to enter upon adjacent 							property or any road drainage ditch, natural watercourse, or natural or 						man-made body of water.

18.	Pollution of Water and Disposal of Wastes

No person shall conduct any well drilling, production or transmission 						operation that contaminates or pollutes the land surface or any surface

or subsurface water.  No saltwater (brine), sludge, frac-water or any

other oil field wastes shall be deposited or discharged in the Township for

any purpose.  No person shall vary or change the waste disposal plan or 						method initially submitted without prior approval of the Zoning

Inspector.  The owner shall maintain a record at the drill site of the name, address and telephone number of the person, corporation or firm disposing of the waste, the location of the disposal site, and the method of disposal being used and the date, time and license plate number of the last vehicle to have left the drill site hauling waste.

19.	Plats

The owner and/or operator of all transmission lines shall provide the 						Zoning Inspector with a plat drawing to scale of all transmission lines 						within one thousand feet of the well.  All transmission lines buried or 						above ground, shall be marked with permanent markers.  All lines 							crossing public highways shall be marked with permanent markers at 						each side of the right-of-way.

20.	Damage to Township Roads

Prior to drilling, the Lawrence Township Road Supervisor and a 							representative of the owner shall inspect all township roads in the 						vicinity of the drilling site and over which the owner expects to move

equipment and/or vehicles.  Upon the completion of drilling, any and all

damage to township roads shall be assessed by the Road Supervisor and

a representative of the owner and the estimated cost of repairs, if any,

shall be deducted from bond deposited with the Zoning Inspector.  At the

end of each twenty-four-hour period during drilling operations, the driller

shall keep all township roads in the vicinity of the drilling site and over

which the driller expects to move equipment and/or vehicles, free from

accumulations of mud.  In the event that the amount of bond exceeds the

estimated cost of repairs required; the remainder of the bond shall be 		returned to the owner.

21.	Cleaning of Pits and Disposal of Liners

All gas and oil production liquid wastes, including but not limited to brine 						sludge, frac-water and/or all fluids contained in gas and oil production 						storage and brine storage pits and all pit liners shall be removed and 						disposed of as soon as practical after the completion of the drilling 						operation or within two weeks after completion of the drilling 							operation, which is sooner.  None such wastes shall be disposed of or 						deposited in Lawrence Township for any purpose.

22.	Lightning Rods

Each permanent production structure, including but not limited to 						separator units and storage tanks shall be equipped with property 						grounded lightning rods.

23.	Zoning Inspection

The Lawrence Township Zoning Inspector may inspect oil and gas wells 						and storage facilities at any time to ensure compliance with local zoning 						regulations.  The Zoning Inspector may order any storage tanks, or 						supports, guys, braces, anchors, or any other articles or furnishings, to be 						painted or refurbished at least once each year.  All storage tanks shall be 						maintained in a safe condition.

24.	Fire Inspection

The Chief Fire Prevention Officer of Lawrence Township may inspect oil 						and gas wells and storage facilities at any time to ensure compliance with 					local fire regulations.

25.	All applicable township zoning regulations must be followed, including 						but not limited to:

a.	Storm Water Management per Section 210

b.	Environment Performance Standards per Section 211

c.	Floodplain regulations per Section 311

E.	Granny Cottage (Accessory Dwelling Unit)

1.	Purpose

The purpose of these regulations is to provide flexible household living 						arrangements, expand affordable housing opportunities, and provide a 						variety of housing types to meet the needs of residents while maintaining 					the aesthetics and residential use, compatible with homes in the 							neighborhood.  The establishment of an accessory dwelling unit in 						existing single-family dwellings shall be allowed as specified in the 						Residential District sections of this Resolution, subject to specific 							development, design and owner-occupancy standards.

2.	Establishment

An accessory dwelling unit may be established by any one of the 							following methods:

a.	Alteration of interior space of an existing dwelling

b.	Conversion of an attic, basement, garage or other previously 							uninhabited portion of a dwelling

c.	Addition of a separate unit onto an existing dwelling

d.	Construction of a free-standing unit.

3.	Standards and Criteria

No accessory dwelling unit may be established unless it complies with the 					following standards:

a.	An accessory dwelling unit may only be constructed on a lot 							whose area is equal to or greater than the minimum lot area 							required in the zoning district.

b.	The accessory dwelling unit shall be a complete, separate 								housekeeping unit (including kitchen and bathroom facilities) that 							functions as a separate unit from the original unit.

c.	Only one accessory dwelling unit may be created within or added 							to an existing single-family residence.

d.	The accessory dwelling unit shall be clearly a subordinate part of 							the single-family dwelling.  It shall be no greater than nine

hundred square feet of floor area.

e.	The development standards, front yard, rear yard and side yard 							setbacks, and height limits for the attached accessory dwelling 							unit shall be the same as those required for the principal single-							family dwelling unit.

f.	The principal single-family dwelling unit and the accessory 								dwelling unit shall be under single ownership at all times.  Either 							the principal dwelling unit or the accessory dwelling unit shall be 							occupied by the property owner at all times.  A notarized affidavit 							certifying that the owner is and will remain an occupant of either 							the principal or the accessory dwelling unit shall be submitted as 							part of the application for a Conditional Use Zoning Certificate and 						said statement shall become a deed restriction for this use.  Proof 							of filing said deed restriction shall be provided to the Zoning 							Inspector after the BZA has approved the conditional use.

g.	The accessory dwelling unit shall be designed so that, to the 							degree reasonably feasible, the appearance of the building 							with the principal dwelling unit or use as a separate entrance.  							However, a separate entrance shall only be located on the rear or 							side of the principal dwelling.

h.	The accessory dwelling unit shall comply with applicable health 							standards and shall be approved by the Tuscarawas County Health 						Department.

i.	One additional off-street parking space shall be required for 							the accessory dwelling unit.  The additional space requirement 							may be met by using the garage, carport or driveway of the 							principal dwelling unit as long as the off-street parking 								requirements can be maintained for the principal dwelling in 							compliance with Article V.

F.	Adult Group Homes

1.	Adult group homes, small, shall be permitted in any Residential District.  					2.	Adult group homes, large, shall be conditionally permitted in an R-2 or R-

3 Residential district subject to compliance with the following conditions:

a.	An adult group home shall be located on a lot of at least one 							acre in area in an R-2, and R-3 Residential districts; and at 								least two acres when located in an R-4 Residential district.

b.	All activities, programs, and other events shall be directly related 							to the conditional use permit so granted, and shall be adequately 							and property supervised as to prevent any hazard and to assure 							against any disturbance or nuisance to surrounding properties, 							residents, or the community in general.

c.	No such group home shall be located within a one thousand feet 							radius of another residential facility for the developmentally 							disabled licensed under OAC Section 523.2-3.

d.	Signage for an adult group home shall conform to the								requirements of Article IV.

e.	Adult group homes shall maintain in all respects, the exterior 							appearance of a single-family home in the R-1 and R-4 Residential 							districts

f.	Adult group homes shall comply with all applicable licensing 							requirements, building code requirements, fire code

requirements and health requirements.

g.	If not specifically addressed in this Section, all buildings shall 							conform in all other ways to the requirements of the zoning 							district in which they are located, and be compatible with 								adjoining land uses and the residential character of the 								neighborhood.

h.	Parking spaces shall be provided in accordance with Article V of 							this Resolution.

i.	Parking areas and access drives shall be paved with an impervious 							material such as asphaltic concrete or asphalt.

j.	All refuse areas shall be enclosed.

k.	Safety Inspection:

1.	An adult group home shall comply with the requirements 								of the ICC, National Fire Prevention Code, and any 									amendments thereto and all other applicable safety 								codes.

2.	Inspection, for the purpose of fire safety, shall be 									conducted by the Bolivar Volunteer Fire Department prior 								to occupancy by residents and periodic, unannounced 								inspections shall be conducted a minimum of one time 								per year thereafter.

3.	Owners shall display proof of such inspection and 									compliance in a conspicuous place.

4.	Emotionally disturbed, alcohol or chemically dependent 								individuals/adults are prohibited.

G.	Recycling Plant and Transportation Terminals shall comply with the following:

1.	The collection and/or storage of automobiles, trucks, major household appliances, and any self-propelled type vehicles, or parts thereof, shall be prohibited.

2.	Vehicular approaches to the property shall be so designed to not create an interference with traffic on surrounding public streets or roads.

3.	Paved, off-street parking and service areas shall be required. All parking and service areas shall be paved, improved with gravel, concrete, asphalt, or equivalent. Paved vehicular access drives of at least ten (10) feet in width shall be required for parking areas of ten (10) vehicles or less capacity, and two-way drives of twenty (20) feet paving width minimum shall be required for parking areas of eleven (11) or more vehicle capacity.

4.	Such uses should be located on an arterial street, adjacent to nonresidential uses such as commerce, industry, or recreation, or adjacent to sparsely settled residential uses.

H.	Congregate Living Facility shall comply with the following:

1.	The following type of residential facilities may be included within the Congregate Living Facility and must have common social, recreational, dining and food preparation facilities:

a.	Independent living with congregates dining facilities,

b.	Congregate Living,

c.	Assisted Living,

d.	Nursing Home.

2.	Such use should not be located on a local residential street.

3.	The development plan shall indicate the parking and emergency entrances or exits and other safety precautions.

4.	The maximum height of the building shall be one story, not to exceed twenty (20) feet, in the R districts.

5.	Screening with a minimum of ten (10) feet shall be provided around the perimeter of the property where it abuts a ‘C’ or ‘I’ district. Screening shall comply with Section 209 of these regulations and shall be implemented within six (6) months of completion of each building or phase of the development which abuts a common boundary. A building phase or parking area shall be considered complete the day it is first used for the purpose intended. Changes thereto shall be approved by the Zoning Inspector prior to completion.

I.	Congregate Living Development shall comply with the following:

The development must contain a Congregate Living Facility and Single Family detached or Single Family attached dwelling units and complies with the following:

1.	Congregate Living Facility:

a.	The Congregate Living Facility shall comply with the criteria per Section 601.13(H).

b.	The minimum front, side, and rear building setback for the Congregate Living Facility from a newly constructed interior public or private street shall be 25 ft.

c.	The minimum front, side, and rear parking setback for the Congregate Living Facility from a newly constructed interior public or private street shall be 10 ft.

2.	Single Family detached and single family attached dwelling units:

a.	In an R district the maximum number of units permitted to be attached shall be two (2).

b.	Must have common social and recreational facilities.

c.	Density shall not exceed the maximum number of units set forth for the district in which it is located. The total number of dwelling units permitted shall be calculated by multiplying the total project area set aside for single family detached or single family attached dwelling units exclusive of land area devoted to public right-of-way existing at the time an application is submitted.

d.	Minimum front building setback from an existing road right-of-way:

1.	Must comply with the district in which it is located in.

e.	Minimum building setbacks from project boundary:

1.	Thirty-five (35) feet to a project boundary that abuts a ‘C’ or ‘I’ district.

2.	Twenty-five (25) feet to a project boundary that abuts any other district.

f.	Minimum building setback from an interior street:

1.	Twenty-five (25) feet to a newly constructed public right-of-way.

2.	Twenty (20) feet to the pavement of a private street.

g.	Minimum distance between buildings. In order to ensure reasonable privacy and separation, individual buildings including terraces, decks and patios shall be separated by at least the minimum distance specified, below:

1.	Twenty (20) feet between two detached single-family units.

2..	Thirty (30) feet between a detached single-family unit and attached single-family units and between two attached single-family units.

h.	Each dwelling unit must have at least two off-street parking spaces.

3.	Circulation shall be provided for the project in compliance with the following:

a.	Paved, off-street parking and service areas shall be required. All parking and service areas shall be paved with concrete, asphalt, or equivalent.

b.	At dead ends, turnarounds shall consist of forty-four (44) foot paved radius, measured from the center of roadways, for adequate movement of safety vehicles.

c.	If the development has more than fifty (50) dwelling units at least two (2) access points to public streets shall be provided.

4.	The use, placement and dimensions of all buildings, driveways, sidewalks, parking areas and curb cuts shall conform to and be substantially constructed in accordance with the approved development.

5.	Screening with a minimum of ten (10) feet shall be provided around the perimeter of the property where it abuts a ‘C‘or ‘I’ district. Screening shall comply with Section 209 of these regulations and be implemented within six (6) months of completion of each building or phase of the development which abuts a common boundary. A building phase or parking area shall be considered complete the day it is first used for the purpose intended. Changes thereto shall be approved by the Zoning Inspector prior to completion.

J.	Licensed Group Homes for Handicapped Persons shall comply with the following:

1.	Such homes shall be licensed according to ORC 5123.19 and evidence shall be presented indicating that the facility meets the certification, licensing, or approval requirements of the appropriate state agency.

2.	All activities, programs and other events shall be directly related to the conditional use permit so granted, and shall be adequately and properly supervised so as to prevent any hazard and to assure against any disturbance or nuisance to surrounding properties, residents or to the community in general.

3.	No such group home shall be located within a thousand (1,000) foot radius of another residential facility for the developmentally disabled licensed under ORC 5123.19.

4.	The architectural design and site layout of a group home licensed under ORC Section 5123.19 and the height of any walls, screens, or fences connected with any said group home shall be compatible with adjoining land uses and the residential character of the neighborhood.

K.	Institutional Housing

1.	The maximum density of institutional housing shall be 7 units or 7 beds per acre, whichever is less.

2.	In a residential district, there shall be a maximum of 21 units or beds per site

3.	All buildings shall be set back 50 feet from all lot lines.

4.	All units shall be rented or sold to the elderly, the handicapped, or the disabled in order to qualify as institutional housing. Mixed development, such as a retirement village with some independent living, shall be built under the multi-family dwelling regulations or as part of a planned development.

5.	No part of a parking area shall be closer than 5 feet to the side and rear lot line unless it is adjacent to a residential district where it shall be set back 50 feet from the property line adjacent to the residential district. All setback areas shall be landscaped with grass and planting areas and shall be well maintained

L.	Day Care Center, Adult or Child and Family Day Care Type A shall comply with the following:

1.	The development plan shall indicate the dimensions of the day-care home, or center, open space areas, parking and emergency entrances or exits and other safety precautions.

2.	Property shall be maintained in a neat, orderly and safe condition.

3.	All activities shall be directly related to the conditional use permit so granted, and shall be adequately and properly supervised to prevent any hazard, disturbance or nuisance.

4.	Sixty (60) square feet of outdoor open space and thirty-five (35) feet of indoor open space per occupant shall be provided.

5.	All outdoor activity areas shall be enclosed by a fence or wall having a height of at least five (5) feet but not exceeding six (6) feet. An entry gate shall be securely fastened.

6.	Additional regulations for Family Day Care Type A.

a.	There shall be a minimum of 600 feet between one Type A family day care home and another Type A family day care home.

b.	There shall be no more than one (1) on-premises sign that shall not exceed four square feet in area.

7.	Additional regulations for day care center. No portion of a day-care center site may be located within 600 feet of gasoline pumps, underground gasoline storage tanks, or any other storage area for explosives or hazardous materials.

M.	Agricultural Uses

As provided in ORC 519.21, the following provisions are established to regulate agricultural uses on lots less than five (5) acres:

1.	On lots of one acre or less, a minimum setback of fifty (50) feet from all property lines is required for all buildings, structures, and pasture/feeding areas for agricultural animals and storing areas incidental to the use of land for agricultural purposes, including apiary (Bee Keeping). Buildings and structures must comply with the building height and size regulations for an accessory building or structure applicable to the district in which it is located. Fencing for agricultural animals shall be 50 ft. from all property lines and the maximum height shall not exceed 8 ft.

2.	On lots greater than one acre but not greater than five (5) acres, all buildings or structures incidental to the use of land for agricultural purposes shall comply with the required building setback lines, height and size regulations for an accessory structure applicable to the district in which the use is located. Fencing for agricultural animals shall be a minimum of 10 ft. from all property lines and the maximum height shall not exceed 8 ft.

3.	A pasture/feeding area that is provided for agricultural animals shall be completely surrounding by a fence that is adequate to contain the animal(s).

4.	For the purpose of this Section, an agricultural animal is a domestic animal, including, but not limited to, a horse, cow, llama, goat, hog, mink, and fowl, but does not include an animal commonly kept indoors as a household pet, such as a dog or cat.

5.	A township zoning resolution, or an amendment to such resolution, may in any platted subdivision approved under section 711.05, 711.09, or 711.10 of the Revised Code, or in any area consisting of fifteen or more lots approved under section 711.131 of the Revised Code that are contiguous to one another, or some of which are contiguous to one another and adjacent to one side of a dedicated public road, and the balance of which are contiguous to one another and adjacent to the opposite side of the same dedicated public road regulate:

a.	Agriculture on lots of one acre or less;

b.	Buildings or structures incident to the use of land for agricultural purposes on lots greater than one acre but not greater than five acres by: set back building lines; height; and size;

c.	Dairying and animal and poultry husbandry on lots greater than one acre but not greater than five acres when at least thirty-five per cent of the lots in the subdivision are developed with at least one building, structure, or improvement that is subject to real property taxation or that is subject to the tax on manufactured and mobile homes under section 4503.06 of the Revised Code. After thirty-five per cent of the lots are so developed, dairying and animal and poultry husbandry shall be considered nonconforming use of land and buildings or structures pursuant to section 519.19 of the Revised Code.

Division (B) of this section (519.21) confers no power on any township zoning commission, board of township trustees, or board of zoning appeals to regulate agriculture, buildings or structures, and dairying and animal and poultry husbandry on lots greater than five acres.

N.	Bed and Breakfast Facilities

Bed and Breakfast facilities including inns and residential bed and breakfast shall comply with the following:

1.	Guestrooms shall not contain cooking facilities. A common lounge area may be provided for guests.

2.	Only retail sales customary to overnight facilities shall be permitted.

3.	Bed and Breakfast, Inn facilities shall comply with the following:

a.	Shall have a minimum of four (4) and a maximum of eight (8) guestrooms.

b.	Shall contain a commercial kitchen and shall obtain all required health department food service permits.

4.	Bed and Breakfast, Residential facilities shall comply with the following:

a.	The building shall be compatible with surrounding land use and shall not exceed three (3) guestrooms.

b.	Food or laundry deliveries shall be made at the rear of the building and shall be conducted during daytime hours.

c.	The building shall not contain a commercial kitchen.

O.	Clubs shall comply with the following:

1.	All activities, programs and other events shall be directly related to the conditional use permit so granted, and shall be adequately and properly supervised so as to prevent any hazard and to assure against any disturbance or nuisance to surrounding properties, residents or to the community in general.

2.	 No outdoor activities shall be permitted.

P.	Community Facilities such as Church and Other Places of Worship, Private Elementary and Secondary Schools and Related Facilities, Libraries and Museums shall comply with the following:

1	 Such uses should be located on an arterial or collector street or have direct access to an arterial or collector street without going through a residential neighborhood to lessen the impact on the residential area.

2.	All outdoor children’s activity areas shall be enclosed by a fence or wall having a height of at least five (5) feet but not exceeding six (6) feet. An entry gate shall be securely fastened.

3.	Churches and other places of worship may be erected to a height not to exceed seventy-five (75) feet, except as provided for in Sec. 203.5B2b, if the building is set back from each lot line one (1) foot for each foot of additional building height in excess of the district limitation.

4.	All activities, programs and other events shall be directly related to the conditional use permit so granted, and shall be adequately and properly supervised so as to prevent any hazard and to assure against any disturbance or nuisance to surrounding properties, residents or to the community in general.

5.	The development plan shall indicate the parking and emergency entrances or exits and other safety precautions.

Q.	Landscaping Business shall comply with the following:

1.	Vehicles and Equipment used in conjunction with the business may be stored on the property.

2.	Dismantled or inoperable vehicles or equipment shall not be maintained or stored on the property, unless in compliance with Sec. 203.4E2.

3.	Outdoor storage areas shall be located in the side or rear yard in compliance with Sec.203.8 and 308.5.

4.	Materials, such as mulch or topsoil, may be stockpiled on the site provided:

a.	The materials are maintained in compliance with all Tuscarawas County Health Department regulations.

b.	Such materials shall not be processed on the site.

c.	Such materials shall not emit any odor that is discernible on adjoining property.

d.	Outdoor retail sales shall comply with Sec.203.8 and 308.5 and shall not be conducted in required parking or landscaping areas.

R.	Landscaping and Construction Vehicle and Equipment Storage when not used in conjunction with an onsite business shall be permitted to be stored in compliance with the following:

1.	Dismantled or inoperable vehicles or equipment shall not be maintained or stored on the property, unless in compliance with Sec. 203.4E2

3.	Minor repair of vehicles or equipment shall be permitted.

S.	Mini-Storage Facilities:

1.	Paved, off-street parking and service areas shall be required. All parking and service areas shall be paved, improved with gravel, concrete, asphalt or equivalent.

2.	The maximum size of individual storage compartments shall be 500 square feet.

3.	Such uses should be located on an arterial street, adjacent to nonresidential uses or in sparsely settled residential areas.

4.	The spacing of storage buildings shall comply with Section 306.6, 307.6, 308.7 or 309.6.

T.	Parking Lots

Parking lots as principal uses shall be located in areas least disruptive to pedestrian, bicycle or vehicular traffic.

U.	Porches, Decks, Patios, and Sidewalks:

1.	Accessory structures such as porches, balconies and decks, as part of the principal or accessory building shall meet the setbacks of the principal or accessory building.

2.	Accessory structures such as porches, patios, and decks with a roof, and balconies as part of a principal or an accessory building shall be counted as part of the square footage for the principal or the accessory building.

3.	The installation sidewalks shall not be regulated and do not require a permit.

4.	Freestanding Decks (detached from principal or accessory building) as an accessory to a boat dock shall be a minimum of 5 ft. from the side property line. Freestanding decks as an accessory to a boat dock that are greater than 120 sq. ft. shall be a minimum of 10 ft. from the side property line.

5.	Freestanding Decks (detached from principal building or accessory building) or decks as an accessory to an above ground swimming pool shall be a minimum of 10 ft. from the side and rear property line.

V.	Automobile, Truck, Boat, Trailer and Farm Implement Storage or Repair, shall be operated so that all activities, except those required to be performed at fuel pumps, shall be carried on inside a building; if work is performed on a vehicle, said vehicle shall be entirely within a building.

W.	Swimming Pool - Commercial.

Swimming pools, hot tubs or whirlpools shall be either contained in a completely enclosed building or surrounded by a fence with a minimum height of eight (8) feet. Fences shall be installed in conjunction with pool construction.

X.	TEMPORARY STORAGE POD/ Portable Storage Container

1. 	No more than 1 portable storage container having a storage capacity no greater than 1088 cubic feet shall be permitted on a zoning lot.

2.	Up to 3 storage containers may be allowed on a zoning lot, provided that each individual container has a storage capacity of no greater than 350 cubic feet.

3.	One sign no more than 6 square feet may be displayed on any portable storage container.

4.	No container shall be located closer than 5 feet to any side or rear lot line or on any portion of a zoning lot, except behind the nearest portion of the principal building adjacent to a public street.

5.	On lots where there is no principal structure, the containers shall comply with the front yard setback for that zoning district. (Mostly applies to sites under construction.)

6.	No portable storage container shall have dimensions greater than 16 feet in length, 8 feet in width, or 8 ½ feet in height.

7.	If site conditions prevent locating portable containers in conformity, containers may be placed in a driveway provided the 2 required parking spaces are unobstructed and the container is closer than 10 feet to the paved portion of any street. However, if site conditions prevent these requirements from being met, the portable container may be placed on any portion of the driveway.

8.	Permit Requirements

a.	Portable storage containers shall be allowed only upon issuance of a permit by the Zoning Inspector, except when used in connection with an agricultural activity in an agricultural zoning district.

b.	Containers are allowed for a period not to exceed 16 consecutive days in a 6-month period.

c.	During construction, reconstruction, alteration, or renovation of a principal structure 24 hours before and after of such activity, containers will be allowed for a period not to exceed 30 days within a 6-month period.

d.	Permits shall be displayed on the outside of the container in plain view from the nearest street.

e.	Permits cost subject to Section 1001.4A.

f.	No permits will be required for containers located on a zoning lot less than 72 hours. However, the container companies will be responsible for displaying a permit card on the container, which must include the delivery and removal dates on the permit card.

9.	Agricultural Zoning District

a.	No permit shall be required for containers placed on a lot where the principal use is a bonafide agricultural activity.

b.	On lots where the principal use is other than an agricultural use, the requirements of a residential district shall apply.

10.	Business and Industrial Districts

a.	Containers shall be allowed pursuant to a conditional use permit​​ for bulk storage per the district regulations, unless the following occurs:

1.	A bona fide construction activity on a site and/or within a structure, whereas, portable containers shall be allowed for a period of 24 hours before and after such activity.

2.	When used in connection with moving or relocation of a commercial establishment, containers shall be allowed for a period not exceeding 16 days.

3.	A permit shall be required for each container placed on the site.

4.	There are no limitations as to the number of containers.

b.	Under no circumstances are portable containers allowed to obstruct required parking spaces for any use(s) on a site.

c.	Shall not block fire lanes.

d.	Permit required for each pod.

e.	Not to exceed six (6) months per permit.

Y.	Shipping containers

1.	It is the intent of this chapter to limit, except as provided herein, the placement and use of any shipping container as an accessory building, storage building, or living unit on residentially zoned and other zoned land where residential uses are established. This limitation is to protect the public health and safety and the aesthetic quality of the township.

2.	No person shall place or cause to be placed or use or permit the use of any shipping container as an accessory building, storage building, or living unit on residentially zoned land and/or land used for residential purposes. Licensed and bonded contractors may use shipping containers for temporary housing of equipment and materials during construction as authorized by a township Conditional Use permit per Article VI

3.	All shipping containers shall be covered or hidden from view by a fence at least as high as the shipping container in any Commercial or Industrial zoned district

Z.	Drive-thru Facilities shall comply with the following:

1.	Such facilities shall be located on an arterial or collector street in an area least disruptive to pedestrian or vehicular traffic.

2.	Any proposed loud speaker system shall be approved as part of the development plan.

AA.	Bulk storage Share Link

  - Print

  - Download (docx)

  - Email

  - Compare

Yard

In addition to general requirements, the following special requirements and limitations shall apply to bulk storage yards in districts in which they are generally permitted:

1.	Fencing. The storage yard shall be completely enclosed, except for necessary openings for ingress and egress, by a fence not less than six (6) feet in height, or the necessary height to hide the units, except where prohibited.

2.	Lighting. All outdoor lights shall be shielded to direct light and glare onto the premises; said lighting and glare shall be deflected, shaded, and focused away from all adjoining property. Any outdoor lighting fixtures shall not be erected any higher than fourteen (14) feet.

3.	No sale or processing of scrap, salvage, or secondhand material shall be permitted in such yards; and, provided further that additional requirements as listed.

Sec. 601.14	Dangerous, Exotic and Wild Animals

The owning, harboring, keeping or breeding of any dangerous, exotic or wild animal 				may be considered within the I-1 Industrial District on lots five acres or greater.  The 				owning, harboring, keeping or breeding of any dangerous, exotic or wild animal shall not 			be permitted in other zoning districts.  For the purpose of this Section “Dangerous,

Exotic and Wild Animals” shall be defined in accordance with Section 935.01 of the Ohio Revised Code.

A.	All animals must be kept within a fenced area that is adequate to contain the 					animal(s).

B.	The keeping, harboring or breeding of animals, including buildings in which 					animals are kept, shall be located no closer than two hundred feet to any C-1,

C-2, or C-3 District, or one thousand feet to any residential district, church, park, 					preschool or school.  If a church, park, preschool or school is located in the C-1,

C-2, or C-3 District the one thousand feet setback shall apply.

C.	The keeping, harboring or breeding of animals, including buildings in which 					animals are kept shall be located no closer than fifty feet to any property line 					within an I-1 Industrial District.

D.	The keeping of dangerous, exotic and wild animals as a pet is prohibited in all 					residential, C-1, C-2, and C-3 Districts.

E.	Such animals may be temporarily displayed in any zoning district during a special 					event such as a circus or carnival, or during educational visits to schools, nursing 					homes, or other institutions, upon application and issuance of a zoning permit.

F.	For the purpose of this Section a dangerous, exotic or wild animal shall not 					include any domestic animal including, but not limited to, a horse, cow, llama, 					goat, hog, mink, fowl or an animal commonly kept indoors as a household pet, 					such as a dog or cat.

G.	The provisions of this Section shall not apply to the keeping of dangerous, exotic 					and wild animals in the following cases:

1.	Animals in zoos or zoological parks operated by political subdivision of

the State of Ohio.

2.	Animals in a bona fide, licensed veterinary hospital for treatment.

3.	Law enforcement agencies and on-profit animal protection organizations 						housing an exotic animal at the written request of the animal control 						authority to keep a dangerous wild animal or restricted snake.

H.	Shall comply with Ohio Revised Code 935 (S.B. 310) and show proof of State 					Certification.

Sec. 601.15	Small Wind Farm

A.	The purpose of this Section is to establish general guidelines for the location of 					wind turbine generators (sometimes referred to herein as “WTG”) and 						anemometer towers in Lawrence Township, Tuscarawas County, Ohio (the 					“Township”).  This Section is consistent with the stated primary purpose of the 					Lawrence Township Zoning Resolution: “Protecting the public health, safety, 					comfort and general welfare” of the Lawrence Township residents.  The

Township recognizes in some specific instances, under carefully controlled

circumstances, it may be in the public interest to permit the placement of wind

turbine generators in certain areas of the Township.  The Township also

recognizes the need to protect the scenic beautify of the Township from unnecessary and unreasonable visual interference, noise radiation, and that wind turbine generators may have negative health, safety, welfare and aesthetic impacts upon adjoining and neighboring uses.  As such, this Section seeks to:

1.	Protect residential and agricultural areas from potential adverse impact 						of wind turbine generators.

2.	Permit wind turbine generators in selected areas, subject to the terms, 						conditions and provision hereof.

3.	Ensure the public health, welfare and safety of the Township’s residents

in connection with wind turbine generators; and

4.	Avoid potential damage to real and personal property from the wind 						turbine generators or anemometer towers or the failure of such

structures and related operations.

B.	Procedure

Any proposed construction, erection or siting of a commercial wind 						turbine generator or anemometer would result in characterization of the 						subject real property as an I-1 District.  Therefore, such proposed use shall 					be a permitted use in the I-1 Industrial District, or by issuance of a 							Conditional Use Permit in accordance with Article VI of this Resolution, as 					amended hereof in the R-4 Residential District.

C.	Application

In addition to the requirements set forth in Section 601 of the Zoning 						Resolution, every application for conditional use for a commercial wind 						turbine generator or anemometer tower permit shall be made in writing 						to the Chairperson of the Board of Zoning Appeals on the forms provided 						by the Zoning Board and shall be accompanied by any and all filing fees 						prescribed by the Township Zoning Resolution and this Section.  The 						permit application shall include but is not limited to the requirements set 						forth in Section 601 and the following information:

1.	Name and address of the applicant

2.	Evidence that the applicant is the owner of the property involved 							or has written permission of the owner to make such application

3.	A plot and development plan drawn in sufficient detail to clearly 							describe the following proposed details:

a.	Physical dimensions of the property, existing structures, 								and proposed structures

b.	Location of existing and proposed structures

c.	Location of existing and proposed electrical lines and 								facilities

d.	Existing topography

e.	Proposed grading and removal of natural vegetation

f.	Wind resource study as described in Section 									601.15(D) (1)

g.	Setbacks as described in Section 601.15(D) (3)

h.	Ingress and egress identifying location and distance to the 								nearest County and Township maintained road

4.	A description of the access route from the nearest County and 							Township maintained road to include:

a.	Road surface material stating the type and amount of 								surface cover

b.	Width and length of access route

c.	Dust control procedures

d.	A road maintenance schedule and program

5.	Utilization of the property under the requested permit

6.	Utility interconnection data and a copy of written notification to 							any utility of the proposed interconnection

7.	Specific information of the type, size, height, rotor material, rated 							power output, performance, safety, and noise characteristics of 							each WTG mode, tower and electrical transmission equipment

8.	A soil boring report as described in Section 601.15(D)(17)

9.	A location map to scale of all dwellings within two miles of the 							boundary of the property upon which the WTG’s are to be located

10.	An analysis and written proposal to reduce air navigation clutter 							on airport radar facilities

11.	One or more detailed computer or photographic simulation 							drawings showing the site fully developed with all proposed WTG’s 						and accessory structures

12.	A copy of written notification to the Federal Aviation 								Administration

13.	Any proposed WTG’s which are located within two miles of any 							microwave communications link shall be accompanied by a copy 							of a written notification to the operator of the link

14.	Any proposed WTG’s which are located within a one-hundred-year flood 						plain area shall be accompanied by a detailed report which shall 							address the potential for wind erosion, water erosion, 								sedimentation and flooding, and which shall propose mitigation 							measures for such impacts.  A permit is required from the County 							Floodplain office.

15.	A separate application for conditional use permit shall be 								submitted for each proposed WTG or anemometer tower.

16.	A cost budget for the proposed project, together with 								estimates/proposals from third parties for the services or 								materials to substantiate the budget, including without limitation 							the following:

a.	The costs of site preparation, excavation, access roads or 								drives, and clearing of vegetation

b.	The cost of the sub-grade materials, equipment and 								material costs for the foundation of the proposed structure

c.	The costs of the structure, including labor, equipment and 								materials

d.	The costs of all ancillary equipment, connection devices, 								lines and related matters for the project

D.	Wind Turbine Generators and Anemometer Towers

Wind turbine generators and anemometer towers shall comply with all of 					the following standards as a condition precedent for a conditional use 						permit to be issued:

1.	Sufficient Wind Resources

The proposed site shall have documented annual wind resources 							sufficient for the operation of the proposed wind turbine 								generator; provided, however, this standard shall not apply to an 							anemometer tower.  No wind turbine generator shall be approved 						without submission of a wind resource study documenting wind 							resources on the site over a minimum of two years.  Said study 							shall comprehensively articulate the long-term commercial 							economic viability of the project.  Anemometers to be placed shall 						be calibrated regularly to ensure a measurement of error of one percent 						or less.  All anemometers shall be placed at the expected hub height of 						the wind turbine to be used.  Sufficient wind resources, as described by 						the U.S. Department of Energy, include areas with a wind power class

four or higher.  The Township shall retain the services of an independent 						expert to review the results of the wind resource study prior to acting on 						the application for conditional use permit.  All costs incurred by the 						Township for the services of the independent expert shall be borne by

the applicant and shall come from the requisite deposit mandated under 						Section 601.15(D) (26) hereof.

2.	Minimum Site Area

The minimum site area for a wind turbine generator or an 								anemometer tower erected prior to a wind turbine generator shall 						be the number of acres required to meet the required Adjoining 							Lot Line setbacks set forth in Section 601.15(D)(3) and any other 							provisions of this Section.

3.	Setbacks

For health, safety and welfare purposes, each proposed wind 							turbine generator or anemometer tower shall meet the following 							applicable setback requirements:

a.	Adjoining Lot Lines – Each proposed wind turbine 									generator shall be setback from any and all adjoining lot 								lines a minimum distance of nine hundred fifty feet.  The Zoning 							Board of Appeals may reduce this setback no less than eight 							hundred feet from any and all adjoining lot lines; provided, 							however, the amount of setback relief approved by the Board of 							Zoning Appeals will be based on data provided by the applicant 							establishing that the lesser setback from an adjoining lot line shall 							not adversely affect the health, safety or welfare of any person or 							animal.  Such data shall be subject to review by the Township’s 							independent expert.  All cost incurred by the Township for the 							services of the independent expert shall be borne by the applicant 						and shall come from the requisite deposit mandated under

Section 601.15(D) (26) hereof.

b.	Building or Structures – Each proposed WTG shall be 								setback from any building or structure, as those terms are 								defined under the Township Resolution at Article II, a 								minimum distance of one thousand four hundred fifty feet.

c.	Road of Right-of-Ways – In addition to the above, a wind 								turbine generator shall, in all cases, be setback from a 								public or private road right-of-way or easement a 									minimum distance equal to four times the height of the 								proposed wind turbine generator or anemometer tower as 							defined in this Section.  This requirement is to further 								promote health, safety and welfare as “shadow flicker” 								could affect driver visibility on roads in the Township.

4.	Wind Access Buffer

For any newly proposed wind turbine generator or anemometer 							tower, a Wind Access Buffer equal to a minimum of five rotor 							diameters shall be observed from any existing off-site wind turbine 						generator tower.

5.	Sensitive Environment Areas

Sensitive environmental areas shall have a setback of a specified 							distance that is determined by the Township Zoning Board and the 						Ohio Department of Natural Resources.

6.	Scenic Areas

Scenic areas, including parks, highways, recreational areas and 							others as determined by the Township, shall have a setback of not 						less than one mile; the specific distance of the setback shall be 							determined by the Township.

7.	Maximum Height

The maximum wind turbine generator or anemometer tower 							height from the base to the tip of the blade at its highest point 							shall not exceed two hundred feet.  The Zoning Board may approve an 						increased height for a wind turbine generator tower, not to exceed two 						hundred sixty feet from the base to the tip of the blade, if all of the 						following conditions are met:

a.	The increased height will result in the preservation of a 								substantial stand of trees, existing land forms or structures 							that would otherwise be removed to increase wind 									velocity.

b.	The increased height will not result in increased intensity 								on lighting of the tower to FAA requirements.

8.	Minimum Rotor Wind Vane or Blade Clearance

The lowest point of the arc created by rotating wind vanes or 							blades on a wind turbine generator shall be no less than eighty feet 						measured from the highest point of the terrain within one								blade radius from the base of the tower.

9.	Maximum Noise Levels

The audible noise radiation due to wind turbine operations shall 							not be created which causes the noise level at the boundary of the 						proposed project site to exceed thirty dBA for more than five 							minutes out of any one hour time period or to exceed thirty five dBA						for any time period; provided, however, if the Natural Ambient 							Noise Level without the commercial wind turbine generators is 							greater than thirty dBA, the audible noise radiation shall not exceed 						five dbA above the Natural Ambient Noise Level.  A commercial 							wind energy facility shall not be operated so that impulsive sound 							below twenty Hz adversely affects the habitability or use of any 							dwelling unit, hospital, school, nursing home, or other sensitive noise 						receptor.

10.	Maximum Vibrations

Any proposed wind turbine generator shall not produce vibrations 						humanly perceptible beyond the boundaries of the property on 							which it is located.

11.	Endangered or Threatened Species

Development and operation of a commercial wind energy facility 							shall not have a significant adverse impact on the endangered or 							threatened fish, wildlife, or plant species or their critical habitats, 							or other significant habitats identified by the Department of 							Natural Resources and/or the studies of the U.S. Fish and Wildlife 							Service.  Commercial wind energy facilities must adhere to the 							guidelines set forth by the U.S. Fish and Wildlife Service, 								“Guidelines to avoid and Minimize Wildlife Impacts from Wind 							Turbines”, Federal Register: July 10, 2003 (Volume 68, Number 							132).

12.	Migratory Birds

Development and operation of a commercial wind energy facility 							shall not have an adverse impact on migratory bird species.

13.	Transmission Lines

All electrical transmission lines connecting any wind turbine 							generator to the public utility electricity distribution system shall 							be located underground.

14.	Electromagnetic Interference

Any wind turbine generators shall be constructed and operated so 						that they do not interfere with television, telephone (including 							cellular and land line), microwave, navigation, or radio reception 							to neighboring areas.  The applicant and/or operator of the facility 						shall be responsible for the full cost of any remediation necessary 							to correct any problems, including relocation or removal of the 							facility, caused or exacerbated by the operation of such equipment 						and any and all related transmission lines, transformers, and other 						components related thereto.  The applicant for commercial wind 							turbine generators shall pay for testing of the above reception for 							all properties within two miles prior to construction and will 							pay to correct reception for landowners with degradation of these 						signals.  The entity or agency that conducts the test shall be 							selected by the Zoning Board.  The cost incurred by the Township 							for such tests shall be borne by the applicant and shall come from 							the requisite deposit mandated under Section 601.15(D) (26)							hereof.

15.	Landscaping

Each proposed wind turbine generator or anemometer tower 							erected prior to a wind turbine generator shall meet the following 						landscaping requirements:

a.	The base of the wind turbine generator or anemometer 								tower erected prior to a wind turbine generator shall be 								landscaped with a buffer of plant materials that effectively 							screens the view of the bases of these facilities from 								adjacent property use for residential purposes.  The 								standard buffer shall consist of a landscaped strip at least 								four feet wide outside the perimeter of the facilities.

b.	Existing natural land forms on the site which effectively 								screens the base of the wind turbine generator or 									anemometer tower erected prior to a wind turbine 									generator from adjacent property used for residential 								purposes shall be preserved to the maximum extent 								possible.

c.	Landscaping shall be designed to counter the effects of 								“shadow flicker” on any neighboring residences or 									roadways caused by the rotor rotation in the sunlight.

d.	To ensure compliance with these landscaping standards, 								the Zoning Board may require additional landscaping on 								the site after the installation of the wind turbine 									generator or anemometer tower.

16.	State or Federal Requirements

Any proposed wind turbine generator anemometer tower shall 							meet or exceed any standards and regulations of the F.A.A., the 							Public Utilities Commission of Ohio, National Electric Safety Code, 							U.S. Fish and Wildlife Service and any other agency of the state or 							federal government with the authority to regulate wind turbine 							generators or other tall structures in effect at the time the 								conditional use permit is applied for.

17.	Soil Conditions

A proposal for any wind turbine generator or anemometer tower 							shall be accompanied by a report of the soils present on the site 							based on soil borings, prepared by a firm which specializes in soil 							borings and is approved to perform such work in the State of Ohio.  						The report shall include soil and geologic characteristics of the site 						based upon on-site sampling and testing.  The soil boring reports 							and the proposed plans for the foundation shall be certified by a 							registered Professional Engineer licensed in the State of Ohio, who 						is practicing in his or her area of competency.

18.	Aesthetics and Lightening

Any proposed wind turbine generator or anemometer tower shall 							meet the following requirements:

a.	Each wind turbine generator or anemometer tower shall 								either maintain a galvanized steel finish or, subject to any 								applicable standards of the F.A.A., be painted a neutral 								color so as to reduce visual obtrusiveness and shall be so 								maintained as to be in continuous compliance with this 								paragraph and to prevent any visible oxidation or 									corrosion.

b.	Each wind turbine generator, including all accessory 								structures or anemometer tower shall, to the extent 								possible, use materials, and colors that will blend them 								into the natural setting and surrounding buildings.  A 								medium gray shade is the preferred color for any wind 								generator or anemometer tower; however, the Zoning 								Board may approve an alternate color if the facility is 								suspected to be located within an avian migratory route or 							if an alternate color would otherwise benefit the 									community.

c.	Each wind turbine generator or anemometer tower shall 								not be artificially lighted, unless required by the F.A.A. or 								other applicable governmental authority.  If lighting is 								required, the lighting alternatives and design chosen:

1.	Shall be the lowest intensity allowable under F.A.A. 								regulations

2.	Shall not be strobe lighting or any other 										intermittent white lighting fixtures, unless expressly 								required by the F.A.A.  Such intermittent lighting 									shall be alternated with steady red lights at night if 									acceptable to the F.A.A. and the Township

3.	Shall be a red stop light that does not pulsate or 									blink

4.	All tower lighting required by the F.A.A. shall be 									shielded to the extent possible and acceptable to 									the F.A.A. to reduce glare and visibility from the 									ground

5.	Each wind turbine generator or anemometer tower 								shall be sited on the property in a location that 									reduces to the maximum extent possible any 									adverse impacts on view corridors from adjacent 									properties

6.	Each wind turbine generator or anemometer tower 								shall be a monopole or monotube style construction 								(as distinguished from a lattice-style tower) and 									shall not utilize guide wires

19.	Sign

A sign no more than four square feet in area displaying an 								address and toll-free telephone number, answered by a person 							twenty-four hours per day, seven days per week, for emergency 							calls and information inquiries shall be posted at the proposed 							wind turbine generator or anemometer tower erected prior to a 							wind turbine generator.  No wind turbine generator tower or 							anemometer tower or site shall include any advertising sign.

20.	Maintenance

The owner or operator of the WTG shall furnish an operation and 							maintenance report to the Township on an annual basis.  Any 							physical modification to the WTG that alters the mechanical load, 							mechanical load path, or major electrical components shall require 						re-application for conditional use under this Section.  Like-kind 							replacements shall not require re-application.  Prior to making any 						physical modification (other than a like-kind replacement), the 							owner or operator shall request, in writing, a determination from 							the Township Board of Zoning Appeals whether the physical 							modification requires re-application for conditional use permit.  							The Board of Zoning Appeals shall, in its sole and absolute 								discretion, make such determination.

21.	Local Fire Department

The applicant, owner or operator shall submit to the local Fire 							Department a copy of the site plan.  Upon request by the local Fire 						Department, the owner or operator shall cooperate with the local 							Fire Department to develop the Fire Department’s emergency 							response plan.  Nothing in this Section of this Section shall 								alleviate the need to comply with all other applicable fire laws and 						regulations.  Upon request by the local Fire Department, the 							owner or operator shall on a yearly basis, participate in High Angle 						Rescue using the WTG tower.

22.	Hazard Planning

An application for a wind turbine generator shall be accompanied 							by a hazard prevention plan.  Such plan shall address the following 						at a minimum:

a.	Certification that the electrical wiring between turbines 								and between turbines and the utility right-of-way does not 							pose a fire hazard shall be signed by a registered 									Professional Engineer licensed in the State of Ohio, who is 								practicing in his or her area of competency.

b.	The landscape plan accompanying the application shall be 								designed to avoid spread of fire from any source on the 								turbine; such preventative measures may address the types 							and locations of vegetation below the turbine and on the 								site.

c.	The following shall be submitted with the application for a 								conditional use permit for a wind turbine generator:

1.	A listing of any hazardous fluids that may be used 									on site shall be provided.

2.	Certification that the turbine has been designed to 									contain any hazardous fluids shall be provided.  									This certification shall be signed by a registered 									Professional Engineer licensed in the State of Ohio, 									who is practicing in his or her area of competency.

3.	A statement certifying that the turbine shall be 									routinely inspected to ensure that no fluids are 									released or leaked from the turbine or any other 									equipment or appurtenances on the site.

4.	A Hazardous Materials Waste Plan, complying with 									all federal, state and county laws and regulations 									shall be prepared and filed.  Approval by all of the 									above parties shall be a condition to be met prior to 								the issuance of any permit.  Further, approvals or 									waivers, by the state Department of Environment 									Quality, and the state Department of Natural 									Resources and/or the Corp. of Army Engineers shall 								also be submitted prior to the issuance of any 									permit.

23.	Decommissioning Plan

The applicant must formulate and submit a Decommissioning Plan 						to ensure that the WTG project is property decommissioned.  The 							Decommissioning Plan shall include:

a.	Provision describing the triggering events for 									decommissioning the WTG project.

b.	Removal of structures, debris, access roads, and electrical 								cabling, including transmission lines below the soil surface, 							as specified in each individual WTG contract with the 								landowner(s).

c.	Provisions for the restoration of the soil and vegetation.

d.	An estimate of the decommissioning costs certified by a 								Professional Engineer approved by the Lawrence Township 							Trustees.

e.	A cash bond secured by the owner/operator, for the 								purpose of adequately performing the decommissioning, 								costs plus anticipated inflation.

f.	Identification of and procedures for Township access to the 							cash bond.

g.	A provision that the terms of the Commissioning Plan shall 								be binding upon the owner or operator and any of their 								successors, assigns, or heirs.

24.	Removal of Abandoned Wind Turbine Generators or Anemometer 						Towers

Any wind turbine generator or anemometer tower that is not 							operated for continuous nine months shall be considered 								abandoned and the owner of such wind turbine generator or 							anemometer tower shall remove the same within ninety days of 							receipt of notice from the Township.  In addition to removing the 							wind turbine generator, or anemometer tower, the owner shall 							restore the site of the wind turbine generator or anemometer 							tower to its original condition prior to location of the wind turbine 						generator or anemometer tower.  Any foundation associated with 						a wind generator or anemometer tower shall be totally removed 							and the site restored to its original state including the planting of 							any grasses or cover crops, which may have been present prior to 							construction.  Any and all transmission equipment, buildings and 							fences shall also be removed.  Failure to remove an abandoned 							wind turbine generator or anemometer tower within the ninety day 						period provided in this subsection, or in the event that the owner 							or operator of the wind turbine generator or anemometer tower is 						no longer financially capable, or fails to respond to mail sent to its 							last address on file with the Township, shall be grounds for the 							Township to remove the wind turbine generator or anemometer 							tower structure and all associated equipment or appurtenances at 						the owner’s expense.  The Township may sell any salvageable 							material, deducting the balance due from the cash bond, which 							the Township shall require.

25.	Cash Bond

The Township shall further require the applicant to post a cash 							bond equal to fifty percent of the original cost of the above-grade tower 						structure and attendant structures to assure the removal of such 							abandoned structures and remediation of any toxic or hazardous 							materials left on the site as a condition of a special use permit 							given pursuant to this section.  This shall be in addition to the 							amount referred to in Section 601.15(D) (17) Soil Conditions.  In the 						event that a surety bond is to be substituted for a cash bond, it shall be 						prepaid for a period of five years, with the insurance carrier instructed to 						notify the Township of any delinquency in payment within thirty days of 						the occurrence of such delinquency.  Such delinquency shall be

considered abandonment and full and sufficient grounds for the

Township to terminate the conditional use permit and dispose of the

equipment and appurtenances as stated above.

26.	Security Deposit for Expenses and Fees

The applicant shall deposit, with the Township, a sum of twenty-five 						thousand dollars cash as partial payment for the Zoning Board expenses 						in hiring consultants and experts as the Zoning Board shall, at its sole 						discretion, deem desirable.  If at any time the balance of this fund shall 						fall below ten thousand dollars, the applicant shall submit an additional 						fifteen thousand dollars so that the Township’s full and actual expenses

of examining and verifying the data presented shall be covered in total by 					the applicant.  This deposit shall accompany the initial application and be 						considered a part thereof.  If at any time the balance of this fund shall fall 					below ten thousand dollars for a period of thirty days, the application 						shall be considered to have been withdrawn. The applicant shall be 						entitled to no refund or reimbursement for any monies expended by the 						Zoning Board in connection with the application; if, however, the 							application is withdrawn, rejected or denied, applicant shall be entitled to 					receive the balance of any unused deposit funds as set forth herein, 						provided applicant requests same in writing within ninety days of 							withdrawal, rejection or denial of the application.

27.	Infrastructure Deposit

The applicant shall deposit, with the Township, a sum of eighty thousand 						dollars 	cash as security and anticipated partial payment for any damage 						to Township infrastructure, including roads, bridges and overpasses.  If at 						any time the balance of the deposit falls below sixty thousand dollars, the 					applicant shall submit an additional twenty thousand dollars to the 						Township as security for maintenance and repair of infrastructure.  This 						deposit shall accompany the initial application and be considered a part 						thereof.  An application will not be accepted if the security deposit is not 						tendered therewith.  If at any time the balance of this fund shall fall

below sixty thousand dollars for period of thirty days, the application shall be considered to have been withdrawn.  The applicant shall be entitled to receive the balance of any unused deposit funds as set forth herein, provided applicant requests same in writing within ninety days of withdrawal, rejection or denial of the application.

28.	Severance Clause

If any section, clause, or provision of the Section is declared 							unconstitutional or otherwise invalid by a court of competent 							jurisdiction, said declaration shall not affect the validity of the 							remainder of the Sections as whole or any part thereof, other than 						the part so declared to be unconstitutional or invalid.

29.	Agriculture

Nothing in this Section is intended to limit agriculture.

30.	Default and Remedies

The applicant’s, owner’s or operator’s failure to materially comply 						with any of the above provisions, including any conditional use 							permit granted, shall constitute a default under this Section.  The 							appropriate Township body shall provide written notice to the 							owner and operator, setting forth the default(s).  Such written 							notice shall provide the owner and operator a reasonable time 							period, not to exceed sixty days, to cure the default and make full 							restitution, as mandated by the appropriate Township body.  If

the Township determines in its sole and absolute discretion, that the 						default has not been timely cured or full restitution timely made.  							Any costs incurred by the Township relative to a default shall come 						out of the cash security deposit under Section 601.15(D) (25).

ARTICLE VII

NONCONFORMING USES AND STRUCTURES

Sec. 700	Regulations

The provisions of this Article are intended to regulate the continued existence of structures and 		uses of land and buildings established prior to the effective date of this Resolution that no 			longer conform to the regulations of this Resolution.  Such structures and uses of land and 			buildings may be continued although such use does not conform with the provisions of this 			Resolution, provided the following conditions are met:

Sec. 701.1	Alterations

A nonconforming building or structure may be altered, improved, or reconstructed, but 				not enlarged or extended, provided such work does not exceed in aggregate cost fifty 				percent of the total replacement value of the building or structure.

Sec. 701.2	Extensions

A.	A nonconforming use of land or buildings shall not be enlarged or extended to 					occupy a greater land or building floor area, or altered or moved in whole or in 					part to any other portions of the lot.  No additional structures shall be 						constructed to house a nonconforming use, but the extension of a use which 					conforms with the provisions of this Resolution to any portion of a 						nonconforming structure shall not be considered an extension of a 						nonconforming use.

B.	No nonconforming structure shall be moved, extended, enlarged or altered in a 					way which increases their nonconformity.  Such structures may be extended, 					enlarged, or altered where such work does not increase their nonconformity.

Sec. 701.3	Construction Approved Prior to Resolution

Nothing in the Resolution shall prohibit the completion of construction and use of a 				nonconforming building for which a building permit has been issued prior to the 					effective date of this Resolution, provided that construction is commenced within ninety 			days after the issuance of such permit; that construction is carried on without 					interruption for a continuous period in excess of thirty days; and that the entire building 			shall have been completed within one year after the issuance of said building permit.

Sec. 701.4	Displacement

No nonconforming use shall displace a conforming use.

Sec 701.5	Discontinuance or Abandonment

Wherever a nonconforming use has been discontinued for a period of one year or 				more, such discontinuance shall be considered conclusive evidence of an intention to 				legally abandon the nonconforming use.  At the end of the one-year period of 					abandonment the nonconforming use shall not be re-established, and any further use 				shall be in conformity with the provisions of this Resolution.

Sec. 701.6	Restoration

Any nonconforming building which has been destroyed or damaged by fire, explosion, 				Act of God, or by public enemy to the extent of fifty percent or more of its’ assessed 				valuation, exclusive of the foundation at the time such damage occurred, shall 					thereafter by made to conform with the provisions of this resolution.  If such damage is 				less than fifty percent of its assessed valuation before said damage occurred, exclusive 				of the foundation, then such structure may be restored to the same nonconforming use 				as existed before such damage, provided that such restoration shall be subject to the 				approval of the Board of Appeals.  Said restoration shall be completed within one year 				of the date of such partial destruction.

Sec. 701.7	Zoning Certificate for Nonconforming Uses

A Zoning Certificate shall be required for all nonconforming uses.  Application for the 				Zoning Certificate for nonconforming uses shall be filed in accordance with the 					provisions of Article X.  Such zoning certificates shall be used to establish the legality of 				such nonconformity as of a specified date.  It shall be the burden of the owner of a 				nonconforming use to show that the nonconformity lawfully exists under the provisions 				of this resolution.

ARTICLE VIII

AMUSEMENT ARCADES

Sec. 801	Purpose and Findings

Sec. 801.1	Purpose

That the purpose of this Article is to establish reasonable and uniform regulations to 				minimize and control the negative effects of amusement arcades within the Township in 			order to promote health, safety and welfare of the citizens of the Township.  It is not the 			purpose or intent of this Article to restrict or deny access to recreational and skilled-				based amusement machines.

Sec. 801.2	Findings

A.	The State of Ohio currently allows certain games of skill while games of chance 					are deemed illegal.  Based upon experience within the County, certain arcades 					identified as operating games of skill, have generated various complaints by 					citizens involving thefts, questionable pay practices, considerable sums of cash 					being transacted, food management concerns, fire code and access, and other 					issues of compliance with local and state laws.

B.	These businesses could be located throughout the Township in general business 					districts, in areas that attract adults and children.  It is the specific finding that 					games of skill operated electronically may easily be altered to illegal games of 					chance.  Further, that the Township has a duty to its citizens to require 						businesses open to the public to maintain safe ingress and egress, to maintain 					adequate security where a considerable volume of cash is transacted, and to 					otherwise operate within the bounds of the law.

Sec. 802	A.	Definitions

As used in this Article, except where the context clearly indicates a different meaning:

1.	Amusement arcade means any place of business where three or more 						amusement devises are located for the use or entertainment of persons 						patronizing the place of business.

2.	Amusement device means any machine, device, or instrument which either may 					be activated for play by a third party, person or device, or upon the insertion of 					paper money, a coin, token, slug, or card, operates or may be operated as a 					game, contest, or test of skill, or other amusement of any kind.  Amusement 					device does not include vending machines.

3.	Game machine means any amusement device.

4.	Good moral character means not having been convicted of a crime involving 					moral turpitude within five years next proceeding the date of the application.

5.	Malfunction means failure to operate in accordance with design.

6.	Moral turpitude means a conviction for a theft offense, fraud, falsification, drug 					offense, sex offense, an offense involving gambling, or a felony.

7.	Operator means any individual, corporation, or other entity conducting the 					business of an amusement arcade.

8.	Owner means any individual, corporation or other entity owning title to any 					amusement device or the real property at which an amusement arcade is 						operated.

9.	Playing area means that portion of the premises where the primary use is for 					customer play on amusement devices.

10.	Skill-based amusement machine means a skill-based amusement device such as 					a mechanical, electronic, video, or digital device, or machine, whether or not the 					skill-based amusement machine requires payment for use through a coin or bill 					valuator or other payment of consideration or value to participate in the 						machine’s offering or to activate the machine, provided that all of the following 					apply:

a.	The machine involves a task, game, play, contest, competition, or 							tournament in which the player actively participates in the task, game, 						play, contest, competition, or tournament.

b.	The outcome of an individual’s play and participation is not determined 						largely or wholly by chance.  For purposes of this Article “largely or 						wholly” means at least by fifty one percent.

c.	The outcome of play during a game is not controlled by a person not 						actively participating in the game.

d.	The machine only charges one price to play a task, game, play, contest, 						competition, or tournament.

B.	All of the following apply to any machine that is operated as described in Article VIII				hereof:

1.	As used in this Section, task, game and play mean one event from the initial 					activation of the machine until the results of play are determined without 					payment of additional consideration.  An individual utilizing a machine that 					involves a single task, game, play, contest, competition, or tournament may be 					awarded prizes based on the results of play, the prizes or rewards shall be 					established prior to the individual placing a wager, and the individual shall be 					aware of what prize or reward will occur prior to the start of play.

2.	Advance play for a single task, game, play, contest, competition, or tournament 					participation may be purchased.  The cost of the contest, competition, or 						tournament participation may be greater than a single non-contest, competition, 				or tournament play.

3.	To the extent that the machine is used in a contest, competition, or tournament, 					that contest, competition, or tournament has a defined starting and ending date, 				and is open to participants in competition for scoring and ranking results toward 					the awarding of prizes without payment of additional consideration.

4.	No machine shall have a guessing game at the end of a successfully completed 					task, game, play, contest, competition, or tournament.

Sec 802.1	Operation; License Required

A.	No individual, corporation or other entity shall be an operator of an amusement 					arcade at any place of business unless such operator holds a valid amusement 					arcade license for the place of business where such amusement arcade is 						operated.

B.	No individual, corporation or other entity shall permit or cause to be permitted 					any amusement machine, game machine, or amusement device to be operated, 					placed into operation, moved onto the area of play, or played, without a current 					and valid license for that machine displayed thereon.

C.	No individual, corporation, or other entity shall play any amusement device 					unless it is validly licensed by the township.

Sec. 802.2	Application Information

The original and renewal application for an amusement arcade license and the license 				for each skill-based amusement machine, game machine, or amusement device shall be 				upon a form prescribed by the Zoning Commission and shall set forth therein 					information such as the name and address of the operator, the address of the place of 				business which is to be the licensed amusement arcade, the year for which the license is 			sought, the number of skill-based game machines or amusement devices located at such 			place of business, the name and address of the owner of each machine, a detailed 				explanation of the machine operation, applicable software license authorization, player 				skills, and training required qualifying each machine as a game of skill, and such other 				information as the Zoning Commission reasonably requires.  The application shall be 				signed by the operator in whose name the Township licenses are to be issued as well as 				the owner of the real property.

Sec. 802.3	Corporations, Trusts, and Partnerships

A.	If the operator filing the application for a license under this Article is a 						corporation, the application shall list the names and addresses of all officers and 					directors and any individual, corporation or other entity owning twenty five 					percent or more of the issued and outstanding shares of every class of stock of 					such corporation.

B.	If the operator filing the application for a license is a partnership, the application 					shall list the names and addresses of all partners.

C.	If the operator filing the application for a license is a trust, the application shall 					list the names and addresses of all trustees and/or co-trustees.

D.	The listing required of any corporation, trust, or partnership shall be repeated an 				further repeated for any corporation, partnership or other entity who or which 					appears as a shareholder, trustee, co-trustee, or partner on the application

Sec. 802.4	Affidavit Required

The application for a license under this Article shall be submitted on forms provided by 				the Zoning Inspector and be accompanied by an affidavit attesting that the operator and 			all employees and agents of the operation have not been convicted of a crime of moral 				turpitude and to the truth of the matters set forth in such application.  No person shall 				swear falsely in any affidavit required to be filed under this section.

Sec. 802.5	License Issuance; Effective Period; Fee

The Zoning Inspector is hereby authorized to issue amusement arcade licenses and 				amusement and/or game machine licenses, in such form as he or she determines to be 				appropriate, for a period of up to one year, upon satisfaction of all the following 					conditions:

A.	The operator of the amusement arcade has properly filed the application 						required by this Article.  The owner of the game machines and the real 						property owner shall sign the application.

B.	The operator or any employee of the operator has not been convicted of a crime 				of moral turpitude within the past five years.

C.	The Zoning Inspector has determined that no other reasonable cause exists to 					deny the issuance of such license.

D.	Compliance with Section 802.7.

E.	License fees are non-refundable except upon approval by this Board.

F.	Current fees are available at the township building at the fiscal officer’s office.

Sec. 802.6	License Administration

A.	It shall be the duty of the Zoning Inspector or his designee to administer the 					licensing regulations of this Article.

B.	The Zoning Inspector is hereby empowered to enforce such rules and regulations 				relating to any matter or thing pertaining to the issuance, administration, and 					enforcement of this Article.

C.	The burden shall rest on the owner, applicant, operator, and/or agent of the 					operator, owner, or applicant to timely produce the complete, accurate, and 					true records, documents, programs source codes or other data or objects 					necessary to substantiate the licensing requirements of this Article.  Absent 					such substantiation, the decision of the Zoning Inspector shall be final subject to 					Section 802.13.

D.	A license shall be issued within forty-five days of receipt of complete application 					and compliance with this Article.

E.	No arcade license shall be issued until all individual amusement devices are 					licensed.

Sec. 802.7	Submission of Source Code for Approval

A.	Each applicant, within forty-five days of submitting an application for an 						amusement arcade license, shall submit to the Zoning Inspector:

1.	A written report prepared by an independent laboratory satisfactory to 						the Zoning Commission stating that the specific amusement device, 						including but not limited to the source code, has been tested and 							examined under the requirements of the Ohio Revised Code and this 						Article and the specific amusement device is largely or wholly a game of 						skill.

2.	A report must be prepared for each amusement device to be licensed.

3.	An exact and sealed copy of the source code for each device shall 							accompany the report.  Such sealed code shall be deemed proprietary 						and shall be held by the Zoning Inspector or his designee until the license 						is expired, revoked, or there are reasonable grounds to believe a violation 					of this Article occurred.  If reasonable grounds exist, the sealed source 						code will be provided to law enforcement for investigation.  Otherwise, 						the source code will be returned to the licensee.

B.	The Zoning Inspector shall provide the names of at least two laboratories 						deemed satisfactory.

Sec. 802.8	License Validity and Display

Each license under this Article shall be valid for only so long as the amusement arcade 				is operated by the operator listed on such license, at the place of business listed 					thereon.  Each skill-based amusement machine and/or amusement device shall be valid 				for operation or use only so long as the game machine and/or amusement device has 				displayed on it a current license, or until the license is revoked by the Township or until 				such machines are determined to be games of chance or otherwise deemed illegal by 				the State of Ohio or a court of competent jurisdiction.

Sec. 802.9	Operation of Arcade

A.	No person under the age of eighteen years shall be permitted on the premises.

B.	No door shall be locked preventing ingress or egress by members of the public 					while patrons are on the premises.

C.	The operator shall adopt and enforce a “No Loitering” policy on the premises.

D.	The premises of every amusement arcade shall be equipped with exterior lights 					of sufficient intensity to illuminate every means of ingress and egress and 					adjacent parking area.

E.	Each arcade shall be maintained so that it is handicap accessible throughout.

F.	Each existing arcade shall be a “No Smoking” area.

G.	Each arcade shall be operated with a glass storefront allowing full visibility at the 				sidewalk or right-of-way from the front through the playing area to the rear of 					the facility, exclusive of restrooms.  No machines shall be placed in restrooms.

H.	Windows shall be clear and free of tint.  No obstructions shall prevent observing 					at least fifty percent of the arcade games from outside the front door.

I.	The operator shall maintain a record of each game machine taken out of play for 					any reason, including but not limited to, machine malfunction.  The record shall 					include, but not be limited to the following:  name of operator taking the device 					out of play, name and address of player who last played, the amount reflected a 					won but not paid or lost by the arcade, a description of the malfunction, a 					description of how the game machine was designed to operate, time and date of 				removal from play, make, model, and serial number of the game machine.   Said 					record shall be maintained on the premises for at least two years from date 					of removal.  Further, the record shall be available for inspection to the Zoning 					Inspector, his agents, and designee during regular business hours.

J.	No arcade shall operate during the hours of 1:00 a.m. to 7:00 a.m.

K.	Each operator shall maintain a record of the full name, address, telephone 					number, date, tax identification number, and gross value amount for each player 				receiving consideration or anything of value exceeding two hundred dollars.  This 				record shall be filed with the Township Fiscal Officer for Lawrence Township at 					least quarterly.  Further, such record shall be available during regular business 					hours for a period of two years.

L.	Each operator shall clearly post in a conspicuous place all circumstances in which 				a player may not “cash out”, be reimbursed, or receive a cash payment for the 					value of winnings, credits, rewards, or prepayments.

M.	Each operator, employee, and agent thereof shall wear a full name identification 					nameplate during working hours.

N.	All on premises food service shall comply with state and local health regulations.

O.	Each operator shall conspicuously display by posting with each skill-based 					amusement machine the established prize or reward for each play.  This 						information shall be posted so that the player can observe it prior to and during 					play.

P.	Each operator shall make available and have on display forms as prescribed by 					the Zoning Inspector for the recovery of losses pursuant to O.R.C.3763.

Q.	No weapons, firearms, or dangerous ordinances are permitted on the premises.

R.	No arcade shall operate within one thousand feet from the boundaries of a 					parcel of real estate having situated on it a school, child day care center, church, 					library, governmental park, another arcade business or property zoned R-1 					through and including R-4 or otherwise zoned residential, or C-1 under the 					Lawrence Township Zoning Code, or within one hundred feet of the boundaries 					of a real estate parcel or on the same real estate parcel containing a bar, tavern 					or other business serving or selling liquor, beer, wine or other alcoholic 						beverages.

Sec. 802.10	License Revocation

It shall be cause for revocation of any license required under this Article, by the issuing 				authority, or for non-renewal of such license, for an operator or operator’s officers, 				directors, agents, or employees, trustee, twenty five percent of the shareholders of an 				operator, or any other person to:

A.	Operate an amusement arcade without a valid license.

B.	Operate or permit to be operated an amusement device or game machine 					without a valid license for that machine or device.

C.	Fail to display any license required by this Article.

D.	Provide any false or misleading information in the material submitted during the 					application process.

E.	Permit any violation of:

1.	A Resolution or regulation of the Township

2.	Regulation of the County, including but not limited to rules of the 							Tuscarawas County Department of Health

3.	Statue of the State, any for which a criminal penalty may be invoked, to 						take place at any amusement arcade operated by such operator.

F.	Knowingly allows gambling on the premises.

G.	Transfer or altar any license issued under this Article.

H.	Failure to comply with any provision of this Article.

I.	Be convicted of a crime involving moral turpitude.

Sec. 802.11	Revocation Process

A.	The Zoning Inspector shall notify the licensee in writing, at the address of the 					amusement arcade, of the reason for revocation.  Service shall be made by 					regular first-class mail with proof of service or personally.

B.	When the Township revokes a license, the licensee shall not be issued another 					license for one year from the date the revocation became effective.  If the 					Township finds, subsequent to revocation, that the basis for the revocation has 					been corrected or abated, the applicant may be granted a license if at least 					ninety days have elapsed since the date the revocation became effective.

Sec 802.12	Transfer of License

A. 	An amusement arcade license is not transferable from one licensee to another or 				from one location to another.  Any purported transfer of an amusement arcade 					license shall automatically and immediately revoke that license.

B.	A skill-based amusement machine or amusement device license is not 						transferable from one machine or device to another or to a machine or device 					moved to a different location.  Any purported such transfer of a license shall 					automatically and immediately revoke that license.

Sec. 802.13	Inspection

A.	The Sheriff’s Department shall, from time to time, inspect that portion of the 					arcade business open to the public licensed hereunder in order to assess 						compliance with the provisions of this Article.

B.	The Zoning Inspector shall, from time to time, request the Tuscarawas County 					Health Department to inspect that portion of each arcade business open to the 					public and licensed hereunder, in order to assess compliance with the provisions 					of this Article and relevant Health Department regulations.

C.	That the Fire Marshall and Building Official shall, from time to time, and at least 					four times a year, inspect that portion of the arcade business open to the 						public and licensed hereunder in order to assess compliance with all applicable 					fire, building, and zoning code regulations.

Sec. 802.14	Nuisance

A violation of this Article shall constitute a nuisance and is subject to civil proceedings, 				including an injunction, in addition to prosecution for criminal violations of the State of 				Ohio and the Zoning Resolution of Lawrence Township.

Sec. 802.15	Effect of Partial Invalidity

If any section, subsection, or clause of this Article shall be deemed to be 						unconstitutional or otherwise invalid, the validity and enforcement of the remaining 				sections, subsections, and clauses shall not be affected.

Sec. 802.16	Fee Collected

All license fees collected shall be used for criminal investigations, police weapons, 				training and equipment, electronic and gaming experts, consultants, and other costs 				related to gaming activities.  Up to twenty five percent of such fees collected shall be 				allocated to the Fire Department for training.

Sec. 802.17	Penalty

Whoever violates or fails to comply with any of the provisions of this Article, for which 				no penalty is otherwise provided, is guilty of a second-degree misdemeanor and shall be 			subject to the penalties set forth in O.R.C. Section 2929.  A separate offense shall be 				deemed committed each day during or on which a violation or noncompliance occurs.

ARTICLE IX

WELLHEAD PROTECTION OVERLAY DISTRICTS

Sec. 900 	“WP-1” INNER ZONE OVERLAY DISTRICT

Sec. 900.1	Purpose

The “WP-1” Inner Zone Overlay District is designed to safeguard the public health, 				safety, and welfare of citizens and institutions that are customers of the Tuscarawas 				County Metropolitan Sewer District (hereinafter referred to as County) – Wilkshire Hills 				Community water system by regulating future land use and storage, handling, use, or 				production of regulated substances within the zone of influence.  The intent of this 				designation is to protect the community’s potable water supply against contamination.  				This zone consists of land within the one-year time of travel zone as designated by the 				Ohio Environmental Protection Agency and shown on the Wellhead Protection Zone 				map located in the official Zoning Maps, held at the Lawrence Township offices.  Also 				see Appendix B.

Sec. 900.2	Permitted Uses

The permitted uses within the “WP-1” Inner Zone Overlay District are restricted to 				public utilities, roads and county water supply and treatment facilities.

Sec. 900.3	Conditional Use

The conditional uses within the “WP-1” Inner Zone Overlay District are:

A.	Agriculture

Use is restricted based on crops grown, tillage practices, and chemicals used for 					fertilizer, herbicides and pesticides.  All proposed conditional uses must be 					approved by the County.

B.	Residential housing

Use restricted to dwelling units where public water and sewer service is utilized.

Sec. 900.4	Lot, Height, Bulk, Yard, and Buffer Regulations

The lot size requirements, bulk regulations and buffer strip requirements for the “WP-1” 			Inner Zone Overlay District shall conform to the requirements of the use permitted.

Sec. 901	“WP-2” Outer Zone Overlay District

Sec. 901.1	Purpose

The “WP-2” Outer Zone Overlay District is designed to safeguard the public health, 				safety, and welfare of citizens and institutions that are customers of the county water 				system by regulating the land use and the storage, handling, use, and production of 				regulated substances.  The land within the outer protection zone is described as the 				land area within Lawrence Township which lies within the five year and ten-year 					time of travel areas as designated by the Ohio Environmental Protection Agency (OEPA) 				and shown on the Wellhead Protection Zone (WP-2) map located in the Zoning maps 				held at the Lawrence Township offices.   The intent of this designation is to protect the 				community’s potable water supply against contamination.  Also see Appendix C.

Sec. 901.2	Applicability to Underlying Zoning Districts

The provisions of Section 901.1 through 901.10, inclusive, shall be applicable to all lands 			shown as being located within the boundaries of the “WP-2” Outer Zone Overlay District 			on the zoning map and shall be supplemental to the regulations of the underlying zoning 			district.  Where the requirements of this section are in conflict with the regulations of 				the underlying district, the more restrictive regulations shall apply.

Sec. 901.3	Permitted Uses, Bulk and Yard Regulations

The permitted uses bulk and yard regulations within the “WP-2” Outer Zone Overlay 				District shall be those of the underlying zoning district.

Sec. 901.4	Conditional Uses

The following uses are conditional uses within the “WP-2” Outer Zone Overlay District:

A.	The conditional uses with the “WP-2” Outer Zone Overlay District shall be those 					of the underlying zoning district, except as specified in Sections 901.1 and 901.5.

B.	The excavation, extraction, mining, or processing of sand, gravel, and limestone 					from the earth for resale shall remain as Conditional Uses in the “WP-2” Outer 					Zone Overlay District subject to Board of Zoning Appeals approval of an 						excavation and facilities plan, prepared in accordance with all applicable 						requirements of OAC 1501:14 and ORC Title 15, Chapter 1514, that includes, but 					is not limited to:

1.	An existing site plan with topographic detail at two feet contour 							intervals, all planimetric information, depth to ground water and flood 						plain characteristics where applicable.

2.	The proposed extent and depth of excavation.

3.	Slope angle of excavation walls (any final slopes shall be at the angle of 						repose for the remaining material).

4.	Use and disposition of the soil and/or overburden materials from the 						excavations including a landscaping and vegetation plan to stabilize any 						disturbed material.

5.	Surface drainage plan:

a.	Drainage into on-site excavations from proximate off-site 								transportation facilities such as roadways and roadbeds and off-							site watercourses is prohibited unless the applicant provides a 							plan which otherwise protects the excavations from off-site 							waterborne regulated substances

b.	The final on-site grading shall minimize all surface drainage in the 							excavations

6.	A post-excavation and operation land use plan; or restoration of site to 						conditions at a minimum similar to those prior to mining activates

7.	A security plan (unauthorized access shall be strictly prohibited as long as 						any excavations remain on-site)									C.	Oil and gas wells

1.	Existing gas and oil wells must be maintained in compliance with the 						requirements of OAC 1501:9 and ORC Title 15, Chapter 1509:1

2.	Installation of new oil and gas wells shall remain as conditional uses in 						the “WP-2” Outer Zone Overlay District subject to Board of Zoning 						Appeals approval of a facilities plan, prepared in accordance with all 						applicable requirements of OAC 1501:9 and ORC Title 15, Chapter 1509.1.

Sec. 901.5	Prohibited Uses

A.	Sanitary landfills, construction demolition and debris landfills and their facilities 					as defined per OAC 3745, underground injection wells, drywells, or other non-					approved matter, junkyards and new or replacement underground fuel/chemical 				storage tanks (UST) are prohibited within the “WP-2” Outer Zone Overlay 						District.  Existing USTs are permitted so long as their construction, leak detection 					and maintenance programs meet all local, state, and/or federal UST rules and 					regulations.

B.	The bulk storage of any hazardous material except if used for agricultural 						purposes, in which case it may not be stored for a period greater than thirty 					days.  The storage of any hazardous material must be reported to the County.

Sec. 901.6	Best Management Practices and Exemptions

A.	To the maximum extent practical, owners and operators within all Wellhead 					Protective Overlay Districts shall implement best management practices (BMPs) 					to reduce risk of release and pollution of the environment.  BMPs are defined as 					“schedules of activities, prohibition of practices, maintenance procedures, and 					other management practices to prevent or reduce the risk of a release”.

1.	 BMPs include, but are not limited to, the following:									a.	Treatment techniques											b.	Operating Procedures												c.	 Practices to control runoff, spills, and leaks

2.	Spill control measures shall include, but are not limited to, the following:

a.	Secondary and tertiary containment systems, including the use of 							containment during chemical storage, transfer, and use.  The 							containment system shall be designed to meet the most recent 							requirements of the state or federal government.

b.	Adopting standardized spill response protocols and providing 							training to employees to help ensure response protocols are 							enacted if a spill or release occurs.

c.	Drums and other types of containers holding Regulated 								Substances and wastes of such substances shall be stored within 							secondary containment.

B.	Exiting containment systems and procedures shall not be removed, nor shall 					their ability to contain spills be compromised, so long as regulated substances 					are stored, transferred or used within the containment areas.  Containment 					systems shall be maintained in good working order.  Improvements and/or 					additions to containment systems may be performed so long as the ability to 					contain a spill is not compromised.  Temporary approved containment systems 					may be required during maintenance and/or improvement activities.

C.	The following types of products and volumes are exempt from BMP 						requirements within the “WP-1” Inner Zone Overlay District:

1.	Cleaning agents packaged for personal or household use

2.	Prescription medications for individual use

3.	Construction materials stored at or being transported to a permitted 						construction site which do not pose real or present danger of causing 						contamination

4.	Office supplies packaged for personal or office use

5.	Refrigerants contained in on-site cooling equipment or contained in 						household appliances, including refrigeration repair service storage 						vessels

6.	The through transport of a regulated substance, so long as the 							transporting vehicle is not stopped within the “WP-1” Inner Zone Overlay 						District for longer than two hours or the “WP-2” Outer Zone Overlay 						District for longer than twelve hours.

7.	Properly maintained liquid filled transformers

8.	Motor vehicle fuels and other liquids that are stored on and are an 						integral part of an operable motor vehicle or boat and used specifically 						and solely for the operation of the vehicle in which the substances are 						contained in conjunction with permitted or approved conditional use.  						This does not include the tanker portion of a tractor-trailer or similarly 						purposed vehicle.

9.	Heating oil for residential uses stores in tanks with a total capacity of less 						than five hundred gallons per residential lot

10.	Agricultural BMP per tillage practices, crop and/or

11.	Fertilizer, Herbicide, Pesticide

D.	Substances to be regulated, hereinafter referred to as regulated substances, are 					chemicals and mixtures of chemicals, which are health hazards.  Regulated 					substances include:

1.	Chemicals for which there is scientific evidence that acute or chronic 						health effects may result from exposure including carcinogens, toxic and 						highly toxic agents, reproductive toxins, irritants, corrosives, sensitizers, 						hepotoxins, nephrotoxins, neurotoxins, agents which act on the 							hematopoietic system and agents which damage the lungs, skin, eyes, or 						mucous membranes.

2.	Mixtures of chemicals, which have been tested as a whole, and have 						been, determined to be a health hazard.

3.	Ingredients of mixtures prepared within the “WP-1” Inner Zone Overlay 						District in cases where such ingredients are health hazards but comprise 						less than one-tenth percent of the mixture (on a weight per unit 							weight basis) if carcinogenic or less than one percent of the mixture 						(on a weight per unit weight basis) if noncarcinogenic.

4.	Petroleum and nonsolid petroleum derivatives (except non-PCB dielectric 					fluids).

E.	Wastewater Disposal

All uses within the Wellhead Protective Overlay Districts shall be connected to 					the public wastewater disposal system, if or when available.  If not available, all 					on-lot wastewater disposal systems shall be inspected and approved by the 					Tuscarawas County General Health District to meet criteria for on-lot wastewater 				disposal systems located within a wellhead protection area.

NOTE:  THIS REGULATION DOES NOT RESTRICT THE USE OF AGRICULTURAL CHEMICALS APPLIED IN 	ACCORDANCE WITH BEST MANAGEMENT PRACTICES AND/OR LABEL DIRECTIONS.  THE AVAILABLE 	BEST MANAGEMENT PLAN MUST BE SUBMITTED FOR REVIEW BY THE COUNTY ANNUALLY.

Sec. 901.7	Enforcement

A.	Enforcement Provisions

The enforcement provisions within Section 901.7 through Section 901.10, 					inclusive, of this Zoning Resolution are incorporated herein by reference and 					apply to all land within the “WP-1” Inner Zone Overlay District and the “WP-2” 					Outer Zone Overlay District, as defined.

B.	Management of Regulated Substances

No person shall place, deposit, or permit to be deposited, store, process, use, 					produce, dispose of, transport, or discharge, hereinafter referred to as “handle”, 					any regulated substance on public or private property within the “WP-2” Outer 					Zone Overlay District except as provided by law, statue, ordinance, rule or 					regulation.  With the exception of single-family or two-family residences wherein 				the regulated substances are for the normal and customary maintenance of the 					residence or vehicles under control of the occupant, the use of any land, 						building, or structure in the “WP-2” Outer Zone Overlay District or the “WP-1” 					Inner Zone Overlay District in which any regulated substances are handled and 					for which an occupancy certificate has not been issued is hereby determined to 					be a nuisance and must be abated.

C.	Public Water Supply Protection Authority

1.	If any activity or use of regulated substance is deemed by the Zoning 						Inspector to pose a real and present danger of contaminating surface 						and/or groundwater, which would normally enter the public water 						supply, the Zoning Inspector, is hereby authorized to:

a.	Cause cessation of said activity or use of the regulated substances

b.	Require the provision of administrative controls and/or facilities 							sufficient to mitigate said danger; and/or

c.	Cause the provision of pollution control and/or abatement 							activities.

2.	When considering the exercise of any of the above authorities or actions, 						the Zoning Inspector shall consult with the County Sanitary Engineer 						and/or Director.  Such consultation shall determine what measures need 						to be taken to ensure the public water supply is reasonably and 							adequately protected from contamination for the present and the future.  					The Zoning Inspector may take into consideration any evidence 							represented by the entity regarding cost effectiveness and the economic 						effectiveness and the economic impact imposed by the requirements or 						actions.

D.	Inspections

Subject to applicable provisions of law, the Zoning Inspector or authorized 					designee bearing proper identification shall be permitted to enter private 					property at any reasonable time, with reasonable cause or with prior 						notification, for such purposes as inspection, observation, measurement, 						sampling, and records examination pertaining to the requirements of this 					section.  Upon request of the entity which is the subject of the inspection, and if 					permitted by the state public records law, information obtained as a result of the 				inspection shall be maintained as confidential.  If the owner or tenant does not 					consent to the entry of the Zoning Inspector, the Zoning Inspector may apply to a 				court of competent jurisdiction for an appropriate warrant or other authority to 					enter said property; but no consent is necessary for entry into areas then open 					to the public or to customers.

E.	Technical Consultants

Upon application for a zoning certificate and/or occupancy certificate for a use 					within this outer protection zone, the Zoning Inspector may employ such 						technical expertise as needed, which may include, but not limited to, geologists, 					hydro-geologists, engineers, agronomists and soils scientists, to ensure 						compliance with the provision of these regulations.  All reasonable costs, as 					determined by the Lawrence Township Zoning Board, incurred in the compliance 				review process shall be passed through to the applicant and shall be in addition 					to those fees normally charged to review an application for a zoning certificate 					and/or occupancy certificate.

Sec. 901.8	Reporting Regulated Substance Spills, Leaks, or Discharges

A.	Notification Required

Any person with direct knowledge of a spill, leak or discharge of a regulated 					substance, in any volume, that escapes containment or contacts a pervious 					ground surface within either of the Wellhead Protection Zone Overlay Districts 					and such spill, leak or discharge is not immediately and completely remediated, 					shall give notice to the County by telephone within thirty minutes.  The 						notification shall include at a minimum, a location of the incident, name and 					telephone number, date and time thereof, type of substance(s), concentration 					and volume, and control or corrective action taken.  Such notification shall in no 					way alleviate other local, state, and federal reporting obligations as required by 					law.

B.	Application of Agriculture Chemicals Not a Spill, Leak or Discharge

The application of agricultural chemicals, fertilizers, mineral acids, organic sulfur 					compounds, etc. used in routine agricultural  operations, including plant 						nutrients and crop protection materials, applied under Best Management 					Practices as indicated by soil tests, agricultural experts, or label directions 					approved by the United States EPA or the Ohio Department of Agriculture, shall 					not be considered a spill, leak, or discharge subject to the reporting provisions of 				this paragraph.

C.	Liability and Required Documentation

Any entity or person who spills, leaks or discharges said substance(s) shall be 					liable for any reasonable expense, loss or damages incurred by the County in 					response to such an incident, in addition to the amount of any fines imposed on 					account thereof under state and federal law.  Said entity or person shall 						document and maintain sufficient records so as to reflect accurately the 						circumstances related to any such incident and develop and implement 						procedures to substantially eliminate the likelihood of reoccurrence of such 					spills, leaks or discharges as soon as practicable following the incident, but no 					later than one hundred eighty days after the incident.

D.	Falsifying Information

No person shall make any false statement, representation, or certification in any 					report or other document filed or required to be maintained pursuant to this 					section.  Any person who knowingly withholds relevant information will be 					subject to prosecution.

Sec. 901.9	Exemption of Regulated Substances

The Zoning Inspector is authorized to exclude certain regulated substances that pose no 				threat to ground water, from the provisions of these regulations.  Prior to authorizing 				the exemption of any regulated substance, the Zoning Inspector shall have such request 				for exemption reviewed by the County.

Sec. 901.10	Clean Closure Requirements

Except in the case of a seasonal discontinuation of operation, the owner or operator of 				any non-residential property that becomes unoccupied or has discontinued operation 				for a period of ninety consecutive days shall remove all regulated substances from the 				property other than those used exclusively for heating, cooling, and providing electrical 				lighting for the premises within ninety days after the date upon which the property  				initially became unoccupied or the operation discontinued per OAC 3745-352 and ORC 				3752, Cessation of Regulated Operation (CRO).  The County will be provided with OEPA 				inspection reports documenting applicable CRO regulations have been met.  Except as 				noted above, regulated substances that are excluded from reporting requirements shall 				be removed by the date specified above.  The owner or operator shall secure the 				regulated substances on the property until they have been removed.  The owner or 				operator shall notify the County Sanitary Engineer in writing of the date of the cessation 				of operation or the property becoming unoccupied no later than the day upon which 				the operation actually ceases or the property becomes unoccupied, and such 					notification shall include the owner’s name, phone number, and address and the 				operator’s name, phone number and new address.

ARTICLE X

ADMINISTRATION AND ENFORCEMENT

Sec. 1001	Administration

Sec. 1001.1	Zoning Inspector

A Zoning Inspector designated by the Board of Township Trustees shall administer and 				enforce this resolution.  He may be provided with the assistance of such other persons 				as the Board of Township Trustees may direct.  If the zoning inspector shall find that any 			of the provisions of this resolution are being violated, he shall notify in writing the 				persons responsible for such violations, indicating the nature of the violation and 				ordering the action necessary to correct it.  He shall order discontinuance of illegal use 				of land, buildings, or structures; removal of illegal buildings or structures or of illegal 				additions, alternations, or structural changes discontinuance of any illegal work being 				done; or shall take any other action authorized by this resolution to ensure compliance 				with or to prevent violation of its provisions.

Sec. 1001.2	Submission of Application

All applications for zoning certificates shall be submitted to the Zoning Inspector who 				may issue zoning certificates when all applicable provisions of this Resolution have been 			met.  All applications for Conditional Use Zoning Certificates shall be made to the Zoning 			Inspector and submitted to the Board of Zoning Appeals which may issue Conditional 				Use Zoning Certificates in accordance with Article VI of this Resolution.

Sec 1001.3	Zoning Certificates Required

A.	Before constructing, changing the use of, or altering any buildings, or changing 					the use of any premises, application shall be made to the Zoning Inspector for a 					zoning certificate.  Failure to obtain a zoning certificate shall be a violation of this 				resolution.  The application shall include the following information:

1.	A plot plan drawn to scale showing the exact dimension of the lot to be 						built upon including size and location of any existing structures.

2.	The location, dimensions, height and bulk of structures to be erected.

3.	The intended use.

4.	The yard, open area and parking space dimensions.

5.	Evidence that the responsible health authority has approved the 							proposed sanitary sewage disposal facilities for the use for which the 						zoning certificate has been requested.

6.	Any other pertinent data as may be necessary to determine and provide 						for the enforcement of this Resolution.

7.	For commercial and industrial facilities, two complete sets of building 						plans stamped and approved by the State of Ohio shall be provided when 					applying for a building permit.

B.	No zoning certificate shall be granted to build any structure within the confines 					of the unincorporated area of the Township which has ingress and egress to the 					highway until the owner of such property has secured a permit from the State 					Department of Transportation, the county Engineer, or the proper Township 					official (whichever authority has jurisdiction), for permission to install a culvert 					of the proper size and specifications required by said State Department of 					Transportation, County Engineer, or proper local Township official, (whichever 					authority has jurisdiction).

C.	Within ten days after receipt of application the Zoning Inspector shall issue a 					zoning certificate if the application complies with the requirements of this 					Resolution and the application is accompanied by the proper fee as indicated in 					Section 1001.4.

D.	The zoning certificate shall become void at the expiration of twelve months after 				date of issuance unless construction is started.   If work is not completed within 					one year of start of construction, a new permit must be obtained.

Sec. 1001.4	Fees

A.	Any application for an amendment to this resolution, request for a zoning 					certificate, review of a planned development proposal, request for a variance, or 					other request for other action pursuant to the regulations set forth in this 					resolution shall be accompanied by a fee.  Fees shall be established by resolution 				of the Board of Trustees.

B.	Fees for a conditional zoning certificate shall be established by the Board of 					Trustees in consultation with the Board of Zoning Appeals.

C.	Current fees are available at the township building at the Fiscal Officer’s office.

D.	The fees for a nonconforming zoning certificate shall be determined by the 					Township Trustees.

E.	When a Board or Commission finds it necessary to maintain a strict record of 					public hearing procedures, or when a Board or Commission had deemed it 					necessary to require special studies to be made, the applicant shall bear all direct 				and related costs.

F.	Failure to Acquire Zoning Certificate.  An amount equal to double the normal 					schedule fee shall be assessed for failure to acquire any or all Zoning Permit(s) as 				required by this resolution.

Sec. 1001.5	Board of Zoning Appeals

The Board of Township Trustees of any township proceeding under Sections 519.01 to 				519.99 of the Ohio Revised Code shall create and establish a Township Board of Zoning 				Appeals.  The Board of Zoning Appeals shall be composed of five members who reside in 			the unincorporated area of the township, to be appointed by the board.  The Board of 				Township Trustees may appoint two alternate members to the township Board of 				Zoning Appeals, for terms to be determined by the Board of the Township Trustees.  An 				alternate member shall take the place of an absent regular member at any meeting of 				the township Board of Zoning Appeals, according to procedures prescribed by resolution 			by the Board of Township Trustees.  An alternate member shall meet the same 					appointment criteria as a regular member.  When attending a meeting on behalf of an 				absent member, the alternate member may vote on any matter on which the absent 				member is authorized to vote.

A.	Powers and Duties

The Board of Zoning Appeals shall have the following powers:

1.	To hear and decide appeals where it is alleged there is any order, 							requirement, decision, or determination made by an administrative 						official in the enforcement of Section 519.01 to 519.99 inclusive of the 						Revised Code, or of this Resolution which is adopted pursuant thereto.

2.	To authorize, upon appeal, in specific cases, such variance from the terms 					of the Zoning Resolution as will not be contrary to the public interest, 						where, owning to special condition, a literal enforcement of the 							Resolution will result in unnecessary hardship, and so that the spirit of 						the Resolution shall be observed and substantial justice done.

3.	To grant Conditional Use Zoning Certificates for the use of land, buildings 						or other structures if such certificates for specific uses are provided for in 						the Zoning Resolution.

In exercising the above-mentioned powers, such Board may, in 							conformity with such sections, reverse or affirm, wholly or partly or may 						modify the order, requirements, decision or determination as ought to be 					made, and to that end shall have all powers of the officer from whom the 					appeal is taken.

4.	An appeal stays all proceedings in furtherance of the action appealed 						from, unless the Zoning Inspector from whom the appeal is taken 							certifies to the Board of Zoning Appeals after the notice of appeal is filed 						with him, that by reason of facts stated in the certificate, a stay would, in 						his opinion, cause imminent peril to life and property.  In such case 						proceedings shall not be stayed other than by a restraining order which 						may be granted by the Board of Zoning Appeals or by a court of record on 					application, on notice to the Zoning Inspector from whom the appeal is 						taken and on due cause shown.

B.	Appeals and Applications

Appeals to the Board of Zoning Appeals may be taken by any person aggrieved or 				by an officer of the township affected by any decision of the administrative 					officer.  Such appeal shall be taken within twenty days after the decision, by 					filing with the officer from whom the appeal is taken and with the Board of 					Zoning Appeals, a notice of appeal.  All appeals and applications made to the 					Board of Zoning Appeals shall be in writing and on forms prescribed therefore. 					Every appeal or application shall refer to the specified provision of the 						Resolution, and shall set forth the interpretation that is claimed by the Board, 					the details of the variance that is applied for and the grounds on which it is 					claimed that the variance should be granted as the case may be.  Every decision 					of the Board shall be by resolution, each of which shall contain a full record of 					the findings of the Board of Zoning Appeals by case number under one or 						another of the following headings:

1.	Interpretation

2.	Conditional Use Zoning Certificates

3.	Variance

C.	Variances

To authorize upon appeal in specific cases such variance from the terms of this 					resolution as will not be contrary to the public interest, where owing to special 					conditions, a literal enforcement of the provisions of this resolution would result 					in unnecessary hardship.  No nonconforming use of neighboring lands, 						structures, or buildings in the same district and no permitted nonconforming use 				of lands, structures, or buildings in other districts shall be considered grounds for 				the issuance of a variance.  A variance from the terms of this resolution shall not 					be granted by the board of zoning appeals unless and until:

1.	A written application for a variance is submitted demonstrating:

a.	That special conditions and circumstances exist which are peculiar 						to the land, structure, or building involved and which are not 							applicable to other lands, structures, or buildings in the same 							district

b.	That literal interpretation of this resolution would deprive the 							applicant of rights commonly enjoyed by other properties in the 							same district under the terms of this resolution

c.	That the special conditions and circumstances do not result from 							the actions of the applicant

d.	That granting the variance requested will not confer on the 							applicant any special privilege that is denied by this resolution, to 							other lands, structures, or buildings in the same district

2.	Notice of public hearing shall be given as in Section 601.4E of this 							resolution.

3.	The public hearing shall be held.   Any party may appear in person, or by 						agent or attorney.

4.	The Board of Zoning Appeals shall make findings that the requirements of 					Section 1001.5D or 1001.5E have been met by the applicant for variance.

5.	The Board of Zoning Appeals shall further make a finding that the reasons 					set forth in the application justify the granting of the variance, and that 						the variance is the minimum variance that will make possible the 							reasonable use of the land, building, or structure.

6.	The Board of Zoning Appeals shall further make a finding that the 							granting of the variance will be in harmony with the general purpose and 						intent of this resolution and will not be injurious to the neighborhood, or 						otherwise detrimental to the public welfare.

D.	Use Variance Standard: Unnecessary Hardship

1.	The variance requested stems from a condition which is unique to the 						property at issue and not ordinarily found in the same zone or district.

2.	The hardship condition is not created by actions of the applicant.

3.	The granting of the variance will not adversely affect the rights of 							adjacent owners.

4.	The granting of the variance will not adversely affect the public health, 						safety, or general welfare.

5.	The variance will be consistent with the general spirit and intent of the 						zoning code.

6.	The variance sought is the minimum which will afford relief to the 							applicant.

7.	There is no other economically viable use which is permitted in the 						zoning district.

E	Non-Use Variance Standard:  Practical Difficulty

1.	The property in question will not yield a reasonable return without a 						variance or there cannot be any beneficial use of the property without 						the variance.

2.	The variance is not substantial.

3.	The essential character of the neighborhood is not substantially altered 						and the adjoining properties will not suffer substantial detriment as a 						result of the variance.

4.	The variance will not adversely affect the delivery of governmental 						services.

5.	The property owner’s predicament feasibly cannot be obviated through 						some method other than a variance.

6.	The spirit and intent behind the zoning requirement is observed and 						substantial justice done by granting a variance.

F.	Stay of Proceedings

An appeal to the Board of Zoning Appeals prevents further enforcement of the 					decision that is being appealed, unless the Zoning Inspector certifies to the 					Board of Zoning Appeals after the notice of appeal is filed that by reason of facts 					stated in writing to the Board of Zoning Appeals that a stay, would, in the 						opinion of the Zoning Inspector, cause imminent peril to life or property.  In such 				case, proceedings shall not be stayed other than by an injunction granted by the 					court of Common Pleas.

Sec. 1001.6	Zoning Commission – Powers and Duties

The Board of Township Trustees of any township proceeding under Sections 519.01 to 				519.99 of the Ohio Revised Code shall create and establish a Township Zoning 					Commission.  The Commission shall be composed of five members who reside in the 				unincorporated area of the township, to be appointed by the board.  The Board of 				Township Trustees may appoint two alternate members to the township Zoning 					Commission, for terms to be determined by the Board of the Township Trustees.  An 				alternate member shall take the place of an absent regular member at any meeting of 				the township Zoning Commission, according to procedures prescribed by resolution by 				the Board of Township Trustees.  An alternate member shall meet the same 					appointment criteria as a regular member.  When attending a meeting on behalf of an 				absent member, the alternate member may vote on any matter on which the absent 				member is authorized to vote.

A.	Develop the Zoning Resolution and Map

1.	The Township Zoning Commission shall submit a plan, including 							both text and maps, representing the recommendations of the zoning 						commission for the carrying out by the board of township trustees of 						powers, purposes, and provisions set forth in Section 519.01 to 519.99, 						inclusive, of the Revised Code, including additions to territory in which a 						township zoning plan is in effect.

2.	The Zoning Commission, may within the limits of the money							appropriated by the Board for the purpose, employ or contract with such 						planning consultants and executive and other assistants as it deems 						necessary.  The Zoning Commission, shall organize, adopt rules for the 						transaction of its business and keep a record of its actions and 							determinations.  Members of the Zoning Commission may be allowed 						their expenses, or such compensation, or both, as the Board may approve 					and provide.  No township trustee shall be employed by the zoning 						commission of his township.

a.	The Zoning Commission shall make use of such information and 							counsel as is available from appropriate pubic officials, 								departments, and agencies and such officials, departments, and 							agencies having information, maps and data pertinent to 								township zoning shall make them available for the use of the 							zoning commission.

b.	In any county where there is a county or regional planning 								commission, the Zoning Commission may request such planning 							commission to prepare or make available to the Zoning 								Commission a zoning plan, including text and maps, for the 							unincorporated area of the township or any portion of the same.

B.	Hold Public Hearings on Zoning Amendments and Make Recommendations to 					the Trustees

Before certifying its recommendations of a zoning plan to the Board of Township 				Trustees, the township Zoning Commission shall hold at least one public hearing, 				notice of which shall be given by one publication in one or more newspapers of 					general circulation in the township at least thirty days before the date of such 					hearing.  The notice shall state the place and time at which the text and maps of 					the proposed zoning resolution may be examined.

C.	Review Matters Referred by the Board of Trustees

Amendments to the Zoning Resolution may be initiated by motion of the 						Township Zoning Commission, by the passage of a resolution by the Board of 					Township Trustees, or by the filing of an application by one or more of the 					owners or lessees of property within the area proposed to be changed or 						affected by the proposed amendment with the Township Zoning Commission.  					The Board of Township Trustees may require that the owner or lessee of 						property filing an application to amend the zoning resolution pay a fee to defray 					the cost of advertising, mailing, filing with the county recorder, and other 						expenses.  If the township trustees require such a fee, it shall be required 						generally for each application.  The Board of Township Trustees shall upon the 					passage of such resolution certify it to the Township Zoning Commission.

D.	Make Reports on the Operation of the Zoning Resolution.

E.	Approve Proposed Development within Planned Unit Developments

1.	A township zoning resolution or amendment adopted in accordance with 						this article may establish or modify planned-unit developments.  Planned-						unit development regulations shall apply to property only at the election 						of the property owner and shall include standards to be used by the 						Board of Township Trustees, or if the board so chooses, by the Township 						Zoning Commission, in determining whether to approve or disapprove 						any proposed development within a planned-unit development.  The 						planned-unit development shall further the purpose of promoting the 						general public welfare, encouraging the efficient use of land and 							resources, promoting greater efficiency in providing public and utility 						services and encouraging innovation in the planning and building of all 						types of development.  Within a planned-unit development, the township 					zoning regulations, where applicable, need not be uniform, but may vary 						in order to accommodate unified development and to promote the public 					health, safety, morals and other purposes of this section.

2.	Pursuant to Section 519.12 of the Revised Code, the Board of Township 						Trustees may adopt planned-unit development regulations and amend 						the zoning map to rezone property as planned-unit developments.  Any 						other zoning regulations and zoning district that exist at the time a 						planned-unit development district is established under this division 						continue to apply within the planned-unit development district unless the 					Board or the Township Zoning Commission approves an application of an 						owner of property within the district to subject the owner’s property to 						planned-unit development regulations under this division.  Such an 						application shall be made in accordance with the planned-unit 							development regulations and shall include a development plan that 						complies with the planned-unit development regulations.  Upon receiving 					such an application, the Board of Township Trustees or Township Zoning 						Commission, as applicable, shall determine whether the application and 						plan comply with the planned-unit development regulations.  The Board’s 					or Commission’s determination shall not be considered to be an 							amendment to a township zoning resolution for purposes of Section 						519.12 of the Revised Code, but may be appealed pursuant to Chapter 						2506, of the Revised Code.  If the Board or Commission makes a final 						determination that the plan included in the application complies with the 					planned-unit development regulations or, if the Board’s or Commission’s 						final determination is one of non-compliance then if a court of 							competent jurisdiction makes a final non-appealable order finding 						compliance, the Board or Commission , as applicable, shall approve the 						application and upon approval shall cause the zoning map to be changed 						so that any other zoning district that applied to the property that is the 						subject of the owner’s application no longer applies to that property.  The 					removal of the prior zoning district from the zoning map is a ministerial 						act and shall not be considered to be an amendment or supplement to a 						township zoning resolution for the purposes of Section 519.12 of the 						Revised Code and may not be appealed pursuant to Chapter 2506 of the 						Revised Code.

F.	Similar Use Provision

1.	Where there is a proposed use that is not currently listed in this 							Resolution, the Zoning Commission may review the use to determine the 						appropriate zoning districts, if any, where the use may be permitted.

2.	The Zoning Commission should consider the nature, operation, and 						function of the use in its determination of an appropriate district and 						user’s similarity to other uses permitted in the respective Zone District.

3.	The Zoning Commission may find that the use is not compatible with any 						existing zoning district and not permit the use under the current 							Resolution or, as an alternative, the Zoning Commission may request that 					a new district or new provisions be adopted, through the zoning text and 						map amendment procedure, pursuant to Section 1001.8.

Sec. 1001.7	Board of Trustees – Powers and Duties

A.	Adopt the Zoning Resolution

After receiving the recommended zoning plan from the township zoning 						commission and hold the public hearing provided for by Section 519.08 of the 					Revised Code, the Board of Township Trustees shall consider such 							recommendations and vote upon the adoption of the zoning regulations.

B.	Adopt the Zoning Text and Map Amendments

1.	No change in or departure from the text or maps, as certified by the 						Township Zoning Commission, shall be made by the Board of Township 						Trustees unless it is first resubmitted to the commission for approval, 						disapproval, or suggestions.  Upon receipt of the recommendations of the 					Township Rural Zoning Commission regarding the proposed changes, the 						Board of Township Trustees shall hold a second public hearing, at least 						ten-day notice of the time and place of which shall be given by one 						publication in one or more newspapers of general circulation in 							the township affected.  If such changes are disapproved by the Zoning 						Commission, the provision so disapproved must receive the favorable 						vote of the entire membership of the Board of Township Trustees in 						order to be adopted.

2.	Within twenty days after such public hearing, the board shall either adopt 					or deny the recommendations of the Zoning Commission or adopt some 						modification of them.  If the Board denies or modifies the 								recommendation of the Township Zoning Commission, the unanimous 						vote of the Board shall be required.

3.	Such amendment adopted by the Board shall become effective in thirty 						days after the date of such adoption, unless, within thirty days after the 						adoption of the amendment, there is presented to the Board of Township 					Trustees a petition, signed by a number of registered electors residing in 						the unincorporated area of the township or part of that unincorporated 						area included in the zoning plan equal to not less than eight percent of 						the total vote cast for all candidates for governor in such area at the most 					recent general election at which a governor was elected, requesting the 						Board of Township Trustees to submit the amendment to the electors of 						such area for approval or rejection at a special election to be held on the 						day of the next primary or general election.  Each part of this petition 						shall contain the number and the full and correct title, if any, of the 						zoning amendment resolution, motion, or application, furnishing the 						name by which the amendment is known and a brief summary if its 						contents.  In addition to meeting the requirements of this section, each 						petition shall be governed by the rules specified in Section 3501.38 of the 					Revised Code.

C.	Set Permit and Application Fees

The Township Trustee shall set the permit and application fees as needed.  These 				fees shall be kept in the Township Clerk’s office, and may be reviewed there.  					These fees may be amended as needed without prior notice.

D.	Appoint Zoning Commission and Board of Zoning Appeals Members

1.	The Board of Township Trustees of any township proceeding under 						Sections 519.01 to 519.99 of the Revised Code shall create and establish a 					Township Zoning Commission.  The Commission shall be composed of 						five members who reside in the unincorporated area of the township, 						to be appointed by the Board.  The Board of Township Trustees may 						appoint two alternate members to the Township Zoning Commission, for 						terms to be determined by the Board of Township Trustees.  An alternate 					member shall take the place of an absent regular member at any meeting 					of the Township Zoning Commission, according to procedures prescribed 						by resolution by the Board of Township Trustees.  An alternate member 						shall meet the same appointment criteria as a regular member.  When 						attending a meeting on behalf of an absent member, the alternate 						member may vote on any matter on which the absent member is 							authorized to vote.

2.	The Board of Township Trustees of any township proceeding under 						Section 519.01 to 519.99 of the Revised Code shall create and establish a 						township Board of Zoning Appeals.  Refer to Section 1001.5 of this 						resolution to see the powers and duties of the Board of Zoning Appeals.

Sec. 1001.8	Amendments

A.	Amendments or supplements to this Resolution may be initiated by:

1.	Motion of the Township Zoning Commission

2.	Passage of a resolution by the Board of Township Trustees

3.	Filing of an application by one or more of the owners or leases of 							property within the area proposed to be changed or affected by the 						proposed amendment or supplement with the Township Zoning 							Commission

4.	At the time that an application for a change of zoning districts is filed with 					the Zoning Commission, as provided herein, a fee shall be paid to 							Lawrence Township for investigation, legal notices, and other expenses 						incidental to the determination of the zoning change.  Said fee shall be 						for one lot or part of one lot.  An additional fee shall be deposited for 						each additional lot or part of an additional lot or part of a lot to be 						adjacent to each other.

5.	The fee schedule shall be posted at the township office.

B.	Upon the adoption of such motion, certification of such resolution or filing of 					such application the Township Zoning Commission shall pursuant to Chapter 519 					of the Ohio Revised Code:

1.	Set a date for public hearing, which date shall be not less than twenty 						days or more than forty days after adopting, certifying or filing the 						initiating actions

a.	Notice of said hearing shall be given by the Township Zoning 							Commission by publication in one or more newspapers of 								general circulation in the Township at least ten days before 							the date of the hearing.  										2.	If the property to rezone or redistrict is less than ten parcels of land, 						notice shall be mailed by first class mail to all property owners within, 						contiguous to, or across the street or roadway from the area proposed to 					be changed at least ten days prior to the date of the hearing.  Said 						notice shall be mailed to such owner at the addresses appearing on the 						current tax duplicate of the county.  The failure of delivery of such notice 						shall not invalidate any amendment or supplement.

3.	Within five days after the adoption, certification or filing for 							amendment of supplement, the Township Zoning Commission shall 						transmit a copy thereof together with text and map pertaining thereto, to 					the Regional Planning Commission.

The Regional Planning Commission shall recommend the approval or 						denial of the proposed amendment or supplement, or the approval of 						some modification thereof and shall submit such recommendation to the 						Township Zoning Commission.  Such recommendation shall be 							considered at the public hearing on the proposed amendment or 							supplement.  Failure of the Planning Commission to make a 							recommendation shall in no way invalidate the proposed amendment or 						supplement.

C.	After such hearing, the Township Zoning Commission shall within thirty days, 					recommend approval or denial of the proposed amendment or supplement or 					the approval of some modification thereof and certify such recommendation 					together with the motion, resolution, application, and text and map pertaining 					thereto and the recommendation of the Regional Planning Commission to the 					Board of Township Trustees.

D.	The Board of Township Trustees shall upon receipt of such recommendation:

1.	Set a date for public hearing on such proposed amendment or 							supplement that shall be not more than thirty days from receipt thereof.

2.	Give notice of such public hearing by publication in one or more 							newspapers of general circulation in the township at least ten days 						before the date of such hearing in accordance with Section 519.12F and 						519.12G of the Ohio Revised Code, as applicable.

3.	Within twenty days after such public hearing, the Board shall either 						adopt or deny the recommendation of the Zoning Commission or adopt 						some modification thereof. In the event the Board denies or modifies the 						recommendation of the Zoning Commission; a unanimous vote of the 						Board shall be required.

E.	Effective Date, Referendum

1.	The amendment or supplement as adopted by the Board of Trustees shall 					become effective thirty days after the date of such adoption unless 						within thirty days after its adoption there is presented to the Board of 						Township Trustees a petition signed by a number of qualified voters 						residing in the unincorporated area of the Township or part of that 						unincorporated area included in the Zoning Plan equal to not less than 						eight percent of the total vote cast for all candidates for governor in such 						area at the last preceding general election at which a governor was 						elected, requesting the Board of Township Trustees to submit the 							amended or supplement at the next primary or general election.

2.	No amendment or supplement for which a referendum vote has been 						requested shall be put into effect unless a majority of the vote cast on 						the issue is in favor of the amendment or supplement.

3.	Upon certification by the Board of Election that the amendment has been 					approved by the voters, it shall be effective immediately.

SEC. 1001.9	 Permitted Use Certificate

A.	Prior to the occupancy of any nonresidential structure or nonresidential portion of an otherwise residential structure, a Permitted Use Certificate shall be obtained from the Zoning Inspector. Any change in use from one use to another, addition of a new use or from one owner/tenant to another shall require a new Permitted Use Certificate. A Permitted use Certificate shall be issued when all aspects of the building, structure or use comply with the zoning certificate previously issued.

B.	It shall be unlawful to use, occupy, or permit the use or occupancy of any building, premises, or both, or part thereof hereafter created, erected, changed, converted, or wholly or partly altered or enlarged in its use or structure until a Certificate of Use is issued by the Zoning Inspector stating that the proposed use, structure, or land conforms to the requirements of this resolution

Sec. 1002	Enforcement

Sec. 1002.1	Violations

Buildings erected, altered, moved, razed, converted, or any use of land or premises 				carried on in violation of any provision of this Resolution is declared to be a nuisance per 			Section 203.3.

Sec 1002.2	Inspection

The Zoning Inspector shall inspect each alleged violation and shall, in writing, order 				correction of all conditions that are found to be in violation of this Resolution.

Sec. 1002.3	Correction Period

All violations shall be corrected within a period of thirty days after written order is 				issued or for a longer period of time as indicated by the Zoning Inspector.  Any violations 			not corrected within a specified time period shall be reported to the County Prosecutor 				who shall initiate prosecution procedures.

Sec. 1002.4	Penalties

The owner or owners of any building or premises or part thereof where anything in 				violation of the Resolution shall be placed or shall exist, and any tenant or occupant of 				such building or premises, and any architect, builder, or contractor who shall assist in 				the commission of any such violation, any person who shall violate any of the provisions 				of this Resolution or fail to comply therewith shall for each violation of noncompliance is 			deemed to be guilty of a misdemeanor and upon conviction thereof, per O.R.C. statute.  				Each day such violation or failure to comply shall exist shall constitute a separate 					offense.

Sec. 1002.5	Prevention of Violations

In case any building is or is proposed to be located, erected,  constructed, 					reconstructed, enlarged, changed, maintained, or used or any land is proposed to be 				used in violation of Section 519.01 to 519.99, inclusive, of the Revised Code of Ohio, or 				any regulation or provision adopted by the Township Trustees or Township Boards, the 				prosecuting attorney of the county, the township zoning inspector, or any adjacent or 				neighboring property owner who would be especially damaged by such violation, in 				addition other remedies provided by law, may institute injunction, mandamus, 					abatement, or any other appropriate action or proceeding to prevent, enjoin, abate, or 				remove such unlawful location, erection, construction, reconstruction, enlargement, 				change, maintenance, or use.  The Board of Township Trustees may employ special 				counsel to represent it in any proceeding or to prosecute any actions brought under this 			section.

ARTICLE XI

VALIDITY AND SEPARABILITY

It is hereby declared to be the legislative intent that, if any provision or provisions of this Resolution, or the application thereof to any zoning lot, building, or other structure, or tract of land, are declared by a court of competent jurisdiction to be invalid or ineffective in whole or in part, or to be inapplicable to any person or situation, the effectiveness of such decision to be invalid or ineffective, or to the zoning lot, building, or other structure, or tract of land immediately involved in the controversy.  All other provisions of this Resolution shall continue to be separate and fully effective, and the application of any such provision to other persons or situations shall not be affected.

ARTICLE XIII

EFFECTIVE DATE

# APPENDIX ‘A’ - SCHEMATIC FOR FIRE LANES

# APPENDIX ‘B’ - WP-1

# APPENDIX ‘C’ - WP-2

# APPENDIX ‘D’

# - LOT AREAS

# APPENDIX ‘G’ - BASEMENT & STORY

When “A” is less than “B”,

“C” is a Basement.

Story

s

Average Grade

A															B		C				                     Basement

Basement

When “A” is greater than “B”,

“C” is a Story

Story

A			C								B		          Basement		                     Basement

Average Grade

# APPENDIX ‘I’ - TYPES OF SIGNS

# APPENDIX ‘J’ - NONCONFORMING USE

| Appendix "K" |  | Official Schedule of District Regulations |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | of Lawrence Township |  |  |  |  |  |  |  |  |
|  | R-1 | R-2 | R-3 | R-4 |  | C-1 | C-2 | C-3 | I-1 | F-1 |
|  | LOW DENSITY | MEDIUM DENSITY | HIGH DENSITY | RURAL RESIDENTIAL |  | SERVICE DISTRICT | LOCAL COMMERICIAL | HIGHWAY | LIGHT INDUSTRIAL | FLOOD PLAIN |
| Minimum Lot Size | PROHIBITED | PROHIBITED | PROHIBITED | 65,340 sq. ft. |  | 40,000 sq. ft. | NONE | NONE | Two (2) Acres | (15) |
| (WITHOUT CENTRAL SEWAGE TREATMENT) |  |  |  |  |  |  |  |  |  |  |
| Minimum Lot Size | 11,250 sq. ft. | 10,800 sq. ft. (3,4) | 7,500 sq. ft. (5) |  | 13,600 sq. ft. | 30,000 sq. ft. | NONE | NONE | Two (2) Acres | (15) |
| (WITH CENTRAL SEWAGE TREATMENT) |  |  |  |  |  |  |  |  |  |  |
| Minimum Lot Size | PROHIBITED | PROHIBITED | PROHIBITED | 65,340 sq. ft. |  | 60,000 sq. ft. | NONE | NONE | Three (3) Acres | (15) |
| (CORNER LOT W/OUT CENTRAL SEWAGE)) |  |  |  |  |  |  |  |  |  |  |
| Minimum Lot Size | 16,875 sq. ft. | 16,200 sq. ft. (3,4) | 11,250 sq. ft. (5) |  | 20,400 sq. ft. | 45,000 sq. ft. | NONE | NONE | Three (3) Acres | (15) |
| (WITH CENTRAL SEWAGE TREATMENT) |  |  |  |  |  |  |  |  |  |  |
| Minimum Lot Width at Building | 75' |  | 60' (6) | 150' | 85' | NONE | NONE | NONE | 100' | (15) |
| Minimum Lot Width at Street | 50' (1) | 50' | 50' (7) | 75' (9) | 50' | NONE | NONE | NONE | 80' | (15) |
| Minimum Lot Width at Street (Corner Lot) | 90' | 90' | 90' | 105' | 90' | NONE | NONE | NONE | 80' | (15) |
| Minimum Front Yard Depth from Street Right-of-Way | 30' (2) | 30' | Ref. 303.3D | 70' (10) | 30' (2) | Equal to Immediate Vicinity "R" District | NONE (11) | 50' | 50' | (15) |
| Minimum Side Yard (One Side) | 10' | 10' | 8' | 20' | 10' | None: Except 30' Adjacent to "R" District | None: Except 30' Adjacent to "R" District | None: Except 30' Adjacent to "R" District | 25' (14) | (15) |
|  | 25' | 25' | 20' (8) | 50' | 25' |  |  |  |  | (15) |
| Minimum Rear Yard | 40' | 40' | 40' | 40' |  | 25' | 25' | NONE | 25' | (15) |
| Minimum Floor Area | 1200 sq. ft. | Ref. 302.4F | Ref. 303.3H | 1200 sq. ft. |  | 1200 sq. ft. |  |  |  | (15) |
| Maximum Height Principle Building | 35' | 35' | 50' | 35' |  | 50' | 50' | 50' | 50' | (15) |
| Number Permissible Stories | 2.5 |  |  | 2.5 |  |  |  |  |  | (15) |

| Appendix "K" |  | Official Schedule of District Regulations |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | of Lawrence Township |  |  |  |  |  |  |  |  |
|  | R-1 | R-2 | R-3 | R-4 |  | C-1 | C-2 | C-3 | I-1 | F-1 |
|  | LOW DENSITY | MEDIUM DENSITY | HIGH DENSITY | RURAL RESIDENTIAL |  | SERVICE DISTRICT | LOCAL COMMERICIAL | HIGHWAY | LIGHT INDUSTRIAL | FLOOD PLAIN |
| Accessory Building (s) | 1200 sq. ft. Max | 1200 sq. ft. Max | 1200 sq. ft. Max | Not to exceed 40% of lot size minus principle building size |  |  |  |  |  | (15) |
| Maximum Height | 20' | 20' | 20' | 20' | 20' | 20' | 20' | 20' | 20' | (15) |
| Side Yard Set Back | 10' | 10' | 10' | 10' | 10' | 10' | 10' | 10' | 10' | (15) |
| Rear Yard Set Back | 10' | 10' | 10' | 10' | 10' | 10' | 10' | 10' | 10' | (15) |
| Principle Building Set Back | 15' | 15' | 15' | 15' | 15' | 15' | 15' | 15' | 15' | (15) |
| Maximum Lot Coverage (percentage) Principle and Accessory Buildings | 40% | 40% | 50% | 40% |  | 40% | 50% | 70% | 50% | (15) |
| (1) Cul-De-Sac 45' |  |  | (6) Two Family 80'; Multi-Family 100' |  |  |  | (11) 40' Non-Established Right-of-Way |  |  |  |
| (2) 60' Non-Established Right-of-Way |  |  | (7) Cul-De-Sac 40' |  |  |  | (12) 60' Non-Established Right-of-Way |  |  |  |
| (3) Single Family Individual Lot |  |  | (8) Multi-Reference 303.3 E2 & 3 |  |  |  | (13) 90' Non-Established Right-of-Way |  |  |  |
| (4) 8,000 sq. ft. Single Family Attached |  |  | (9) Cul-De-Sac 50' |  |  |  | (14) 40' Non-Established Right-of-Way |  |  |  |
| (5) Two Family 10,000 sq. ft.; Three Family 12,000 sq. ft.; Four or more Family 2,500 sq. ft. per Family |  |  | (10) 100' Non-established Right-of-Way |  |  |  | (15) In Accordance With Army Corps of Engineers and Parcel Deed Restrictions |  |  |  |

APPENDIX “L” – LOT TYPES

STREET

INTERIOR

LOT

CORNER

LOT

INTERIOR

LOT

S

T

R

E

E

T

PANHANDLE

LOT

DOUBLE

FRONTAGE

LOT

CORNER

LOT

LOT

DEPTHH

LOT

WIDTH

CORNER

LOT

INTERIOR

LOT

STREET

APPENDIX “M” – LOCATION OF LOT LINES AND SETBACKS/YARD ON A TYPICAL LOT

REAR

LOT

LINE

MINIMUM SIDE

YARD SETBACK

MINIMUM

REAR YARD

SETBACK

MINIMUM

ACCESSORY

USE

SETBACK

SIDE LOT

LINE

SIDE

LOT

LINE

MINIMUM

FRONT YARD

SETBACK

FRONT LOT LINE

ULTIMATE RIGHT-OF-WAY

STREET

DEFINITIONS

For the purposes of this Resolution, certain terms are hereby defined.  Words used in the present tense shall include the future; the singular number shall include the plural and the plural the singular; the word “shall” is mandatory and not directory; the word “building” shall include the word “structure”; the word “used” shall include the words “arranged”, “designed ”,”constructed” ,”altered” ,”converted”, or “intended to be used” and a “person” shall mean, in addition to an individual, a firm, corporation, association, or any legal entity which may own and/or use land or buildings.

A

ABANDONED CELLULAR COMMUNICATION TOWER:  A tower erected for cellular communications which is no longer being used for that purpose, and which has not been used for that purpose for uninterrupted period of one hundred and eighty days or more.

ABANDONDED VEHICLE:

1.	 Any vehicle left on a public right-of-way for more than seventy-two hours (or less, 				depending on the weight of the vehicle and its location) or

2.	 on private property without the consent of the property owner.

ACCESSORY BUILDINGS OR USE:  A subordinate building for use customarily incidental to and located upon the same lot occupied by the main building and use.

ACRE:  Land area, equal to forty-three thousand five hundred and sixty square feet, measured on the horizontal plane, and including land occupied by all natural and manmade features of the landscaping.

ADJOINING LOT LINE:  The property boundary lines between the real property for the proposed sitting of a wind turbine generator or anemometer tower subject of the application and real property owned by another person, persons or entity.

ADULT CARE FACILITY:  An adult family home or an adult group home.  For the purposes of this Resolution, a residence, facility, institution, hotel, congregate housing project, or similar facility that provides accommodations and supervision to three to sixteen unrelated adults, at least three of whom are provided personal care services, is an adult care facility regardless of how the facility holds itself out to the public.  Adult Care Facility does not include:

1.	A facility operated by a hospice care program licensed under Section 3712.04 of the Revised 			Code that is used exclusively for care of hospice patients;

2.	A nursing home, residential care facility, or home for the aging as defined in Section 3721.01 of 			the Revised Code;

3.	A community alternative home;

4.	An alcohol and drug addiction program as defined in Section 5119.01 of the Revised Code;

5.	A residential facility for the mentally ill licensed by the Department of Mental Health under 			Section 5123.19 of the Revised Code;

6.	A facility licensed to provide methadone treatment under Section 5119.391 of the Revised

Code;

7.	A residential facility licensed under Section 5123.19 of the Revised Code or otherwise regulated 		by the Department of Mental Retardation and Developmental Disabilities;

8.	Any residence, institution, hotel, congregate housing project, or similar facility that provides 			personal care services to fewer than three residents or that provides, for any number of 				residents, only housing, housekeeping, laundry, meal preparation, social or recreational 				activities, maintenance, security, transportation, and similar services that are not personal care 			services or skilled nursing care;

9.	Any facility that receives funding for operating costs from the Department of Development 			under any program established to provide emergency shelter housing or transitional housing

for the homeless;

10.	A terminal care facility for the homeless that has entered into an agreement with a hospice care 		program under Section 3712.07 of the Revised Code;

11.	A facility approved by the Veteran's Administration under Section 104(a) of the Veterans Health 		Care Amendments of 1983, 97 Stat. 993, 38 U.S.C.A. 630, as amended, and used exclusively for 			the placement and care of veterans;

12.	Until January 1, 1994, the portion of a facility in which care is provided exclusively to members 			of a religious order if the facility is owned by or part of a nonprofit institution of higher 				education authorized to award degrees by the Ohio Board of Regents under Chapter 1713 of

the Revised Code.

ADULT DAY CARE FACILITY:  An establishment that during any part of the normal business day provides supervised educational, recreational and social activities to elderly and/or handicapped adults, but not including person suffering from acute or chronic alcoholism or other drug dependence and persons who regularly require restraint.

ADULT FAMILY HOME:  A residence or facility as defined and regulated in Chapter 5119 of the Ohio Revised Code, which provides accommodations to three to five unrelated adults and supervision and personal care services to at least three of those adults.

ADULT FAMILY HOME – MANAGER:  The person responsible for the daily operation of an adult care facility.  The manager and the owner of the facility may be the same person.

ADULT FAMILY HOME – OWNER:  The person(s) who owns the business and who ultimately controls the operation of an adult care facility and to whom the manager, if different from the owner, is responsible.

ADULT GROUP HOME – LARGE:  A resident or facility, as defined and regulated in Chapter 5119 of the Ohio Revised Code, which provides accommodations to at least nine but not more than sixteen unrelated adults and provides supervision and personal care services to at least three of the unrelated adults.

ADULT GROUP HOME – SMALL:  A residence or facility, as defined and regulated in Chapter 5119 of the Ohio Revised Code, which provides accommodations to at least six but not more than eight unrelated adults and provides supervision and personal care services to at least three of the unrelated adults.

ADULT RETIREMENT COMMUNITY:  A residential facility to provide for the needs of individuals who are elderly or handicapped, who do not require medical care and the intensive services of a nursing home, but desire the security and safety of a residential environment of peers, which can accommodate productive care and independent living.  The facility shall consist of independent residential dwelling units designed specifically for the elderly or handicapped, and have common social, recreational, dining and food preparation facilities.  Any change in use of such a facility shall require the issuance of a new zoning permit by the Zoning Inspector after review by the Zoning Inspector for determination that the parking required for such other use is met.

ADVERTISEMENT:  The act of using the techniques of radio, television films, handbills, etc.

ADVERTISING:  The use of text, logos, art, the spoken word, or any other device for the purpose of offering a good, product, or service for sale, trade or barter.  The provisions of this section shall not restrict vehicles from featuring advertising, but shall prohibit the use of those vehicles as unpermitted permanent or semi-permanent stationary signs.

AGRICULTURE:  The science and art of farming, the work of cultivating soil, producing and marketing crops, and the raising of livestock.

AGRICULTURAL USE:  Farming, ranching, aquaculture, alga culture meaning the farming of algae, apiculture and related apicultural activates, production of honey, beeswax, honeycomb, and other related products; horticulture; viticulture, winemaking, and related activates; animal husbandry, including but not limited to, the care and raising of livestock, equine, and fur-bearing animals, poultry husbandry and the production of poultry and poultry products; dairy production; the production of field crops, tobacco, fruit, vegetables, nursery stock, ornamental shrubs, ornamental trees, flowers, sod, or mushrooms; timber; pasturage; any combination of the foregoing; the processing, drying, storage, and marketing of agricultural products when those activities are conducted in conjunction with, but are secondary to, such husbandry or production; and any additions or modifications to the foregoing made by the director of agriculture by rule adopted in accordance with Chapter 119 of the ORC.

AGRICULTURE PUBLIC PARK:  Where there is the teaching and displays of agriculture, natural resources, and land management.  This can be accomplished through seminars, hands on experience, and guidance to educate in such areas as, but not limited to, horticulture, land management, turf grass management, agricultural science, small animal care, machine and shop classes, health and nutrition, livestock management, biology courses, etc.

ALLEY:  A public or private right-of-way affording secondary means of access to abutting property.

ANEMOMETER:  An instrument for measuring and recording the speed of the wind.

ANEMOMETER TOWER:  A structure, including all accessory facilities, temporarily erected for no more than two years, on which an anemometer is mounted for the purposes of documenting whether a site has wind resources sufficient for the operation of a wind turbine generator.

ANIMAL HUSBANDRY:  The keeping or raising of domestic animals incidental to the use of land for agricultural purposes permitted under the above definition of agriculture.

APARTMENT:  A self-contained housing unit that occupies only part of a building, commonly referred to as an apartment building or apartment house.

APARTMENT HOUSE:  The building containing individual living spaces know as apartments.

APPLICANT:  The entity or person who submits to the Chairperson of the Township Board of Zoning Appeals and application.

ARTERIAL THOROUGHFARE:  A highway primarily for through vehicular traffic usually on a continuous route.  This thoroughfare provides for through traffic movement between areas across the county and to and from expressways.  Arterial streets shall be as designated in the records of Tuscarawas County Regional Planning Commission, including any amendments thereto.

AUTOMOTIVE HULK:  Any wrecked, dismantled or inoperative motor vehicle or part thereof which cannot be made an operative motor vehicle without the addition of vital parts or mechanisms and the application of substantial amount of labor to effect repairs.

AUTOMOTIVE REPAIR:  The repair, rebuilding or reconditioning of motor vehicles or parts thereof, including collision service, painting and steam cleaning of vehicles.

AUTOMOBILE SERVICE STATION:  A place where gasoline, kerosene, or any other motor fuel or lubricating oil or grease for operating motor vehicles is offered for sale to the public and deliveries are made directly into motor vehicles, including greasing and oiling on the premises.

AUTOMOTIVE WRECKING, SALVAGE, OR JUNK FACILITIES:  The dismantling or wrecking of vehicles at a facility as defined and regulated in ORC Section 4738.  Such uses may include the storage or sale of junk vehicles, inoperable vehicles, or parts.

B

BAKERY:  The processing, assembling, packaging, and distributing of baked goods.

BASEMENT:  A story having more than one-half of its height below average grade.  A basement shall not be counted as a story for the purpose of height regulations.

BED AND BREAKFAST:  A home where the owner lives yet offers a tourist bed room in that home to stay for a short period of time.  The owner prepares a breakfast for the tourist.

BERM:  A continuous, raised earthen mound with flattened top and sloped sides, used especially as a site, sound and dust barrier, capable of supporting live landscaping materials, and with a height and width that complies with the requirements of this zoning resolution.

BILLBOARD:  Same as “Outdoor Advertising Sign”, See Section 401.2B.

BOARD:  The Board of Zoning Appeals as created by the Resolution.

BOARDING HOUSE:   A building other than a hotel or motel where for compensation by the week or month, meals and/or lodging are provided for at least three but not more than twenty persons.

BUFFERYARD:  A strip of ground at least ten feet in width running along the property line between adjacent districts designed to improve the quality of the district and to protect any adjacent neighbor from offensive, unsafe or unhealthy conditions.

BUILDING:  Any structure having a roof supported by columns or by walls and intended for the shelter, housing, or enclosure of persons, animals or chattels.

BUILDING, HEIGHT OF:  The vertical distance measured from the average elevation of the proposed finished grade at the front of the building to the highest point of the roof for flat roofs, to the deck line for mansard roofs, and the mean height between eaves and ridge for gable, hip and gambrel roofs.

BUILDING LINES:  The line defining the minimum front, side, and yard requirements outside of which no building or structure may be located, except as otherwise provided herein.

BUILDING, PRICIPAL:  The building on a lot used to accommodate the primary use to which the premises are devoted.

BUILDING, SIGNS:  A sign which identifies the business or profession conducted or the principal products sold upon the premises.

C

CARPORT:  A covered vehicle parking space not completely enclosed by walls or doors.  A carport shall be subject to all the provisions prescribed in these regulations for a private garage or accessory building.

CEMETERY:  Land used or intended to be used for the burial of human or animal dead and dedicated for cemetery purposes, including crematories, mausoleums and mortuaries, if operated in connection with and within the boundaries of such cemetery.

CENTRAL SEWER SYSTEM/CENTRAL SEWAGE TREATMENT:  A system of pipes and associated facilities for the collection and disposal of sewage from more than one lot.

CERTIFICATE OF OCCUPANCY:  Certificate required to be obtained from the Zoning Inspector before the occupancy or change of occupancy of and use is permitted in the business or industrial district of Lawrence Township.

CHILD DAY-CARE CENTER: Any place in which child day-care is provided for thirteen or more children at one time or any place that is not the permanent residence of the licensee or administrator in which child day-care is provided for seven to twelve children at one time. In counting children for the purposes of this division, any children under six years of age who are related to a licensee, administrator, or employee and who are on the premises of the center shall be counted. “Child day-care center” and “center” do not include a place located in and operated by a hospital, as defined in ORC §3727.01, in which the needs of children are administered to, if all the children whose needs are being administered to are monitored under the on-site supervision of a physician licensed under ORC Chapter 4731 or a registered nurse licensed under ORC Chapter 4723, and the services are provided only for children who, in the opinion of the child’s parent, guardian, or custodian, are exhibiting symptoms of a communicable disease or other illness or are injured. “Child day-care center” and “center” do not include any child day camp.

CHURCH: A fellowship of believers, congregation, society, corporation, convention, or association that is formed primarily or exclusively for religious purposes and that is not formed for the private profit of any person. (ORC§5709.07)

CLINIC:	 Any building or other structure devoted to the medical diagnoses, treatment, and care of human outpatients.

CLUB: A group of people organized for a common purpose to pursue common goals, interests or activities and usually characterized by certain membership qualifications, payment of fees and dues, regular meetings, and a constitution and by-laws.

CO-LOCATION:  The use of a wireless telecommunications facility by more than one wireless telecommunications provider.

COMMEMORATIVE PLAQUE:  To serve as a memorial.

COMMERCIAL VEHICLE:  Any self-propelled or towed motor vehicle used on a highway in interstate commerce to transport passengers or property when the vehicle:

(1)	Has a gross vehicle weight rating (GVWR) (Can be located on the Vehicle Identification Plate on 			the driver’s door of most vehicles) or gross combination weight rating, or gross vehicle weight

or gross combination weight of four thousand five hundred thirty-six kg (ten thousand and one 			pound) or more, whichever is greater, or

(2)	Is designed or used to transport more than eight passengers (including the driver) for 				compensation; or

(3)	Is designed or used to transport more than fifteen passengers, including the driver, and is not 			used to transport passengers for compensation; or

(4)	Is used in transporting material found by the Secretary of Transportation to be hazardous under 		49 U.S.C. 5013 and transported in a quantity requiring placarding under regulations prescribed 			by the Secretary under 49 CFR, Subtitle B, Chapter I, Subchapter C.

COMMISSION:  The Township Zoning Commission.

COMMUNICATIONS TOWER:  Any tower used for the purpose of communications.

COMPLEX STRUCTURES:  Two or more primary business uses within a single structure located in C-1, C-2, C-3 or I-1 Districts.

CONDITIONAL USE:  A use permitted within a district other than principally permitted use, requiring a conditional use certificate and approval of the Board of Zoning Appeals.  Conditional uses permitted in each district are listed.

CONDOMINIUM:  A form of real property ownership in which a declaration has been filed submitting the property to the condominium form of ownership under which each owner has an individual ownership interest in a unit with the right to exclusive possession of that unit and an undivided ownership interest with the other unit owners in the common elements of the condominium property.

CONGREGATE LIVING DEVELOPMENT: A residential development that consists of a congregate living facility (See definition of congregate living facility) and single family detached and/or single family attached dwelling units to provide for the needs of individuals who are elderly or handicapped with common social and recreational facilities.

CONGREGATE LIVING FACILITY: A residential facility consisting of independent units, congregate living, assisted living and/or nursing home within the facility itself, designed specifically to provide for the needs of individuals who are elderly or handicapped, and have common social, recreational, dining and food preparation facilities.

COURT:  An open, unoccupied space other than a yard on the same lot with a building, unobstructed from the lowest level to the sky.

D

DANGEROUS, EXOTIC & WILD ANIMALS:  As defined in the Ohio Revised Code, Section 935.01.

DECORATIVE WALL:   A wall made from stone, concrete, or bricks designed for decorative purposes.

DEDICATED PRIVATE DRIVE:  A dedicated private drive provides legal access to a tract of land where the dedicated privates drive fonts upon or abuts an existing public road.  The dedicated private drive shall be located in an area that can be developed for access by emergency equipment (fire trucks, ambulances).  A dedicated private drive provides legal access to land.  A dedicated private drive may be used as an alternate to pipe stem lots.

DEMOLITION MATERIALS:  Waste materials removed from the alteration, construction, destruction or repair of any man-made physical structures including, but not limited to houses, buildings and industrial or commercial facilities or roadways.  This does not exclusively include paving materials, or materials recycled on the premises.  This does not include solid wastes or hazardous wastes as pursuant to ORC 3734.

DENISTY:  The number of families residing on, or dwelling units developed on, an acre of land.

DESIGNATED DRIVEWAY:  Clearly defined roadway leading from the street which is surfaced by asphalt, concrete, gravel or similar material not to exceed twenty-four feet in width or such width as may be allowed in any other development standards adopted by Lawrence Township, accordingly to Article V of the Zoning Resolution.

DEVELOPMENTAL DISABILITY:  As defined in ORC 5123.011(Q), means a severe, chronic disability that is characterized by all of the following:

(1)	It is attributable to a mental or physical impairment or a combination of metal and physical 			impairments, other than a mental or physical impairment solely caused by mental illness as 			defined in division (A) of Section 5122.01 of the Revised Code.

(2)	It is manifested before age twenty-two.

(3)	It is likely to continue indefinitely.

(4)	It results in one of the following:

a.	In the case of a person under three years of age, at least one developmental delay or a 				diagnosed physical or mental condition that has a high probability of resulting in a 				developmental delay;

b.	In the case of a person at least three years of age but under six years of age, at least two 			developmental delays;

c.	In the case of a person six years of age or older, a substantial functional limitation in at 				least three of the following areas of major life activity, as appropriate for the person’s 				age:  self-care, receptive and expressive language, learning, mobility, self-direction, 				capacity for independent living, and, if the person is at least sixteen years of age, 					capacity for economic self-sufficiency.

(5)	It causes the person to need a combination and sequence of special, interdisciplinary, or other 			type of care, treatment, or provision of services for an extended period of time that is 				individually planned and coordinated for the person.

DISCARDED MOTOR VEHICLES:  Any inoperable motor propelled vehicle or accessory to same, which is in the process of being wrecked, dismantled or stored and which does not have a license thereon which is valid or was valid not more than six months previous.

DISTRICTS:  A section or sections of the unincorporated territory of Lawrence Township for which the regulations governing use of building and premises or the height and areas of buildings are uniform.

DOUBLE PARKING:  The standing or stopping of a vehicle within the line of traffic for purposes other than observing normal traffic laws and regulations:

(1)	On the roadway side of a parked vehicle; or

(2)	On the roadway side of the space in which a vehicle may be parked.

DRIVE-THRU FACILITY: Any portion of a building or structure from which business is transacted, or is capable of being transacted, directly with customers located in a motor vehicle during such business transactions. The term “drive-thru” shall also include a “drive-up” and “drive-in.”

DWELLING:  Any building, or structure, or portion thereof, which is designed or used primarily for residence purposes, including one-family, two-family, and multi-family, manufactured, modular and factory-built homes, but not including house trailers, mobile homes, basement dwellings, hotels, motels, boarding houses, lodging houses, and tourist dwellings.  An attached garage shall be considered part of the dwelling.  All such dwellings, whether built onsite or off premises, shall be constructed in accordance with local and/or state building codes and shall be certified for permanent residential purposes.

DWELLING GROUP:  More than one dwelling, whether single-family, two-family or a combination thereof, located on one lot or tract of ground and having common open space and/or parking and driveway facilities.

DWELLING, INDUSTRIALIZE UNIT:  An assembly of materials or products comprising all or part of a total structure which, when constructed, is self-sufficient or substantially self-sufficient and when installed, constitutes a dwelling unit, except for necessary preparations for its placement, and including a modular or sectional unit but not a mobile home.

DWELLING, SINGLE-FAMILY:  A dwelling designed for or used exclusively for residence purposes by one family.

DWELLING, STUDIO UNIT:  A small apartment consisting typically of a main room, kitchenette and bathroom.

DWELLING, TWO-FAMILY:  A building designed and used exclusively by two families living independently of each other.

DWELLING UNIT:  A single unit providing complete, independent living facilities for one or more persons, including permanent provisions for living, sleeping, eating, cooking and sanitation.

E

EMPLOYEE:  A person who works for another person or company in exchange for pay, and will also include volunteers and interns.

ENFORCEMENT OFFICER:  Lawrence Township Zoning Inspector, his or her designee, or a Tuscarawas County or Lawrence Township Deputy Sheriff.

ESSENTIAL SERVICES:  Services provided by public and private utilities necessary for the exercise of the principal use or service of the principal structure and for the public health, safety and general welfare.  These services include underground, surface, overhead gas, electrical, steam, water, transmission or distribution system, sanitary sewage, storm water drainage, and communication systems and accessories thereto, such as poles, towers, wires, mains, drains, vaults, culverts, laterals, sewers, pipes, catch basins, water storage tanks, conduits, cables, fire alarm boxes, police call boxes, traffic signals, pumps, lift stations, and hydrants, but not including buildings.

EVERGREEN SHRUBS:  A shrub that remains green throughout the year and does not lose its leaves or needles.

EVERGREEN TREE:  A tree that remains green throughout the year.

EXCAVATION:  The process of altering the natural grade/elevation by cutting, filling, or moving the earth, or any activity by which soil or rock is cut, dug, quarried, uncovered, removed, displaced, or relocated.

EXCAVATION – PRELIMARY:  Earth moving activity which does not exceed either:

(1)	five feet in vertical depth at its deepest point; or

(2)	a total of one-half acre of work area.

EXOTIC ANIMALS:	As defined in Section 935.01 of the Ohio Revised Code.

F

FACTORY BUILT HOUSING:  A residential dwelling unit designed for long-term residential use, the components of which are essentially constructed or assembled prior to its delivery, to and installation upon a site.  For the purpose of this Resolution, “factory-built housing” shall include manufactured home and/or modular home.

FAMILY:  One or more persons occupying a dwelling and living as a single housekeeping unit, provided that unless all members are related to each other by blood, adoption, or marriage, no such family shall contain over five persons.

FAMILY DAY-CARE HOME:  Administering to the needs of infants, toddlers, pre-school children, and school children outside of school hours by person other than their parents or guardians, custodians, or relatives by blood, marriage, or adoption for any part of the twenty-four-hour day in a place or residence other than the child’s own home.  The following are child day-care facilities:

TYPE A:	Seven to twelve children (or four to twelve if four children are under two years of age) 			cared for in the provider’s home.  The provider’s own children six years of age and 				under must be included in the total count.

TYPE B:	Homes serving children through the publicly funded child care program.  One to six 				children cared for in the provider’s personal home.  No more than three children may 				be under the age of two.  The provider’s own children under six years of age must 				be included in the total count.  Type B homes are only inspected by ODJFS if they 				serve, or intend to serve, children through the publicly funded child care program. (It 				should be noted anyone can operate a Type B home without a license.) (ORC§5104.01)

FAMILY HOME:  Means a residential facility that provided room and board, personal care, habilitation services and supervision in a family setting for not more than eight persons with developmental disabilities.  ORC 5123.

FARM MARKET:  See Roadside Stand

FENCE:  An artificially constructed barrier of wood, masonry, stone, wire, metal, vinyl, or other manufactured material or combination of material erected to enclose, screen, or separate areas.

FENCE – BARBED WIRE:  One or more strands of wire or other material having intermittent sharp points of wire or metal that may puncture, cut, tear, or snag persons, clothing or animals.  The term “barbed wire” as used herein excludes razor ribbon.

FENCE – CHAIN LINK:  An open mesh fence made entirely of wire woven in squares of approximately one- and one-half inches with vertical supports not less than one- and one-half inches in diameter spaced not less than six feet, and not more than eight feet, apart.

FENCE – DECORATIVE:  A designed open or solid fence or wall that meets all of the following:

(1)	It contributes to the identification and beauty of the principal use;

(2)	It is not erected to satisfy any other provision of this code;

(3)	It does not act as a retaining structure;

(4)	It is not a privacy or stockade fence;

(5)	A fence hedge used for decorative purposes only and not used to confine or enclose an area.

FENCE – FUNCTIONAL:  A barrier fence or hedge used to confine or enclose an area.

FENCE – HEIGHT:  The height above the horizontal property grade that represents not less than sixty percent of the property line.

FENCE – PRIVACY:  A fence no more than six feet in height intended to inhibit public view and provide seclusion.  When viewed at right angles has less than sixty six percent of its area open to light and air.  Examples of privacy fences include but are not limited to:

(1)	Basket weave or woven fences – made of interwoven strips or slats of flexible material in which 			the pattern has the appearance of a “basket weave”.

(2)	Louver or ventilating fences – Made of a series of slats placed at an angle or positioned so as to 			provide air but to deflect light perpendicular to its plane.

(3)	Board on board fence – A fence made of vertical wood planks supported by horizontal framing 			with the vertical planks usually mounted on alternating sides of the framing.  The planks may or 		may not be placed with a space between.

(4)	Stockade Fence – A fence made of board-on-board construction using rounded vertical planks 			with sharpened tops.  The planks are usually placed with no space between.

(5)	Masonry block.

FENCE – SECURITY:  A type of fencing used mostly on industrial or commercial property with the purpose of providing enhanced protection for the assets, for the storage areas as well as the open areas on the property.

FENCE – TEMPORARY:  Fences erected for a specific function and limited time duration.

FILL OR FILLING:  Any artificial or mechanical act by which earth, sand, gravel, rock or any other material is placed, pushed, dumped, pulled, transported or moved to a new location above the natural surface of the ground or on top of the stripped surface and shall include the conditions resulting there from; the difference in elevation between a point on the original ground and a designated point of higher altitude on the filled grade, the material used to make a fill.

FLOOD PLAIN:  That portion of a river or creek valley adjacent to the river or creek channel which is covered with water when the river or creek overflows its banks at flood stage.

FLOODPLAIN MANAGEMENT:  The following terms used in this Resolution are defined as set forth in the regulations governing the National Flood Insurance Program (44 CFR Section 59.1).  Wherever there exists a conflict between the following definitions, the federal definitions shall apply.

Base Flood:  The flood having a one percent chance of being equaled or exceed in any given 				year.

Flood or Flooding:

(1)  A general and temporary condition of partial or complete inundation of normally 					dry land areas from:

a.	The overflow of inland or tidal waters.

b.	The unusual and rapid accumulation or runoff of surface waters from any source.

c.	Mudslides (i.e., mud-flows) which are proximately caused by flooding as defined 					in 1.b above and are akin to a river of liquid flowing mud on the surfaces of 					normally dry land areas, as when earth is carried by a current of water 						and deposited along the path of the current.

(2)	The collapse or subsidence of land along the shore of a lake or other body 					of water as a result of erosion or undermining caused by waves or 						currents of water exceeding anticipated cyclical levels or suddenly caused 					by an unusually high water level in a natural body of water, accompanied 						by a severe storm, or by an unanticipated force of nature, such as a flash 						flood or an abnormal tidal surge, or by some similarly usual and 							unforeseeable event which results in flooding as defined in 1.a of this 						definition.

Floodplain or Flood-prone area:  Land area susceptible to inundation by water from any source.	Floodway:  The channel of a river or other watercourse and the adjacent land areas that must 				be reserved in order to discharge the base flood without cumulatively increasing the water 			surface elevation more than a designated height.

Floodway Fringe:  Those portions of the floodplain, other than the floodway, which can be filled 			or otherwise obstructed without causing substantially higher flood levels or flow velocities.

Zone A:  An area inundated by one-hundred-year flooding, for which no base flood elevation has been established.

Zone AE:  An area inundated by one-hundred-year flooding, for which the base flood elevation 				has been established.

Zone AH:  An area inundated by one-hundred-year flooding (usually in an area of ponding), for 				which the base flood elevations have been determined.  The flood depths range from

one to three feet.

Zone AO:  An area inundated by one hundred-year-old flooding (usually sheet flow on sloping 				terrain), for which average depths have been determined; flood depths range from one 				to three feet.

FLOOR AREA:  The sum of the gross horizontal areas of the one or several floors of a building, measured from the exterior faces of exterior walls or from the centerline of common walls separating two buildings.  Floor area, for the purposes of these regulations, shall not include basement, elevator and stair bulkheads, attic space, terraces, breezeways, open porches, and uncovered steps.

FOUNDATION – PERMANENT:  That portion of a structure extending below grade beyond frost line, and supporting the upper portion of a structure and extending in a continuous perimeter of the structure, consisting of materials such as concrete, brick, block or tile.

FOUNDATION – TEMPORARY:  Materials supporting a structure not embedded below the frost line and not being continuous to the perimeter of the structure.

FREESTANDING WIND ENERGY CONVERSTION SYSTEM (WECS):  A machine consisting of one wind turbine, one tower and associated control electronics that converts the kinetic energy in the wind into a usable form (commonly known as a “wind turbine” or “windmill”).  The turbine or windmill may be on a horizontal or vertical axis, rotor or propeller.  WECS terms:

DECOMMISSIONING:  The process of terminating the operation of the WECS by completely 				removing the entire WECS and all related buildings, structures, foundations, supports, and 			equipment.

NACELLE:  The enclosure located at the top of a wind turbine tower that houses the gearbox, 				generator and other equipment.

POWER CENTER:  Serves as the central connection point for the electrical components in the 				systems and provides a number of necessary control functions.

ROTOR:  The rotating part of the turbine, including the blades.

TOWER:  The support structure, including guyed, monopole and lattice types, upon which a wind 			turbine or other mechanical device is mounted.

TOWER HEIGHT:  The height of the tower, measured from the natural grade surrounding the 				support pad to the tip of the blade at its highest point along the vertical axis of the tower.

WIND TURBINE:  A device that converts kinetic wind energy into rotational energy that drives an 			electrical generator.  A wind turbine typically consists of a tower, nacelle body, power 				center and rotor with two or more blades.

FRONTAGE:  The portion of a lot nearest the street right-of-way.  For the purpose of these regulations, corner lots and double frontage lots, all sides of the lot adjacent to streets shall be considered frontage, for the purpose of determining yard requirements only.

G

GARAGE – AUTO SERVICE SHOP:  Buildings and premises where gasoline, oil, grease, batteries, tires, and motor vehicle accessories may be supplied and dispensed at retail, and where in addition, the following services may be rendered and sales are made:

1.	Sales and services of spark plugs, batteries and distributor parts

2.	Tire services and repair, but not recapping or re-grooving

3.	Replacement of mufflers and tail pipes, water hoses, fan belts, brake fluid, light bulbs, fuses, 			floor mats, seat covers, windshield wipers and blades, grease containers, wheel bearings, 			mirrors, and the like

4.	Radiator cleaning and flushing

5.	Radiator wending and repair

6.	Greasing and lubrication

7.	Providing and repairing fuel pumps, oil pumps and lines

8.	Minor servicing and repair of carburetors

9.	Adjusting and repairing brakes

10.	Minor motor adjustment not involving removal of the head of crankcase or racing the motor

11.	Sales of cold drinks, packaged food, tobacco and similar convenience goods to service station 			customers, as accessory and incidental to principal operation

12.	Provisions of road maps and other informational materials to customers, provision of restroom 			facilities

13.	Warranty maintenance and safety inspections

Uses permissible at a service station do not include major mechanical and bodywork, straightening of 	body parts, painting, welding, and storage of automobiles not in operating condition.   A service station 	is neither a repair garage nor a body shop.

GARAGE – PRIVATE:  An accessory building or an accessory portion of the main building, enclosed on all sides and designed or used for the shelter or storage of passenger vehicles or personal property and located on the same lot as the dwelling to which it is accessory.  Garages and/or storage buildings containing more than one thousand two hundred square feet shall require a Conditional Use Zoning Certificate in an R-1, R-2 and R-3 District.

GARAGE – PUBLIC:  A building, or portion of a building, in which more than four motor vehicles are, or are intended to be, housed under arrangements with the patrons for renting or leasing such space and accommodation, and in which no repair work is carried on.

GASOLINE FUELING PUMP:  Each fueling location within a gasoline station.  The number of gasoline pumps shall be calculated based upon the number of vehicles capable of refueling at the same time within the gasoline station.

GASOLINE STATION, RETAIL:  An individual business, often in the form of a convenience store, which dispenses fuel for motor vehicles to the general public.

GASOLINE STATION, WHOLESALE:  A business often associated with a larger business located on site, which dispenses fuel for motor vehicles at a discounted rate through the use of member pricing and/or club cards.

GOVERNMENT BUILDINGS AND PUBLIC USE:  A building or land used and/or controlled exclusively for governmental or public purposes by any department or branch of government including township, state, county, or other recognized public entity.  Such use may include, but is not limited to, township offices, public works, libraries, post offices, and other uses not defined separately within this Resolution.  Government and public use shall not include schools or other educational facilities as defined elsewhere in this Resolution.

GRADE, FINISHED:  For buildings abutting one street only, the elevation of the sidewalk at the center of the wall facing the street (or the elevation of the centerline of the street where no sidewalk exists); for buildings having walls facing more than one street, the average elevation of the sidewalk and the centers of all walls facing the street; for buildings having no walls facing the street, the average level of the finished surface of the ground adjacent to the exterior walls of the buildings.  (Any wall approximately parallel to a street line is to be considered as facing the street.)

GRADE, NATURAL:  The elevation of the undisturbed natural surface of the ground prior to any excavation or fill.

GRANNY COTTAGE: (e.g. ACCESSORY DWELLING UNIT) A dwelling unit accessory to the principal dwelling unit on a lot.

GREENHOUSE:  A glass or plastic enclosed structure in which plants requiring controlled temperature are grown or stored.

GREENHOUSE – COMMERCIAL:  A greenhouse for storage and sale of plants as a commercial business.

GREENHOUSE – PERSONAL:  A greenhouse used for personal use only with no products for sale.

GREENHOUSE – TEMPORARY:  A greenhouse used for the seasonal sales of plants and plant products for a period of no more than six months in a one-year period.

GROSS FLOOR AREA:  Expressed in square feet and calculated by the applicant, and shall include the area within the exterior building walls of all floors of a building or structure.  Gross floor area shall include all occupiable areas minus vehicular parking and loading areas within the structure.

GROSS LEASEABLE AREA:  Expressed in square feet and calculated by the applicant, and shall mean the gross floor area minus the following floor area deductions:

1.	Elevator shafts and stairways

2.	Public restrooms

3.	Public lobbies, common mall areas, atriums and courtyards provided solely for pedestrians 			access to the building from the exterior, and/or for aesthetic enhancement or natural lighting 			purposes

4.	Permanently designed corridors (i.e. not subject to relocation by the requirements of a specific 			space)

GROUP DWELLING DEVELOPMENT:  Two or more structures of multi-family dwellings located on the one lot.

GROUP HOME:  A residential facility that provides room and board, personal care, habitation services and supervision in a family setting for at least nine but no more than sixteen persons with developmental disabilities.  ORC 5123.19.

H

HAZARDOUS/INFECTIOUS WASTE:  As defined under Chapter 3745-51 and 3745-27, respectively, of the Ohio Administrative Code.

HEDGE:  A fence of bushes, a protecting barrier planted by the property owner, excluding any form of trees.

HOME OCCUPATION:  Any use or profession customarily incidental and subordinate to a residential dwelling’s use for residential purposes which does not change the essential character for appearance of residential properties and neighborhoods.

HOSPITAL:  Any building or other structure containing beds for at least four patients and devoted to the medical diagnosis, treatment, or other care of human ailments.

HOTEL:  Any building in which lodging is provided and offered to the public for compensation and which is designed primarily for use by transient guests, as distinguished from a boarding house or a lodging house.

I

INCINERATOR BURNING:  As defined under Chapter 3745-17 of the Ohio Administrative Code.

INDUSTRIALIZED UNIT:  A building unit or assembly of closed construction fabricated in an off-site facility, that is substantially self-sufficient as a unit or as part of a greater structure, and that requires transportation to the site of intended use.  “Industrialized unit” includes units installed on the site as independent units, as part of a group of units, or incorporated with standard construction methods to form a completed structural entity.  “Industrialized unit” does not include a manufactured home as defined by ORC 3781.06(c)(4) or a mobile home as defined by division (0) of Section 4501.01 of the Revised Code.

INOPERABLE VEHICLE:  A vehicle which is apparently inoperable or which requires repairs in order to be operated legally on the public roads, such as:  repair or replacement of a window, windshield, wheel, tire, motor or transmission.

INSTITUTION:  Buildings and/or land designed to aid individuals in need of mental, therapeutic, rehabilitative counseling or other correctional services.

INSTITUTIONAL HOUSING:  Housing for the elderly or infirm in which three or more unrelated individuals may live on a short-term or long-term basis and where both food and care are provided for compensation.  Institutional housing includes, but is not limited to elderly housing, nursing homes, assisted living facilities, and hospices.  Institutional housing shall not include hospitals, medical offices/clinics, or similar institutions devoted primarily to the diagnosis and treatment of the sick or injured.

INSTORE BAKERY DISTRIBUTION CENTER:  The facility within a retail establishment which not only processes, assembles, packages and sells baked goods for its own purposes, but for the purposes of distributing baked goods to off-premises establishments owned by the proprietor of the store in which the bakery is located.

J

JUNK/SCRAP YARD:  Is the use of more than twenty five square feet of any land, building or structure, whether for private and/or commercial purposes, where waste, discarded or salvaged materials such as scrap metals, used building materials, used lumber, used glass, discarded motor vehicles or parts of motor vehicles, plastic, iron, paper, rags, rubber, cordage, barrels or other similar materials, are sold, stored, bought, exchanged, baled, packed, sorted, disassembled, dismantled, or handled, for more than fifteen days.  This includes all non-hazardous demolition materials.

JUNK VEHICLE:  Any vehicle including, but not limited to, automobiles, commercial vehicles, boats and trailers, motorcycles, and recreational vehicles certified under ORC 505.173(E) and meeting all of the following criteria:

1.	Three model years old, or older;

2.	Apparently inoperable;

3.	Extensively damaged, including, but not limited to, any of the following:  missing wheels, tires, 			engine, or transmission, rust or body damage covering more than ten percent of the vehicles 			exterior surface, excluding undercarriage.

K

KENNEL:  An establishment where dogs or cats are bred, trained, or boarded.

L

LANDOMINIUM:  A development where an individual dwelling unit and the land it sits on is owned privately but where the landscaping and lawn is maintained by a homeowner’s or other community association.

LANDSCAPE STRIP:  The area of ground required between nonresidential properties consisting of grass, flowers, shrubs, trees or other vegetation.  Gravel alone is not adequate landscape material.

LANDSCAPED:   Sodden, seeded, mulched, shrubs, and other appropriate material.

LARGE EVENT:  Events which would utilize (or are expected to utilize) the total number of spaces dedicated to a specific use, or will generate significant short-term traffic immediately prior to or following the event.  These events shall include sporting events such as high school football games, or playoff games and community events such as outdoor concerts.  Events requiring a special events permit, including large funerals or weddings, may be required to meet this provision.

LATTICE TOWER:  A support structure constructed of vertical metal struts and cross braces forming a triangular or square structure which often tapers from the foundation to the top.

LICENSED VEHICLE DEALER:   An individual or company licensed by the State of Ohio according to the provision of ORC 4517.  Any individual or company selling more than five vehicles in a twelve-month period shall be required to be a licensed vehicle dealer.

LIGHT INDUSTRIAL USE:  The manufacturing, processing, or assembly of products within a fully enclosed structure where noise, odor, light, or vibration is not noticeable from the adjacent properties.

LITTER CONTROL:  Fence screening and/or landscaped mounding of suitable design and installation shall be utilized where required necessary to control against fugitive waste being transported off the site of a permitted use facility by wind or water forces.  Any and all wastes that escape the use facility site by such means shall be recollected and contained by the owner/operator of the facility for proper required disposal and the use site facility shall be kept in a neat and well-kept condition at all times.

LOADING SPACE:  An off-street space or berth on the same lot with a building, or contiguous to a group of buildings, for the temporary parking of a commercial vehicle while loading or unloading merchandise or materials, and which abuts upon a street, alley, or other appropriate means of access.

LODGING HOUSE:  A building other than a motel or hotel, where for compensation by the week or month, lodging is provided for at least three but not more than twenty persons.

LOT:  A parcel of land occupied or intended for occupancy by a use permitted in this Resolution and the open spaces required by this Resolution and having its principal frontage upon a street or place.

LOT AREA:  The total area within the lot lines of a lot, excluding any street right-of-way or other legal public dedication.

LOT, CORNER:  A lot abutting upon two or more streets at their intersection or upon two parts of the same street, and in either case forming an interior angle of less than one hundred thirty-five degrees. Appendix “L”.

LOT COVERAGE:  That portion of a lot that is covered by the principal and accessory building, structures, and surfaces that prevent the passage or absorption of storm water including paving and driveways.

LOT DEPTH:  The mean horizontal distance between the front and rear lot lines.  See Appendix “M”.

LOT DOUBLE FRONTAGE:  A lot having a frontage on two nonintersecting streets, as distinguished from a corner lot.  See Appendix “L”.

LOT FRONTAGE:  The dimension of a lot abutting a public street measured along the street right-of-way line.

LOT INTERIOR:  A lot other than a corner lot, or a double frontage lot.

LOT LINE, FRONT:  The front property line, which is coterminous with the street right-of-way.  A front lot line is generally parallel to or less than forty-five degrees to the rear lot line.  The front lot line is generally opposite the rear lot line.   A corner lot or double frontage lot has more than one front lot line.  See Appendix “M”.

LOT LINE, REAR:  An internal lot line opposite a front yard.  A rear lot line is generally parallel to or less than forty-five degrees to the front street right-of-way line.  A lot line greater than forty-five degrees from the front street right-of-way line would be a side lot line.  See Appendix “M”.

LOT LINE, SIDE:  An internal lot line generally extending perpendicular to the front and rear lot lines.  The side lot line extends between the front lot line and the rear lot line.  See Appendix “M”.

LOT LINES:  The property lines bounding the lot.

LOT OF RECORD:  A lot which is part of a subdivision, the map of which has been recorded in the office of the Recorder of Tuscarawas County; or a parcel of land, the deed to which was of record on or prior to the effective date of these regulations.

LOT, PANHANDLE:  A lot not fronting or abutting a public street and where access to the public street is limited to a narrow strip of land.  See Appendix “L”.

LOT WIDTH:  The horizontal distance between side lot lines measured at the required front setback or at the building line for any irregularly shaped lot.  See Appendix “L”.

M

MANUFACTURE:  The process of making something from raw or semi-finished materials whether by hand or by mechanized process.  These regulations also include producing, assembling, fabricating, alloying, and metal and chrome plating.

MANUFACTURED HOME:  Any non-self-propelled vehicle transportable in one or more sections which in the traveling mode is eight feet or more in width or forty feet or more in length or, when erected on site is eight hundred twenty or more square feet, and which built on a permanent chassis and designed to be used as a permanent dwelling unit with or without a permanent foundation when connected to the required utilities, and includes the plumbing, heating, air conditioning, and electrical systems contained therein, and which bears a label certifying that is built in compliance with federal manufactured housing construction and safety standards.

MANUFACTURED HOME PARK:  Any tract of land upon which three or more manufactured or mobile homes used for habitation are parked, either free of charge or for revenue purposes, and includes any roadway, building, structure, vehicle, or enclosure used or intended for use as a part of the facilities of the park per ORC 3733.01.

MANUFACTURED HOUSING:  Manufactured homes and mobile homes.

MANUFACTURING EXTRACTIVE:  Any mining, quarrying, excavating, processing, storing, separating, cleaning or marketing of any mineral natural resource.

MANUFACTURING HEAVY:  Manufacturing, processing, assembling, storing, testing and similar industrial uses which are generally major operations and extensive in character; require large sites, open storage and service areas, extensive services and facilities, ready access to regional transportation; and normally generate some nuisances such as smoke, noise, vibration, dust, glare, air discharge, and water discharge, but not beyond the district boundary.

MANUFACTURING LIGHT:  Manufacturing or other industrial uses which are usually controlled operations; relatively clean, quiet and free of objectionable or hazardous elements such as smoke, noise, odor or dust; operating and storing within enclosed structures; and generating little industrial traffic.

METEOROLOGICAL TOWER:  A facility consisting of a tower and related wind-measuring devices, which is used solely to measure winds preliminary to construction of a small wind energy conversion system.  Meteorological towers shall not be allowed for time periods in excess of six months, and shall be removed prior to the installation of the wind energy conversion system for which they are measured.  A request to install a meteorological tower shall be included in the application to install a small wind energy conversion system.

MINERAL EXTRACTION:  Any artificial or mechanical act by which earth, sand, rock or any other similar material is dug into, cut, quarried, uncovered, removed, displaced, relocated or moved and shall include the conditions resulting there from, including but not limited to gravel pits and not including the impacts of such operation.

MINERALS:  Sand, gravel, clay, shale, gypsum, halite, limestone, dolomite, sandstone, other stone, metalliferous, non-metalliferous ore, other material or substance of commercial value excavated in a solid state from natural deposits on or in the earth, but does not include coal, peat, or top soil.

MINIMUM BUILDING SETBACK LINE:  A line parallel to the street right-of-way line and at a distance there from equal to the required depth of the front yard, and extending across the full width of the lot.

MINI STORAGE:  A limited storage facility for private and commercial use with limited access.  Maximum compartment storage to be no more than five hundred feet.

Minor Street:  The Street normally carrying the lower volume of vehicular traffic.

MOBILE HOME:  A building unit or assembly of closed construction that is fabricated in an offsite facility, is more than thirty-five body feet in length or, when erected on site, is three hundred twenty or more square feet, is built on a permanent chassis, is transportable in one or more sections, and does not qualify as a manufactured home as defined in division (C) (4) of Section 3781.06 of the Revised Code or as an Industrialized Unit as defined in division (C) (3) of Section 3781.06 of the Revised Code.

MODULAR HOME:  Factory built housing certified as meeting the local or state building code as applicable to modular housing.  Once certified by the state, modular homes shall be subject to the same standards as site-built homes.

MONOPOLE:   A support structure constructed of a single, self-supporting hollow metal tube securely anchored to a foundation.

MORTUARY:  	An establishment with facilities for the preparation of human dead for burial or cremation at a place other than the subject premises, for the viewing of the body, and for funerals.

MOTEL:  Any building or group of buildings containing sleeping rooms, with or without cooking facilities, designed primarily as overnight sleeping quarters for automobile tourists or transients, with garage attached or parking space conveniently located to each unit, including auto courts, motor lodges, and tourist courts.

MOTOR VEHICLE:  Any vehicle, including a recreational vehicle, propelled by power other than muscular power or power collected from overhead electric trolley wires.

MOTOR VEHICLE SALVAGE DEALER:  Any person who engages in business primarily for the purpose of selling salvage motor vehicle parts and secondarily for the purpose of selling at retail salvage motor vehicles or manufacturing or selling a product of gradable scrap metal.

MULTI-FAMILY:  Multiple separate housing units for residential (i.e. non-commercial) inhabitants contained within one building or several buildings within one complex.

N

NATURAL ABIENT NOISE LEVEL:  The normal and predominant noise level absent any industrial or commercial noise radiation, excluding any noise resulting from any agricultural operations.

NON-COMMERCIAL MOTOR VEHICLES:  Any motor vehicle, including a farm truck as defined in Section 4503-04 of the Revised Code, designed by the manufacturer to carry a load of no more than one ton and used exclusively for residential family dwelling purposes and not for engaging in business for profit.

NON-CONFORMING USE:  Any building or land lawfully occupied by a use on the effective date of these regulations or any amendment or supplement thereto, which does not conform to the Use Regulations of the District in which it is situated.

NON-RESIDENTIAL BUILDING:  Non-residential building means any building that is not a residential building or a manufactured or mobile home.

O

OCCASIONAL OR TEMPORARY VISITOR:  A non-residential family member or guest who has an established residence elsewhere who is visiting for not more than thirty days in a twelve-month period.

OFFICE:  A room, set of rooms, or building used as a place for commercial, professional, or bureaucratic work. A use, conducted within a building or part of a building, involving the conduct of the administrative, executive, management, or clerical affairs of a business, service, industry, or government; or the activities of a profession; and generally furnished with desks, tables, files, and communication equipment.

OPEN SPACE:  An area substantially open to the sky, which may be on the same lot with a building or buildings.  The area may include along with the natural environmental features, water areas, swimming pools, walking or bike paths, and tennis courts, or any other similar recreational facilities provided within the development to the occupants of such a development.  Streets, parking areas, structures for habitation, trash collection sites and the like shall not be included.  Land devoted to conservation or recreational purposes and/or lands designated by the Township to remain undeveloped.

OPEN SPACE RESIDENTIAL DEVELOPMENT:  A development where the lot sizes and other site development standards are reduced to allow for the preservation of open space.

ORC shall mean the Ohio Revised Code.

OUTDOOR ADVERTISING SIGN (BILLBOARD):  A fixed or portable off premises sign which advertises a business, profession or service not conducted or offered upon the premises where such sign is placed.  See Article IV.

OUTDOOR STORAGE:  The keeping of any goods, materials, merchandise, or vehicles outside of a structure or building for more than forty-eight hours.  Outdoor storage shall not include car lots, tool rental establishments, greenhouses, or other uses where the sale of the merchandise is the primary use of the property pursuant to Section 203.8 (Outdoor Display, Sales, and Storage).

OUTDOOR WOOD FURNACE:  Any equipment, device, appliance or apparatus, or any part thereof, which is installed, affixed or situated outdoors for the primary purpose of combustion of fuel to produce heat or energy used as a component of a heating system providing heat for any interior space or water source.  An Outdoor Wood Furnace may also be referred to as an Outdoor Wood Boiler or Outdoor Wood Hydronic Heater.

OVERBURDEN:  All of the earth materials, which cover a natural deposit of minerals, coal and peat.  Also means such earth and other materials after removal from their natural state in the process of surface and strip mining.

P

PARK - ACTIVE AND RECREATIONAL FACILITY:  Any park or recreational facility that requires grading of the land, construction of facilities, lighting, or is developed for ball fields, tennis courts, swimming pools, skate parks, disc golf, golf courses and other active sports facilities with the exception of bike and hike trails.

PARK – PASSIVE, RECREATIONAL FACILITY AND CONSERVATION AREA:  Any park or recreational facility where there is no grading of the land, construction of facilities, lighting, or development of ball fields with the exception that passive parks, recreational facilities, and conservation areas may include the development of trails and sidewalks.

PARK – PUBLIC:  Land owned by a government entity which has been designated for park or recreational activities including, but not limited to, a park, playground, nature trails, swimming pool, reservoir, athletic field, basketball or tennis courts, pedestrian/bicycle paths, open space, wilderness areas, or similar public land within the township which is under the control; operation, or management of the township, county or state.

PARKING AREA:  Includes the parking spaces together with driveway and the access to public streets.

PARKING LIFT:  A hydraulic or mechanical system designed to raise or lower parked vehicles in order to create additional parking space, typically above or below the lift and within an enclosed building.

PARKING LOT:  An off-street parking area where the principal uses of the tract, or lot, is for vehicular parking.

PARKING SPACE:  An off-street space or berth for the temporary parking of one vehicle for a period longer than required to load or unload persons or goods, the dimension of a parking space shall be in accordance with the Lawrence Township development standards.  Ratios of compact spaces and ADA accessible spaces shall be determined upon application and in accordance with the Ohio Building Code.

PEAT:  Partially carbonized vegetable matter formed by partial decomposition of various plant life in water in a natural habitat.

PERMANENT FOUNDATION:  Permanent foundation means permanent masonry, concrete, or a footing or foundation approved by the manufactured homes commission pursuant to ORC 4781.01(I) and ORC 3781.06(c)(5) to which a manufactured or mobile home may be affixed.

PERMANENTLY SITED MANUFACTURED HOME:  Permanently sited manufactured home means a manufactured home that meets all of the following criteria:

1.	The structure is affixed to a permanent foundation and is connected to appropriate facilities;

2.	The structure, excluding any addition, has a width of at least twenty-two feet at one point, a 			length of at least twenty-two feet at one point, and a total living area, excluding garages, 			porches, or attachments, of at least nine hundred square feet;

3.	The structure has a minimum of three to twelve residential roof pitches, conventional 				residential siding, and a six-inch minimum eave overhang, including appropriate guttering;

4.	The structure was manufactured after January 1, 1995;

5.	The structure is not located in a manufactured home park as defined by Section 4781.01(D) of 			the Revised Code.

PERSONAL SERVICES:  Establishments that are primarily engaged in providing services generally involving the care of the person or person’s possessions.  Personal services may include, but are not limited to, laundry and dry-cleaning services, barber shops, beauty salons, health and fitness studios, music schools, informational and instructional services, tanning salons, and portrait studios.

PLANNED COMMERICAL COMPLEX:  Two or more primary use structures placed on a common lot in C-1, C-2, C-3 or I-1 Districts.

PLANNED OFFICE COMPLEX:  Two or more primary use structures placed on the common lot in C-1, C-2, C-3 or I-1 Districts.

PLANTS, INVASIVE:  Plants that are invasive to Ohio as listed in the following link:  http://www.invasivespeciesinfo.gov/plants/main.shtml.

PLANTS, NATIVE:  Plants that are native to Ohio and should have first priority when choosing plants as listed in the following link:  http://ohioline.osu.edu/b865/b865_01.html.

PRINCIPAL BUILDING:  The building on a lot used to accommodate the primary use to which the premises are devoted.

PRIVATE DRIVE:  A shared means of vehicular ingress and egress located within an easement of access serving rear or panhandle lots, not dedicated to the County by recorded instrument that is maintained by the party or parties using such private drive for private access.

PROFESSIONAL ENGINEER:  A qualified individual who is licensed as a Paraprofessional Engineer in the State of Ohio.

PROFESSIONAL OR BUSINESS OFFICE:  Establishments providing executive, management, administrative, or professional services including, but not limited to, real estate, architecture, legal, travel, medical, dental, employment, advertising, design, engineering, accounting, and similar uses.

PUBLIC BUILDING:  A building owned by the Lawrence Township Board of Trustees.

PUBLIC PARK:  Land owned by a governmental entity which has been designed for park or recreational activities including but not limited to, a park, playground, nature trails, swimming pool, reservoir, athletic field, basketball or tennis courts, pedestrian/bicycle paths, open space, wilderness areas, or similar public land within the township which is under the control, operation, or management of the township, county, or state.

PUBLIC UTILITY:  An entity deemed a public utility for the purpose of ORC519.211.

Q

R

RATED NAMEPLATE CAPACITY:  The maximum rated output of electric power production equipment for a small wind energy conversion system.  This output is typically specified by the manufacturer with a “nameplate” on the equipment.

RECREATIONAL EVENTS:  Temporary activities including fairs, festivals, block parties and various entertainments.

RECREATIONAL FACILITY/USE:  A place designed and equipped for the conduct of sports, leisure time activities and other customary and usual recreational activities or entertainment activities, but not including those uses otherwise specifically defined or regulated in this resolution.  This does not include a recreational use that is an accessory to a residence.  As provided in this resolution, recreational facilities are classified into four types:

RECREATIONAL FACILITIES/USE, TYPE A:  Outdoor recreation facilities/uses, usually require a large space, which are relatively quiet and compatible with residential uses, including, but not limited to, picnic area, park, playground, trails for walking, bicycling or horse-riding (excluding trails used by motorized vehicles), fishing, boating, observation of nature, non-commercial field and court sports and similar facilities and activities.

RECREATIONAL FACILITIES/USE, TYPE B: Outdoor recreation facilities/uses for the commercial use of 			intense sports and athletic activities, and including, but not limited to, field sports, baseball 			field, soccer field, swimming pools, tennis courts, basketball court, miniature golf course, a 			regulation golf course and related facilities and golf driving range.

RECREATIONAL FACILITIES/USE, TYPE C:  Indoor recreation and entertainment facilities/uses, including, 			but not limited to, those required for indoor court sports & field sports, athletic clubs, bowling 			alleys, theaters, auditoriums, lodge halls, health and fitness spa, miniature golf course, 				swimming pool, and social clubs.

RECREATIONAL FACILITIES/USE, TYPE D:  Recreational facilities/uses which are not classified as Type A, 			Type B, Type C which may generate noise and may otherwise not be compatible with

residential uses, including, but not limited to, shooting range, amusement park, water park,

race track for motorized vehicles, animal race track, amphitheaters, paintball games and

campgrounds.

RECREATIONAL PRIVATE USE:  Privately owned and utilized recreation facilities located on a lot with a dwelling and used by the occupants and of the dwelling and their non-paying guests thereof as an accessory to the dwelling use of the property.

RECREATIONAL VEHICLES:  Vehicular portable structure that meets all of the following conditions:

1.	It is designed for the sole purpose of recreational travel;

2.	It is not used for the purpose of engaging in business for profit;

3.	It is not used for the purpose of engaging in intrastate commerce;

4.	It is not used for the purpose of commerce as defined in 49 C.F.R. 383.5, as amended;

5.	It is not regulated by the public utilities commission pursuant to Chapter 4919, 4921, or 4923, 			of the Revised Code;

6.	It is classed as one of the following:

Travel Trailer:  means a non-self-propelled recreational vehicle not exceeding an overall 					length of thirty-five feet, exclusive of bumper and tongue or coupling, and 					includes a tent type fold out camping trailer as defined in division(s) of Section 					4517.01 of the Revised Code.

Motor Home:  means a self –propelled recreational vehicle constructed with 						permanently installed facilities for cold storage, cooking and consuming of food 					and for sleep.

Truck Camper:  means a non-self-propelled recreational vehicle, without wheels for road 				use, and designed to be place upon and attached to a motor vehicle.  Truck 					camper does not include truck covers, which consists of walls and roof but does 					not have floors and facilities for using same as a dwelling.

Fifth Wheel Trailer:  means a vehicle that is of such size and weight as to be movable 					without a special highway permit, that has a gross trailer area of four hundred 					square feet or less, that is constructed with a raised forward section that allows 					a bi-level floor plan and that is designed to be towed by a vehicle equipped with 					a fifth-wheel hitch or ordinary installed in the bed of a truck.

Park Trailer:  means a vehicle that is commonly known as a park model recreational 					vehicle, meets the American National Standard Institute Standard A119.5 (1988) 					for park trailers, is built on a single chassis, has a gross trailer area of four 						hundred square feet or less when set up, is designed for seasonal or temporary 					living quarters, and may be connected to utilities necessary for the operation of 					installed features and appliances.

RECYCLING FACILITIES: included are Recycling Center or Transfer Facility, Recycling Plant, and Recycling Convenience Center.

RECYCLING TRANSFER FACILITY:  A facility for the collection of products such as paper, glass, plastic, metals, rubber, wood and synthetic products.

RESEARCH FACILITIES:  Research, development and testing related to such fields as chemical, pharmaceutical, medical, electrical, transportation and engineering.  All research, testing and development shall be carried on within entirely enclosed buildings, and no noise, smoke, glare, vibration or odor shall be detected outside of such building.

RESIDENTIAL CLUSTER DEVELOPMENT:  The grouping of residential properties on a development site in order to use the extra land as open space, recreation or agriculture.

RESIDENTIAL FACILITY:  A home or facility, as defined and regulated in Section 5123.19 of the Ohio Revised Code, as amended.

RESIDENTIAL STRUCTURE:  A structure containing a one-family, two-family, or three-family dwelling unit, and any accessory structure incidental to that dwelling unit and any dwelling unit that is used as a model to promote the sale of a similar dwelling unit.  Residential structure does not include an industrialized unit as defined by these regulations, a manufactured home as defined by these regulations, or a mobile home as defined by these regulations.

RESIDUAL SOLID WASTE OR RESIDUAL WASTE:  As defined under Chapter 3745-27-01 of the Ohio Administrative Code.

RESTAURANT, DRIVE THRU – An establishment in which there is no seating inside or outside the establishment for customers and meals or beverages are served at a drive-up window only.

RESTAURANT TYPE A:  An establishment, with an occupancy load of greater than seventy-five (75) occupants per the Occupancy Permit, which is located in a building that is primarily engaged in the preparation and serving of meals and beverages which are consumed on its premises by customers seated at tables and/or counters either inside or outside the establishment and engage in providing customers with take-out service of food and beverages for off-site consumption.  This definition shall include taverns, lounges, bars, clubs and lodges.

RESTAURANT TYPE B:  An establishment, with an occupancy load of seventy-five (75) or les Occupancy Permit, which is located in a building that is primarily engaged in the preparation and serving of meals and beverages which are consumed on its premises by customers seated at tables and/or counters either inside or outside the establishment and engage in providing customers with take-out service of food and beverages for off-site consumption.  This definition shall include taverns, lounges, bars, clubs and lodges.

RETAIL COMMERICAL SERVICES:  Establishments primarily engaged in the sale of goods and materials to the general public.  Retail commercial uses may include, but are not limited to, bookstores, antique stores, convenience stores, bakeries, grocery stores, and other similar uses.

ROADSIDE STAND (FARM MARKET):  A temporary vehicle or temporary stand without foundation used for the sale of agricultural produce where fifty percent or more of the gross income received from the market is derived from produce raised on farms owned or operated by the market operator in a normal crop year, in accordance with Section 519.21.

ROTOR DIAMETER:  The length as measured across the center of the full spine of the rotors of a SWECS turbine.

S

SANITARY LANDFILL:  A land disposal site employing a method of disposing of solid wastes on land in a manner intended to minimize environmental hazards by spreading the solid wastes in thin layers, compacting the solid wastes to the smallest practical volume, and applying and compacting cover material daily.  Does not include hazardous materials.

SATELLITE SIGNAL RECEIVER: “Dish-type Satellite Signal-Receiving Antennas”, “earth stations”, or “ground stations” whether functioning as part of a basic service system, direct broadcast satellite system, or multi-point distribution service system designed, constructed or modified to bring in or receive satellite television signals.

SCHOOL:  Buildings for the education of children grades K-12, and colleges and universities.  Includes both general public use and private institutions, excluding; however, trade and other types of schools not listed herein.

SCREEN:  A permanent or semi-permanent sight-obscurity device, typically affixed to the ground or a structure, designed to minimize the visual impact of certain features on a site or group of sites.  Screens may include, but are not limited to, vegetation, fences, or walls, but shall not include temporary devices such as tarps or vehicle covers.

SCREENING:  A continuous fence, wall, trees, shrubs, mound or combination thereof that effectively buffers the property and is broken only by access drives and walks.

SEASONAL USE:  A use dependent on a particular season of the year.

SEMI-TRAILERS:  Any vehicle of the trailer type without motive power so designed or used with another and separate motor vehicle that in operation a part of its own weight or that of its load, or both, rests upon and is carried by such other vehicle furnishing the motive power for propelling itself and the vehicle referred to in this division and includes, for the purpose only of registration and taxation under such chapters, any vehicle of the dolly type, such as a trailer dolly designed or used for the conversion of a semi-trailer into a trailer.

SENSITIVE ENVIRONMENTAL AREAS:  Any areas determined by the Ohio Department of Natural Resources, any other state or federal governmental agency, or the Township that consists of unique or sensitive, ecological, biological or related ecosystems.

SEPTAGE:  Means the liquid or solid material removed from a septic tank or similar domestic or commercial sewage treatment system when the system is serviced.

SERVICE COMMERCIAL USE:  Establishments that primarily engage in rendering services to businesses including, but not limited to, printers, equipment rental, protective services, mailing, photo finishing, and other similar uses.

SEXUALLY ORIENTED-ADULT CABARET:  Refer to Lawrence Township Resolution 19-09.

SHIPPING CONTAINER:  is a container with strength suitable to withstand shipment, storage, and handling. Shipping containers range from large reusable steel boxes used for intermodal shipments to the ubiquitous corrugated boxes. In the context of international shipping trade, "container" or "shipping container" is virtually synonymous with "intermodal freight container," a container designed to be moved from one mode of transport to another without unloading and reload.

SIGN:  Any visual communication display, object, device, graphic structure, or part, situated indoors or outdoors, or attached to, painted on, or displayed from a building or structure, in order to direct or attract attention to, or to announce or promote, an object, person, service, product, event, location, organization or the like, by means of letters, words, designs, colors, symbols, fixtures, images or illuminations.  Refer to Article V Section 501.1.

SKILL-BASED AMUSEMENT MACHINE(S):  As defined in Ohio Revised Code 2915.01.

SLUDGE:  Thick suspension of solid matter in a liquid, e.g., the mud on a riverbed, or carbonaceous mixture of oil as the waste product of an internal-combustion engine; the treated solid matter of sewage after drying used as a fertilizer; floating (partly melted) ice or snow sludge.

SMALL WIND ENERGY CONVERSION SYSTEMS (SWECS):  A wind energy conversion system consisting of a wind turbine, tower, and associated control or conversion electronics that generates power for an individual property for the purpose of reducing on-site energy consumption with a rated nameplate capacity of one hundred kW or less.  This includes, but is not limited to, storage, electrical collection and supply equipment, and transformers.  Excess electrical power generated, and not presently needed for onsite use, may be utilized by the utility company.  SWECS shall include Horizontal Axis Wind Turbines (HAWTs), Vertical Axis Wind Turbines (VAWTs), and Blade Tip Power System Turbines (BTPSs).

SMALL STRUCTURE MOUNTED WIND ENERGY CONVERSION SYTEM (SSM-WECS):  A structure mounted wind energy system that converts wind energy into electricity through the use of equipment (e.g. base, blade, rotor, foundation, generator, nacelle, tower, transformer, vane, wire, inverter, batteries, etc.).  SSM-WECS’s are attached to a structure’s roof, walls or another elevated surface.  SSM-WECS’s have nameplate capacities that do not exceed to ten kilowatts.  SSM-WECS definitions:

NACELLA:  The enclosure located at the top of a wind turbine tower that houses the gearbox, generator 		and other equipment.

POWER CENTER:  Serves as the central connection point for the electrical components in the system

and provides a number of necessary control functions.

ROTOR:  The rotating part of the turbine, including the blades.

TOWER:  The support structure, including guyed, monopole and lattice types, upon which a wind 			turbine or other mechanical device is mounted.

TOWER HEIGHT:  The height of the tower, measured from the natural grade surrounding the support 			pad to the tip of the blade in a vertical position along the vertical axis of the tower.

WIND TURBINE:  A device that converts kinetic wind energy into rotational energy that drives an 			electrical generator.  A wind turbine typically consists of a tower, nacelle body, power center 			and rotor with two or more blades.

SNACK BAR:  A public eating place where snacks are served usually at a counter.

SOLOR PANEL:  A solar photovoltaic panel, or solar hot air or water panel collector device, which relies upon solar radiation as an energy source for the generation of electricity or transfer of stored heat.

SOLID WASTE:  Unwanted residual solid or semisolid material as results from industrial, commercial, agricultural, and community operations, excluding earth or material from construction, mining, or demolition operations, or other waste materials of the type that normally would be included in demolition debris, nontoxic fly ash and bottom ash, including at least ash that results from the combustion of coal and ash that results from the combustion of coal in combination with scrap tires where scrap tires comprise not more than fifty percent of heat input in any month, spent nontoxic foundry sand, and slag and other substances that are not harmful or inimical to public health, and includes, but is not limited to, garbage, scrap tires, combustible and noncombustible materials, street dirt, and debris.  Solid waste does not include any material that is an infectious waste or hazardous waste.

SOLID WASTE DISPOSAL FACILITY:  As it relates to the Solid Waste District of this Resolution means any site, location, tract of land, installation, or building used for incineration, composting, sanitary land filling, or other methods of disposal of solid wastes or, if the solid wastes consist of scrap tires, for the collection, storage or processing of the solid wastes; for the transfer of solid wastes; for the treatment of infectious wastes; or for the storage, treatment, or disposal of hazardous waste.

SOLID WASTE TRANSFER FACILITY:  Means any site, location, tract of land, installation, or building that is used or intended to be used primarily for the purpose of transferring solid wastes that were generated off the premises of the facility from vehicles or containers into other vehicles for transportation to a solid waste disposal facility as defined in Section 3734 of the Ohio Revised Code.  Solid waste transfer facility does not include any facility that consists solely of portable containers that have an aggregate volume of fifty cubic yards or less nor any facility where legitimate recycling activities are conducted.

SPECIAL EVENT VENDOR PERMIT:  A permit issued for a temporary place of business for the sale of goods or services at fairs or festivals within the township.

SPOIL BANK:  A deposit of removed overburden.

STACKING SPACE:  A lane or area that is specifically designated for cars to stack in while utilizing drive-up or drive-through services at uses that may include, but are not limited to, car washes, restaurants, and financial institutions.

STORAGE:  When used in collection with hazardous waste, means the holding of hazardous waste for a temporary period in such a manner that it remains retrievable and substantially unchanged physically and chemically and, at the end of the period, is treated; disposed of; stored elsewhere; or reused, recycled, or reclaimed in a beneficial manner.  When used in connection with solid wastes that consist of scrap tires, storage means the holding of scrap tires for a temporary period in such a manner that they remain retrievable and, at the end of that period, are beneficially used; stored elsewhere; placed in a scrap tire monocell or monofill facility licensed under section 3734.81 of the Revised Code; processed at a scrap tire recovery facility licensed under that section or a solid waste incineration or energy recovery facility subject to regulation under this chapter; or transported to a scrap tire monocell, monofill, or recovery facility, any other solid waste facility authorized to dispose of scrap tires, or a facility that will beneficially use the scrap tires, that is located in another state and is operating in compliance with the laws of the state in which the facility is located.

STORAGE FACILITY:  A building or group of buildings in a controlled access compound that contains equal or varying sizes of individual, compartmentalized, and controlled access stalls or lockers for the storage of residential or commercial customer’s goods or wares.  Such facilities do not include sales, service, or storage of hazardous materials.

STORY:  That portion of a building included between the surface of any floor and the surface of the next floor above it, or if there is no floor above it, then the space between the floor and the ceiling next above it.

STORY, HALF:  A space under a sloping roof which the line of intersection of roof decking and wall face not more than three feet above the top floor level and in space not more than two-thirds of the floor areas is finished off for use.

STREET RIGHT-OF-WAY LINES:  A dividing line between a lot, tract, or parcel of land and contiguous street.   Where the lot, tract or parcel of land has been conveyed to the center of the street, the street right-of-way line then becomes the inside line of land reserved for street purposes or if no right-of-way is established, the right-of-way shall be assumed to be sixty feet.

STREET TYPES:

ACCESS STREET:  Minor street emptying into a collector or other primary street and connecting with a 			cul-de-sac, or other street facility.

ARTERIAL STREET:  A major street or highway primarily for through traffic, carrying large volumes of 			traffic and usually on a continuous route.

CIRCLE STREET:  Minor street in a circular or curved form, each street end connecting with another 			street having housing units only on the outside of the curve.

COLLECTOR STREET:  A primary street serving residential, commercial or industrial areas and carrying 			traffic flow from loop, circle, cul-de-sac or other minor streets generally to other areas of the 			community and to arterial streets.

CUL-DE-SAC:  Minor street terminating in a dead-end space enabling automobile traffic to turn around 			and serving several individual home sites grouped around the turnaround.

LOOP STREET:  Minor street in a looped or half circular form with each end connected to a collector 			street and serving housing units located on both sides of the street.

PRIVATE: A thoroughfare which affords principal means of access to abutting property, but which 			has not been dedicated to the public, or subject to public easements.

PUBLIC:  Public or private dedicated thoroughfares subject to public easement thereto, and which 			affords the principal means of access to abutting property.

STRIP MINING:  All or any part of the process followed in the production of coal from a natural deposit whereby the coal may be extracted after removing the overburden.

STRUCTURAL ALTERATIONS:  Any change on the supporting members of a building, such as bearing walls or partitions, columns, beams or girders or any increase in the area or cubic contents of the building.

STRUCTURE:  Anything constructed or erected, the use of which requires a permanent or temporary location on the ground or attached to something having a permanent or temporary location on the ground, including advertising signs, billboards, and farmers roadside stands.

SURFACE MINING:  All or any part of a process followed in the production of minerals or peat from the earth or from the surface of the land by surface excavation methods such as open pit mining, dredging, and placering or quarrying.

SWIMMING POOL, COMMERCIAL:  A body of water in an artificial receptacle or other container, whether located indoors or outdoors, used or intended to be used for public, semi-public, or private swimming by adults and/or children, whether or not any charge or fee is imposed upon adults or children, operated and maintained by any person as herein defined whether they be an owner, lessee, operator, licensee, or concessionaire, exclusive of a family pool as defined herein, and shall include all structures, appurtenances, equipment, appliances, and other facilities appurtenant to and intended for the operation and maintenance of a swimming pool, and also all swimming pools operated and maintained in conjunction with or by clubs, motels, hotels and community associations.

SWIMMING POOL, FAMILY:  A swimming pool used or intended to be used solely by the owner or lessee thereof and his family, and by friends invited to use it without payment of any fee.

SWIMMING POOL, PRIVATE:  Exclusively used without paying an additional charge for admission by the residents and guests of a single household, a multi-family development or a community, the members and guests of a club, or the patrons of a motel or hotel; an accessory use.

T

TELECOMMUNICATION:  The technology which enables information to be exchanged through the transmission of voice, video or data signals by means of electrical or electromagnetic systems.

TEMPORARY:  Intended to last only a short time unless otherwise specified in these regulations.

TEMPORARY:  As related to tents, uses not to exceed thirty days as related to signs not to exceed one hundred eighty days per calendar year.

TEMPORARY BUILDINGS OR STRUCTURES:  A building or structure intended for a limited duration for uses at special events, including but not limited to, fairs and festivals, which does not have a permanent foundation.

TEMPORARY CONSTRUCTION TRAILERS: A trailer for uses incidental to construction work intended for a limited duration that does not have a permanent foundation; however, such temporary trailer shall be removed within 30 days of the completion or abandonment of the construction work.

TEMPORARY SALES/OFFICE TRAILER: A trailer for sales/office use for a limited duration that does not have a permanent foundation incidental to the proposed principal use of the property; however, such temporary trailer shall be removed within 30 days upon completion of principal use.

TEMPORARY STORAGE CONTAINERS:  shall mean a portable structure or container that allows for storage of goods or materials, on or off-site and which is not permanently affixed to a foundation. Temporary storage containers can include semi-tractor trailers if they are used for storage and not transport.

TEMPORARY STORAGE POD/ PORTABLE STORAGE CONTAINER: 	A portable, weather-resistant receptacle designed and used for the storage or shipment of household goods, wares, building materials or merchandise.
This term shall not include roll-off containers or storage containers having storage capacity of less than 150 cubic feet that has no permanent foundation in which a permit is issued for the temporary location not to exceed six (6) months in a commercial district or two (2) months in a residential district.

(Example of a roll-off container: city yard waste & debris containers or open/unclosed containers.)

THOROUGHFARE:  A street or alley.

TOPSOIL:  Superficial soil capable of sustaining plant life indigenous to this area, ordinarily rich in organic matter or humus debris.

TOPSOIL REMOVAL:  Removal of topsoil from the premises.

TOURIST:  A person visiting, touring.

TOURIST DWELLING:  A dwelling where overnight accommodations are provided for tourists.

TOWER:  The support structure, including guyed, monopole and lattice types, upon which a wind turbine or other mechanical device is mounted.

TOWNHOUSE:  A single-family dwelling constructed in a group of three or more attached units in which each extends from the foundation to the roof and with open space on at least two sides.

TRADE SCHOOLS:  Schools for the education in a trade or for the furtherance in obtaining employment.

TRADE/COMMERICAL OR CONSTRUCTION TRAILER:  The conducting of any business, trade, or occupation, or use as a selling or advertising device, or use for storage or conveyance for tools, equipment or machinery, and so designed that it is or may be mounted on wheels and used as a conveyance on highways and streets, propelled or drawn by its own or other motive power.

TRAILER PARK OR MOBILE HOME PARK:  A tract of land open to the public upon which spaces for trailers or mobile homes are provided for a consideration, whether for overnight, by the day, the week, the month, or longer.

TRAILERS:  Any vehicle without motive power designed or used for carrying property wholly on its own structure and for being drawn by a motor vehicle, and includes any such vehicle when formed by or operated as a combination of a semi-trailer and a vehicle of dolly type such as that commonly known as a trailer dolly, or a vehicle used to transport agricultural produce or agricultural production materials between a local place of storage or supply and the farm when drawn or towed on a public road or highway at a speed greater than twenty-five miles per hour.  Trailer does not include a manufactured home or travel trailer.

TRANSFER STATION:  A facility for the purpose of temporary collection of solid waste or recyclable materials, (garbage, demolition materials, aluminum, plastic and/or paper for the purpose of recycling; to be transferred to a permanent location for disposal).  Also known as Transfer Facility

TRANSPORTATION TERMINAL:  The use of land, buildings or structures for the purpose of storing, servicing, repairing, or loading trucks, transport trailers and/or buses, but does not include automobile service stations or transportation sales or rental outlets.

TRUCK TERMINAL:  A facility where freight is unloaded from interstate trucks and/or intermodal trailers and containers carried on the railroad and loaded onto local delivery trucks.

TRUCK TRACTOR: Any vehicle with greater than three (3) axles, and a fifth wheel.

TRUSTEES:  The Board of Trustees of Lawrence Township.

U

USE:  The purpose for which a building or premises is or may be occupied.  In the classification of uses, a use maybe a use as commonly understood or the name of an occupation, business, activity, or operation carried on, or intended to be carried on, in a building or on the premises, or the name of a building, place, or thing which name indicates that use or intended use.

V

VARIANCE:  A variance from the terms of the Zoning Resolution as will not be contrary to the public interest, where, owing to special conditions, a literal enforcement of the resolution will result in unnecessary hardship and so that the spirit of the resolution shall be observed and substantial justice done.

VEHICLE:  A motorized conveyance that transports people or objects; including everything on wheels, runners or tracks including motorized bicycles.

VETERINARY CLINIC:  Any building or structure devoted to the veterinarian diagnosis, treatment and care of non-human outpatients.

VETERINARY HOSPITAL:  A place used for care, grooming, diagnosis, and treatment of sick, ailing, or injured animals, including overnight accommodations and boarding, if incidental to the primary activity.

W

WALL:  A structure of brick, stone, concrete, etc., serving as a fence.

WAREHOUSE:  Structures used for the storage of distribution of goods where there is no sale of items to retailers or the general public unless permitted as an accessory use to the warehouse.

WASTE TRANSFER STATION:  A facility that receives some portion of its waste directly from collection vehicles, then consolidates and reloads the waste onto larger vehicles for deliver to a final disposal facility.  Also known as a Transfer Station.

WELL:  Any borehole, whether drilled or bored, within the Township, for production, extraction or injection of any gas or liquid mineral, excluding potable water to be used as such, but including natural or artificial brines and oil field waters.  The following definitions are related to wells:

BRINE:  All saline geological formation water resulting obtained or produced in connection with the 			exploration, drilling, or production of oil and gas.

DRILLING UNIT:  The minimum acreage on which one well may be drilled, but does not apply to a well 			for injecting gas into or removing gas from a gas storage reservoir.

GAS:	All-natural gas and all other fluid hydrocarbons not defined above as oil, including 				condensate.

OIL:	Crude petroleum oil and all other hydrocarbons, regardless of gravity, that are produced in 			liquid form by ordinary production methods, but do not include hydrocarbons that were 			originally in a gaseous phase in the reservoir.

OWNER:  The person who has the right to drill on a tract or drilling unit and to drill into and produce 			from a pool and to appropriate the oil or gas that he produces therefrom either for himself or 			for others.

POOL:  An underground reservoir containing a common accumulation of oil or gas, or both, but does 			not include a gas storage reservoir.  Each zone of a geological structure that is completely 			separated from any other zone in the same structure may contain a separate pool.

TRACT:  A single, individually taxed parcel of land appearing on the tax list.

WASTE:  Includes:

1.	Physical waste, as such term is generally understood in the oil and gas industry.

2.	Inefficient storing of oil or gas.

3.	Locating, drilling, equipping, operating or producing an oil or gas well in a manner that 				reduces or tends to reduce the quality of oil or gas ultimately recoverable under 					prudent and proper operations from the pool into which it is drilled, or that causes or 				tends to cause unnecessary or excessive surface loss or destruction of oil or gas.

4.	Other underground or surface waste in the production therefrom either for himself or 				for others.

WHOLESALE COMMERICAL USE:  The sale of merchandise to retail and service commercial uses, office uses, or institutional uses, or to other wholesalers.  Wholesale commercial uses may also mean acting as an agent or broker in the buying or selling of merchandise; but not selling to the general public.

WIND ACCESS BUFFER:  The distance between the closest points of any rotor diameters of two or more wind turbine generators.

WIND ENERGY CONVERSTION SYSTESM (WECS):  A machine consisting of one wind turbine, one tower and associated control electronics that converts the kinetic energy in the wind into a useable form (commonly known as a “wind turbine” or “windmill”).  The turbine or windmill may be on a horizontal or vertical axis, rotor or propeller.

WIND TURBINE GENERATOR (WTG):  A tower, pylon, or other structure, including all accessory facilities, upon which any, all or some combination of the following are mounted:

1.  	A wind vane, blade, or series of wind vanes or blades, or other devices mounted on a rotor for 			the purpose of converting wind into electrical or mechanical energy.

2.	A shaft, gear, belt, or coupling device used to connect the rotor to a generator, alternator, or 			other electrical or mechanical energy producing device.

3.	A generator, alternator, or other device used to convert the energy created by the rotation of 			the rotor into electrical or mechanical energy.

WIND TURBINE GENERATOR TOWER HEIGHT:

1.	Horizontal Axis Wind Turbine Rotors:  The distance between the ground and the highest point 			of the wind turbine generator, plus the length by which the rotor wind vanes or blades 				mounted on a horizontal axis wind turbine rotor exceeds the height of the wind turbine 				generator.

2.	Vertical Axis Wind Turbine:  The distance between the ground and the highest point of the wind 		turbine generator.

WIND TURBINE GENERATOR CONTRACT:  The agreement between the applicant and the landowner(s).

WIND TURBINE GENERATOR FACILITY:  All necessary devices that together convert wind energy into electricity, including the rotor, nacelle generator, WTG tower, electrical components, WTG foundation, transformer, substation, and electrical cabling from the WTG tower to the substation.

WIND TURBINE GENERATOR OPERATOR:  The entity responsible for the day-to-day operation and maintenance of the WTG, including their respective successors and assigns.  Owner does not mean the property owner from whom land is leased for locating the WTG unless the property owner has an equity interest in the WTG or if any person holding a security interest in the WTG solely to secure an extension of credit, or a person foreclosing on such security interest provided that after foreclosure, such person seeks to sell the WTG at the earliest practicable date.

WIRELESS TELECOMMUNICATIONS ANTENNA:  The physical device through which electromagnetic, wireless telecommunications signals authorized by the Federal Communications Commission are transmitted or received.

WIRELESS TELECOMMUNICATIONS EQUIPMENT SHELTER:  The structure in which the electronic receiving the relay equipment for a wireless telecommunications facility is housed.

WIRELESS TELECOMMUNICATIONS FACILITY:  A facility consisting of the equipment and structures involved in receiving telecommunications or radio signals from a mobile radio communications source and transmitting those signals to a central switching computer which connect the mobile unit with the land-based telephone lines.

WIRELESS TELECOMMUNICATIONS TOWER:  A structure intended to support equipment used to transmit and/or receive telecommunications signals including monopoles, and lattice construction steel structures.  The tower shall be self-supporting and no guy wires are permitted.

X

Y

YARD:  An open space on the same lot with a building, unoccupied and unobstructed by any portion of a structure from ground upward, except as otherwise provided herein.

YARD, FRONT:  A yard extending across the full width of a lot and being the perpendicular distance between the street right-of-way line and the nearest portion of any building or structure existing or proposed for construction on said lot.

YARD, REAR:  A yard extending across the full width of a lot between the side lot lines and being the perpendicular distance between the rear lot line and the nearest portions of any building or structure existing or proposed to be constructed on said lot.

YARD, SIDE:  A yard between the nearest portion of any building or structure existing or proposed to be constructed on said lot and the side lines of the lot and extending from the front yard to the rear yard.

Z

ZONING CERTIFICATE:  Document issued by the Township Zoning Inspector authorizing the use of lots or structures in accordance with the Zoning Resolution.

ZONING MAP:  The “Zoning Map of Lawrence Township, Tuscarawas County, Ohio”.